3GPP TSG-RAN WG4 Meeting #112-bis	R4-2415156

# Hefei , CN, Oct 14 – 18, 2024

Title: 	Discussion on conditional Intra-CU LTM for Rel-19 mobility enhancements

Source: 	CATT

Agenda item:	6.24.2.2

Document for:	Discussion

- Introduction

A new RAN4 leading work item for Rel-19 mobility enhancements was approved in RAN#102 [1], and the latest revised version was approved in RAN#105. The core part objectives of the work item are as following [2]:

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1]

  - Specify UE evaluated conditions for triggering LTM

  - Aim to support conditional LTM including subsequent LTM

  - Limit specifying the conditional LTM to the scenario where the UE is in non-DC

  - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

- Specify RRM requirements related to the above objectives as necessary [RAN4]

This document will discuss the general RRM issues about conditional Intra-CU LTM, and give our understanding and proposals.

- Discussion

For legacy L3 mobility, conditional handover (CHO) and other conditional mobility procedures (CPAC, SCPAC) were developed to achieve high robustness by enabling the procedure to be executed without necessitating a signalling exchange with source cell beforehand.

However, LTM introduced in Rel-18 offers short interruption time but not with the same level of robustness as the conditional L3 mobility procedures. Therefore, enhancements should be specified so that the system can benefit from both the high robustness and short interruption in Rel-19.

- General

According to the WID [2], for the scenario of conditional LTM, RAN4 will only define the related requirements for intra-CU and prioritize to consider non-DC.

For legacy L3 handover, the requirements for NR Conditional Handover were defined for handover in clause 6.1.4 in TS 38.133. For Rel-18 LTM, the related requirements were defined in a new clause 6.3 in TS 38.133[3]. In our view, for specification structure, we believe that the requirements for conditional LTM can be defined in the clauses of Rel-18 LTM.

Proposal 1: For the scenario of Rel-19 conditional LTM, RAN4 will only define the related requirements for intra-CU and prioritize to consider non-DC.

Proposal 2: For specification structure, the requirements for conditional LTM can be defined in the clauses of Rel-18 LTM.

- The requirements for conditional LTM

For how to define the specific requirments of conditional LTM, we believe that the legacy requirments for L3 CHO can be a starting point, and RAN4 need to further study the LTM related features, and the following applicable scenarios for SA can be the same:

- PCell switch to a neighboring LTM candidate cell

- FR1 cell to FR1 cell

- FR1 cell to FR2 cell

- FR2 cell to FR2 cell

- FR2 cell to FR1 cell

- PCell switch to an LTM candidate cell that is a serving SCell in MCG

- FR1 cell to FR1 cell

- FR2 cell to FR2 cell

Proposal 3: At least the following applicable scenarios can be considered for conditional LTM:

- PCell switch to a neighboring LTM candidate cell

- FR1 cell to FR1 cell

- FR1 cell to FR2 cell

- FR2 cell to FR2 cell

- FR2 cell to FR1 cell

- PCell switch to an LTM candidate cell that is a serving SCell in MCG

- FR1 cell to FR1 cell

- FR2 cell to FR2 cell

In detail, in legacy L3 CHO requirements, the handover delay, the measurement time, the preparation time and the interruption time were defined. Compared to L3 HO delay requirements, the CHO delay requirement takes into account the delay uncertainty (the time from when the UE successfully decodes a conditional handover command until a condition exists at the measurement reference point which will trigger the conditional handover), the measurements time and the conditional execution preparation time in addition to the RRC procedure delay and the interruption time.

Therefore, for Rel-19 conditional LTM, we believe that the delay uncertainty, the measurements time and the conditional execution preparation time also need to be considered based on the LTM Cell Switch delay requirements. For the interruption time in conditional LTM Cell Switch delay, the interruption time defined in Rel-18 LTM Cell Switch delay can be a baseline.

In addition, since the whole procedure will be discussed in parallel in RAN1/RAN2, and the requirements for conditional LTM are related to the procedure, so RAN4 also need to wait for RAN2 more progress.

Observation 1: Compared to L3 HO delay requirements, the CHO delay requirement takes into account the delay uncertainty, the measurements time and the conditional execution preparation time in addition to the RRC procedure delay and the interruption time.

Observation 2: The whole procedure will be discussed in parallel in RAN2, and the requirements for conditional LTM are related to the procedure.

Proposal 4: The legacy requirments for L3 CHO can be a starting point, and RAN4 need to further study the LTM related features based on the legacy requirments.

- Wait for RAN1/RAN2 more progress.

Proposal 5: The delay uncertainty, the measurements time and the conditional execution preparation time also need to be considered for Rel-19 conditional LTM Cell Switch delay.

Proposal 6: For the interruption time in Rel-19 conditional LTM Cell Switch delay, the interruption time defined in Rel-18 LTM Cell Switch delay can be a baseline.

- Conditional LTM including subsequent LTM

In Rel-18 mobility WI, subsequent CPAC were introduced for L3 mobility, so that there is no need to reconfigure and re-trigger the CPC/CPA process, but to perform a continuous cell group change after a cell group change. That is, by pre-configuring candidate SCGs once, subsequent SCG changes can be achieved, thereby reducing signaling overhead and interruption time. The requirements were defined for the following NR-DC scenarios [3]:

- NR Conditional Handover including target MCG and target SCG

- NR Conditional Handover including target MCG and candidate SCG

However, according to the WID, RAN4 will limit specifying the conditional LTM to the scenario where the UE is in non-DC. For conditional LTM including subsequent LTM [2], the requirements applicapable scenario is different with legacy subsequent CPAC, so we believe that RAN4 should firstly clarify the scenario for conditional LTM including subsequent LTM.

Proposal 7: RAN4 should further clarify the scenario for conditional LTM including subsequent LTM.

- Conclusion

This document discussed preliminary the issues for conditional Intra-CU LTM and presented following proposals and observations:

Proposal 1: For the scenario of Rel-19 conditional LTM, RAN4 will only define the related requirements for intra-CU and prioritize to consider non-DC.

Proposal 2: For specification structure, the requirements for conditional LTM can be defined in the clauses of Rel-18 LTM.

Proposal 3: At least the following applicable scenarios can be considered for conditional LTM:

- PCell switch to a neighboring LTM candidate cell

- FR1 cell to FR1 cell

- FR1 cell to FR2 cell

- FR2 cell to FR2 cell

- FR2 cell to FR1 cell

- PCell switch to an LTM candidate cell that is a serving SCell in MCG

- FR1 cell to FR1 cell

- FR2 cell to FR2 cell

Observation 1: Compared to L3 HO delay requirements, the CHO delay requirement takes into account the delay uncertainty, the measurements time and the conditional execution preparation time in addition to the RRC procedure delay and the interruption time.

Observation 2: The whole procedure will be discussed in parallel in RAN2, and the requirements for conditional LTM are related to the procedure.

Proposal 4: The legacy requirments for L3 CHO can be a starting point, and RAN4 need to further study the LTM related features based on the legacy requirments.

- Wait for RAN2 more progress.

Proposal 5: The delay uncertainty, the measurements time and the conditional execution preparation time also need to be considered for Rel-19 conditional LTM Cell Switch delay.

Proposal 6: For the interruption time in Rel-19 conditional LTM Cell Switch delay, the interruption time defined in Rel-18 LTM Cell Switch delay can be a baseline.

Proposal 7: RAN4 should further clarify the scenario for conditional LTM including subsequent LTM.

- Reference

[1]     RP-234036, New WID: NR mobility enhancements Phase 4, Intel

[2]     RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom

[3]     3GPP TS 38.133, NR Requirements for support of radio resource management