3GPP TSG-RAN WG4 Meeting #112-bis	R4-2415157

# Hefei , CN, Oct 14 – 18, 2024

Title: 	Discussion on (e)RedCap RRM requirements for Rel-19 NTN phase3

Source: 	CATT

Agenda item:	6.26.4.1

Document for:	Discussion

- Introduction

A new RAN4 leading work item for Non-Terrestrial Networks (NTN) for NR Phase 3 was approved in RAN#102, and the WID have been revised in RAN#105.The current objectives of the work item which are related to (e)RedCap UEs are copied as following [1]:

- Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1] - For full-duplex and half duplex FDD RedCap and eRedCap UEs, define the RF and RRM requirements [RAN4] - For HD-FDD RedCap UEs and eRedCap UEs, specify enhancements for mitigating issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB [RAN1] - Enhancements are targeted for the following HD collisions cases (cases 3 and 4): - Semi-statically configured DL reception collides with semi-statically configured UL transmission - Dynamically scheduled DL reception collides with dynamic scheduled UL transmission - Note: enhancements for other HD collision cases are not targeted. Enhancements for HD collision cases 3 and 4 that may improve other HD collision cases are not precluded. - Note: potential work in RAN2, if any, starts after RAN1 has decided on the solution. - Notes for this objective: - GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.

This document will discuss (e)RedCap RRM requirements for Rel-19 NTN phase3, and give our understanding and proposals.

- Discussion

2.1 Applicability clarification

- 2.1.1 The bandwidth of (e)Redcap UE with FR1-NTN

For the bandwidth assumption of (e)RedCap UE, the maximum bandwidth is reduced to 20MHz in FR1 for R17 RedCap UEs, and the maximum bandwidth can be further reduced to 5MHz in FR1 for R18 eRedCap UEs. Generally speaking, both 20MHz maximum bandwidth and 5MHz maximum bandwidth will be considered for R19 (e)RedCap UE with FR1-NTN, and the related agreements had already been reached in RF session in RAN4#110-bis meeting [2]:

## Sub-topic 1-5: eRedCap BW reduction Agreement: Specify both eRedCap variants, i.e. with and without bandwidth reduction.

For RRM requirements, even though both 20MHz maximum bandwidth and 5MHz maximum bandwidth will be considered, further bandwidth reduction to 5MHz had no RRM impacts in R18 eRedCap WI.

Observation 1: Both 20MHz maximum bandwidth and 5MHz maximum bandwidth will be considered for (e)RedCap UE with FR1-NTN bands.

Observation 2: There is no RRM impact on the further bandwidth reduction to 5MHz for R18 eRedCap UEs.

- 2.1.2 The network scenario for (e)Redcap UE with FR1-NTN

The Way Forward on the network scenario for (e)RedCap UE with FR1-NTN is copied as below [3]:

<Way Forward> - For scenario considered for (e)RedCap UE with FR1-NTN bands, RAN4 only supports NR SA operation mode with single carrier.

We believe that RAN4 should follow the network scenario in NTN for (e)RedCap UE with FR1-NTN bands, that is, RAN4 only to support NR SA operation mode with single carrier.

Proposal 1: For scenario considered for (e)RedCap UE with FR1-NTN bands, RAN4 only supports NR SA operation mode with single carrier.

  - General consideration

- 2.2.1 What RRM requirements are defined for (e)RedCap UE with FR1-NTN

In the last meeting, RAN4 discussed what RRM requirements should be defined for (e)RedCap UE with FR1-NTN and had some agreements, but there was still some requirements for FFS. The Way Forward are copied as below [3]:

<Way Forward> - FFS for the following requirements: - Minimization of Drive Tests (MDT) in RRC_IDLE state and RRC_INACTIVE state - NR Rel-17/18 Conditional Handover, and RACH-less Handover - Note: further check whether they are supported for RedCap NTN in RAN1/2 spec. - FFS: Send LS to RAN1/2 to ask the status. RAN4 does not trigger the RAN1/2 discussion to support these features. - Configured Grant based Small Data Transmissions (CG-SDT) - Random access based Small Data Transmissions (RA-SDT)

Firstly, in the agreed requirements in the last meeting, we would like to further clarify some requirements.

Cell re-selection for RRC_IDLE/RRC_INACTIVE state mobility

For the cell re-selection for RRC_IDLE/RRC_INACTIVE state mobility, the requirements for both NTN-NTN and NTN-TN(from NTN to TN and from TN to NTN) will be defined for (e)RedCap UE with FR1-NTN bands.

Proposal 2: For the requirements of cell re-selection for RRC_IDLE/RRC_INACTIVE state mobility, both NTN-NTN and NTN-TN (from NTN to TN and from TN to NTN) will be defined for (e)RedCap UE with FR1-NTN bands.

L3 intra- and inter-frequency measurements

For L3 measurements, since intra-frequency measurements with/without gaps and inter-frequency measurements with/without gaps were supported for both existing NTN requirements and (e)RedCap UEs with TN, so we believe that the same supported type of L3 measurements can be considered and the related measurement requirements will be defined for (e)RedCap UEs with FR1-NTN.

Proposal 3: RAN4 to define the requirements of the following types of L3 measurements for (e)RedCap UEs with FR1-NTN:

- Intra-frequency measurements without measurement gaps

- Intra-frequency measurements with measurement gaps

- Inter-frequency measurement without measurement gaps

- Inter-frequency measurement with measurement gaps

For the requirements which were still FFS, we would like to further provide our following views.

NR Rel-17/18 CHO and RACH-less Handover

Regarding the requirements of NR Rel-17/18 Conditional Handover and RACH-less Handover, FFS was kept in the last meeting because companies thought that they are also related to whether RAN1/RAN2 can support the corresponding mechanisms. The definition of (e)RedCap UE defined in clauses 4.2.21.1 and 4.2.22.1 in TS 38.306 are copied as below [5]:

CA, MR-DC, DAPS, CPAC and IAB (i.e., the RedCap UE is not expected to act as IAB node) related UE features and corresponding capabilities are not supported by RedCap UEs. All other feature groups or components of the feature groups as captured in TR 38.822 [24] as well as capabilities specified in this specification remain applicable for RedCap UEs same as non-RedCap UEs, unless indicated otherwise.

According to the above, with the exception of CA, MR-DC, DAPS, CPAC, and IAB-related UE features and corresponding capabilities, all other feature groups or components of feature groups are not excluded from (e)RedCap UE, so it means that CHO and RACH-less Handover are not precluded for (e)RedCap UE from RAN2 perspective.

From this point of view, both NR Rel-17/18 CHO and RACH-less Handover can be supported for (e)RedCap UEs in the same way as non-RedCap UE, and we believe that the requirements for NR Rel-17/18 Conditional Handover and RACH-less Handover can be considered for (e)RedCap UE with FR1-NTN in RAN4. Consideriing that the new HD-FDD impacts are not clear and they are discussed in RAN1 currently, so at least FD-FDD (e)RedCap UE with FR1-NTN bands can consider to defiine the related requirements.

Observation 3: The current RAN2 spec does not preclude supporting NR Rel-17/18 Conditional Handover and RACH-less Handover for (e) RedCap UE.

Proposal 4: The requirements for NR Rel-17/18 Conditional Handover and RACH-less Handover can be defined in RAN4 without the work of other group.

CG-SDT and RA-SDT

Regarding SDT, R17 introduced SDT to allow UE to transmit small data via CG-SDT and RA-SDT in the RRC_INACTIVE state, and reduce power consumption and signaling overhead, aiming to solve the efficiency problem of the network and UE in small data transmission scenarios.

In RedCap WI, considering that small data transmission is a typical scenario for RedCap UEs, the related requirements including CG-SDT and RA-SDT were defined for RedCap UEs.

However, SDT related requirements were not defined in current NTN requirements. If there is no other WG impacts, CG-SDT and RA-SDT can be defined for (e)RedCap UE with FR1-NTN bands.

Observation 4: CG-SDT and RA-SDT are only supported for RedCap UEs, and related processes are not currently supported in NTN.

Proposal 5: If there is no other WG impacts, CG-SDT and RA-SDT can be defined for (e)RedCap UE with FR1-NTN bands.

MDT

Regarding MDT in RRC_IDLE state and RRC_INACTIVE state, the related requirements were not defined for (e)RedCap UEs in TN network. In NTN, the MDT feature is optional, and the mobility from NTN to TN and TN to NTN in RRC_IDLE state and RRC_INACTIVE state has been supported, so (e)RedCap UEs will meet the MDT requirements in NTN but will have no MDT requirements in TN, it is irrational.

Also considering the workload, we prefer to prioritize MDT for (e)RedCap UE with FR1-NTN bands.

Observation 5: MDT requirements were not defined for (e)RedCap UEs in TN network.

Observation 6: If MDT is defined for (e)RedCap UE with FR1-NTN bands, (e)RedCap UEs will meet the MDT requirements in NTN but will have no MDT requirements in TN, it is irrational.

Proposal 6: Deprioritize to define the requirements of MDT for (e)RedCap UE with FR1-NTN bands.

- 2.2.2 Features to be considered for (e)RedCap UE with FR1-NTN

In the last meeting, the Way Forward on the features to be considered for (e)RedCap UE with FR1-NTN are coppied as below [3]:

<Way Forward> - RAN4 to analyze the RRM requirement impacts from following features for (e)RedCap UE with FR1-NTN case by case: - 1Rx - FFS: NCD-SSB - HD-FDD - FFS: R17 relaxed measurement - FFS: RRC_IDLE/RRC_INACTIVE mode eDRX enhancement - FFS: Determine if CSSF and concurrent gap is supported in Redcap NTN - FFS: FH in PRS measurement - Single cell search and measurement engine

RAN4 has agreed not to consider the RRM impacts caused by the features of HD-FDD, and agreed to consider the RRM impacts caused by the features of 1Rx, HD-FDD and single cell search and measurement engine in the last meeting.

R17 relaxed measurements

For R17 relaxed measurements, they were introduced for R17 RedCap UEs in TN network, including stationary/ low mobility/not-at-cell-edge criteria and their combinations in RRC_IDLE/RRC_INACTIVE mode. In current NTN requirements, only R16 relaxed measurements were introduced in RRC_IDLE mode, whether to consider these R17 enhenced relaxed measurements for (e)RedCap UEs with FR1-NTN bands should be discussed according to the actual power saving demand.

Proposal 7: Whether to consider the R17 enhenced relaxed measurements in RRC_IDLE/RRC_INACTIVE mode for (e)RedCap UEs with FR1-NTN bands should be discussed according to the actual power saving demand.

RRC_IDLE/RRC_INACTIVE mode eDRX

In Rel-17 RedCap and Rel-18 eRedCap, the eDRX enhancement has been introduced, and the relevant enhancements were only have an impact on the RRM requirements for RRC_IDLE/RRC_INACTIVE state mobility. However, the requirements related to eDRX wasn’t been defined in NTN, so whether and how to introduce the eDRX for (e)RedCap UEs with FR1-NTN bands should be discussed.

In the last meeting, RAN4 discussed the related issue, but there was no consensus among the companies. The candidate solutions are copied as below [3]:

#### Issue 6-5-1: How to consider the impact of eDRX enhancement for (e)Redcap UE with FR1-NTN? <Way Forward> : Further discuss the following proposal - Proposal 1 (CATT, CMCC, Samsung): - The eDRX enhancement introduced for (e)RedCap UEs should be considered when defining RRC_IDLE or/and RRC_INACTIVE state mobility requirements for (e)RedCap UEs with FR1-NTN bands. - Proposal 1a (CATT): RAN4 need to discuss whether the eDRX enhancement for both Rel-17 RedCap and Rel-18 eRedCap UEs should be introduced for (e)RedCap UEs with FR1-NTN bands. - Option 1: Only introduce the eDRX enhancement for Rel-17 RedCap UEs. - Option 2: Introduce the eDRX enhancement for both Rel-17 RedCap and Rel-18 eRedCap UEs. - Proposal 1b (CMCC): Support eDRX configuration for Redcap over NTN, and define following applicability rule for each deployment scenario: - For GEO deployment, all DRX and eDRX cycle can be supported. - For earth-fixed LEO deployment, requirements are applicable for up to 10.24s eDRX cycle - For earth-moving LEO deployment, requirements are not applicable for eDRX cycle - Proposal 1c (Samsung): - RAN4 to discuss and specify the requirements related to eDRX for GSO and NGSO (LEO) separately. Not all eDRX cycle can be applicable for NGSO (LEO) scenario. - Proposal 1d (QC): For eRedCap UE, RAN4 can consider the following aspect for RRM requirement definition if necessary and applicable for NR NTN support. - Enhanced eDRX in RRC INACTIVE - Proposal 2 (LG): - For NTN RRM requirements for RedCap UE, - For IDLE/INACTIVE mode: Preclude eDRX_IDLE cycle configuration for RedCap NTN

In Rel-17 RedCap WI, requirements were introduced for eDRX_IDLE up to 10485.76s and eDRX_INACTIVE cycles which up to 10.24s. In Rel-18 eRedCap WI, enhanced requirements were introduced for eDRX_INACTIVE up to 10485.76s [4].

In our view, we should discuss the related issue case by case. In GEO deployment, techenically speaking, all eDRX cycle can be supported, which includes all of eDRX enhancement introduced for (e)RedCap UEs in TN. However, we believe that RAN4 should further check the necessity of applicability in this scenario, e.g., whether there is such a typical scenario in a real NTN deployment that needs such a large eDRX cycle. In LEO deployment, we support that not all eDRX cycle can be applicable, even up to 10.24s eDRX cycle may feasible in earth-fixed LEO deployment, we still prefer not to introduce any eDRX cycle for (e)RedCap UE with FR1-NTN.

Observation 7: Techenically speaking, all eDRX cycle can be supported in GEO deployment, which includes Rel-17/18 eDRX enhancement introduced for (e)RedCap UEs in TN.

Proposal 8: RAN4 should further check the necessity of applicability in GEO deployment, e.g., whether there is such a typical scenario in a real NTN deployment that needs such a large eDRX cycle.

Observation 8: Techenically speaking, up to 10.24s eDRX cycle may feasible in earth-fixed LEO deployment.

Proposal 9: Prefer not to introduce any eDRX cycles for (e)RedCap UE with FR1-NTN in LEO deployment.

The number of searchers and CSSF

Regarding the searcher assumption and CSSF for (e)RedCap UEs in TN network, 1Rx and 2Rx (e)RedCap UEs use only a single cell search and measurement engine, with no CA in mind. The single searcher was shared by all of intra- and inter-frequency layers. For CSSF, its values are categorized into CSSFoutside_gap,i and CSSFwithin_gap,i. The legacy design of CSSFwithin gap were reused for (e)RedCap UEs in TN, and CSSFoutside_gap,I were defined as follows:

For UE in SA operation mode, the carrier-specific scaling factor CSSFoutside_gap_RedCap,i for intra-frequency SSB-based measurements, inter-frequency SSB-based measurements performed outside measurements gaps, will be specified as follows: CSSFoutside_gap,i =1, if only one measurement object is configured to be measured outside of MG for RedCap. CSSFoutside_gap,i =2 for intra-frequency measurement, and CSSFoutside_gap,i = 2*Y for inter-frequency measurement with no measurement gap, Y is the number of configured inter-frequency MOs without MG that are being measured outside of MG. If intra-frequency measurement is with measurement gap, CSSFoutside_gap,i = Y for inter-frequency measurement with no measurement gap, Y is the number of configured inter-frequency MOs without MG that are being measured outside of MG.

For NTN requirements, the legacy searcher assumption and CSSF were reused for normal UE. In our view, a single searcher and CSSF defined for (e)RedCap UEs in TN will be reused for (e)Redcap UE with FR1-NTN since they are related to the capability of the UE to process and depend on the type of UEs.

Observation 9: The legacy searcher assumption and CSSF are related to the capability of the UE to process and depend on the type of UEs.

Proposal 10: Single searcher assumption and CSSF defined for (e)RedCap UEs in TN will be reused for (e)Redcap UE with FR1-NTN.

Type of gap

For (e)RedCap UEs in TN network, only both per-UE and per-FR gap related requirements were defiined, and other enhanced gaps were not supported, such as NeedForGap, NCSG, concurrent gaps and so on.

In current NTN requirements, both per-UE and per-FR gap related requirements were defiined, and concurrent gaps was also introduced. However, we believe that the supported types of gaps should consider the types of UEs, and introducing enhanced gap will bring more complexity for (e)RedCap UE with FR1-NTN, so we prefer to reuse the gap defined for (e)RedCap UEs in TN.

Observation 10: The supported types of gaps are related to the types of UEs, and introducing the enhanced gap will bring more complexity for (e)Redcap UE with FR1-NTN.

Proposal 11: For (e)Redcap UE with FR1-NTN, prefer to reuse the gap defined for (e)RedCap UEs in TN and not to introduce the enhanced gap, such as concurrent gaps.

  - Reduction in the number of UE Rx branches

In the last meeting, RAN4 has discissed the principle for defining the requirements for 2Rx/1Rx (e)Redcap UE with FR1-NTN and reached the following agreements [3]:

<Agreement> - For RRM requirements, the number of Rx considered for (e)Redcap UE with FR1-NTN bands support include both 1Rx and 2Rx. - RAN4 should define separate sets of RRM requirements for 1Rx and 2 Rx (e)Redcap UE, if needed. - As baseline, follow the same principle for (e)RedCap UE in TN network. - For 2Rx (e)RedCap UEs with FR1-NTN: Reuse the existing requirements for NTN as a baseline. - Meanwhile, consider the following difference between 2Rx (e)RedCap and non-RedCap UE: - The number of searchers - Others if any

Acording to the above, RAN4 has agreed to follow the same principle for (e)RedCap UE in TN network as baseline, so we suggest that the following requirements need to be defined for 1Rx and 2Rx (e)RedCap UEs respectively with FR1-NTN bands, and other requirements not mentioned can be defined as a common set of RRM requirements for 1Rx and 2 Rx (e)Redcap UE.

Table 1 RRM Requirements need to be defined for 1Rx and 2Rx (e)RedCap UEs respectively

SA: RRC_IDLE/ RRC_INACTIVE state mobility

- Cell Selection in RRC_IDLE state - Cell Re-selection in RRC_IDLE/ RRC_INACTIVE state

SA: RRC_CONNECTED state mobility

- NR Handover - NR FR1 - NR FR1 Handover - RRC Connection Mobility Control - SA: RRC Re-establishment - Random access - SA: RRC Connection Release with Redirection - NR SAN Satellite switching with re-synchronization

Signalling characteristics

- Radio Link Monitoring - Link Recovery Procedures

Measurement Procedure

- NR intra-frequency measurements - NR inter-frequency measurements - L1-RSRP measurements for Reporting - NR measurements for positioning (Network verified UE location)

Note: Other requirements not listed are not distinguished between 1Rx and 2 Rx (e)Redcap UE

How to relax the requirements for 1Rx (e)RedCap UEs with FR1-NTN ?

The Way Forward on how to relax the requirements for 1Rx (e)RedCap UEs with FR1-NTN is copied as follows [3]:

<Way Forward> : Further discuss the following proposal - Proposal 1 (vivo, ZTE): - Introduce corresponding RRM requirements for 1 Rx and current requirements for Redcap UE with 1Rx can be the reference. - Proposal 2 (Xiaomi): - For 1RX RedCap: Relax measurement requirements from the aspects of extending the number of measurement samples or relaxing the measurement accuracy. - Proposal 3 (ZTE): - RAN4 shall define the Qout requirements for RedCap UEs in FR1-NTN bands. The evaluation period for 2Rx shall be reused and the evaluation period shall be extended for 1Rx by factor 2. Qin shall be reused. - RAN4 shall reuse the 1dB offset as legacy for 1Rx RedCap UE in NR NTN. - Proposal 4 (QC): - For 1Rx RedCap NTN UE, RAN4 to adopt the same requirement relaxations and modifications as follows: - Extended delays for PSS/SSS detection and SSB index identification (PBCH decoding) - Handover delay Extension - Extended RLM OOS and BFD evaluation periods - Aggregation level of 16 for RLM/BFD hypothetical PDCCH parameters - Measurement accuracy relaxation for SS-RSRP, SS-RSRQ, SS-SINR, L1-RSRP

Based on our proposed framework, we would like to further analyze the specific impact on different RRM requirements for 1Rx(e)RedCap UEs with FR1-NTN bands.

- 2.3.1 Mobility in RRC_IDLE / RRC_INACTIVE state

For the mobility requirements in RRC_IDLE / RRC_INACTIVE state, firstly RAN4 need to consider the offset for the relative cell-specific RSRP thresholds. In TN network, -1 dB offset was used for the thresholds  of Qrxlevmin and Qqualmin. We support to reuse the same offset for 1Rx(e)RedCap UEs with FR1-NTN bands.

Proposal 12: RAN4 to consider the offset for the reletive cell-specific RSRP thresholds for 1Rx(e)RedCap UEs with FR1-NTN bands, including Qrxlevmin and Qqualmin:

- -1 dB offset can be reused as a baseline.

For the UE measurement capability for 1 Rx (e)RedCap UEs with FR1-NTN, the number of NR inter-frequency carriers that UE shall at least be capable of monitoring should be reduced. RAN4 can refer to the capability for 1 Rx (e)RedCap UEs in TN and 6 carriers can be reused.

Proposal 13: RAN4 to reduce the number of NR inter-frequency carriers that UE shall at least be capable of monitoring for 1Rx(e)RedCap UEs with FR1-NTN bands.

- 6 NR inter-frequency carriers can be reused.

For the measurement and evaluation of serving cell for (e)RedCap with FR1-NTN, Nserv_RedCap will be not distinguished between 1Rx and 2 Rx (e)Redcap UEs.

For the measurements of intra- and inter-frequency NR cells for (e)RedCap with FR1-NTN, Tdetect,NR_Intra/Inter_RedCap, Tmeasure,NR_Intra/Inter_RedCap and Tevaluate,NR_Intra/Inter_RedCap will be not distinguished between 1Rx and 2 Rx (e)Redcap UE. For the threshold of absThreshSS-BlocksConsolidation, +1 dB offset was used for the 1Rx (e)RedCap UEs in TN network, and we support to reuse the same offset for 1Rx (e)RedCap UEs with FR1-NTN bands. For the cases of rangeToBestCell is not configured and rangeToBestCell is configured, +1 dB offset was used for better cell ranked for 1Rx (e)RedCap UEs in TN network, and the same offset can be reused for 1Rx (e)RedCap UEs with FR1-NTN bands.

Proposal 14: The paremeters of Nserv_RedCap, Tdetect,NR_Intra/Inter_RedCap, Tmeasure,NR_Intra/Inter_RedCap and Tevaluate,NR_Intra/Inter_RedCap defined for cell re-selection requirements will be not distinguished between 1Rx and 2 Rx (e)Redcap UE.

Proposal 15: For the measurements of intra- and inter-frequency NR cells,

- +1 dB offset on the threshold of absThreshSS-BlocksConsolidation can be reused as a baseline for 1Rx(e)RedCap UEs with FR1-NTN bands.

- For the cases of rangeToBestCell is not configured and rangeToBestCell is configured, +1 dB offset can be reused for better cell ranked for 1Rx (e)RedCap UEs with FR1-NTN bands.

For measurements of inter-frequency NR cells in RRC_IDLE / RRC_INACTIVE state for (e)RedCap with FR1-NTN, the same +1 dB offset can be reused for a margin in FR1 for reselections when UE evaluating whether a newly detectable inter-frequency cell meets the reselection criteria, which are as follows [4]:

For 1 Rx RedCap by a margin of at least - 6 dB in FR1 for reselections based on ranking or - 7 dB in FR1 for SS-RSRP reselections based on absolute priorities or - 5 dB in FR1 for SS-RSRQ reselections based on absolute priorities.

The above margin can also be reused for 1Rx (e)RedCap with FR1-NTN.

Proposal 16: In RRC_IDLE / RRC_INACTIVE state, +1 dB offset can be reused for a margin in FR1 for reselections when UE evaluating whether a newly detectable inter-frequency cell meets the reselection criteria for 1Rx (e)RedCap with FR1-NTN:

- For 1 Rx RedCap by a margin of at least

- 6 dB in FR1 for reselections based on ranking or

- 7 dB in FR1 for SS-RSRP reselections based on absolute priorities or

- 5 dB in FR1 for SS-RSRQ reselections based on absolute priorities.

- 2.3.2 Mobility in RRC_CONNECTED state

Handover

For NR FR1 – NR FR1 handover requirements, handover delay extension has been considered for 1 Rx RedCap UE in TN, and the relaxations on Tsearch for unknown intra/inter- frequency cell were introduced in HO requirements in TS 38.133 are as following [4]:

If the target cell is an unknown intra-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch = 1*Trs ms if UE is capable of 2 Rx antennas; Tsearch = 2*Trs ms if UE is only required to support 1 Rx antenna.

- the target cell is an unknown inter-frequency cell. In this case, if the target cell Es/Iot≥-2 dB, then Tsearch = 3* Trs ms if the UE is operating with 2 Rx antennas; Tsearch = 5*Trs ms if UE is operating with only 1 Rx antenna.

Since the values of Tsearch in NR SAN FR1 – NR SAN FR1 handover requirements are the same as those in TN network for normal UE, so we believe that the same relaxation on Tsearch for unknown intra/inter-frequency cell in HO requirements can be reused as baseline for (e)RedCap with FR1-NTN.

Proposal 17: Handover delays should be extended for 1Rx(e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

- Define the relaxation on Tsearch for unknown intra/inter-frequency cell.

- The same relaxation for 1 Rx RedCap UE in TN network can be reused as baseline.

- Tsearch = 2*Trs for intra-frequency HO

- Tsearch = 5* Trs for inter-frequency HO

RRC Connection Mobility Control

For RRC Re-establishment requirements, UE re-establishment delay (TUE_re-establish_delay) extension has been considered for 1 Rx RedCap UE in TN, and the relaxations on Tidentify_intra_NR and Tidentify_inter_NR, i in TS 38.133 are as following [4]:

The requirements in clause 6.2.1 shall apply when RedCap UE is capable of 2 Rx. When UE is only required to support 1 Rx antenna, the requirements defined in clause 6.2.1 shall apply except that: - Tidentify_intra_NR as specified in Table 6.2.1B.2-1. - Tidentify_inter_NR, i as specified in Table 6.2.1B.2-2. Table 6.2.1B.2-1: Time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell Serving cell; FR of target NR; Tidentify_intra_NR [ms]. SSB Ês/Iot (dB); cell; Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (200 ms, 6 x TSMTC); MAX (800 ms, [11] x TSMTC). < -8; FR1; N/A; 800Note1. Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB.. Table 6.2.1B.2-2: Time to identify target NR cell for RRC connection re-establishment to NR inter-frequency cell Serving cell SSB Ês/Iot (dB); FR of target NR cell; Tidentify_inter_NR, i [ms]. Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (200 ms, 7 x TSMTC, i); MAX (800 ms, [14] x TSMTC, i). < -8; FR1; N/A; 800Note1. Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC,i > 20 ms and serving cell SSB Ês/Iot < -8 dB..

Since the values of Tidentify_intra_NR and Tidentify_inter_NR, i in UE Re-establishment delay requirements in NTN network are the same as those in TN network for normal UE, so we believe that the same relaxation on Tidentify_intra_NR and Tidentify_inter_NR, i for known and unknown intra-/inter-frequency NR cell in re-establishment delay requirements can be reused as baseline, that is, RAN4 to relax the Tidentify for (e)RedCap UEs with FR1-NTN by 1 more samples for FR1.

Proposal 18: UE RRC re-establishment delay (TUE_re-establish_delay) should be extended for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

- Define the relaxation on Tidentify for known and unknown intra/inter-frequency cell.

- The same relaxation for 1 Rx RedCap UE in TN network can be reused as baseline.

- Relax the Tidentify_intra_NR and Tidentify_inter_NR, i by 1 more samples for FR1 known and unknown intra/inter-frequency cell.

For random access requirements, the offsets for the following relative cell-specific RSRP thresholds are defined in TN network in TS 38.133 as below [4]:

The 1 Rx RedCap UE for performing the random access procedure defined in clause 5.1 [7] applies: - rsrp-ThresholdSSB as the signaled value of rsrp-ThresholdSSB [2] + 1 dB. - msgA-RSRP-ThresholdSSB as the signaled value of msgA-RSRP-ThresholdSSB [2] + 1 dB. - msgA-RSRP-Threshold as the signaled value of msgA-RSRP-Threshold [2] + 1 dB. - rsrp-ThresholdMsg3 as the signaled value of rsrp-ThresholdMsg3 [2] - 1 dB.

We believe that the same consideration and same offset can be reused as a baseline for 1Rx(e)RedCap UEs with FR1-NTN bands.

Proposal 19: For random access requirements, RAN4 to consider the offset for the following relative cell-specific RSRP thresholds.

- +1 dB offset can be reused as a baseline for rsrp-ThresholdSSB, msgA-RSRP-ThresholdSSB and msgA-RSRP-Threshold.

- -1 dB offset can be reused as a baseline for rsrp-ThresholdMsg3.

RRC connection release with redirection

For RRC connection release with redirection to NR requirements, the time delay (Tconnection_release_redirect_NR) extension has been considered for 1 Rx RedCap UE in TN, and the relaxation on Tidentify-NR in TS 38.133 is as following [4]:

The requirements in clause 6.2.3.2.1 shall apply when RedCap UE is capable of 2 Rx. When UE is only required to support 1 Rx antenna, the requirements defined in clause 6.2.3.2.1 shall apply except that: - Tidentify-NR as specified in Table 6.2.3A.2.1-1. Table 6.2.3A.2.1-1: Time to identify target NR cell for RRC connection release with redirection to NR FR of target NR cell; Tidentify-NR. FR1; MAX (680 ms, [12] x Trs). Note: If the UE has been provided with higher layer signaling of smtc2 specified in TS 38.331 [2] prior to the redirection command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell..

Since the value of Tidentify-NR in RRC connection release with redirection to NR requirements in NTN network is the same as those in TN network for normal UE, so we believe that the same relaxation on Tidentify-NR for FR1 can be reused as baseline, that is, RAN4 to relax the Tidentify-NR for (e)RedCap UEs with FR1-NTN by 1 more samples for FR1.

Proposal 20: RRC connection release with redirection to NR delay (Tconnection_release_redirect_NR) should be extended for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

- Define the relaxation on Tidentify-NR for FR1.

- The same relaxation for 1 Rx RedCap UE in TN network can be reused as baseline.

- Relax the Tidentify-NR by 1 more samples for FR1.

- 2.3.3 Timing

In TN network, timing requirements are not distinguished between 1Rx and 2 Rx (e)Redcap UE, so there is also no need to distinguish the timing requirements for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands.

Proposal 21: There is no need to define the following timing requirements for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands respectively.

- UE transmit timing

- UE timer accuracy

- Timing Advance

- 2.3.4 Signalling characteristics

RLM and BFD

For SSB and CSI-RS based radio link monitoring/beam failure detection requirements, some PDCCH transmission parameters for out-of-sync/in-sync evaluation and beam failure instance were defined for 1Rx and 2Rx RedCap UE respectively in TN network, including aggregation level (CCE) and bandwidth (PRBs). In detail, 16 was used for 1Rx RedCap UE for aggregation level (CCE), and 48 was used for 1Rx RedCap UE for bandwidth (PRBs).

Since the values of aggregation level (CCE) and bandwidth (PRBs) in RLM/BFD hypothetical PDCCH parameters in NTN network are the same as those in TN network for normal UE, so we believe that the same values of aggregation level (CCE) and bandwidth (PRBs) can be reused as baseline, that is, RAN4 to support aggregation level (CCE) of 16 and bandwidth (PRBs) of 48 for 1Rx (e)RedCap UEs with FR1-NTN.

In addition, considering that the impact on bandwidth for SSB based and CSI-RS based RLM/BFD, SCS=60kHz was not applicable for FR1 for RedCap UE in TN network. The same applicability should be apply for (e)RedCap UEs with FR1-NTN.

Proposal 22: For SSB based and CSI-RS based radio link monitoring/beam failure detection requirements, some of hypothetical PDCCH parameters should be defined for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN respectively, including aggregation level (CCE) and bandwidth (PRBs).

- The same values for 1 Rx RedCap UE in TN network can be reused as baseline.

- Aggregation level (CCE) of 16.

- Bandwidth (PRBs) of 48.

Proposal 23: For SSB based and CSI-RS based RLM/BFD requirements, SCS=60kHz is not applicable for (e)RedCap UEs with FR1-NTN.

In additon, RLM OOS and BFD evaluation periods extension has been considered for 1 Rx RedCap UE in TN, and the relaxations on TEvaluate_out/in_SSB,RedCap, TEvaluate_out/in_CSI-RS,RedCap,TEvaluate_BFD_SSB _Redcap and TEvaluate_BFD_CSI-RS_Redcap in TS 38.133 are as following [4]:

Table 8.1B.2.2-3: Evaluation period TEvaluate_out_SSB,RedCap and TEvaluate_in_SSB,RedCap for FR1 for 1Rx RedCap UE Configuration; TEvaluate_out_SSB,RedCap (ms); TEvaluate_in_SSB,RedCap (ms). no DRX; Max(400, Ceil(20 P) TSSB); Max(100, Ceil(5 P) TSSB). DRX cycle≤320ms; Max(400, Ceil(30 P) Max(TDRX,TSSB)); Max(100, Ceil(7.5 P) Max(TDRX,TSSB)). DRX cycle>320ms; Ceil(20 P) TDRX; Ceil(5 P) TDRX. NOTE: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length.. Table 8.1B.3.2-3: Evaluation period TEvaluate_out_CSI-RS,RedCap and TEvaluate_in_CSI-RS,RedCap for FR1 for 1Rx RedCap UE Configuration; TEvaluate_out_CSI-RS,RedCap (ms); TEvaluate_in_CSI-RS,RedCap (ms). no DRX; Max(400, Ceil(Mout,RedCap×P)×TCSI-RS); Max(100, Ceil(Min,RedCap×P) × TCSI-RS). DRX ≤ 320ms; Max(400, Ceil(1.5×Mout,RedCap ×P)× Max(TDRX, TCSI-RS)); Max(100, Ceil(1.5×Min,RedCap×P)× Max(TDRX, TCSI-RS)). DRX > 320ms; Ceil(Mout,RedCap ×P) × TDRX; Ceil(Min,RedCap ×P) × TDRX. NOTE: TCSI-RS is the periodicity of the CSI-RS resource configured for RLM. The requirements in this table apply for TCSI-RS equal to 5 ms, 10ms, 20 ms or 40 ms. TDRX is the DRX cycle length.. The values of Mout,RedCap and Min,RedCap used in Table 8.1B.3.2-1 and Table 8.1B.3.2-2 for UE with 2Rx are defined as: - Mout,RedCap = 20 and Min,RedCap = 10, if the CSI-RS resource configured for RLM is transmitted with higher layer CSI-RS parameter density [6, clause 7.4.1] set to 3 and over the bandwidth ≥ 24 PRBs. The values of Mout,RedCap and Min,RedCap used in Table 8.1B.3.2-3 for RedCap UE with 1Rx are defined as: - Mout,RedCap = 40 and Min,RedCap = 10, if the CSI-RS resource configured for RLM is transmitted with higher layer CSI-RS parameter density [6, clause 7.4.1] set to 3 and over the bandwidth ≥ 24 PRBs. Table 8.5B.2.2-1: Evaluation period TEvaluate_BFD_SSB_Redcap for FR1 Configuration; TEvaluate_BFD_SSB_Redcap (ms) for Redcap UE with 2 Rx; TEvaluate_BFD_SSB_Redcap (ms) for 1 Rx Redcap. no DRX; Max(50, Ceil(5 P) TSSB); [ Max(50, Ceil(10 P) TSSB)]. DRX cycle ≤ 320ms; Max(50, Ceil(7.5 P) Max(TDRX,TSSB)); [ Max(50, Ceil(15 P) Max(TDRX,TSSB)) ]. DRX cycle > 320ms; Ceil(5 P) TDRX; [ Ceil(10 P) TDRX]. Note: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length.. Table 8.5B.3.2-1: Evaluation period TEvaluate_BFD_CSI-RS_Redcap for FR1 Configuration; TEvaluate_BFD_CSI-RS_Redcap (ms) for Redcap UE with 2 Rx; TEvaluate_BFD_CSI-RS_Redcap (ms) for 1 Rx Redcap. no DRX; Max(50, Ceil(MBFD P PBFD) TCSI-RS); Max(50, Ceil(2MBFD P PBFD) TCSI-RS). DRX cycle ≤ 320ms; Max(50, Ceil(1.5 × MBFD P PBFD) Max(TDRX, TCSI-RS)); Max(50, Ceil(21.5 × MBFD P PBFD) Max(TDRX, TCSI-RS)). DRX cycle > 320ms; Ceil(MBFD P PBFD) TDRX; Ceil(2MBFD P PBFD) TDRX. Note: TCSI-RS is the periodicity of CSI-RS resource in the set . TDRX is the DRX cycle length..

Since the values of TEvaluate_out/in_SSB, TEvaluate_out/in_CSI-RS,TEvaluate_BFD_SSB and TEvaluate_BFD_CSI-RS in RLM/BFD requirements in NTN network are the same as TN network for normal UE, so we believe that the same extension for SSB and CSI-RS based RLM OOS and BFD evaluation periods can be reused as baseline for 1Rx (e)RedCap UEs with FR1-NTN, that is,

- For RLM evaluation periods,

- RAN4 not to extend the TEvaluate_in_SSB/CSI-RS for FR1.

- RAN4 to extend the TEvaluate_out_SSB/CSI-RS by factor 2 for FR1.

- For BFD evaluation periods,

- RAN4 to extend the TEvaluate_BFD_SSB/CSI-RS by factor 2 for FR1.

Proposal 24: The SSB and CSI-RS based RLM OOS and BFD evaluation periods should be extended for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

- The same extension for 1 Rx RedCap UE in TN network can be reused as baseline.

- For RLM evaluation periods,

- RAN4 not to extend the TEvaluate_in_SSB/CSI-RS for FR1.

- RAN4 to extend the TEvaluate_out_SSB/CSI-RS by factor 2 for FR1.

- For BFD evaluation periods,

- RAN4 to extend the TEvaluate_BFD_SSB/CSI-RS by factor 2 for FR1.

In TN network, the following requirements were not distinguished between 1Rx and 2 Rx (e)Redcap UE:

- CBD evaluation period TEvaluate_CBD_SSB_Redcap

- Active BWP switch delay

- Active TCI state switching delay

- UE-specific CBW change

Therefore, there is also no need to distinguish these requirements for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands, and reuse these requirements defined in NTN for (e)RedCap UEs with FR1-NTN.

Proposal 25: The following requirements defined for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same.

- Active BWP switch delay

- Active TCI state switching delay

- UE-specific CBW change

- 2.3.5 Measurement Procedure

NR intra/inter-frequency L3 measurements

For the measurement requirements, the relaxation on the time period for PSS/SSS detection, SSB index identification (PBCH decoding) and measurement period have been considered for 1Rx (e)RedCap UEs compared to those defined for 2Rx (e)RedCap UEs in TN network, According to TS 38.133, the specific relaxation principles for 1Rx (e)RedCap UEs compared to 2Rx (e)RedCap UEs in TN are listed as following:

|  | Intra-f without measurement gaps | Intra-f with measurement gaps | Inter-f without measurement gaps | Inter-f with measurement gaps |
| --- | --- | --- | --- | --- |
| Time period for PSS/SSS detection | Relax for 1Rx - Increase number of attempts by 2 SSB samples: from ‘5’ to ‘7’ - Not to extend the lower boundary ‘600ms’ | Relax for 1Rx - Increase number of attempts by 2 SSB samples: from ‘5’ to ‘7’ - Not to extend the lower boundary ‘600ms’ | Relax for 1Rx - Increase number of attempts by 2 SSB samples: from ‘5’ to ‘7’ - Not to extend the lower boundary ‘600ms’ | Relax for 1Rx - Increase number of attempts by 2 SSB samples: from ‘8’ to ‘10’ - Not to extend the lower boundary ‘600ms’ |
| Time period for SSB time index detection | Relax for 1Rx - Extend the lower boundary from ‘120 ms’ to ‘160ms’ - Increase number of attempts by 3 SSB samples: from ‘3’ to ‘6’ | Relax for 1Rx - Extend the lower boundary from ‘120 ms’ to ‘160ms’ - Increase number of attempts by 3 SSB samples: from ‘3’ to ‘6’ | Relax for 1Rx - Extend the lower boundary from ‘120 ms’ to ‘160ms’ - Increase number of attempts by 3 SSB samples: from ‘3’ to ‘6’ | Relax for 1Rx - Extend the lower boundary from ‘120 ms’ to ‘160ms’ - Increase number of attempts by 3 SSB samples: from ‘3’ to ‘6’ |
| Measurement period | No relaxation | No relaxation | No relaxation | Relax for 1Rx - Extend the lower boundary from ‘200 ms’ to ‘400ms’ - Keep the same number of samples. |

According to the above, we believe that the same relaxation can be reused as baseline for 1Rx (e)RedCap UEs with FR1-NTN.

Proposal 26: The time period for PSS/SSS detection defined for intra/inter-frequency measurement requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

- Reuse the same extensions for 1Rx RedCap UE in TN network.

- Increase the number of attempts by 2 SSB samples for TPSS/SSS_sync_intra and TPSS/SSS_sync_inter.

- Not to extend the lower boundary.

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600ms, ceil( 7 x Kp) x SMTC period )Note 1 x CSSFintra_RedCap |
| DRX cycle≤ 320ms | max(600ms, ceil(1.5 x 7 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra_RedCap |
| DRX cycle>320ms | ceil(7 x Kp) x DRX cycle x CSSFintra_RedCap |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

| Condition NOTE1 | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | Max(600ms, 10 Max(MGRP, SMTC period)) CSSFinter_RedCap |
| DRX cycle ≤ 320ms | Max(600ms, Ceil(10 *1.5) Max(MGRP, SMTC period, DRX cycle)) CSSFinter_RedCap |
| DRX cycle > 320ms | 10 DRX cycle CSSFinter_RedCap |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 |  |

Proposal 27: The time period for SSB index identification (PBCH decoding) defined in intra/inter-frequency measurement requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

- Reuse the same extensions for 1Rx RedCap UE in TN network.

- Increase number of attempts by 3 SSB samples for TSSB_time_index_intra and TSSB_time_index_inter.

- Extend the lower boundary from ‘120 ms’ to ‘160ms’.

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(160ms, ceil( 6 x Kp ) x SMTC period)Note 1 x CSSFintra_RedCap |
| DRX cycle≤ 320ms | max(160ms, ceil (1.5 x 6 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra_RedCap |
| DRX cycle>320ms | Ceil(6 x Kp) x DRX cycle x CSSFintra_RedCap |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

| Condition NOTE1 | TSSB_time_index_inter |
| --- | --- |
| No DRX | Max(160ms, 6 Max(MGRP, SMTC period)) CSSFinter_RedCap |
| DRX cycle ≤ 320ms | Max(160ms, Ceil(6 1.5) Max(MGRP, SMTC period, DRX cycle)) CSSFinter_RedCap |
| DRX cycle > 320ms | 6 DRX cycle CSSFinter_RedCap |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 |  |

Proposal 28: The measurement period defined in inter-frequency measurement with measurement gaps requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

- Reuse the same extensions for 1Rx RedCap UE in TN network.

- Extend the lower boundary from ‘200 ms’ to ‘400ms’

- Keep the same number of samples for T SSB_measurement_period_inter.

| Condition NOTE1 | T SSB_measurement_period_inter |
| --- | --- |
| No DRX | Max(400ms, 8 Max(MGRP, SMTC period)) CSSFinter_RedCap |
| DRX cycle ≤ 320ms | Max(400ms, ceil(1.5 8) Max(MGRP, SMTC period, DRX cycle)) CSSFinter_RedCap |
| DRX cycle > 320ms | 8 DRX cycle CSSFinter_RedCap |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 |  |

Proposal 29: The measurement period defined in intra-frequency measurement with/without gap and inter-frequency measurement without gaps requirements for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same.

L1-RSRP measurements for Reporting

For SSB and CSI-RS based L1-RSRP measurement requirements, there were no extensions on the measurement period TL1-RSRP_Measurement_Period_SSB_RedCap and TL1-RSRP_Measurement_Period_CSI-RS_RedCap for FR1 for 1 Rx RedCap UE in TN network [4]. Therefore, we believe that RAN4 doesn’t need to extend the measurement period TL1-RSRP_Measurement_Period_SSB and TL1-RSRP_Measurement_Period_CSI-RS for 1Rx (e)RedCap UEs with FR1-NTN bands, that is, reuse those defined for 2Rx.

Proposal 30: The SSB and CSI-RS based L1-RSRP measurement requirements for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same.

Measurement accuracy requirements

The specific measurement accuracy requirements were defined in clauses 10.1A and 10.2A in TS 38.133[4] for (e)RedCap UEs in TN network, including NR measurements for RedCap and E-UTRAN measurements for RedCap. For 2Rx (e)RedCap UEs in TN network, the legacy accuracy requirements were reused. For 1Rx (e)RedCap UEs in TN network, 1 dB relaxation on SS-RSRP/ SS-RSRQ/ SS-SINR accuracy requirements was considered for L3 intra- and inter-frequency measurement in FR1, and 3dB relaxation on L1-RSRP accuracy requirements was considered for SSB based and CSI-RS based L1 measurement in FR1.

In current NTN requirements, the specific measurement accuracy requirements were also defined with the suffix C in TS 38.133[4]. SS-RSRP/ SS-RSRQ/ SS-SINR accuracy requirements were considered for L3 intra- and inter-frequency measurement in FR1, and SSB based L1-RSRP accuracy requirements was considered in FR1. All of the values of accuracy are the same as the legacy.

Therefore, we believe that RAN4 can reuse the accuracy requirements defined in NTN for 2Rx (e)RedCap UEs with FR1-NTN bands, and consider the same relaxation as TN for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

Proposal 31: RAN4 to reuse the accuracy requirements defined in NTN for 2Rx(e)RedCap UEs with FR1-NTN bands, and consider the same relaxation as TN for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

For SSB_RP and SSB Ês/Iot, we believe that RAN4 can reuse the conditions defined in NTN for a corresponding Band. For 1Rx (e)RedCap UEs and 2Rx (e)RedCap UEs, the values for SSB_RP and SSB Ês/Iot are the same.

Proposal 32: RAN4 to reuse the conditions defined in NTN for a corresponding Band, and the values for SSB_RP and SSB Ês/Iot are the same for 1Rx (e)RedCap UEs and 2Rx (e)RedCap UEs with FR1-NTN bands.

  - HD-FDD

FD-FDD and HD-FDD mode have been considered for (e)RedCap UE. However, HD-FDD (e)RedCap UEs can’t transimit the UL data and receive DL data at the same time, so RAN4 also need to discuss the imapct on RRM requirements for HD-FDD (e)RedCap UE with FR1-NTN bands. In the last meeting, RAN4 discussed the specific impact of HD-FDD and the agreement is copied as belows [3]:

#### Issue 6-4-1: The specific impact of HD-FDD for (e)Redcap UE with FR1-NTN Online agreement <Agreement> - RAN4 to consider the impact of HD-FDD for (e)Redcap UE with FR1-NTN. - The TN HD-FDD related requirements and applicable conditions defined for (e)RedCap UE can be used as a starting point, and RAN4 take into account NTN specific aspects if any.

RAN4 has agreed that the TN HD-FDD related requirements and applicable conditions defined for (e)RedCap UE can be used as a starting point. According to the RedCap WI, the following  RRM requirements are affected by HD-FDD for (e)RedCap UE in TN network, so the same requirements will be affected by HD-FDD for (e)RedCap UE with FR1-NTN.

- Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state

- Handover interruption time

- Random access

- SA: RRC Connection Release with Redirection

- Minimum requirement for L1 indication for RLM and LR

- MAC-CE/DCI/RRC based uplink spatial relation switch delay

- Scheduling availability of UE performing intra/inter measurements

- Scheduling availability of UE during L1-RSRP measurement

Proposal 33: The following requirements will be affected by HD-FDD for (e)RedCap UE with FR1-NTN:

- Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state

- Handover interruption time

- Random access

- SA: RRC Connection Release with Redirection

- Minimum requirement for L1 indication for RLM and LR

- MAC-CE/DCI/RRC based uplink spatial relation switch delay

- Scheduling availability of UE performing intra/inter measurements

- Scheduling availability of UE during L1-RSRP measurement

For the specific impact on above RRM requirements, RAN4 should consider the requirement applicability for HD-FDD (e)RedCap UE with FR1-NTN bands as legacy, and the legacy applicability can be used as baseline. Since RAN1 is still discussing the enhancements for mitigating issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB, and there is no much conclusions, the HD-FDD related enhancement on legacy requirement applicability should wait for more RAN1 progress.

Observation : RAN1 is still discussing the enhancements for mitigating issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB, and there is no much conclusions.

Proposal 34: The HD-FDD related enhancement on legacy requirement applicability in NTN should wait for more RAN1 progress.

  - Specification structure：

To facilitate the writing of CR, the specification structure also should be discussed. In the last meeting, the candidate solutions about specification structure are copied as below [3]:

<Way Forward> : Further discuss the following proposal - Proposal 1 (CATT, CMCC): - Use the following principles to define the RRM requirements for (e)RedCap UEs with FR1-NTN bands: - Define them in the new sections of section number with new suffix X in the specification. - Utilize the reference method for the requirements that can be reused. - Proposal 2 (CMCC): - Slightly prefer to introduce the Redcap over NTN requirements in new subsections in TS 38.133 or new subclauses in current NTN section.

We support to define the requirements for (e)RedCap UEs with FR1-NTN in the new sections of section number with new suffix X in the specification, like current way used for NTN requirements and (e)RedCap requirements. It is more clear and simple way. In addition, RAN4 will utilize the reference method for the requirements that can be reused, thereby reducing the redundancy of the spec to a certain extent.

For the method of introducing new subclauses in current NTN section, one question is that both current NTN requirements and (e)RedCap requirements already had their own suffixes [4], so a new suffix after NTN suffix may need to be added, and we’re not sure it is a permitted methods.

Observation 11: For introducing new subclauses in current NTN section, one question is that both current NTN requirements and (e)RedCap requirements already had their own suffixes, so a new suffix after NTN suffix may need to be added.

Proposal 35: To use the following principles to define the RRM requirements for (e)RedCap UEs with FR1-NTN bands:

- Define them in the new sections of section number with new suffix X in the specification.

- Utilize the reference method for the requirements that can be reused.

- Conclusion

This document discussed the issues for (e)RedCap RRM requirements for R19 NR NTN and presented following observations and proposals:

Observation 1: Both 20MHz maximum bandwidth and 5MHz maximum bandwidth will be considered for (e)RedCap UE with FR1-NTN bands.

Observation 2: There is no RRM impact on the further bandwidth reduction to 5MHz for R18 eRedCap UEs.

Proposal 1: For scenario considered for (e)RedCap UE with FR1-NTN bands, RAN4 only supports NR SA operation mode with single carrier.

Proposal 2: For the requirements of cell re-selection for RRC_IDLE/RRC_INACTIVE state mobility, both NTN-NTN and NTN-TN (from NTN to TN and from TN to NTN) will be defined for (e)RedCap UE with FR1-NTN bands.

Proposal 3: RAN4 to define the requirements of the following types of L3 measurements for (e)RedCap UEs with FR1-NTN:

- Intra-frequency measurements without measurement gaps

- Intra-frequency measurements with measurement gaps

- Inter-frequency measurement without measurement gaps

- Inter-frequency measurement with measurement gaps

Observation 3: The current RAN2 spec does not preclude supporting NR Rel-17/18 Conditional Handover and RACH-less Handover for (e) RedCap UE.

Proposal 4: The requirements for NR Rel-17/18 Conditional Handover and RACH-less Handover can be defined in RAN4 without the work of other group.

Observation 4: CG-SDT and RA-SDT are only supported for RedCap UEs, and related processes are not currently supported in NTN.

Proposal 5: If there is no other WG impacts, CG-SDT and RA-SDT can be defined for (e)RedCap UE with FR1-NTN bands.

Observation 5: MDT requirements were not defined for (e)RedCap UEs in TN network.

Observation 6: If MDT is defined for (e)RedCap UE with FR1-NTN bands, (e)RedCap UEs will meet the MDT requirements in NTN but will have no MDT requirements in TN, it is irrational.

Proposal 6: Deprioritize to define the requirements of MDT for (e)RedCap UE with FR1-NTN bands.

Proposal 7: Whether to consider the R17 enhenced relaxed measurements in RRC_IDLE/RRC_INACTIVE mode for (e)RedCap UEs with FR1-NTN bands should be discussed according to the actual power saving demand.

Observation 7: Techenically speaking, all eDRX cycle can be supported in GEO deployment, which includes Rel-17/18 eDRX enhancement introduced for (e)RedCap UEs in TN.

Proposal 8: RAN4 should further check the necessity of applicability in GEO deployment, e.g., whether there is such a typical scenario in a real NTN deployment that needs such a large eDRX cycle.

Observation 8: Techenically speaking, up to 10.24s eDRX cycle may feasible in earth-fixed LEO deployment.

Proposal 9: Prefer not to introduce any eDRX cycles for (e)RedCap UE with FR1-NTN in LEO deployment.

Observation 9: The legacy searcher assumption and CSSF are related to the capability of the UE to process and depend on the type of UEs.

Proposal 10: Single searcher assumption and CSSF defined for (e)RedCap UEs in TN will be reused for (e)Redcap UE with FR1-NTN.

Observation 10: The supported types of gaps are related to the types of UEs, and introducing the enhanced gap will bring more complexity for (e)Redcap UE with FR1-NTN.

Proposal 11: For (e)Redcap UE with FR1-NTN, prefer to reuse the gap defined for (e)RedCap UEs in TN and not to introduce the enhanced gap, such as concurrent gaps.

Proposal 12: RAN4 to consider the offset for the reletive cell-specific RSRP thresholds for 1Rx(e)RedCap UEs with FR1-NTN bands, including Qrxlevmin and Qqualmin:

- -1 dB offset can be reused as a baseline.

Proposal 13: RAN4 to reduce the number of NR inter-frequency carriers that UE shall at least be capable of monitoring for 1Rx (e)RedCap UEs with FR1-NTN bands.

- 6 NR inter-frequency carriers can be reused.

Proposal 14: The paremeters of Nserv_RedCap, Tdetect,NR_Intra/Inter_RedCap, Tmeasure,NR_Intra/Inter_RedCap and Tevaluate,NR_Intra/Inter_RedCap defined for cell re-selection requirements will be not distinguished between 1Rx and 2 Rx (e)RedCap UE.

Proposal 15: For the measurements of intra- and inter-frequency NR cells,

- +1 dB offset on the threshold of absThreshSS-BlocksConsolidation can be reused as a baseline for 1Rx(e)RedCap UEs with FR1-NTN bands.

- For the cases of rangeToBestCell is not configured and rangeToBestCell is configured, +1 dB offset can be reused for better cell ranked for 1Rx (e)RedCap UEs with FR1-NTN bands.

Proposal 16: In RRC_IDLE / RRC_INACTIVE state, +1 dB offset can be reused for a margin in FR1 for reselections when UE evaluating whether a newly detectable inter-frequency cell meets the reselection criteria for 1Rx (e)RedCap with FR1-NTN:

- For 1 Rx RedCap by a margin of at least

- 6 dB in FR1 for reselections based on ranking or

- 7 dB in FR1 for SS-RSRP reselections based on absolute priorities or

- 5 dB in FR1 for SS-RSRQ reselections based on absolute priorities.

Proposal 17: Handover delays should be extended for 1Rx(e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

- Define the relaxation on Tsearch for unknown intra/inter-frequency cell.

- The same relaxation for 1 Rx RedCap UE in TN network can be reused as baseline.

- Tsearch = 2*Trs for intra-frequency HO

- Tsearch = 5* Trs for inter-frequency HO

Proposal 18: UE RRC re-establishment delay (TUE_re-establish_delay) should be extended for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

- Define the relaxation on Tidentify for known and unknown intra/inter-frequency cell.

- The same relaxation for 1 Rx RedCap UE in TN network can be reused as baseline.

- Relax the Tidentify_intra_NR and Tidentify_inter_NR, i by 1 more samples for FR1 known and unknown intra/inter-frequency cell.

Proposal 19: For random access requirements, RAN4 to consider the offset for the following relative cell-specific RSRP thresholds.

- +1 dB offset can be reused as a baseline for rsrp-ThresholdSSB, msgA-RSRP-ThresholdSSB and msgA-RSRP-Threshold.

- -1 dB offset can be reused as a baseline for rsrp-ThresholdMsg3.

Proposal 20: RRC connection release with redirection to NR delay (Tconnection_release_redirect_NR) should be extended for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

- Define the relaxation on Tidentify-NR for FR1.

- The same relaxation for 1 Rx RedCap UE in TN network can be reused as baseline.

- Relax the Tidentify-NR by 1 more samples for FR1.

Proposal 21: There is no need to define the following timing requirements for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands respectively.

- UE transmit timing

- UE timer accuracy

- Timing Advance

Proposal 22: For SSB based and CSI-RS based radio link monitoring/beam failure detection requirements, some of hypothetical PDCCH parameters should be defined for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN respectively, including aggregation level (CCE) and bandwidth (PRBs).

- The same values for 1 Rx RedCap UE in TN network can be reused as baseline.

- Aggregation level (CCE) of 16.

- Bandwidth (PRBs) of 48.

Proposal 23: For SSB based and CSI-RS based RLM/BFD requirements, SCS=60kHz is not applicable for (e)RedCap UEs with FR1-NTN.

Proposal 24: The SSB and CSI-RS based RLM OOS and BFD evaluation periods should be extended for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

- The same extension for 1 Rx RedCap UE in TN network can be reused as baseline.

- For RLM evaluation periods,

- RAN4 not to extend the TEvaluate_in_SSB/CSI-RS for FR1.

- RAN4 to extend the TEvaluate_out_SSB/CSI-RS by factor 2 for FR1.

- For BFD evaluation periods,

- RAN4 to extend the TEvaluate_BFD_SSB/CSI-RS by factor 2 for FR1.

Proposal 25: The following requirements defined for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same.

- Active BWP switch delay

- Active TCI state switching delay

- UE-specific CBW change

Proposal 26: The time period for PSS/SSS detection defined for intra/inter-frequency measurement requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

- Reuse the same extensions for 1Rx RedCap UE in TN network.

- Increase the number of attempts by 2 SSB samples for TPSS/SSS_sync_intra and TPSS/SSS_sync_inter.

- Not to extend the lower boundary.

Proposal 27: The time period for SSB index identification (PBCH decoding) defined in intra/inter-frequency measurement requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

- Reuse the same extensions for 1Rx RedCap UE in TN network.

- Increase number of attempts by 3 SSB samples for TSSB_time_index_intra and TSSB_time_index_inter.

- Extend the lower boundary from ‘120 ms’ to ‘160ms’.

Proposal 28: The measurement period defined in inter-frequency measurement with measurement gaps requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

- Reuse the same extensions for 1Rx RedCap UE in TN network.

- Extend the lower boundary from ‘200 ms’ to ‘400ms’

- Keep the same number of samples for T SSB_measurement_period_inter.

Proposal 29: The measurement period defined in intra-frequency measurement with/without gap and inter-frequency measurement without gaps requirements for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same.

Proposal 30: The SSB and CSI-RS based L1-RSRP measurement requirements for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same.

Proposal 31: RAN4 to reuse the accuracy requirements defined in NTN for 2Rx(e)RedCap UEs with FR1-NTN bands, and consider the same relaxation as TN for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

Proposal 32: RAN4 to reuse the conditions defined in NTN for a corresponding Band, and the values for SSB_RP and SSB Ês/Iot are the same for 1Rx (e)RedCap UEs and 2Rx (e)RedCap UEs with FR1-NTN bands.

Proposal 33: The following requirements will be affected by HD-FDD for (e)RedCap UE with FR1-NTN:

- Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state

- Handover interruption time

- Random access

- SA: RRC Connection Release with Redirection

- Minimum requirement for L1 indication for RLM and LR

- MAC-CE/DCI/RRC based uplink spatial relation switch delay

- Scheduling availability of UE performing intra/inter measurements

- Scheduling availability of UE during L1-RSRP measurement

Proposal 34: The HD-FDD related enhancement on legacy requirement applicability in NTN should wait for more RAN1 progress.

Observation 11: For introducing new subclauses in current NTN section, one question is that both current NTN requirements and (e)RedCap requirements already had their own suffixes, so a new suffix after NTN suffix may need to be added.

Proposal 35: To use the following principles to define the RRM requirements for (e)RedCap UEs with FR1-NTN bands:

- Define them in the new sections of section number with new suffix X in the specification.

- Utilize the reference method for the requirements that can be reused.

- Reference

- RP-241789 Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Thales, CATT

- R4-2406610, WF on NR_NTN_Ph3_UERF, Qualcomm Inc.

- R4-2414037, The WF on RRM requirements for Rel-19 NR NTN phase3, CATT

- 3GPP TS 38.133, NR Requirements for support of radio resource management

- 3GPP TS 38.306, NR User Equipment (UE) radio access capabilities