3GPP RAN WG4 Meeting #112bis	R4-2415165

Hefei, China, October 14 – October 18, 2024

Title: 	Discussion on Core requirements of RedCap and BW aggregation Pos

Source: 	CATT

Agenda item:	4.2.1.4

Document for:	Discussion

Introduction

In RAN4#112 meeting, RAN4 discussed the core requirements for RedCap and the interruption requirements due to SRS aggregation transmission and the agreements were captured in the WF [1]. In this paper, we will further discuss some open issues.

Discussion

Core requirement on N_hop for RedCap Positioning

In last RAN4 meeting, an open issue is how to update Nhops,effect to cover the impacts of muting patterns. The options are copied below:

Proposals: - Option 1: the parameter NrepPRS shall take into account the number of unmuted inter-slot PRS repetitions that overlap with the sampling window within each hop - Option 2: Nhops,effect shall take into account the number of unmuted inter-slot PRS repetitions that overlap with the sampling windows of the hopping pattern with Nhopsslot hops per slot, within a single MG occasion, excluding the gap retuning times. - Option 3: Other options are not precluded.

In our understanding, two options have no huge difference and Option 2 provided more details. Besides, it is necessary to consider the value of  Nhopsslot in each hop because the value may be different in different hops. After some study, we think the solution proposed in [2] can solve the issue caused by muting patterns with some small simplifications.

Before giving our solution, we would like to explain the main logic first. The current situation is RAN4 needs to define the number of effective hops in a single MG occasion, while the muting pattern makes the issue more complex because the configurations of PRS resources and resource sets can be very flexible in time domain and so is the muting pattern, which may result in different value of  Nhopsslot in each hop. Currently, RAN4 has defined a table to capture applicable number of hops per slot with different PRS configurations, switching periods and sampling window duration considered, i.e., Table 9.9A.2.6.1-1. Hence, it is reasonable and quite straightforward to count the number of hops in a sampling window-wise manner in each MG occasion, i.e., in each sampling window, the value of  Nhops,effect is decided by the unmuted PRS resources that fully or partially overlap with this sampling window.

Based on the above discussion and revision to the solution in [2], we prefer the following solution:

Nhops,effect is updated as:

Nhops,effect=hNrepmax

Where

- h is the index of sampling window within a MG instance;

- Nrepmax=2 if Nhopsslot=2, otherwise, Nrepmax=1;

- When deciding the value of Nhopsslot, only the unmuted PRS resources that fully or partially overlap with this sampling window are considered. If all PRS resources within sampling window h are muted, Nrepmax=0.

Proposal 1: Nhops,effect,i,j is updated as:

Nhops,effect,i,j=hNrepmax

Where

- h is the index of sampling window within a MG instance;

- Nrepmax=2 if Nhopsslot=2, otherwise, Nrepmax=1;

- When deciding the value of Nhopsslot, only the unmuted PRS resources that fully or partially overlap with this sampling window are considered. If all PRS resources within sampling window h are muted, Nrepmax=0.

In last meeting, the calculation of  Nhop was also discussed and updated in the end. In current spec, it is derived by:

Nhop, i,  j=minNhops,effect,i, j,N3Lper-hop,i,j,Nhop,max,i

During the discussions, we generally agreed this update since it is reasonable to take into account UE’s processing capability when calculating the number of hops in one MG occasion. However, after careful check on this update, it may need some refinements. Firstly, we provide the definition of N3 below for reference:

processingDuration-r18	SEQUENCE {

processingPRS-SymbolsDurationN3-r18		ENUMERATED {msDot125, msDot25, msDot5, ms1, ms2,

ms4, ms6, ms8, ms12,ms16, ms20, ms25,

ms30, ms32, ms35, ms40, ms45, ms50},

processingDurationT3-r18				ENUMERATED {ms8, ms16, ms20, ms30, ms40, ms80,

ms160, ms320, ms640, ms1280}

- processingDuration: Indicates the duration of DL-PRS symbols N3 in units of ms a UE can process every T3 ms. - processingPRS-SymbolsDurationN3: This field specifies the values for N3. Enumerated values indicate 0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50 ms. - processingDurationT3: This field specifies the values for T3. Enumerated values indicate 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280ms.

Apparently, N3 and T3 are provided in a manner of combinations and N3 makes sense only with T3 considered. Since number of hops is calculated in each MG occasion, therefore the relationship between MGL and T3 should be taken into account. For most of measurement gap configurations, MGL is smaller than T3 and the updated formula can work properly. But for MG#24 and #25, the MGL can be 10ms or 20ms which can be larger than T3 in some circumstances, where the formula may give wrong values for number of hops in one MG occasion. For cases where MGL>T3, the formula can be modified as follows:

Nhop, i,  j=minNhops,effect,i, j,MGL*N3iT3iLper-hop,i,j,Nhop,max,i

Where

- MGL is the measurement gap length of the current MG occasion;

- T3i is indicated via UE capability processingDurationT3-r18 for positioning frequency layer i.

Proposal 2: Nhop, i,  j is updated for cases with MGL>T3 as follows:

Nhop, i,  j=minNhops,effect,i, j,MGL*N3iT3iLper-hop,i,j,Nhop,max,i

Where

- MGL is the measurement gap length of the current MG occasion;

- T3i is indicated via UE capability processingDurationT3-r18 for positioning frequency layer i.

Interruption requirements for SRS aggregation

In RAN4#112 meeting, the principle to define interruption requirements for SRS aggregation was agreed:

Agreements - The interruption length is derived by: guard period + duration of aggregated SRS transmission + guard period. - SRS is transmitted only if SRS duration plus guard period before and after does not collide with other UL transmission or DL reception with higher priority as defined by RAN1 on victim cells - SRS is transmitted only if SRS duration plus guard period before and after does not collide with any NR L3 or L1 measurement on victim cells - The victim cells/carriers can be derived by UE capability 41-4-9 - The interruption length is defined on a symbol level.

According to the above agreements, the interruption requirements will be defined on a symbol level to minimize the interruption length and the specific value is derived by guard period * 2 + duration of aggregated SRS transmission. In current spec, the possible duration of SRS transmission is {1, 2, 4} OFDM symbols, and the guard period is {0, 30, 100, 140, 200 us}. To support all SRS transmission configurations, we propose to convert the guard period into the number of OFDMs taking into different SCS configurations of victim cells into account, and the interruption length is decided by the SRS transmission configuration.

First, the total duration of the guard periods before and after aggregated SRS transmission is converted into number of OFDM symbols in Table 1, considering different SCS configurations of victim cells:

Table 1 Total duration of guard periods before and after aggregated SRS transmission in OFDM symbols

|  | Guard period (us) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SCS of victim cell (kHz) | 0 | 30 | 100 | 140 | 200 |
| 15 | 0 | 1 | 3 | 4 | 6 |
| 30 |  | 2 | 6 | 8 | 12 |
| 60 |  | 4 | 12 | 16 | 23 |
| 120 |  | 7 | 23 | 32 | 45 |
| 480 |  | 27 | 87 | 122 | 174 |
| 960 |  | 53 | 174 | 244 | 348 |

Regarding the number of SRS symbols, a conversion, which is quite straightforward, may also be needed if the SCSs of the aggressor cell and the victim cell(s) are different. The applicable conversions are also provided:

Table 2: Duration of aggregated SRS transmission with nrofSymbols = 1 (OFDM symbols)

|  | SCS of aggressor cell (kHz) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SCS of victim cell (kHz) | 15 | 30 | 60 | 120 | 480 | 960 |
| 15 | 1 | 1 | 1 | 1 | 1 | 1 |
| 30 | 2 | 1 | 1 | 1 | 1 | 1 |
| 60 | 4 | 2 | 1 | 1 | 1 | 1 |
| 120 | 8 | 4 | 2 | 1 | 1 | 1 |
| 480 | 32 | 16 | 8 | 4 | 1 | 1 |
| 960 | 64 | 32 | 16 | 8 | 2 | 1 |

Table 3: Duration of aggregated SRS transmission with nrofSymbols = 2 (OFDM symbols)

|  | SCS of aggressor cell (kHz) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SCS of victim cell (kHz) | 15 | 30 | 60 | 120 | 480 | 960 |
| 15 | 2 | 1 | 1 | 1 | 1 | 1 |
| 30 | 4 | 2 | 1 | 1 | 1 | 1 |
| 60 | 8 | 4 | 2 | 1 | 1 | 1 |
| 120 | 16 | 8 | 4 | 2 | 1 | 1 |
| 480 | 64 | 32 | 16 | 8 | 2 | 1 |
| 960 | 128 | 64 | 32 | 16 | 4 | 2 |

Table 4: Duration of aggregated SRS transmission with nrofSymbols = 4 (OFDM symbols)

|  | SCS of aggressor cell (kHz) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SCS of victim cell (kHz) | 15 | 30 | 60 | 120 | 480 | 960 |
| 15 | 4 | 2 | 1 | 1 | 1 | 1 |
| 30 | 8 | 4 | 2 | 1 | 1 | 1 |
| 60 | 16 | 8 | 4 | 2 | 1 | 1 |
| 120 | 32 | 16 | 8 | 4 | 1 | 1 |
| 480 | 128 | 64 | 16 | 16 | 4 | 1 |
| 960 | 256 | 128 | 64 | 32 | 8 | 4 |

The interruption length can be derived by adding a value in Table 1 and a value in Table 2/3/4.

Proposal 3: The interruption values are defined in the following procedures:

- Total duration of the guard periods before and after aggregated SRS transmission is converted into number of OFDM symbols and captured into spec.

- The final interruption length is decided by the number of configured SRS symbols.

A draft CR to incorporate the interruption requirements into RAN4 RRM spec is also submitted [3].

Core requirements for PRS aggregation

During the discussions in last meeting, RAN4 identified that the basic principle to define delay requirements for BW aggregation measurements may need to be revisited. The candidate options are reproduced below:

Proposals - When UE is indicated by LMF in nr-DL-PRS-JointMeasurementRequest-r18 to perform measurements by aggregating resources from multiple PFLs - Option 1: UE only measures resources indicated by LMF as linked resources. - Remove requirement on Tnon-aggregate from the existing core requirements for PRS aggregation. - Option 2: UE measures resources that are indicated by LMF as linked resources and resources that are not indicated by LMF as linked resources.

For this issue, we checked RAN1 agreements and one agreement achieved in RAN1#113 probably provides an answer to this issue [4].

Agreement (RAN1#113) When the UE receives a request to perform aggregated measurements, - TRP(s) that include PRS aggregation have higher priority than the TRPs that do not include PRS aggregation - If 2 or more TRPs include linked resources, then their priority follows the legacy priority, i.e., sorted in the configuration according to priority - If a PRS resource set is linked for aggregation, then it has higher priority compared to the PRS resource set not linked for aggregation. - If both sets in a PFL are linked for aggregation, then their priority follows the legacy priority, i.e., sorted in the configuration according to priority

According to the RAN1 agreement, the PRS resource set linked for aggregation has higher priority compared to the PRS resource set not linked for aggregation, when UE receives a request to perform aggregated measurements. Hence UE needs to perform aggregated measurements before performing measurements on any PRS resources not linked for aggregation. And from our understanding, whether UE needs to perform non-aggregated measurements depends on UE implementation and it is not mandatory by network. Therefore Option 1 is more preferred.

Proposal 4: When UE receives a request to perform aggregated measurements, UE is requested to measures resources only indicated by LMF as linked resources. RAN4 removes requirements on Tnon-aggregate from the existing core requirements for PRS aggregation.

Conclusions

This paper discussed the core requirements for RedCap and the interruption requirements for SRS BW aggregation, and some proposals are provided:

RedCap:

Proposal 1: Nhops,effect,i,j is updated as:

Nhops,effect,i,j=hNrepmax

Where

- h is the index of sampling window within a MG instance;

- Nrepmax=2 if Nhopsslot=2, otherwise, Nrepmax=1;

- When deciding the value of Nhopsslot, only the unmuted PRS resources that fully or partially overlap with this sampling window are considered. If all PRS resources within sampling window h are muted, Nrepmax=0.

Proposal 2: Nhop, i,  j is updated for cases with MGL>T3 as follows:

Nhop, i,  j=minNhops,effect,i, j,MGL*N3iT3iLper-hop,i,j,Nhop,max,i

Where

- MGL is the measurement gap length of the current MG occasion;

- T3i is indicated via UE capability processingDurationT3-r18 for positioning frequency layer i.

BW aggregation:

Proposal 3: The interruption values are defined in the following procedures:

- Total duration of the guard periods before and after aggregated SRS transmission is converted into number of OFDM symbols and captured into spec.

- The final interruption length is decided by the number of configured SRS symbols.

Proposal 4: When UE receives a request to perform aggregated measurements, UE is requested to measures resources only indicated by LMF as linked resources. RAN4 removes requirements on Tnon-aggregate from the existing core requirements for PRS aggregation.

References

[1] R4-2414032, WF on RedCap positioning and PRS/SRS bandwidth aggregation, Ericsson, RAN4#112.

[2] R4-2412647, Discussion on RedCap positioning and PRS/SRS BWA, Huawei, RAN4#112.

[3] R4-2315161, Draft CR on core requirements for SRS BW aggregation, CATT, RAN4#112bis.

[4] Chair’s note RAN1#113 eom, RAN1#113.