3GPP RAN WG4 Meeting #112bis	R4-2415167

Hefei, China, October 14 – October 18, 2024

Title: 	Discussion on Performance requirements of RedCap UE positioning

Source: 	CATT

Agenda item:	4.2.2.4

Document for:	Discussion

Introduction

In RAN4#112 meeting, fruitful progress regarding the performance requirements was achieved which was captured in the WF [1] and Rel-18 Positioning WI was successfully closed. However, there are some issues which need further discussions. In this contribution, we will discuss these performance issues.

Discussion

Group delay calibration margin for RedCap positioning

The measurement accuracy requirements were defined in the last meeting while group delay calibration margins for RSTD and Rx-Tx time difference measurements are not completely determined yet. The main issue is whether an additional margin is needed for FH cases. The related agreements are copied below:

Agreement: - RAN4 to use the following assumptions for group delay calibration margin for RedCap - Non-FH: existing values for non-RedCap for the applicable BW. - FH: existing values for non-RedCap for per-hop BW [FFS: plus an extra margin (value TBD)].

Proponents suggested that the applicable calibration margin values are limited by the per-hop BW because the UE’s hardware capabilities are still limited and additional margins may be needed since UE’s Rx chain works in different frequencies within one PFL. From measurement point of view, each measurement is performed based on the per-hop BW, therefore it is reasonable to use the margin corresponding to the per-hop BW for the final measurement result after frequency hopping.

Regarding the need for additional margins, considering the margin value corresponding to per-hop BW is larger than that corresponding to the total BW after FH, the jitter due to Rx chain calibration within single PFL could be covered by the large margin values. Therefore no additional margin is added on top of legacy margin values.

Proposal 1: Clarify that margin values are limited by per-hop BW in performance requirements, and no additional margin is added on top of the legacy margin values.

PRS RMC for RedCap positioning TCs

One issue for RedCap positioning TCs is how to configure the bandwidths for RedCap UEs and cells since the legacy configurations may cause misunderstanding to TE vendors since in legacy TCs the same bandwidth applies for both DUT and cells. Some inputs from TE vendors were provided in the WF and reproduced below for reference:

Discussion in AH2: - TE(R&S) input on the earlier tentative agreement from AH1 [For PRS BW for FH TCs, consider the BW in test configuration as UE BW and allow cell BW to be larger.]: - Different BWs for cell and UE have never been used - There will be confusion between OCNG and UE BW, even if we go that way - Challenge for TE: interpret OCNG (and other RMCs) as the UE BW - Two steps could be considered: - Step 1: in the configuration, separately cell BW and UE channel BW are mentioned - Step 2: resolve the OCNG issue - FFS: UE total BW should match a cell BWP (added off-line) - Recommendation: consider the above input in the work on TCs

In order to eliminate the misunderstandings to TE vendors caused by bandwidth configurations, a straightforward way is to explicitly configure the bandwidth of RedCap UE and cells as Step 1 above.

Besides, TE vendors brought up another issue which is the bandwidth of the configured OCNG may be interpreted as the UE BW by TE vendors. Considering that frequency hopping is a UE implementation which means gNB transmits PRS across the configured PRS BW and the unused REs are allocated to virtual UEs, everything remains the same as the TCs without frequency hopping from the gNB point of view. Therefore we do not think there is any sever OCNG issues caused by frequency hopping. To deal with the interpretation issue of OCNG BW brought up by TE vendors, we propose to add a declaration of the OCNG BW in the TCs, i.e., the OCNG BW is the cell BW instead of RedCap UE BW. For the study part, i.e., UE total BW should match a cell BWP, we do not see a strong necessity to add such limit according to the discussion above.

Proposal 2: Explicitly configure BWs for UE and Cell separately and clarify that the OCNG BW is the cell BW in TCs.

Conclusions

This paper discussed the performance requirements for RedCap UE positioning and provided the following proposals:

Proposal 1: Clarify that margin values are limited by per-hop BW in performance requirements, and no additional margin is added on top of the legacy margin values.

Proposal 2: Explicitly configure BWs for UE and Cell separately and clarify that the OCNG BW is the cell BW in TCs.

References

[1] R4-2414032, WF on RedCap positioning and PRS/SRS bandwidth aggregation, Ericsson, RAN4#112.