3GPP RAN WG4 Meeting #112bis	R4-2415170

Hefei, China, October 14 – October 18, 2024

Title: 	Discussion on RRM core requirements for BM

Source: 	CATT

Agenda item:	6.17.4.1

Document for:	Discussion

Introduction

In RAN4#112 meeting, RAN4 discussed on Rel-19 AI/ML for NR air interface and agreements are captured in [1]. Two issues belonging to RRM core requirements for BM were brought up without any agreements. In this contribution, we will present our views on the two issues [2]:

-	UE side beam prediction requirements impact

-	Beam prediction and TCI state related issues

Discussion

UE side beam prediction requirements impact

In last meeting, a table including potential requirement impacts was provided in [2] copied below:

| UE- sided model | Data collection | FFS new clause for data collection for UE-sided model training | FFS RAN4 impact from data collection for UE-sided model training (depending on RAN1/2 input). |
| --- | --- | --- | --- |
|  | Additional assistance information | Added as new side condition: consistency/association shall be guaranteed for inference and perf. monitoring requirement testing | For the testing of UE-sided model inference and monitoring (if any), the consistency of NW-side additional condition across training and inference shall be guaranteed as side condition. |
|  | Inference | New clause for UE-side AI/ML model inference | RAN4 shall specify RRM requirement for UE-side AI/ML model inference: to guarantee (1) accuracy of inference results (2) the delay of reporting inference results (containing the inference latency). |
|  | Perf. monitoring | FFS new clause is needed for Type 1 - Option 1 New clause for Type 1 - Option 2 No RAN4 impact expected for Type 2 | For different types of performance monitoring for UE-sided model, the necessity of RAN4 requirement is provided as: - Type 1, Option 1 (NW-side performance monitoring): The necessity of RAN4 requirement on data collection for monitoring is not significant, because it is similar to data collection for other purposes. - Type 1, Option 2 (UE-assisted performance monitoring): RAN4 requirement on data collection for monitoring can be specified to test the accuracy of performance metrics calculated by UE. - Type 2 (UE-side performance monitoring): No RAN4 requirement is needed because no UE feedback will be performed. |
|  | LCM | New clause for RAN4 RRM requirement for “Network decision, network-initiated” LCM | For UE-sided model LCM, the necessity of RAN4 requirement: - “Network decision, network-initiated” LCM: RAN4 requirement is needed. - FFS the detailed RAN4 requirement (depending on RAN1/2 input). |

Basically, RAN4 needs to define core requirements for inference, performance monitoring and LCM-related procedures. Whether core requirements are needed for other procedures depends on RAN1/2 discussions. For LCM requirements, RAN4 needs to wait for RAN1/2 progress since RAN4 does not have sufficient inputs to start discussions.

Proposal 1: RAN4 postpones the discussions on LCM core requirements until RAN1/2 make sufficient progress.

Measurement and inference delay requirements

For inference, at least delay requirement should be defined and an agreement was achieved in RAN4#111 as below:

Agreement: - Use the following as the starting point - Inference latency can be implicitly captured in the delay of corresponding procedures (e.g. reporting delay) and/or performance requirements. - Inference latency is part of delay of the corresponding procedure and no separate requirement will be specified for inference latency. - Check the above bullets based on per use case, and if other WGs define the inference latency, RAN4 can revisit the above bullets.

And there are two sub-use cases are defined in TR 38.843 [3] including spatial-frequency domain prediction and time- spatial-frequency domain prediction. The details are reproduced below:

The following are selected as representative sub-use cases: - BM-Case1: Spatial-domain Downlink beam prediction for Set A of beams based on measurement results of Set B of beams - Consider: Alt. 1): AI/ML model training and inference at NW side. Alt. 2): AI/ML model training and inference at UE side. - Consider: Alt. i): Set A and Set B are different (Set B is NOT a subset of Set A). Alt. ii): Set B is a subset of Set A. Note: Set A is for DL beam prediction. The codebook construction of Set A and Set B can be clarified by companies. - AI/ML model input consider: Alt 1): Only L1-RSRP measurement based on Set B; Alt.2): L1-RSRP measurement based on Set B and assistance information; Alt. 3): CIR based on Set B; Alt. 4): L1-RSRP measurement based on Set B and the corresponding DL Tx and/or Rx beam ID. - BM-Case2: Temporal Downlink beam prediction for Set A of beams based on the historic measurement results of Set B of beams - Consider: Alt. 1): AI/ML model training and inference at NW side. Alt. 2): AI/ML model training and inference at UE side. - Consider: Alt. i): Set A and Set B are different (Set B is NOT a subset of Set A). Alt. ii): Set B is a subset of Set A (Set A and Set B are not the same). Alt. iii): Set A and Set B are the same. - AI/ML model input consider: measurement results of K (K≥1) latest measurement instances with the following alternatives: Alt. 1): Only L1-RSRP measurement based on Set B; Alt 2): L1-RSRP measurement based on Set B and assistance information; Alt. 3): L1-RSRP measurement based on Set B and the corresponding DL Tx and/or Rx beam ID. - F predictions for F future time instances can be obtained based on the output of AI/ML model, where each prediction is for each time instance. At least F=1. Set B is a set of beams whose measurements are taken as inputs of the AI/ML model.

According to the highlighted contents, BM-Case1 can be regarded as a special case of BM-Case2 in terms of delay requirements, which means it is possible to define unified requirements for both BM sub-use cases. Hence, RAN4 can prioritize defining inference delay requirements for BM-Case2 and modifications can be adopted for BM-Case1.

Proposal 2: RAN4 defines unified inference delay requirements for both BM-Case1 and Case2 and discussion is based on BM-Case2.

As many delay requirements defined in TS 38.133, some key parameters that impact the delay requirements should be considered, such as periodicity of measurement resources, periodicity of SMTC/MG and duration of DRX cycles, etc. In additional, the F and K, which are for BM-case2, also affect the inference delay requirements. The details are FFS after RAN1’s design on inference is completed.

Proposal 3: Inference delay requirements are defined by considering at least the follow parameters:

- Periodicity of measurement resources.

- Periodicity of SMTC/MG.

- Duration of DRX cycles.

- F and K (For BM-Case 2).

Details are FFS and RAN4 waits for more RAN1 inputs.

Performance monitoring delay requirements

Considering that AI/ML models may be deployed in different entities, i.e., UE side or gNB side, and the controllability may belong to an entity that is different from the one where the AI/ML models are deployed. Therefore different performance monitoring procedures should be considered. The possible scenarios are already captured in TR 38.843 [3]. And for BM-Case1 and BM-Case2 with a NW-side AI/ML model, no AI/ML-specific impacts are identified for now.

For BM-Case1 and BM-Case2 with a UE-side AI/ML model, two types of performance monitoring mechanisms are defined and are generally shown in Figure 2.

Figure 2-Two types of performance monitoring mechanisms for BM-Cases with a UE-side AI/ML model

For Type1 network-based performance monitoring, UE needs to measure the configured measurement resources and report either measurement results or performance monitoring metric/event and implements LCM procedures indicated by gNB. Therefore the time duration between the reception of the configuration/signalling for performance monitoring resources and UE reporting can be defined as performance monitoring delay.

For Type2 UE-based performance monitoring, indication/request/report from UE to gNB for performance monitoring and reporting may not be needed for all scenarios which are not clarified yet. Hence, more discussions are required for Type2 performance monitoring to identify the necessity of defining performance monitoring delay requirements.

Proposal 4: For BM cases with NW-side AI/ML model, no AI/ML-specific RRM requirements are needed.

Proposal 5: For Type1 network-based performance monitoring, the time duration between the reception of the configuration/signalling for performance monitoring resources and UE reporting can be defined as performance monitoring delay.

Proposal 6: For Type2 UE-based performance monitoring, RAN4 to further discuss the necessity of performance monitoring delay requirements.

Beam prediction and TCI state related issues

There were also some proposals discussing known/unknown TCI state for predicted beams and options are as below.

- Proposals - Option 1: Tx beam prediction does not imply that UE knows the best UE Rx beam to receive the predicted beam - TCI state for the predicted beam is unknown unless it has been explicitly measured within 1280ms - Option 2: UE knows the Rx beam paired with the predicted Tx beam - FFS whether the TCI state associated with a predicted Tx beam is known or unknown - Option 3: UE knows the Rx beam paired with the predicted Tx beam - TCI state should be considered known , FFS whether side conditions for known TCI state should be modified (relaxed) for this case

According the known conditions for TCI state in current RRM spec [4], it is very clear to judge if a TCI state is known or unknown. And the key point is whether the TCI state is measured by UE in the last 1280 ms as Option 1 mentioned. In other words, the TCI state of the predicted beam is known if the known conditions are met. Otherwise it is unknown. Therefore, in our understanding, RAN4 can directly reuse the known conditions for AI/ML enabled BM prediction

Proposal 7: The TCI state is known if it meets the known conditions defined in current RRM spec. Otherwise, it is unknown.

Conclusions

This paper discussed of RRM core requirement issues related to AI/ML for beam management, and following proposals are provided:

Proposal 1: RAN4 postpones the discussions on LCM core requirements until RAN1/2 make sufficient progress.

Proposal 2: RAN4 defines unified inference delay requirements for both BM-Case1 and Case2 and discussion is based on BM-Case2.

Proposal 3: Inference delay requirements are defined by considering at least the follow parameters:

- Periodicity of measurement resources.

- Periodicity of SMTC/MG.

- Duration of DRX cycles.

- F and K (For BM-Case 2).

Details are FFS and RAN4 waits for more RAN1 inputs.

Proposal 4: For BM cases with NW-side AI/ML model, no AI/ML-specific RRM requirements are needed.

Proposal 5: For Type1 network-based performance monitoring, the time duration between the reception of the configuration/signalling for performance monitoring resources and UE reporting can be defined as performance monitoring delay.

Proposal 6: For Type2 UE-based performance monitoring, RAN4 to further discuss the necessity of performance monitoring delay requirements.

Proposal 7: The TCI state is known if it meets the known conditions defined in current RRM spec. Otherwise, it is unknown.

References

[1] R4-2414323, WF on AI/ML, Qualcomm Incorporated, RAN4#112.

[2] R4-2412831, Topic summary for [112][129] NR_AIML_air, QC, RAN4#112.

[3] TR 38.843, V18.0.0.

[4] TS 38.133, V18.7.0.