3GPP RAN WG4 Meeting #112bis	R4-2415171

Hefei, China, October 14 – October 18, 2024

Title: 	Discussion on testability and interoperability issues for BM

Source: 	CATT

Agenda item:	6.17.4.2

Document for:	Discussion

Introduction

In RAN4#112 meeting, RAN4 discussed on Rel-19 AI/ML for NR air interface and agreements are captured in [1]. There are still some issues related to testability and interoperability for beam management that needs to be further discussed. In this contribution, we will present our views on the following issues:

-	RSRP prediction accuracy

-	Metrics/KPIs for BM Measurement error impact

-	Measurement error impact

Discussion

RSRP prediction accuracy

In last meeting, definitions of absolute and relative RSRP prediction accuracy were recommended in [2]. The option is reproduced below for reference:

Issue 2-1: RSRP Prediction accuracy - Proposals - Option 1: The predicted L1-RSRP difference is similar to the legacy, the absolute RSRP accuracy and the relative RSRP accuracy can be: - Absolute RSRP accuracy= predicted L1-RSRP of beam index i – ideal measured L1-RSRP of beam index I (ground truth as agreed in RAN4#111) - Relative RSRP accuracy= predicted L1-RSRP of beam index i – predicted L1-RSRP of beam index n, where the beam index n owns the largest predicted value. - Accuracy requirements should apply to all predicted beams that satisfy the predefined side conditions, including SNR. - Option 2: others

The definition of absolute RSRP accuracy looks good to us, while the relative RSRP accuracy may need modification because it completely depends on prediction results without any measurement results involved, which may not be very reasonable. In addition, RAN1 also listed candidate options for monitoring metric shown below. RSRP difference is defined in Alt2 and Alt3, both of which consider the measurement results. Therefore the relative RSRP accuracy can be modified as follows:

    - Relative RSRP accuracy = predicted L1-RSRP of beam index i – predicted measured L1-RSRP of predicted beam index n, where the beam index n owns the largest predicted measured value obtained by measuring the predicted beams.

Agreement-RAN1#118: For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), further study at least the following alternatives, including: - Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/resources for monitoring - Alt 2: The L1-RSRP difference information based on actual measurement of the L1-RSRP of one or more of Top K predicted beam, and L1-RSRP measurements from a resource set/resources for monitoring - Alt 3: The RSRP difference information between the predicted RSRP and measured L1-RSRP of corresponding beam(s) of a resource set/resources for monitoring - Note: resources for Set B for monitoring are not precluded and can be study. - Note: this is only applicable when the model can predict RSRP - Alt 4: The probability information of the predicted beam(s) to be the Top 1 or Top K beam - Note: this is only applicable when the model can generate probability information - FFS: for Alt 1/2/3, on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If not, whether/how to obtain the measurement of the predicted Top 1 or Top K beam for calculating the prediction accuracy or the RSRP difference. - For all alternatives, study whether the performance information is calculated per sample (one-shot), or per set of samples (window)

Proposal 1: Relative RSRP accuracy is modified as follows:

    - Relative RSRP accuracy = predicted L1-RSRP of beam index i – predicted measured L1-RSRP of predicted beam index n, where the beam index n owns the largest predicted measured value obtained by measuring the predicted beams.

Metrics/KPIs for BM

There were no agreements on metrics/KPIs for beam management in previous RAN4 meeting. The latest list of options is summarized below for reference:

RAN4#111: For metrics for beam management requirements/tests, the following test metrics are identified and could be considered - Option 1: RSRP accuracy - Option 2: Beam prediction accuracy - Top-1 (%) : the percentage of "the Top-1 strongest beam is Top-1 predicted beam" - Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams" - Top-1/K (%) : the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams" - Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB, - Related measurement accuracy can be considered to determine x - Option 4: combinations of above options RAN4#112: - Option 1: Use Option 1 - Option 2: Use Option 2 - Option 3: Use Option 1,2,3 depending on use case - Option 4: Neither Option 1, 2, 3 is appropriate, a new metric is needed - Option 5: Consider reliability metric based on relative difference in between the predictions due to change in the conditions/inputs. - consider defining multiple tests under different conditions but with the same outcome(same predicted beam) - Option 6: Other

Since RAN4 probably will follow the legacy principle to define absolute RSRP accuracy and relative RSRP accuracy, the metrics need to be discussed separately as well. For absolute RSRP accuracy, the definition is quite clear enough and it can be used as metric directly. The next step that RAN4 needs to do is to define relative accuracy requirements and in test cases RAN4 can reuse the legacy test requirements.

For relative RSRP accuracy, we prefer to combine the Option 1 and 3, both of which are based on RSRP values, listed in RAN4#111 meeting with our proposal 1 considered, and the combined RSRP prediction metric can be:

    - The relative RSRP accuracy shall be less than X dB, where X are relative accuracy requirements and will be defined in performance part according to the agreed definition.

Proposal 2: The definition of absolute RSRP accuracy can be used as test metric directly. For relative RSRP accuracy metric, it can be defined as:

    - The relative RSRP accuracy shall be less than X dB, where X are relative accuracy requirements and will be defined in performance part according to the agreed definition.

In regards to the beam prediction accuracy, all three sub-options are feasible. The main difference is whether a margin shall be considered for beam ID prediction as the figure below illustrate.

Figure1. Illustrations of three sub-options for Option 2

In addition, a common issue to calculate the beam prediction metric is how to determine the Top-1 or Top-K measured beams, if UE does not measure the full set A, which is very likely considering the measurement overheads. We find that RAN1 currently has been discussing monitoring metrics and the alternatives are captured in section 2.1. In our opinion, the RAN1 agreements on beam prediction monitoring metric can be directly reused for tests. Hence, RAN4 can wait for RAN1 progress.

Proposal 3: For beam prediction accuracy, RAN4 waits for RAN1 progress in monitoring metric which could be reused for beam prediction tests directly.

Measurement error impact

The impact of measurement error on the inference accuracy was discussed and agreement is as follows:

Agreement: RAN4 will study the impact of measurement error for the input to inference and dataset for training (set B measurement error) on prediction accuracy, companies should bring proposals on how to proceed with such a study. impact on different prediction accuracy metrics should be taken into account studies already performance by other groups(RAN1) could also be taken into account Note: intention of the study is just to understand the impact of measurement error on prediction accuracy.

According to the note in the agreement, this study is to understanding the impact of measurement error on the prediction accuracy and the study outcomes will not be used as a side condition in accuracy requirements, which means this study has no impacts on test procedures. For example, RAN4 does not need justify how measurement error can be evaluated during tests and which accuracy requirement, i.e., specific value, should be fulfilled.

Regarding the study method, a straightforward method is to model the measurement error with a certain distribution, such as uniform distribution or Gaussian distribution. By changing characteristics of the selected distribution, different amplitudes of measurement errors can be generated and added to the ideal measurement results. Then the impacts of modelled measurement errors on inference accuracy can be evaluated by simulations.

Proposal 4: Measurement errors can be modelled with a certain statistical distribution, e.g., uniform or Gaussian distribution, and added to ideal measurement results. Then the impacts can be evaluated by simulations.

Conclusions

This paper discussed some issues related to AI/ML for beam management, and following proposals are provided:

Proposal 1: Relative RSRP accuracy is modified as follows:

    - Relative RSRP accuracy = predicted L1-RSRP of beam index i – predicted measured L1-RSRP of predicted beam index n, where the beam index n owns the largest predicted measured value obtained by measuring the predicted beams.

Proposal 2: The definition of absolute RSRP accuracy can be used as test metric directly. For relative RSRP accuracy metric, it can be defined as:

    - The relative RSRP accuracy shall be less than X dB, where X are relative accuracy requirements and will be defined in performance part according to the agreed definition.

Proposal 3: For beam prediction accuracy, RAN4 waits for RAN1 progress in monitoring metric which could be reused for beam prediction tests directly.

Proposal 4: Measurement errors can be modelled with a certain statistical distribution, e.g., uniform or Gaussian distribution, and added to ideal measurement results. Then the impacts can be evaluated by simulations.

References

[1] R4-2414323, WF on AI/ML, Qualcomm Incorporated, RAN4#112.

[2] R4-2412831, Topic summary for [112][129] NR_AIML_air, QC, RAN4#112.