3GPP RAN WG4 Meeting #112bis	R4-2415173

Hefei, China, October 14 – October 18, 2024

Title: 	Discussion on testability and interoperability issues for positioning

Source: 	CATT

Agenda item:	6.17.5.2

Document for:	Discussion

Introduction

In RAN4#112 meeting, RAN4 discussed Rel-19 AI/ML for NR air interface and agreements are captured in [1]. There were still some issues related to testability and interoperability for positioning accuracy enhancement that need to be further discussed [2]. In this contribution, we will present our views on the following issues:

-	Requirements for case 1

-	Reported metrics for case 2a

Discussion

Requirements for case 1

In RAN4#112 meeting, no agreements for case 1 were achieved and options are shown below:

- Proposals - Option 1: RAN4 should not define requirements for case 1 because positioning accuracy is not testable - Option 2: RAN4 to further study defining requirements for case 1, companies are invited to bring further analysis: - how to establish the ground truth ( what should the UE report) - how to define accuracy requirements - what are the inputs for the AI/ML model - what side conditions to consider - Option 3: RAN4 should study how to define relative requirements (e.g. how the reported position changes based on changing inputs) - Option 4: Others

Considering UE may be requested to report the inferred position by network and a RAN2 agreement indicates that the current LPP signallings can be used for at least case 1 [3], we prefer to define accuracy requirements and corresponding tests for case 1. Then two aspects need to be discussed to proceed: accuracy requirements and tests.

For tests, a main issue is how to obtain the ground truth of DUT. It is quite challenging in real deployment to get it, while there are solutions to achieve it since TE can control many parameters, which is not applicable in real deployment. One straightforward method is to set reasonable coordinates of DUT and TRPs that transmit PRS resources, and then TE can directly know the ground truth of DUT location, and the coordinates of TRPs can be signalled to DUT for assistance. After DUT reports its inferred position, TE can compare the reported position with the ground truth and then judge if DUT passes the tests. Another merit is DUT cannot cheat in tests because the ground truth is unknown to DUT.

There are many ways to choose a set of reasonable coordinates of DUT and TRPs. For example, RAN4 can fix the positions and radii of TRPs first and then find the intersection, coordinate of which should be DUT’s ideal coordinate.

Proposal 1: Ground truth of DUT can be set in tests as well as the coordinates of TRPs. RAN4 can provide these coordinates by proper designs of test cases.

For accuracy requirements, it is almost impossible to define requirements for coordinates directly. A more reasonable way is to define relative requirements, i.e., UE passes the tests if the 3-D distance between the ground truth and the UE predicted position is smaller than the required distance under specified side conditions, such as SINR, SCS, BW of measurement resources and channel model, etc.

Proposal 2: RAN4 to define relative accuracy requirements based on 3-D distance difference between ground truth and UE predicted position with following side conditions considered:

-	SINR

-	SCS and BW of measurement resources, e.g., PRS

-	Channel model

-	Other key parameters, if identified.

Reported metrics for case 2a

RAN4 discussed whether to define requirements for reported metrics for case 2a in last meeting and the agreement are copied below:

Agreement: - RAN4 will take existing requirements for existing metrics as baseline - FFS whether to revisit these requirements - RAN4 will study how to define requirement for any new reported metrics - FFS how to define requirements for LOS/NLOS

According to RAN1 study outcomes in R18 SI phase, significant improvement of positioning accuracy was observed [4]. Hence, RAN4 can consider tightening the exiting requirements for AI/ML enabled positioning.

In regards with the requirements for LOS/NLOS, considering it is not introduced by AI/ML positioning and it is used for assistance only. Besides, though statistic solutions were proposed by companies, they are more appropriate to be used for performance monitoring. For each LOS/NLOS indicator reporting, we do not see a strong motivation to define requirements.

Proposal 3: RAN4 to consider tightening the existing accuracy requirements for AI/ML enabled positioning.

Proposal 4: RAN4 not to define requirements for LOS/NLOS indicator reporting.

Conclusions

This contribution discussed some issues in regards with testability and interoperability for AI/ML enabled positioning accuracy enhancement and following proposals are provided:

Proposal 1: Ground truth of DUT can be set in tests as well as the coordinates of TRPs. RAN4 can provide these coordinates by proper designs of test cases.

Proposal 2: RAN4 to define relative accuracy requirements based on 3-D distance difference between ground truth and UE predicted position with following side conditions considered:

-	SINR

-	SCS and BW of measurement resources, e.g., PRS

-	Channel model

-	Other key parameters, if identified.

Proposal 3: RAN4 to consider tightening the existing accuracy requirements for AI/ML enabled positioning.

Proposal 4: RAN4 not to define requirements for LOS/NLOS indicator reporting.

References

[1] R4-2414323, WF on AI/ML, Qualcomm Incorporated, RAN4#112.

[2] R4-2412831, Topic summary for [112][129] NR_AIML_air, QC, RAN4#112.

[3] R2_126_ChairNotes_24-05-24_final, RAN2#126.

[4] TR 38.843, V18.0.0.