3GPP RAN WG4 Meeting #112bis	R4-2415177

Hefei, China, October 14 – October 18, 2024

Title: 	Discussion on testability and interoperability issues for AIML mobility

Source: 	CATT

Agenda item:	6.18.3

Document for:	Discussion

Introduction

In RAN#105, RAN4 scope of AIML mobility was clarified and one aspect is to study the testability and interoperability of AI/ML mobility. The details are reproduced below [1]:

- Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.) - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - NOTE 5: Avoid the overlaps with RAN2 work for evaluation - NOTE 1: RAN1/3 work can be triggered via LS NOTE 2: To avoid duplicate study with “AI/ML for NG-RAN” led by RAN3 NOTE 3: Two-sided model is not included

In this contribution, we will present our views on impacts of testability and interoperability of AI/ML mobility.

Discussion

General testability and interoperability issues

According to the SID [1], RAN2 are studying three cases and the progress till August meeting is summarized below:

- Case 1: RRM measurement prediction

  - Completed the 1st round of performance evaluation.

- Case 2: Measurement event prediction

  - Deprioritized.

- Case 3: RLF/HO prediction

  - RLF prediction: Completed the initial discussion on simulation assumptions.

  - HO prediction: Deprioritized.

Therefore RAN4 can focus on the testability and interoperability study of case 1 first. Regarding the model deployment, only one-sided model is adopted for each use case. Hence the reference diagram agreed in AI/ML for NR air WI and captured in TR 38.843 clause 7.3.2.3.1 can be referred to [2].

Proposal 1: RAN4 can focus on the testability and interoperability study of case 1 with UE-sided models as the starting point.

Before discussing AI/ML enabled tests, it is necessary to briefly review the legacy L3 measurement test cases including measurement delay and accuracy tests. In the legacy L3 measurement test cases, one PCell and one neighbour cell are configured, and event reporting delay is verified in delay tests and the accuracy of measurement quantities is verified in accuracy tests. However, more than one neighbour cell should be configured in AI/ML assisted mobility tests because one of the purposes of AI/ML assisted mobility is to predict RSRPs of all candidate cells and possible handovers in the near future and to find a target cell decided by network based on these predicted RSRPs.

Proposal 2: Number of neighbour cells should be larger than 1 in test. The specific value is FFS.

Next issue is the mapping relationship between SSB signals from all cells and AoAs in FR2. Firstly, RAN4 needs to discuss the number of AoAs used in AI/ML based mobility tests. According to the test configurations specified in clause A.3.15 in TS 38.133 [3], up to 3 AoAs are supported in current OTA tests. And in AI/ML for NR air WI, study on the support of more than 3 AoAs is under discussion. Hence, if RAN4 identifies that more than 3 AoAs are required in AI/ML based mobility tests, then RAN4 shall wait for the progress in AI/ML for NR air WI.

Observation 1: Up to 3 AoAs are supported for OTA tests in current spec. And study on the support of more than 3 AoAs is under discussion in AI/ML for NR air WI.

Proposal 3: RAN4 to discuss the required number of AoAs in AI/ML based mobility tests. RAN4 shall wait for progress in AI/ML for NR air WI if it is identified that more than 3 AoAs are needed.

After number of neighbour cells and AoAs are determined, their mapping relationship should also be specified, since the number of neighbour cells could be larger than that of AoAs in a test, which means SSBs from multiple neighbour cells could be transmitted from one probe.

Proposal 4: RAN4 to discuss the mapping relationship between SSB signals and AoAs when a single AoA is multiplexed by multiple cells.

In addition, an important issue is whether beam-level RSRP should be used as input and/or output in test in FR2. If it is adopted in AI/ML mobility tests, it means each cell will transmit multiple DL beams, which bring up a very complex demand for test equipment as AI/ML based beam management use case in AI/ML for NR air WI and the discussion on the test solution is still on-going. Therefore RAN4 can assume to use cell-level RSRP as input/output in tests at current stage and further discuss the necessity of using beam-level RSRP.

Proposal 5: Cell-level RSRP is assumed to be used as input/output in tests and further discuss the necessity of using beam-level RSRP.

Test metrics/KPIs

As mentioned in section 2.1, the test metric in legacy L3 delay and accuracy measurements is event reporting delay and accuracy of measurement quantities respectively. However, new test metrics are needed considering the motivations of introducing AI/ML capabilities into mobility. Some potential metrics are provided in SID [1] for RAN2 evaluations and as well as RAN4 tests.

- The evaluation of the AI/ML aided mobility benefits should consider HO performance KPIs (e.g., Ping-pong HO, HOF/RLF, Time of stay, Handover interruption, prediction accuracy, and measurement reduction) etc.) and complexity tradeoffs [RAN2]

According to the SID, the HO performance KPIs that RAN2 considers during evaluations include:

- Ping-pong HO

- HOF/RLF

- Time of stay

- Handover interruption

- Prediction accuracy

- Measurement reduction

From RAN4 perspective, some of them are not suitable to be test metrics, since RAN4 does not define any requirements for such KPIs, such as Ping-pong HO and time of stay. For the rest of KPIs, they can be classified into two categories: one for delay tests and one for accuracy tests. Apparently, prediction accuracy is used for accuracy tests, resulting in a new issue which is whether to tighten the existing accuracy requirements for RSRP measurements. For delay tests, more RAN4 discussions are needed as well as RAN2 progress.

Proposal 6: For delay test metrics of RRM measurement prediction tests, RAN4 discussions can start from the following options:

- Handover interruption.

- Measurement reduction .

- HOF/RLF.

- Others.

Conclusions

This contribution discussed the testability and interoperability issues in AI/ML mobility and following proposals are provided:

Proposal 1: RAN4 can focus on the testability and interoperability study of case 1 with UE-sided models as the starting point.

Proposal 2: Number of neighbour cells should be larger than 1 in test. The specific value is FFS.

Observation 1: Up to 3 AoAs are supported for OTA tests in current spec. And study on the support of more than 3 AoAs is under discussion in AI/ML for NR air WI.

Proposal 3: RAN4 to discuss the required number of AoAs in AI/ML based mobility tests. RAN4 shall wait for progress in AI/ML for NR air WI if it is identified that more than 3 AoAs are needed.

Proposal 4: RAN4 to discuss the mapping relationship between SSB signals and AoAs when a single AoA is multiplexed by multiple cells.

Proposal 5: Cell-level RSRP is assumed to be used as input/output in tests and further discuss the necessity of using beam-level RSRP.

Proposal 6: For delay test metrics of RRM measurement prediction tests, RAN4 discussions can start from the following options:

- Handover interruption.

- Measurement reduction .

- HOF/RLF.

- Others.

References

[1] RP-242393, Revised SID on AIML for mobility in NR, OPPO, RAN#105.

[2] TR 38.843, V18.0.0.

[3] TS 38.133, V18.7.0.