3GPP TSG-RAN WG4 Meeting #112bis				                                          R4-2415181

Hefei, Anhui, China, 14th – 18th October, 2024

Title:	Discussion on Fast SCell activation for UE supporting Rel-18 EMR

Source:	CATT

Agenda Item:	6.15.3

Document for:	Discussion

1 Introduction

In last RAN4 meeting, the fast SCell activation for UE supporting Rel-18 EMR was discussed. The conclusions were captured in the approved WF [1] and the open issues can be found in the topic summary [2]. In this paper, we continue the discussion on the fast SCell activation with EMR and give our views.

2 Discussion

2.1 General

Applicability of fast SCell activation

In last meeting, we have provided the background for fast SCell activation with Rel-18 eEMR in our paper [3]. After the discussion, it was agreed in [1] that we will not change the Rel-18 eEMR definition in this work item, and the fast SCell activation delay requirements are defined for the case when the UE supports Rel-18 eEMR and the UE has reported valid results on the SCell to be activated before SCell activation command.

#### Issue 1-1-1: Clarification on Rel-18 eEMR Agreements: - Do not change the Rel-18 eEMR definition in this fast SCell activation discussion. #### Issue 1-1-2: Applicability of fast SCell activation delay requirements Agreement: - The fast SCell activation delay requirements are defined for the case when - the UE supports Rel-18 eEMR: - Including both EMR and cell reselection measurement, - Including the case when measIdleValidityDuration-r18 and / or measReselectionValidityDuration-r18 are configured - FFS Including the case when measIdleValidityDuration-r18 and / or measReselectionValidityDuration-r18 are not configured - and the UE has reported valid results on the SCell to be activated before SCell activation command.

There is still one open issue whether the requirements applicability can include the case when the durations for validity check (i.e., measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18) are not configured. According to the following definition in Rel-18, if the duration is not configured, UE is not required to perform the validity check and UE may report measurement results given the measurement results satisfy measurement accuracy requirement at the measurement instance. In such case, it is not clear whether the UE will report the measurement results and even if it reports, it is not clear when the measurements are performed and whether the results can be used for SCell activation, i.e., the results cannot be guaranteed as valid.

For a UE supporting measValidationReportEMR-r18 and idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16, it shall report based on the idle mode measurement which is specified in clause 4.4, and according to the measurement reporting requirements specified in clause 4.7.3. For a UE supporting measValidationReportReselectionMeasurements-r18, it shall report based on the idle mode measurement specified in the clause 4.2.2, and according to the measurement reporting requirements specified in the clause 4.7.3. The measurement results are considered valid if the following conditions are met for the validity check: - the measurements are performed before msg1 transmission for RRC resume/setup request within the last: - measIdleValidityDuration-r18 seconds for carriers configured in measIdleCarrierListNR-r16 or measIdleCarrierListEUTRA-r16, and/or - measReselectionValidityDuration-r18 seconds for carriers configured in measReselectionCarrierListNR-r18, - the measurement results satisfy measurement accuracy requirement at the measurement instance. Otherwise, the measurement results are considered invalid. The UE shall not report invalid measurement results when measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18 is configured. If the measIdleValidityDuration-r18 is not configured, the UE is not required to perform validity check for carriers in measIdleCarrierListNR-r16 and measIdleCarrierListEUTRA-r16, and the UE may report measurement results given the measurement results satisfy measurement accuracy requirement at the measurement instance. If the measReselectionValidityDuration-r18 is not configured, the UE is not required to perform validity check for carriers configured in measReselectionCarrierListNR-r18, and the UE may report measurement results given the measurement results satisfy measurement accuracy requirement at the measurement instance.

But based on the objective description in the WID [4], the target is to reduce the SCell activation delay with valid EMR reporting. If the validity of measurement results cannot be guaranteed, the reduced delay requirements cannot apply. So we understand we do not need to consider the case when the durations for validity check are not configured.

Proposal 1: Do not consider the case when the durations for validity check (i.e., measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18) are not configured.

Scope of fast SCell activation for UE supporting Rel-18 eEMR

In current specification, SCell activation delay requirements are defined for many scenarios including RRC based and MAC CE based, DL SCell and PUCCH SCell, single SCell and multiple SCell etc. In last meeting, the scope of fast SCell activation was discussed and the following agreement was captured in [1]:

#### Issue 1-1-3: Scope of fast SCell activation for UE supporting Rel-18 eEMR Agreement: - Start from the following scope of fast SCell activation for UE supporting Rel-18 EMR: - Direct SCell Activation at SCell addition - FFS: Single MAC CE based DL SCell Activation - FFS: Single MAC CE based PUCCH SCell Activation - RAN4 to discuss whether and how to apply the solutions to other SCell activation procedures after RAN4 has conclusions for the above cases.

Based on the delay requirements defined in clause 8.3 in TS 38.133, we understand the SCell activation procedures for different scenarios are quite similar, especially for the downlink part, so we think the enhancement of fast SCell activation with valid EMR reporting can be generally applied to most of SCell activation procedure. But considering the limit time for this objective and to focus on the solution discussion, we prefer to start from the simplest scenarios, i.e., single DL Cell. For single DL SCell activation, it has been agreed in last meeting that RRC based procedure should be considered, but since direct SCell activation is optional based on UE capability, we think MAC CE based activation procedure which is the Rel-15 baseline should be also considered. With this scope, we understand the two cases would not make much difference or increase the workload since the main difference between them is the time uncertainty between SCell configuration and SCell activation command. We think these two scenarios can be the example to discuss the solution, and for other scenarios, we would suggest to discuss them after the solution is decided. It is expected straightforward to reuse the solution and may not spend much effort.

Proposal 2: Start from the following scope of fast SCell activation for UE supporting Rel-18 EMR, and check the applicability to other SCell activation requirements after the solution is decided:

  - Direct SCell Activation at SCell addition

  - Single MAC CE based DL SCell Activation

2.2 RRM requirements

Regarding how to define the fast SCell activation delay requirements with valid EMR reporting, it was agreed to update the known condition to include the case when UE has valid EMR reporting and how to update the known condition is FFS.

#### Issue 1-2-1: How to define the fast SCell activation delay requirements with valid eEMR reporting Agreement: - RAN4 to update the known condition for SCell activation to include the case when UE has valid EMR reporting before SCell activation command. - FFS how to update the known condition (issue 1-2-1a). - FFS the applicability of valid EMR reporting for the Rel-19 fast SCell activation

In the current specification, the SCell activation delay requirements are defined for known case and unknown case, and the known condition is defined as below for FR1 and FR2:

SCell in FR1 is known if it has been meeting the following conditions: - During the period equal to max(5*measCycleSCell, 5*DRX cycles) for FR1 before the reception of the SCell activation command: - the UE has sent a valid measurement report for the SCell being activated and - the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3. - the SSB measured during the period equal to max(5*measCycleSCell, 5*DRX cycles) also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3. Otherwise SCell in FR1 is unknown. For the first SCell activation in FR2 bands, the SCell is known if it has been meeting the following conditions: - During the period equal to 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4 before UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable): - the UE has sent a valid L3-RSRP measurement report with SSB index, and - SCell activation command is received after L3-RSRP reporting and no later than the time when UE receives MAC-CE command for TCI activation - During the period from L3-RSRP reporting to the valid CQI reporting, the reported SSBs with indexes remain detectable according to the cell identification conditions specified in clauses 9.2 and 9.3, and the TCI state is selected based on one of the latest reported SSB indexes. Otherwise, the first SCell in FR2 band is unknown.

We see the following three main points to define the known condition and can be checked for valid EMR reporting:

- During the period equal to [X] for FR1 before the reception of the SCell activation command;

- UE has sent a valid measurement report for the SCell being activated;

- The SSB measured remains detectable during [X] and SCell activation procedure.

For point 1 which defines the proximity of valid reporting and SCell activation command, and point 3 which defines the availability of SSB measured, we think the existing framework can be reused, because it decides whether the reports can be used in the SCell activation procedure according to the distance in time domain and is not relevant to the report contents. But here since the target is to use the measurement in IDLE/INACTIVE state and the SCell has not been configured, there is no measCycleSCell configuration, and the measurement is not performed during period [X]. So the [X] value for FR1 should be updated and can be defined as a fixed value, e.g., 5s. For FR2, the existing values, i.e., 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4, can be considered to be reused.

For point 2, it is straightforward to be updated to the valid EMR reporting which is based on the validity check.

Another issue raised in last meeting is the applicability of the Rel-18 EMR reporting. According to the existing configuration of measIdleValidityDuration-r18 and measReselectionValidityDuration-r18 as below, the duration for validity check can be up to 100s which means the valid results can be performed 100s before RRC resume/setup request. Considering the mobility, the value may be a little long for the known condition of SCell activation. We are open to discuss the applicable durations for fast SCell activation and the values for FR1 and FR2 can be defined separately.

-- ASN1START -- TAG-MEASUREMENTVALIDITYDURATION-START MeasurementValidityDuration-r18 ::= ENUMERATED {s5, s10, s20, s50, s100, spare3, spare2, spare1} -- TAG-MEASUREMENTVALIDITYDURATION-STOP -- ASN1STOP

According to the discussion above, the example updates are shown as below and a draft CR was provided in [5].

SCell in FR1 is known if it has been meeting the following conditions: - During the period equal to max(5*measCycleSCell, 5*DRX cycles) for FR1 before the reception of the SCell activation command: - the UE has sent a valid measurement report for the SCell being activated and - the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3. - the SSB measured during the period equal to max(5*measCycleSCell, 5*DRX cycles) also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3. For a UE supporting measValidationReportEMR and measIdleValidityDuration-r18 is configured with smaller than [X] or a UE supporting measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured with smaller than [X], besides the conditions above, SCell in FR1 is also known if it has been meeting the following conditions: - During the period equal to [5s] for FR1 before the reception of the SCell activation command: - the UE has sent a valid EMR report according to the reporting requirements in 4.7.3 or 5.8.3 for the SCell being activated and - the SSB measured remains detectable according to the cell reselection conditions specified in 4.2 or the CA/DC measurement conditions specified in 4.4 when UE is in IDLE/INACTIVE state and - the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3 when UE is in CONNECTED mode. - the SSB measured also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3. Otherwise SCell in FR1 is unknown. For the first SCell activation in FR2 bands, the SCell is known if it has been meeting the following conditions: - During the period equal to 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4 before UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable): - the UE has sent a valid L3-RSRP measurement report with SSB index, and - SCell activation command is received after L3-RSRP reporting and no later than the time when UE receives MAC-CE command for TCI activation - During the period from L3-RSRP reporting to the valid CQI reporting, the reported SSBs with indexes remain detectable according to the cell identification conditions specified in clauses 9.2 and 9.3, and the TCI state is selected based on one of the latest reported SSB indexes. For a UE supporting measValidationReportEMR and measIdleValidityDuration-r18 is configured with smaller than [X] or a UE supporting measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured with smaller than [X], besides the conditions above, for the first SCell activation in FR2 bands, the SCell is also known if it has been meeting the following conditions: - During the period equal to [4s] for UE supporting power class 1/5 and [3s] for UE supporting power class 2/3/4 before UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable): - the UE has sent a valid EMR report according to the reporting requirements in 4.7.3 or 5.8.3, and - SCell activation command is received after valid EMR reporting and no later than the time when UE receives MAC-CE command for TCI activation - During the period from valid EMR reporting to the valid CQI reporting, the reported SSBs with indexes remain detectable according to the cell identification conditions specified in clauses 9.2 and 9.3, and the TCI state is selected based on one of the latest reported SSB indexes. Otherwise, the first SCell in FR2 band is unknown.

Proposal 3: Define applicability of the valid EMR reporting, i.e., the fast SCell activation delay requirements apply to the case when configured duration is smaller than a threshold. The threshold is FFS.

Proposal 4: RAN4 to update the known conditions for SCell activation as in draft CR [5] to include the case when UE has valid EMR reporting before SCell activation command.

In last meeting, there were also the following two considerations raised for fast SCell activation with EMR reporting.

#### Issue 1-2-2: Consideration on SINR condition during the whole procedure for known SCell activation Proposals - Option 1: - RAN4 to discuss whether it’s necessary to define a consistent SINR condition during the whole procedure for known SCell activation requirement (including EMR measurement in IDLE/Inactive and known SCell activation in RRC connected mode), e.g., SINR>=-2dB #### Issue 1-2-3: Consideration on measurement period condition of FR1 known SCell activation Proposals - Option 1: - RAN4 to discuss whether and how to change the measurement period condition of FR1 known SCell activation (i.e., 2400ms in current requirement) for fast SCell activation with EMR.

Regarding the SINR condition, the issue is whether to define consistent SINR condition during the known SCell activation requirements since the side condition defined in accuracy requirements for CA/DC Idle Mode Measurements is -4dB while the side condition defined in SCell activation delay requirements is -2dB. In our view, it is not necessary. Based on the existing CA/DC idle mode measurement accuracy definition, the requirements for -4dB and -2dB side condition are the same. So as long as the UE passed the accuracy requirements test, the reported results of CA/DC Idle measurements which meet the accuracy requirements can also be used for SCell activation delay requirements which are based on -2dB side condition. In addition, the situation is same as legacy requirements in which the side condition of deactivated SCell measurement accuracy requirements is -6dB or -4dB while the side condition for SCell activation requirements is -2dB. Therefore we can simply follow the legacy requirements.

Regarding the measurement period condition, the following is defined for FR1 known SCell activation in which one additional SMTC periodicity is required to perform AGC when the SCell measurement period is larger than 2400ms.

If the SCell is known and belongs to FR1, Tactivation_time is: - - TFirstSSB+ 5ms, if the measurement period of the SCell being activated is equal to or smaller than 2400ms. - - TFirstSSB_MAX + Trs + 5ms, if the measurement period of the SCell being activated is larger than 2400ms.

Now for the CA/DC Idle Mode Measurement, based on the requirements in clause 4.4, the minimum measurement period is 3.84s (3*1.28).  For cell reselection requirements, based on the requirements in 4.2, the measurement period is much longer. And considering the measurement for EMR reporting may occur long time before SCell activation and there is time uncertainty between them, it is hard to follow the legacy requirements with measurement period smaller than 2400ms and the measurement occasion for AGC may be always needed.

Based on the analysis above, the legacy requirement with measurement period larger than 2400ms is applied to fast SCell activation requirements with EMR reporting. But we understand this can be derived according to the measurement period calculation even if we do not change the existing requirements, so we think we do not need to change the measurement period condition and can reuse the existing requirements.

Proposal 5: No need to update the SINR condition and measurement period condition for known SCell activation.

The last issue raised in last meeting is the indication to network for fast SCell activation as below:

#### Issue 1-2-4: Whether the indication to network is needed Proposals - Option 1: - The Rel-19 RRM enhancement in fast scell activation for UE support Rel-18 EMR shall not only be RAN4 side condition update, certain activation fast or slow indication to the network is needed.

One company mentioned that network may be not aware of this fast SCell activation enhancement and would not use the EMR reporting to activate the SCell, so a UE indication to network would be helpful. But we understand the valid EMR report according to Rel-18 validity check can already be the indication to network, and what we are trying to do in this work item is to define requirements for the case when UE has this indication. It means the requirements are based on Rel-18 indication and we do not need to introduce a new indication. After getting this report, it is up to network how to use it to configure or activate SCell.

Proposal 6: No need to introduce additional indication to the network for fast or slow SCell activation.

3 Summary

In this paper, we have some further discussions on fast SCell activation with Rel-18 EMR, and the following proposals are given:

Proposal 1: Do not consider the case when the durations for validity check (i.e., measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18) are not configured.

Proposal 2: Start from the following scope of fast SCell activation for UE supporting Rel-18 EMR, and check the applicability to other SCell activation requirements after the solution is decided:

  - Direct SCell Activation at SCell addition

  - Single MAC CE based DL SCell Activation

Proposal 3: Define applicability of the valid EMR reporting, i.e., the fast SCell activation delay requirements apply to the case when configured duration is smaller than a threshold. The threshold is FFS.

Proposal 4: RAN4 to update the known conditions for SCell activation as in draft CR [5] to include the case when UE has valid EMR reporting before SCell activation command.

Proposal 5: No need to update the SINR condition and measurement period condition for known SCell activation.

Proposal 6: No need to introduce additional indication to the network for fast or slow SCell activation.

4 References

- R4-2414041, WF on NR_RRM_Ph5_Part2 - fast SCell activation, CATT, RAN4#112

- R4-2411814, Topic summary for [112][219] NR_RRM_Ph5_Part2, CATT, RAN4#112

- R4-2411359, Discussion on Fast SCell activation for UE supporting Rel-18 EMR, CATT, RAN4#112

- RP-242380, Revised WID: NR Radio Resource Management (RRM) Phase 5, Apple, CATT, RAN#105

- R4-2415182, draftCR on Fast SCell activation for UE supporting Rel-18 EMR, CATT, Apple, RAN4#112bis