3GPP TSG-RAN WG4 Meeting #112bis				                                         R4-2415183

Hefei, Anhui, China, 14th – 18th October, 2024

Title:	Discussion on RRM requirements for on-demand SSB Scell operation

Source:	CATT

Agenda Item:	6.22.2.1

Document for:	Discussion

1 Introduction

In last RAN4 meeting, the RRM requirements for Rel-19 NES enhancement were discussed. The conclusions were captured in the approved WF [1] and the open issues can be found in the summary [2]. In this paper, we further discuss the potential RRM impact for on-demand SSB SCell operation and give our views.

2 Discussion

General

In last meeting, it was agreed to discuss OD-SSB based deactivated SCell measurement requirements and SCell activation requirement, and the following cases are to be considered based on RAN1 definition.

- Case #1 and Scenario #2

- Case #1 and Scenario #2A

- Case #2 and Scenario #2

- Case #2 and Scenario #2A

- Notes:

  - Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321)

  - Scenario #2A refers to

    - “When UE receives SCell activation command (e.g., as defined in TS 38.321)”

  - Case #1: No always-on SSB on the cell

  - Case #2: Always-on SSB is periodically transmitted on the cell

But according to the discussions on SCell activation requirements, the scenarios need to be further clarified especially for scenario #2A since it is unclear whether there are other OD-SSB operations before SCell activation command and it is unclear whether the OD-SSB parameters are changed or not after SCell activation command. We understand the scenario definition in RAN1 is only to clarify the timeline of OD-SSB indication for SSB transmission used in SCell activation procedure and the corresponding SCell activation command. It doesn’t touch the behaviors before SCell activation command, and therefore it is possible that there is another OD-SSB transmission but has been finished before SCell activation command.

Proposal 1: Clarify that the following case should also be considered in the requirements:

- There are other OD-SSB operations finished before SCell activation.

Based on the following RAN1 agreements, the RRC based OD-SSB SCell operation is supported for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration.

RAN1#118: Agreement - For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration. - FFS: Whether to support RRC based signaling for other cases. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A. - Note: Deactivation and adaptation of on-demand SSB transmission can be separately discussed.

We understand this case means the direct SCell activation at SCell addition and the OD-SSB indication is provided when UE receives SCell activation command which is scenario #2A. With this agreement, we think RAN4 can discuss and define the SCell activation delay requirements for this case.

Proposal 2: RAN4 to define OD-SSB based SCell activation delay requirements for direct SCell activation at SCell addition for scenario #2A.

In last meeting, there was also a proposal to discuss whether to define requirements for OD-SSB based L3 neighbor cell measurement. In our view, UE is not aware of the OD-SSB configuration of neighbour cell and the awareness of this configuration needs signalling support which should be discussed in RAN2/3 first. Before that, we think RAN4 should not discuss the OD-SSB based L3 neighbor cell measurement.

Proposal 3: RAN4 not to define OD-SSB based L3 neighbour cell measurement requirements.

Deactivated SCell L3 measurement part

For deactivated SCell L3 measurement, the existing measurement period requirements are defined as below which are based on measCycleSCell.

Table 9.2.5.2-3: Measurement period for intra-frequency measurements without gaps (deactivated SCell) (FR1) DRX cycle; T SSB_measurement_period_intra. No DRX; Ceil(5 x Kp) x measCycleSCell x CSSFintra. DRX cycle≤ 320ms; Ceil(5 x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra. DRX cycle> 320ms; Ceil(5 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra. NOTE 1: The requirements also apply to deactivated SCG SCel. Table 9.2.5.2-4: Measurement period for intra-frequency measurements without gaps (deactivated SCell) (FR2) DRX cycle; T SSB_measurement_period_intra. No DRX; Ceil(Mmeas_period_w/o_gaps x Kp) x measCycleSCell x CSSFintra. DRX cycle≤ 320ms; Ceil(Mmeas_period_w/o_gaps x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra. DRX cycle> 320ms; Ceil(Mmeas_period_w/o_gaps x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra. NOTE 1: The requirements also apply to deactivated SCG SCell..

For OD-SSB based SCell operating, according to the discussion for proposal 1, there is the case when other OD-SSB operations have been performed and finished before SCell activation for both scenario #2 and #2A, so we think when we discuss deactivated SCell L3 measurement requirements, we do not need to differentiate scenario #2 and #2A, but just to differentiate case #1 and case #2.

For case #1 where no always-on SSB is transmitted on the cell, it was agreed that UE is not expected to perform the SSB based deactivated SCell L3 measurement before OD-SSB is triggered. And after OD-SSB is triggered, we understand it is straightforward to follow OD-SSB periodicity to define the requirements since it may not have measCycleSCell configuration. And even if there is measCycleSCell configuration, considering the OD-SSB will be stopped after certain duration and the measurement cycle may be too long for OD-SSB transmission, UE may not perform the measurement before the OD-SSB is stopped. It makes the OD-SSB useless for the SCell.

For case #2 where always-on SSB is periodically transmitted on the cell, before OD-SSB is triggered, the UE follows legacy behaviour to perform L3 measurement based on measCycleSCell configuration. And after OD-SSB is triggered, although UE have different SSB configurations, to maximize the NES gain, similar as case #1, we think UE can also follow OD-SSB periodicity to perform the measurements.

In legacy deactivated SCell measurement requirements, the measurement period is also based on DRX cycle, but for OD-SSB operation, considering the timeliness of OD-SSB, maybe we can consider not to include DRX cycle in the requirements.

Proposal 4: For both case #1 and case #2, after OD-SSB is triggered, the deactivated SCell L3 measurement requirements are based on OD-SSB periodicity.

Proposal 5: For both case #1 and case #2, discuss whether to consider DRX cycle in the OD-SSB based deactivated SCell L3 measurement requirements.

SCell activation part

In the existing specification, the SCell activation delay requirements are defined in clause 8.3.2 for known and unknown cases based on the measurement report before SCell activation command. For OD-SSB based SCell activation delay, we think the similar principle can be used. To discuss the SCell activation delay requirements, the following timeline is used for easy elaboration.

Deactivated SCell

SCell activation procedure

Activated SCell

Point 1: SCell addition

Point 2: SCell activation command

Point 3: CSI report

Phase 1

Phase 2

Phase 3

According to the discussion in deactivated SCell measurement part, in phase 1, it may or may not have SSB configuration provided, and if there is SSB configuration, it can be always-on SSB or OD-SSB. To simplify the requirements, we understand we do not need to differentiate the SSB type when defining the known condition, i.e., the SCell can be known when UE has report the measurement results during certain time before SCell activation command regardless the results are performed by always-on SSB or OD-SSB. And similarly we do not need to differentiate the different cases or scenarios above, i.e., the general requirements can be defined for both case #1 and case #2, and for both scenario #2 and scenario #2A. With this principle, the existing SCell activation requirements can be reused by clarifying that the first SSB after SCell activation command means the first actual transmitted OD-SSB to reflect time uncertainty between OD-SSB indication and actual OD-SSB transmission.

Proposal 6: Define general requirements for all the agreed cases (i.e., including both case #1 and case #2, and both scenario #2 and scenario #2A).

Proposal 7: The existing SCell activation delay requirements can be reused with the following clarification:

- TFirstSSB and TFirstSSB_MAX is the time to the end of first actual transmitted OD-SSB.

RAN1 LS on timeline for On-demand SSB operation on SCell

In last meeting, RAN1 discussed the timeline of OD-SSB transmission and defined the starting point of actual OD-SSB transmission, i.e., time instance A, as below. Based on the agreements, RAN1 sent the LS [3] to ask RAN4 to confirm the minimum value of T.

Agreement The previous RAN1 agreement made in RAN1#117 is revised as follows. - For SSB burst(s) indicated by on-demand SSB SCell operation via MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows. - Alt 3-1: Time instance A is the beginning of the first slot containing [candidate SSB index 0 or the first actually transmitted SSB index] of on-demand SSB burst [the slot boundary of] the first SSB time domain position [of actually transmitted on-demand SSB burst] which is at least T [slots or symbols] after the [slot or symbol] where UE receives a signalling from gNB to indicate on-demand SSB transmission - The SSB time domain positions of on-demand SSB burst are configured by gNB. - FFS: Details of the value of T (≥ 0) including possibility of T comprising of multiple components - Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation - (Working assumption): T is not less than T_min= where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to indicate on-demand SSB transmission ending in slot n, and is as defined in current specification. - RAN4 to confirm that T_min can be equal to - FFS: Whether the value of T is predefined or indicated/configured by gNB - (Working assumption) T=T_min - FFS: Details of “the [slot or symbol] where UE receives a signalling from gNB” or “the [slot or symbol] where UE transmits HARQ-ACK corresponding to a signalling from gNB to trigger on-demand SSB” - Above applies at least for the case where SCell with on demand SSB transmission and cell with signalling transmission have the same numerology.

We understand the value should at least include the time for MAC CE signaling processing and the time waiting for uplink resources which is same as legacy. And according to the RAN1 LS, both aspects have been considered. So we understand it is reasonable to define T_min as m+3Nslotsubframe,  μ+1 and can reply to RAN1 to confirm the understanding.

Proposal 8: Reply LS to RAN1 to confirm that T_min can be equal to m+3Nslotsubframe,  μ+1.

3 Summary

In this paper, we have some further discussions on on-demand SSB SCell operation, and the following proposals are given:

Proposal 1: Clarify that the following case should also be considered in the requirements:

- There are other OD-SSB operations finished before SCell activation.

Proposal 2: RAN4 to define OD-SSB based SCell activation delay requirements for direct SCell activation at SCell addition for scenario #2A.

Proposal 3: RAN4 not to define OD-SSB based L3 neighbour cell measurement requirements.

Proposal 4: For both case #1 and case #2, after OD-SSB is triggered, the deactivated SCell L3 measurement requirements are based on OD-SSB periodicity.

Proposal 5: For both case #1 and case #2, discuss whether to consider DRX cycle in the OD-SSB based deactivated SCell L3 measurement requirements.

Proposal 6: Define general requirements for all the agreed cases (i.e., including both case #1 and case #2, and both scenario #2 and scenario #2A).

Proposal 7: The existing SCell activation delay requirements can be reused with the following clarification:

- TFirstSSB and TFirstSSB_MAX is the time to the end of first actual transmitted OD-SSB.

Proposal 8: Reply LS to RAN1 to confirm that T_min can be equal to m+3Nslotsubframe,  μ+1.

4 References

- R4-2413885, WF on Netw_Energy_NR_enh, Ericsson, RAN4#112

- R4-2411817, Topic summary for [112][222] Netw_Energy_NR_enh, Moderator (Ericsson), RAN4#112

- R1-2407565, LS on timeline for On-demand SSB operation on SCell, Apple (RAN1), RAN1#118