3GPP TSG-RAN WG4 Meeting #112bis                                 R4-2415185

Hefei, Anhui, China, 14th – 18th October, 2024

Title: 	Discussion on RRM requirements for LP-WUS/WUR

Source: 	CATT

Agenda Item:	6.23.4.2

Document for:	Discussion

1. Introduction

In last RAN4 meeting, the RRM requirements for R19 LP-WUS/WUR were further discussed. The conclusions were captured in the approved WF [1] and the detailed issue list can be found in the summary [2].

In this contribution, we have some further discussions on RRM requirements for LP-WUS/WUR and give our proposals.

2. Discussion

Cases/states to be considered for MR and LR

In last meeting, RAN4 discussed the cases to be considered for RRM requirements based on the different ON/OFF status of MR and LR, and the different requirements of serving cell and neighbour cell measurement. According to the conclusions in the WF [1], it was agreed to support case 3 in which both serving cell and neighbour cell measurements are relaxed, but it is open on whether to combine the serving cell measurements derived by MR and LR in such case.

Agreement: - Support case #3 - For serving cell measurement, further discuss: - Option 1: Further discuss whether to combine the measurements across the two radios, i.e., MR and WUR - Option 2: From RAN4 requirement perspective, not consider combining the measurements across the two radios, i.e., MR and WUR - Option 3: It is up to UE implementation whether to combine the measurement across the two radios. - Other options are not precluded. RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #3 Relaxed case b; On with relaxation measurement; On with relaxation measurement; ON. Note: In RAN4 understanding, the discussion of the related measurement criteria (i.e., whether to use the legacy or new criteria) is out of RAN4 responsibility.

From RAN4 perspective, we understand what we care about is whether the final reported measurement results can meet the requirements or not, and seems no need to consider how this result is derived from LR or MR. But RAN2 has the following agreement on entry/exit conditions which means the thresholds for OFDM-based and OOK-based WUR can be different. And considering that the requirements for LR can be OFDM-based and OOK-based which are expected to be different, and can also be different with MR requirements, we would suggest not combining the results.

RAN2#127 agreements: Separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs. Signalling details are FFS.

Proposal 1: From RAN4 requirement perspective, for case 3, not consider combining the measurements across the two radios, i.e., MR and WUR.

Criteria (entry/exit conditions)

The discussion of criteria of entry/exit conditions includes the conditions for LP-WUS monitoring, LP-WUR measurement and MR RRM measurement relaxation. Firstly, RAN4 need to align the understanding on the three procedures and our understandings on the entry of three procedures are as below:

- LP-WUR measurement: LR starts serving cell measurement and meets the measurement requirements regardless MR status.

- MR measurement relaxation: MR performs relaxed measurement and meets relaxed requirements when LR is turned on, i.e., the case #3 above.

- LP-WUS monitoring: LR starts monitoring LP-WUS. MR may stop the legacy PO monitoring before LR receives LP-WUS indicating wake-up.

For the entry/exit condition of LP-WUS monitoring, RAN1 and RAN2 had the following conclusions:

RAN1#116bis meeting: Working Assumption - From RAN1 perspective, for the entry/exit conditions for LP-WUS monitoring in IDLE/inactive mode, - The UE may start LP-WUS monitoring if - the serving cell measurement performed by the MR is above entry threshold(s), if configured by the gNB, and/or - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied - If UE starts LP-WUS monitoring, it may stop the legacy PO monitoring before UE receives LP-WUS indicating wake-up - The UE monitors the legacy PO (and may monitor PEI) and may stop LP-WUS monitoring if - the serving cell measurement performed by the LR is below exit threshold(s), if configured by the gNB, and/or - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied - FFS the serving cell measurement metrics - The entry/exit thresholds can be configured separately for different types of LR - It is left to RAN2 discussion whether the threshold(s) are always configured by the gNB. - Note: This may be revisited based on the RAN2/RAN4 discussion. - RAN2#126 meeting: - Baseline for entry condition definition: If the serving cell quality, e.g. RSRP, RSRQ from MR, is above threshold(s) (if configured), UE may start to monitor LP-WUS, if UE monitors LP-WUS, it may stop monitoring the legacy PO. FFS if any measurement from LR is needed. - Baseline for exit condition definition: If the serving cell measurement result based on LR is below a threshold (if configured), UE monitors PO as in legacy and it may stop monitoring the LP-WUS. - RAN2#127 meeting: - Separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs. Signalling details are FFS. - Working assumption (can revisit if R1/R4 reached different conclusions): If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds. - The metrics for serving cell quality measured by MR/LR for entry condition includes (LP-)RSRP and optional (LP-)RSRQ. - The metrics for serving cell quality measured by LR for exit condition includes (LP-)RSRP and optional (LP-)RSRQ.

And for RRM relaxation and offloading in RRC_IDLE/INACTIVE, RAN2 had the following agreements in RAN2#126 and RAN2#127 meeting:

- RAN2#126 meeting: - For serving cell measurement offloading (i.e., serving cell measurement fully offloaded to LR and no serving cell measurement via MR is required), RAN2 should focus on specifying the offloading criterion for serving cell for UEs supporting LP-WUS, and assume that RAN4 will define the measurement offloading requirements for serving cell. - RAN2 understand that the RRM measurement of the neighboring cell can only be performed by MR. Can discuss again if RAN1 inform us otherwise. - RAN2 will further discuss the neighbour cell measurement relaxation criteria (if the UE is using LR to measure the serving cell), e.g., considering reuse Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’. - RAN2#127 meeting: - RAN2 only discuss RRM measurement offloading/relaxation for LP-WUS UEs. - For serving cell measurement offloading (i.e., there is no serving cell measurement by MR): - The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered. - The exit condition is based on the LR measurement results.

We can see that RAN1/2 already had initial agreements on the criteria for LP-WUS monitoring, RRM measurement relaxation and fully offloading. And from the previous experience, the criteria discussion is usually within RAN2 scope. So we think we can follow the RAN1/2 conclusions and focus on the requirements discussion. Any further discussion can be triggerred by RAN2 LS.

For the entry/exit conditions for LP-WUR measurement which is not discussed in RAN1/2, we think when to turn on the LR and start LP-WUR measurement is UE implementation, and what we should consider is when UE can relax or stop the MR measurements. And even when the measurement is performed by MR, UE is not mandatory to turn off LR and stop LR measurement. Based on such understanding, we think we do not need to define the entry/exit condition for LP-WUR measurement.

Proposal 2: Do not define entry/exit conditions for LP-WUR measurement.

Proposal 3: The criteria discussion for LP-WUS monitoring, RRM measurement relaxation and fully offloading  can be left to RAN2, and RAN4 to focus on the LP-WUR measurement requirements and MR measurement relaxation requirements.

LP-WUR measurement requirements in RRC_IDLE/INACTIVE state

According to the existing cell reselection requirements, the UE shall measure the SS-RSRP and SS-RSRQ level of the serving cell and evaluate the cell selection criterion S defined in TS 38.304 [1] for the serving cell within a certain cycle. But for fully offloading case, the MR measurements are stopped and no neighbour cell measurement is performed. In such case, whether the evaluation of cell selection criterion is still needed should be discussed. If needed, the evaluation would be based on LR measurement and the evaluation results may be one of the exit conditions for fully offloading case.

Proposal 4: RAN4 to discuss whether to define evaluation requirements for cell selection criterion based on LR measurement for fully offloading case.

Besides the above requirements, we need to define measurement period requirements for LR. In previous meeting, it was agreed both LP-SS and SSS based serving cell measurement requirements need to be defined. Since the LP-SS design is still discussing in RAN1, RAN4 can discuss the SSS based measurement requirements firstly and wait for more progress from RAN1 for LP-SS.

For the SSS based measurement, we think the existing intra-frequency cell reselection requirements can be used as baseline. And the side condition can be further discussed following RAN1 conclusions on the SNR.

And considering UE need to evaluate the entry/exit condition for LP-WUS monitoring, MR measurement relaxation and/or fully offloading, the evaluation period requirements may also be needed. We understand the evaluation period requirements can be the same as the measurement requirements and further discuss whether to define measurement filtering (number of samples and separation between samples).

Proposal 5: RAN4 to discuss LP-WUR measurement requirements starting from SSS based measurement and wait for more progress from RAN1 for LP-SS based measurement.

Proposal 6: For SSS based LP-WUR measurement, the existing intra-frequency cell reselection requirements can be used as baseline with the side condition to be further discussed.

Proposal 7: RAN4 to define requirements for entry/exit criteria evaluation for LP-WUS monitoring, MR RRM relaxation and fully offloading based on LP-WUR measurement period, and further discuss whether to define measurement filtering (e.g., number of samples).

LP-WUR measurement accuracy requirements

In the previous meeting, RAN4 discussed the accuracy for LP-WUR requirements and the related agreement is copied as follows:

Agreement: RAN4 requirement for LP-WUR RRM measurement in Idle/inactive states: - RAN4 to discuss the simulation assumptions considering the target SNR, accuracy and number of samples. - RAN4 will define the delay requirement in core part, and further discuss whether/how to define the accuracy requirement. - For information only: - RAN4 requirement for LP-WUR RRM measurement in Idle/inactive states: - Options for further discussion (the options are only for information purpose) - Option 1): Define the dedicated accuracy requirement in the performance section, FFS whether/how it can be verified in the test case. - Option 2): No dedicated accuracy requirement in the performance section, and reflect the accuracy performance as a margin in the core requirement. - Option 2a): FFS accuracy requirement, and reflect the accuracy performance in delay requirement in core part. - Other options are not precluded.

For whether/how to define the accuracy requirement, considering that the LP-WUR RRM measurement results of serving cell may be used in the entry/exit conditions, we believe that the measurement accuracy is very important to guarantee the network performance, we prefer to use the legacy measurement accuracy for CONNECTED mode in Clause 10.1.2 TS 38.133 as baseline to perform the simulation. Regarding how to define the accuracy requirements, we think it can be defined in dedicated section to make it clear. However, considering that UE may not need to report the result but only use it to evaluation the criteria, we can also consider adding it as a margin in the entry/exit conditions and it can be further discussed after the simulation.

Proposal 8: RAN4 to use the legacy measurement accuracy for CONNECTED mode in Clause 10.1.2 TS 38.133 as baseline to perform simulation.

Proposal 9: Define LP-WUR measurement accuracy in dedicated section. Whether/how to reflect it in the core requirements can be further discussed after the simulation by taking the results into account.

MR RRM relaxation for serving cell/neighbour cell

In Rel-16/17 PowSav and RedCap WI, various RRM measurement relaxation schemes in IDLE/INACTIVE mode on neighbouring cell were introduced. For this WI, further relaxation or even no measurement by MR could be considered while serving cell measurements are offloaded to LP-WUR. Based on RAN2 conclusion, they will further discuss the criteria for neighbour cell measurement relaxation, e.g., considering reuse Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’. So for the neighbor cell measurement relaxation, the existing relaxed requirements can be used as baseline.

During the discussion in last meeting, there are proposals to further relax the requirements comparing to legacy relaxation factor. In our understanding, if we only reuse the legacy criteria and do not include any condition of LR measurement, the effectiveness of LP-WUR cannot be guaranteed and it is not reasonable to further relax the neighbour cell measurement requirements. But if new criteria are designed considering the LR measurement, to maximize the power saving gain, the relaxation factor for neighbour cell measurement can be further increased ( e.g., 8).

For the serving cell measurement, there are no relaxed measurement requirements before, but we think the principle when defining neighbor cell measurement relaxation can be reused, i.e., to introduce a relaxation factor based on legacy requirements. And whether to use same relaxation factor with neighbour cell can be further discussed.

Proposal 10: For MR neighbor cell measurement relaxation, the existing relaxed requirements in 4.2.2 can be used as baseline. Whether to further relax the requirements can be discussed after the criteria are defined by RAN2.

Proposal 11: For MR serving cell measurement relaxation, a scaling factor can be introduced on existing serving cell measurement requirements in 4.2.2. Further discuss whether to use same scaling factor as neighbor cell measurement.

LP-WUS operation in RRC_CONNECTED state

Based on the WID as copied below, no LP-WUR measurement is requested and the UE RRM/RLM/BFD/CSI measurements are performed by MR in for CONNECTED mode. So no LP-WUR measurement requirements are needed and no RAN4 impact is expected.

- For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1) - Check in RAN#105 for potential TU adjustment in RAN2 - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR

RAN1 has discussed LP-WUS operation in RRC_CONNECTED state, and has the following agreements:

- RAN1#116: - PDCCH monitoring triggered by LP-WUS is enabled/disabled by gNB RRC signaling - FFS whether to support UE assistance - Minimum time gap between LP-WUS reception and MR to start PDCCH monitoring is introduced considering at least following： - LP-WUS processing time - MR transition time for ramp up - Time/frequency synchronization of MR - RAN1#117: - For RRC CONNECTED mode, support UE capability report for determination of minimum time gap between LP-WUS reception and MR to start PDCCH monitoring.

From the agreements, we can see that the only requirements aspect is the time gap between LP-WUS reception and PDCCH monitoring. And this has been defined as UE capability in RAN1. RAN1 will decide the candidate values and no RAN4 impact is expected so far.

Proposal 12: For LP-WUS operation in RRC_CONNECTED state, no RAN4 impact is expected so far.

Other issues

In this meeting, RAN4 received one LS [3] from RAN1 to check the applicability and specification support needed for LP-WUS reception.

In RAN1, the common understanding is that UE may not support LP-WUS reception on all the bands supported by the UE. RAN1 respectfully asks RAN2 and RAN4 to check if there is any issue and specification support needed for IDLE/INACTIVE UEs.

From RRM perspective, we understand the measurement requirements are band agnostic. But for criteria evaluation for LP-WUS monitoring, RRM measurement relaxation and fully offloading, we think it is not reasonable to use the measurement results on one carrier to decide the behaviour on the other carrier considering the channel conditions may be different. So we understand for criteria evaluation, same carrier shall be assumed for both LR and MR. But whether the LP-WUS reception is supported on all the bands should be discussed in RF.

Proposal 13: For criteria evaluation, RAN4 assumed LR and MR are operating on the same carrier.

Proposal 14: Whether the LP-WUS reception is supported on all the bands depends on RF discussion.

3. Conclusion

In this paper, we provide our views on RRM core requirements for R19 LP-WUS/WUR. From this discussion we have derived the following observations and proposals:

Proposal 1: From RAN4 requirement perspective, for case 3, not consider combining the measurements across the two radios, i.e., MR and WUR.

Proposal 2: Do not define entry/exit conditions for LP-WUR measurement.

Proposal 3: The criteria discussion for LP-WUS monitoring, RRM measurement relaxation and fully offloading  can be left to RAN2, and RAN4 to focus on the LP-WUR measurement requirements and MR measurement relaxation requirements.

Proposal 4: RAN4 to discuss whether to define evaluation requirements for cell selection criterion based on LR measurement for fully offloading case.

Proposal 5: RAN4 to discuss LP-WUR measurement requirements starting from SSS based measurement and wait for more progress from RAN1 for LP-SS based measurement.

Proposal 6: For SSS based LP-WUR measurement, the existing intra-frequency cell reselection requirements can be used as baseline with the side condition to be further discussed.

Proposal 7: RAN4 to define requirements for entry/exit criteria evaluation for LP-WUS monitoring, MR RRM relaxation and fully offloading based on LP-WUR measurement period, and further discuss whether to define measurement filtering (e.g., number of samples).

Proposal 8: RAN4 to use the legacy measurement accuracy for CONNECTED mode in Clause 10.1.2 TS 38.133 as baseline to perform simulation.

Proposal 9: Define LP-WUR measurement accuracy in dedicated section. Whether/how to reflect it in the core requirements can be further discussed after the simulation by taking the results into account.

Proposal 10: For MR neighbor cell measurement relaxation, the existing relaxed requirements in 4.2.2 can be used as baseline. Whether to further relax the requirements can be discussed after the criteria are defined by RAN2.

Proposal 11: For MR serving cell measurement relaxation, a scaling factor can be introduced on existing serving cell measurement requirements in 4.2.2. Further discuss whether to use same scaling factor as neighbor cell measurement.

Proposal 12: For LP-WUS operation in RRC_CONNECTED state, no RAN4 impact is expected so far.

Proposal 13: For criteria evaluation, RAN4 assumed LR and MR are operating on the same carrier.

Proposal 14: Whether the LP-WUS reception is supported on all the bands depends on RF discussion.

4. Reference

- R4-2413899, WF for RRM core requirements for LP-WUSWUR, vivo, RAN4#112

- R4-2411818, Topic summary for [112][223] NR_LPWUS, vivo, RAN4#112

- R1-2407559, LS on LP-WUS operation in IDLE/INACTIVE mode, RAN1, RAN1#118