3GPP TSG RAN WG4 Meeting #112bis	R4-2415186

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:			6.17.1

Source:	vivo

Title:	Discussion on general aspects for AI/ML

Document for:	Discussion

- Introduction

On the general aspects for AI/ML, in the last meeting, RAN4 further discussed the issues on post-deployment testing, channel modeling assumption for requirements/tests and testing goals, etc. In this contribution, we further provide our views on the pending part of the testing goal for AI/ML NR air interface based on the latest outcome reached in the last meeting [1].

- Discussion

- 2.1	Testing goals

In the last meeting, RAN4 further determined the testing goal by refining the current definition as achieving minimal performance/core requirements defined on functionality/feature level, not to consider defining requirements per each AI/ML specific model. The detailed content was reached as follows.

Issue 1-3: Testing goals Agreement: - The testing goal is to verify whether the minimum performance of AI/ML functionality/feature can be achieved - FFS on whether and how many different test configurations/parameters are used for tests. - LCM would be tested

Based on the description of functionality defined in TR38.843 [2], it refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability. Based on this definition, one functionality may be associated with several configurations/scenarios. From testing perspective, RAN4 needs to identify how many independent test cases need to be considered among these supportive configurations/scenarios, which is exactly the issue identified in FFS part in the second bullet. However, the challenge of this issue is that currently there is no clear concept of the granularity of functionality and the associations to scenarios provided by other WGs. It is difficult to reach the specific number of test configurations/parameters in such detail. As the starting point, we think RAN4 can first figure out the key factors that potentially affect inference performance and further determine the detailed configurations/parameters and how many scenarios to test in the test coverage. Next, we would like to initially provide potential key factors that affect inference performance in case-by-case manner.

Use case for Beam management

In terms of the possible configurations/parameters which potential effect on inference performance for BM, they were initially evaluated in generalization performance study by RAN1. And the simulation results are summarized in TR38.843[2]. From the results for UE sided model, it was shown that the unseen scenarios as follows cause significant performance loss in terms of Top-1 beam prediction accuracy for both BMCase-1 and BMCase-2:

- Deployment scenarios: different ISD, UMi/UMa

- Configurations:

- various gNB setting:

  - gNB antenna array dimensions

  - DL Tx beam codebook (e.g., beam width, beam angle, etc)

- various Set B patterns

- various Set A patterns

As the reference, RAN4 can firstly determine the factors to be considered for test design among the above settings for further evaluation. Specific parameters for BMCase-2

- T1 for measurement/T2 for prediction

- UE mobility

Specifically, for BMCase-2, the following factors may have some impacts on performance in some cases, which can be considered as additional factors for further study.

Use case for CSI compression

For CSI compression, the generalization/scalability performances of channel model and configuration have been evaluated by RAN1, which are summarized in TR38.843[2]. It is seen that following factors are to be considered:

- Channel model: InH, UMi, UMa, TDL and CDL

- Generalization/scalability of configurations:

  - various TxRU mappings

  - various CSI payload sizes

  - bandwidths

  - Tx port numbers

Note that even though TDL and CDL are not evaluated in TR, they are also suggested to be considered, since RAN4 may use them as test scenario.

Use case for CSI prediction

For CSI prediction, the generalization performances of channel model and configuration have been evaluated by RAN1. Some results have been studied in R18 and summarized in TR38.843[2], while some results are concluded in R19 study. It is seen that following factors are to be considered:

- Channel model: UMi, UMa, TDL and CDL

- Configurations:

  - various UE speed

  - various carrier frequency

Note that even though TDL and CDL are not evaluated in TR, they are also suggested to be considered, since RAN4 may use them as test scenario.

Use case for Positioning

For AI/ML based positioning, since there are not many discussions about the test configurations in the previous RAN4 meetings and it is not clear whether to introduce tests for positioning based on current progress, we provide some considerations about the factors that may impact the performance of different positioning cases.

Case 1:

The direct positioning case 1 is a very cell-specific case, different channel/scenarios and different TRP deployments will bring a various positioning performance. In that case, following factors are to be considered:

- Channel model:

  - Drop

  - Scenarios: SH, DH, HH, etc

  - LOS/NLOS

  - Delay spread

- TRP deployment:

  - TRP number

  - TRP location

Case 2:

For case 2a, the output is the intermediate feature, e.g., DL RSTD, and the generalization performance is better than case 1. Legacy NR positioning test configurations can be referred to and may be the only difference is the channel model. For AI/ML, NLOS channel are mainly focused on and maybe AWGN is not appropriate in AI test.

For case 2b, the output is the channel measurements to be reported to NW side for model inference. If the channel measurements are to be tested in RAN4, channel model is also importance the AWGN may be not appropriate.

Therefore, following factors could be considered for case 2:

- Channel model:

  - NLOS channel or AWGN

Case 3:

There is no need to define tests for case 3.

Proposal 1: For verifying the minimum performance of AI/ML functionality/feature, RAN4 to first determine the factors to be considered in the test design. For each use case, the candidate configurations/scenarios include:

- For beam management

  - Deployment scenarios: different ISD, UMi/UMa

  - Configurations:

    - various gNB settings:

      - gNB antenna array dimensions

      - DL Tx beam codebook (e.g., beam width, beam angle, etc)

    - various Set B patterns

    - various Set A patterns

- For CSI compression

  - Channel model: InH, UMi, UMa, TDL and CDL

  - Configurations:

    - various TxRU mappings

    - various CSI payload sizes

    - bandwidths

    - Tx port numbers

- For CSI prediction

  - Channel model: UMi, UMa, TDL and CDL

  - Configurations:

    - various UE speed

    - various carrier frequency

- For positioning

  - Case 1:

    - Channel model:

      - Drop

      - Scenarios: SH, DH, HH, etc

      - LOS/NLOS

      - Delay spread

    - TRP deployment:

      - TRP number

      - TRP location

  - Case 2:

    - Channel model:

      - NLOS channel or AWGN

  - Case 3:

    - No need to test

- Summary

In this contribution, we provided our views on general aspects for AI/ML. Based on above analysis, following proposals are presented.

Proposal 1: For verifying the minimum performance of AI/ML functionality/feature, RAN4 to first determine the factors to be considered in the test design. For each use case, the candidate configurations/scenarios include:

- For beam management

  - Deployment scenarios: different ISD, UMi/UMa

  - Configurations:

    - various gNB settings:

      - gNB antenna array dimensions

      - DL Tx beam codebook (e.g., beam width, beam angle, etc)

    - various Set B patterns

    - various Set A patterns

- For CSI compression

  - Channel model: InH, UMi, UMa, TDL and CDL

  - Configurations:

    - various TxRU mappings

    - various CSI payload sizes

    - bandwidths

    - Tx port numbers

- For CSI prediction

  - Channel model: UMi, UMa, TDL and CDL

  - Configurations:

    - various UE speed

    - various carrier frequency

- For positioning

  - Case 1:

    - Channel model:

      - Drop

      - Scenarios: SH, DH, HH, etc

      - LOS/NLOS

      - Delay spread

    - TRP deployment:

      - TRP number

      - TRP location

  - Case 2:

    - Channel model:

      - NLOS channel or AWGN

  - Case 3:

    - No need to test

- References

- R4-2414323, WF on AI/ML, Qualcomm Incorporated