3GPP TSG RAN WG4 Meeting #112bis	R4-2415190

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:			6.17.4.1

Source:	vivo

Title:	Discussion on RRM impacts on beam management

Document for:	Discussion

- Introduction

In RAN4#112 meeting, there were some further discussions on key issues including KPIs related, the impact of measurement error on prediction accuracy and testability. The outcome was recorded in the latest WF [1].

Per the arrangement, in this contribution, we will analyse the impact of measurement error for the inference performance and further provide some simulation results on candidate KPIs/ Test Metrics with consideration of the measurement error modeling.

- The impact of measurement error for inference performance

Issue 2-6: Measurement error impact Agreement: RAN4 will study the impact of measurement error for the input to inference and dataset for training(set B measurement error) on prediction accuracy, companies should bring proposals on how to proceed with such a study. impact on different prediction accuracy metrics should be taken into account studies already performance by other groups (RAN1) could also be taken into account Note: intention of the study is just to understand the impact of measurement error on prediction accuracy.

In the last meeting, RAN4 agreed to study the specific impact of AI inference performance when measurement error is considered in the model input of inference and training. For this part of the study, we initially provided some simulation results and general observations in our contribution [2] in the last meeting. To make it clearer, we would like to provide our further analysis mainly from the following two aspects:

- The impact on prediction accuracy for different metrics for beam management

- The modeling of measurement error

2.1 The impact on prediction accuracy for different metrics for beam management

Firstly, for the beam prediction accuracy, the metrics including Top-1 (%), Top-K/1 (%) and Top-1/K (%) are considered. From the simulation results listed in the first column, it can be observed that except for the accuracy of Top-1 (%), there is over 90% beam prediction accuracy under other metrics. On top of it, we also try to verify the effect of introducing measurement error in training/validation data on prediction performance. The measurement errors are modeled as ±4dB and ±6.5dB truncated Gaussian distribution, respectively.

Table 1 Evaluation results of prediction accuracy for BM-Case1 DL Tx beam prediction by considering measurement error

| Simulation results |  | Prediction accuracy (%) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | No error | ±4dB error in validation data | ±4dB error in training/validation data | ±6.5dB error in validation data | ±6.5dB error in training/validation data |
| Top 1 | +0dB | 77.96 | 47.98 | 48.00 | 35.41 | 33.46 |
|  | +1dB | 93.72 | 67.59 | 70.73 | 51.70 | 52.19 |
| Top 2/1 | +0dB | 90.87 | 75.06 | 76.62 | 63.21 | 65.27 |
|  | +1dB | 93.96 | 82.21 | 82.23 | 69.60 | 69.29 |
| Top 4/1 | +0dB | 94.62 | 89.07 | 90.29 | 83.94 | 86.04 |
|  | +1dB | 94.80 | 89.52 | 90.59 | 84.32 | 86.34 |
| Top 5/1 | +0dB | 96.24 | 91.91 | 93.07 | 87.42 | 89.40 |
|  | +1dB | 96.29 | 92.04 | 93.11 | 87.48 | 89.44 |
| Top 1/2 | +0dB | 91.61 | 71.09 | 71.87 | 57.04 | 58.27 |
|  | +1dB | 96.50 | 83.31 | 85.58 | 69.54 | 71.88 |
| Top 1/4 | +0dB | 97.35 | 88.31 | 88.74 | 80.22 | 80.23 |
|  | +1dB | 99.23 | 95.18 | 95.96 | 88.29 | 89.26 |
| Top 1/5 | +0dB | 98.40 | 91.58 | 92.02 | 84.26 | 84.58 |
|  | +1dB | 99.48 | 96.89 | 97.45 | 91.35 | 92.28 |

On the performance impact of measurement error, based on the simulation results, we have several observations:

- No matter for which metrics, measurement errors will degrade the beam prediction performance with AI/ML. Especially for the metric Top-K/1 and Top-1/K when K≤1,2, the degradation is non-negligible.

- There is no large performance gap between the case in which measurement error is only considered in validation data and the case where measurement error is considered both in validation data and training data

Observation 1: There is a non-negligible performance degradation in beam prediction accuracy when considering measurement error modeling in training/validation data.

Observation 2: There is no large performance gap between the case that measurement error is only considered in validation data and the case that measurement error is considered both in validation data and training data

Based on observation 1, we believe the measurement error is one of the important factors affecting the prediction performance. Thus, it needs to be considered when RAN4 define the requirements for beam prediction accuracy

Proposal 1: RAN4 to define beam prediction accuracy requirements with consideration of the impact on measurement error

Additionally, we also study the performance when [X]dB margin is considered in the beam prediction accuracy. Specifically, in our simulations, the Top-1 with X=1dB margin means that the percentage of the Top-1 predicted beam "whose ideal L1-RSRP is within X=1dB of the ideal L1-RSRP of the Top-1 genie-aided beam".

And for Top-K/1 with X=1dB margin, the criteria for judgment are:

- If the Top-1 genie-aided beam is one of the Top-K predicted beams, recorded as correct prediction

- Else if all the differences between the ideal RSRP of ideal Top-1 beam and the predicted RSRP of predicted Top-K beam are within the X=1dB range, recorded as correct prediction, recorded as correct prediction

- Otherwise, recorded as wrong prediction

Besides, for Top-1/K with X=1dB margin, the criteria for judgment are

- If the Top-1 predicted beam is one of the Top-K genie-aided beams, recorded as correct prediction

- Else if the predicted RSRP of predicted Top-1 beam is within the range of {[ideal RSRP of ideal Top-K beam – 1dB], [ideal RSRP of ideal Top-1 beam + 1dB]}, recorded as correct prediction

- Otherwise, recorded as wrong prediction

From the results, we can observe that if 1dB margin is considered in statistics, it will bring a noticeable improvement in the rate of prediction accuracy, especially for the Top 1 case.

Observation 3: In addition to the rate of correct beam prediction, when additional 1dB RSRP margin is considered (e.g., For the success rate of the difference between the ideal RSRP of ideal Top-1 beam and the predicted RSRP of predicted Top-K beam are within the X=1dB range), it will bring a noticeable improvement in the rate of prediction accuracy.

Furthermore, in order to figure out the impact of difference values of X on prediction accuracy and to facilitate RAN4 discussion on whether and how to determine the appropriate value of X margin, we provide some simulations on the probability that the difference between the RSRP of Top-1 beam and the predicted RSRP of Top-K beam is within the X=[1, 5, 10] dB range. Specifically for the selection of the benchmark (i.e., the RSRP of Top-1 beam), we consider including

1)	Predicted RSRP of predicted Top-1 beam;

2)	Ideal RSRP of ideal Top-1 beam;

3)	Predicted RSRP of predicted Top-1 beam with consideration of 4dB measurement error;

4)	Ideal RSRP of ideal Top-1 beam with consideration of 4dB measurement error

Table 2 The probability that the difference between the RSRP of Top-1 beam and the predicted RSRP of Top-K beam is within the X=1dB range

| Simulation results |  | Probability (%) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Predicted top 1 | Ideal top 1 | ±4dB error for predicted top 1 | ±4dB error for ideal top 1 |
| Top 1 | X=1dB | 100.00 | 91.91 | 100.00 | 46.18 |
| Top 2/1 |  | 87.82 | 76.46 | 62.01 | 26.67 |
| Top 3/1 |  | 48.61 | 44.44 | 26.19 | 13.38 |
| Top 4/1 |  | 3.79 | 4.17 | 4.01 | 2.98 |
| Top 5/1 |  | 1.54 | 1.28 | 0.49 | 0.29 |

Table 3 The probability that the difference between the RSRP of Top-1 beam and the predicted RSRP of Top-K beam is within the X=5dB range

| Simulation results |  | Probability (%) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Predicted top 1 | Ideal top 1 | ±4dB error for predicted top 1 | ±4dB error for ideal top 1 |
| Top 1 | X=5dB | 100.00 | 99.61 | 100.00 | 98.74 |
| Top 2/1 |  | 99.96 | 99.34 | 99.39 | 96.64 |
| Top 3/1 |  | 94.73 | 93.49 | 92.54 | 89.28 |
| Top 4/1 |  | 82.16 | 80.22 | 78.16 | 73.93 |
| Top 5/1 |  | 23.01 | 21.45 | 22.16 | 20.69 |

Table 4 The probability that the difference between the RSRP of Top-1 beam and the predicted RSRP of Top-K beam is within the X=10dB range

| Simulation results |  | Probability (%) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Predicted top 1 | Ideal top 1 | ±4dB error for predicted top 1 | ±4dB error for ideal top 1 |
| Top 1 | X=10dB | 100.00 | 99.95 | 100.00 | 99.91 |
| Top 2/1 |  | 100.00 | 99.92 | 100.00 | 99.79 |
| Top 3/1 |  | 99.94 | 99.64 | 99.65 | 99.26 |
| Top 4/1 |  | 95.01 | 94.63 | 94.97 | 94.26 |
| Top 5/1 |  | 53.37 | 52.92 | 55.07 | 55.34 |

From the Table 2-4, we can observe that:

- As the X value increases, the probability of Top-K falling into this range increases. When X is 1, the probability of Top-2 falling into this range is only 70%-80%

- When [Predicted RSRP of predicted Top-1 beam] and [Ideal RSRP of ideal Top-1 beam] are used as benchmarks, the probability of falling into the X range is similar

- The measurement error has some impact on the probability when the value of X is small

Observation 4: The probability of predicted RSRP of predicted Top-K beam falling into the range of [the RSRP of Top-1 beam± XdB] has significant improvement with the increase of X value

Observation 5: When using predicted RSRP of predicted Top-1 beam and Ideal RSRP of ideal Top-1 beam as the benchmark, the probability that the difference between the RSRP of Top-1 beam and the predicted RSRP of Top-K beam is within the [X]dB range are similar

Besides, based on the simulation results of following metrics, i.e., Top-1, Top-K/1 and Top-1/K, we have the following observations:

- with the increasing number of K (i.e., Top K/1 and Top 1/K), the prediction accuracy increases accordingly

- When measurement error is considered, there is significant performance degradation on Top-1 beam prediction (even lower than 50% accuracy in some cases).

- The performance on prediction accuracy defined as Top-K/1and Top-1/K are closed.

From RRM requirement perspective, RAN4 needs to determine one metric on beam prediction accuracy among these candidate metrics. Firstly, as per the observations above, the metric of Top-1 is lack of certain robustness. And for the metrics of Top-K/1and Top-1/K, the performance is approximate so that RAN4 can only choose one of them as the KPI. In our thinking, considering there is a case that UE just reports beam ID of Top-K without any ordering information. The Top-1 predicted beam may not be known, Top-K/1 is more appropriate as the KPI to figure out even without any rankings.

Proposal 2: RAN4 to use Top-K/1 as the metric of beam prediction accuracy for further performance evaluation

For RSRP accuracy, the following are the two candidate definitions of RSRP difference.

- Alt1: RSRP difference only pertains to the case if AI/ML predicted beam ID is consistent with the corresponding genie beam, or

- Alt2: RSRP difference between predicted Top-K beam with corresponding Top-K genie beam

To evaluate the performance difference between the above two metrics, we will provide the corresponding RSRP accuracy simulation results using Alt1 and Alt2 as the KPI, respectively.

Table 5 and Table 6 show the simulation results of the RSRP accuracy. Table 5 shows the 90%-tile RSRP difference between the predicted L1-RSRP of the Top-1/Top-5 predicted beams and the ideal L1-RSRP of the corresponding genie-aided beam. As for Table 6, it shows the 90%-tile RSRP difference between the predicted L1-RSRP of the Top-1/Top-5 predicted beams and the ideal L1-RSRP of the same beams. Note that the simulation is under the case that the ground truth without measurement error is regarded as the benchmark. The measurement errors are only added in the model input.

Table 5 90%-tile RSRP difference between the predicted L1-RSRP of the Top-1/Top-5 predicted beams and the ideal L1-RSRP of the corresponding genie-aided beam

| Simulation results | RSRP difference (dB) without considering errors |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1st Beam | 2nd Beam | 3rd Beam | 4th Beam | 5th Beam |
| Top 1 | 0.83 |  |  |  |  |
| Top 5/1 | 0.83 | 0.84 | 0.90 | 1.10 | 1.30 |
| Top 1/5 | 0.83 | 1.50 | 2.95 | 5.19 | 10.44 |
| RSRP difference (dB) considering errors ±4dB errors (only add in model input) |  |  |  |  |  |
| Top 1 | 2.90 |  |  |  |  |
| Top 5/1 | 2.90 | 2.92 | 2.90 | 3.05 | 3.66 |
| Top 1/5 | 2.90 | 3.06 | 3.98 | 5.68 | 10.56 |
| RSRP difference (dB) considering errors ±6.5dB errors (only add in model input) |  |  |  |  |  |
| Top 1 | 4.78 |  |  |  |  |
| Top 5/1 | 4.78 | 4.29 | 4.29 | 4.48 | 5.11 |
| Top 1/5 | 4.78 | 4.97 | 5.74 | 7.11 | 11.10 |

Table 6 90%-tile RSRP difference between the predicted L1-RSRP of the Top-1/Top-5 predicted beams and the ideal L1-RSRP of the same beams

| Simulation results | RSRP difference (dB) without considering errors |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1st Beam | 2nd Beam | 3rd Beam | 4th Beam | 5th Beam |
| Top 1 | 1.02 |  |  |  |  |
| Top 5/1 | 1.02 | 1.11 | 1.30 | 1.67 | 1.64 |
| RSRP difference (dB) considering errors ±4dB errors (only add in model input) |  |  |  |  |  |
| Top 1 | 4.26 |  |  |  |  |
| Top 5/1 | 4.26 | 3.90 | 3.96 | 4.45 | 4.83 |
| RSRP difference (dB) considering errors ±6.5dB errors (only add in model input) |  |  |  |  |  |
| Top 1 | 7.73 |  |  |  |  |
| Top 5/1 | 7.73 | 6.12 | 6.11 | 6.63 | 7.06 |

It can be observed from the results above that the performance difference on RSRP accuracy under definition Alt1 and Alt2 is relatively small. Taking an example for Top 5/1 beam with no measurement error, the relative difference of 5th beam RSRP difference values is smaller than around 0.4dB. Even when 6.5dB errors are considered, the maximum difference is smaller than 3dB.

Observation 6: There is no large performance gap on the RSRP accuracy between the following two definitions of RSRP difference

- Alt1: RSRP difference only pertains to the case if AI/ML predicted beam ID is consistent with corresponding genie beam, or

- Alt2: RSRP difference between predicted Top-K beam with corresponding Top-K genie beam

Based on this observation, from the better alignment of legacy definition of L1-RSRP accuracy perspective, we prefer the RSRP accuracy is based on the premise that the beam IDs between predicted RSRP and actual RSRP are the same. If we go with Alt 2, our concern is that there may be with misalignment of beam ID when compared. Consequently, it is quite tricky to compare the corresponding RSRP values if the predicted RSRP of beam 1 is very different from the ideal RSRP of beam 2.

Proposal 3: For metrics/KPIs for Beam Management requirements/tests, RAN4 to align the definition of RSRP accuracy as the difference between the predicted L1-RSRP of predicted beam and the ideal L1-RSRP of the same beam.

Furthermore, based on the objectives in latest WID [3], RAN4 aims to specify necessary core requirements for beam management. For core part, it includes the definition of delay requirements for prediction. Basically, the current measurement period requirement for L1-RSRP can be as the reference and the number of samples sample (i.e., sample =1) can be reused. Specifically for BM case-2, given that it is the prediction in time dimension, the definition of delay requirements is also relevant to prediction methodology (e.g., Case A and Case B specified in RAN1) and related parameters for measurement and prediction (e.g., number of time instances for measurement/prediction, the time duration for beam prediction). For this case, more discussion is needed on how to define corresponding requirements with consideration of characteristics of time dimension prediction.

Proposal 4: For core requirements for beam management, RAN4 to define 1-sample based delay requirements for beam prediction of Spatial-domain in BM case-1. For Temporal beam prediction in BM case-2, the definition of delay requirements may take into account specific methods (e.g., caseA and caseB) and characteristics of time-domain.

2.2 The modeling of measurement errors

2.2.1 Baseband error modelling

RAN1 has already evaluated some impacts of the measurement errors at SI stage [4]. For reference, the modelling methods captured in TR 38.843 could be the starting point to facilitate the discussion in RAN4. We take the following method as an example:

Measured RSRP = Ideal system level simulation RSRP + truncated Gaussian distribution for baseband + truncated Gaussian distribution for RF + others (e.g., quantization error)

This modelling method is a straightforward way for evaluation. Most observations in RAN1 evaluation show that measurement errors will bring a noticeable performance degradation and cannot be ignored. Our simulation results in section 2.1 are also based on this modelling method. Note that the baseband error is only considered in the model input, while the model output is ideal label.

However, this kind of way of modelling error, e.g., probability distribution, is not applicable from RAN4 perspective. In RAN4 RRM simulation, link level simulations (LLS) are usually used to derive the difference between the performance under ideal SNR environment and the performance under a realistic SNR environment. LLS could model the detailed link characteristics, e.g., UE receiver algorithms, modulation methods, etc. System-level simulation (SLS) is incapable of implementing such details. Therefore, at least for the baseband measurement characteristics, it could not be reflected by the probability distribution. Here we give an example of two cases to show the difference between the 32 Tx beams RSRP measurements under the fading channel and truncated Gaussian distribution, which is illustrated in Fig. 1. Note that currently 1Tx beam is used as the configuration in the aligned LLS simulation assumptions for L1-RSRP accuracy, RAN4 may need to discuss whether the simulation assumption is based on the TR 38.843 or the aligned assumption for L1-RSRP accuracy.

Case 1: SNR is assigned as -3dB for LLS and the LLS simulation assumption is based on Table 6.3.1-2 in TR 38.843

Case 2: Truncated Gaussian distribution is modelled between -2.5dB to 2.5dB.

Fig. 1 CDF curve of the RSRP difference for modelling errors from LLS and Gaussian distribution.

From the results shown in Fig. 1, it can be observed that although the RSRP difference in the 5%-quantile is almost the same, the trend and characteristics of the two curves are quite different. For Gaussian distribution (Case 2), the curve is central symmetry, while the curve for Case 1 has some relative offset from the y-axis.

To further study the feasibility of deriving baseband measurement errors from LLS under pre-defined SNR for BM, we provide some simulation results by considering errors modelled as the mentioned 2 cases. The results are shown in Table 7-9.

Table 7 Evaluation results of prediction accuracy for BM-Case1 DL Tx beam prediction by modelling measurement errors as case 1 and case 2

| Simulation results | Prediction accuracy (%) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | No error | Case 1 error in validation data | Case 1 error in training/validation data | Case 2 error in validation data | Case 2 error in training/validation data |
| Top 1 | 77.98 | 69.31 | 61.85 | 59.82 | 53.66 |
| Top 2/1 | 90.93 | 87.78 | 86.61 | 83.24 | 82.79 |
| Top 4/1 | 94.53 | 93.92 | 93.81 | 91.82 | 92.23 |
| Top 5/1 | 96.21 | 95.61 | 95.45 | 94.10 | 94.42 |
| Top 1/2 | 91.58 | 87.41 | 84.29 | 81.71 | 79.13 |
| Top 1/4 | 97.30 | 95.66 | 95.03 | 93.18 | 92.43 |
| Top 1/5 | 98.37 | 97.20 | 96.93 | 95.45 | 95.01 |

Table 8 90%-tile RSRP difference between the predicted L1-RSRP of the Top-1 and Top-5 predicted beams and the ideal L1-RSRP of the corresponding genie-aided beam

| Simulation results | RSRP difference (dB) without considering errors |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1st Beam | 2nd Beam | 3rd Beam | 4th Beam | 5th Beam |
| Top 1 | 0.83 |  |  |  |  |
| Top 5/1 | 0.83 | 0.84 | 0.90 | 1.10 | 1.30 |
| Top 1/5 | 0.83 | 1.50 | 2.95 | 5.19 | 10.44 |
| RSRP difference (dB) considering case 1 errors (only add in model input) |  |  |  |  |  |
| Top 1 | 2.06 |  |  |  |  |
| Top 5/1 | 2.06 | 1.85 | 2.02 | 2.07 | 2.41 |
| Top 1/5 | 2.06 | 2.53 | 3.68 | 5.70 | 11.32 |
| RSRP difference (dB) considering case 2 errors (only add in model input) |  |  |  |  |  |
| Top 1 | 2.08 |  |  |  |  |
| Top 5/1 | 2.08 | 2.15 | 2.12 | 2.34 | 2.81 |
| Top 1/5 | 2.08 | 2.30 | 3.32 | 5.36 | 10.51 |

Table 9 90%-tile RSRP difference between the predicted L1-RSRP of the Top-1 and Top-5 predicted beams and the ideal L1-RSRP of the same beam

| Simulation results | RSRP difference (dB) without considering errors |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1st Beam | 2nd Beam | 3rd Beam | 4th Beam | 5th Beam |
| Top 1 | 1.02 |  |  |  |  |
| Top 5/1 | 1.02 | 1.11 | 1.30 | 1.67 | 1.64 |
| RSRP difference (dB) considering case 1 errors (only add in model input) |  |  |  |  |  |
| Top 1 | 2.60 |  |  |  |  |
| Top 5/1 | 2.60 | 2.39 | 2.59 | 2.82 | 3.11 |
| RSRP difference (dB) considering case 2 errors (only add in model input) |  |  |  |  |  |
| Top 1 | 2.68 |  |  |  |  |
| Top 5/1 | 2.68 | 2.64 | 2.89 | 3.24 | 3.50 |

From Table 7-9, it can be observed that the modelling method in case 1 brings less performance degradation than case 2. That is probably because its dataset has a smaller variance than that in case 2 even the mean value (mathematical expectation) of the error dataset in case 1 has an overall offset. For AI models, datasets with larger variances have a distinct impact on inference performance. The simulation results show that different modelling method has some impact on the performance. From RAN4 perspective, our understanding is that case 1 may be more appropriate for modelling baseband error.

Observation 7: From the simulated CDF curve of the RSRP difference for modelling errors from LLS and Gaussian distribution, the characteristics of baseband errors (e.g., variance, mean value) derived from LLS and Gaussian distribution are different.

Proposal 5: For the LLS-based measurement error modelling, RAN4 to discuss whether the simulation assumption is based on the TR 38.843 or the aligned assumption for L1-RSRP accuracy.

Proposal 6: RAN4 to consider the RSRP difference derived from link level simulation under specific SNR condition as the baseband error, and add the error into the ideal dataset derived from system level simulation so as to build the nonideal dataset for training and inference.

2.2.2 RF error modelling

RF error is another important factor for measurement error modelling. In general, around 4dB error is considered in the current L1-RSRP accuracy requirement in FR2. However, how to model such kind of RF error in AI dataset is a tricky question. Next, we will give a reference modelling method by modelling 4dB RF error could be modelled as an offset value in the original dataset. Note that the RF error is only considered in the model input, while the model output is ideal label. The Table 10-Table 12 show the simulation results in terms of the case that RF error is modelled as a 4dB offset.

Table 10 Evaluation results of prediction accuracy for BM-Case1 DL Tx beam prediction by considering RF errors as 4dB offset

| Simulation results | Prediction accuracy (%) |  |  |
| --- | --- | --- | --- |
|  | No error | 4dB offset in validation data | 4dB offset in training/validation data |
| Top 1 | 77.98 | 77.83 | 78.16 |
| Top 2/1 | 90.93 | 90.85 | 90.97 |
| Top 4/1 | 94.53 | 94.64 | 94.48 |
| Top 5/1 | 96.21 | 96.26 | 96.22 |
| Top 1/2 | 91.58 | 91.63 | 91.59 |
| Top 1/4 | 97.30 | 97.36 | 97.28 |
| Top 1/5 | 98.37 | 98.40 | 98.33 |

Table 11 90%-tile RSRP difference between the predicted L1-RSRP of the Top-1 and Top-5 predicted beams and the ideal L1-RSRP of the corresponding genie-aided beam

| Simulation results | RSRP difference (dB) without considering errors |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1st Beam | 2nd Beam | 3rd Beam | 4th Beam | 5th Beam |
| Top 1 | 0.83 |  |  |  |  |
| Top 5/1 | 0.83 | 0.84 | 0.90 | 1.10 | 1.30 |
| Top 1/5 | 0.83 | 1.50 | 2.95 | 5.19 | 10.44 |
| RSRP difference (dB) considering 4dB offset errors (only add in model input) |  |  |  |  |  |
| Top 1 | 4.50 |  |  |  |  |
| Top 5/1 | 4.50 | 4.47 | 4.41 | 4.36 | 4.54 |
| Top 1/5 | 4.50 | 4.16 | 3.77 | 3.74 | 6.66 |

Table 12 90%-tile RSRP difference between the predicted L1-RSRP of the Top-1 and Top-5 predicted beams and the ideal L1-RSRP of the same beam

| Simulation results | RSRP difference (dB) without considering errors |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1st Beam | 2nd Beam | 3rd Beam | 4th Beam | 5th Beam |
| Top 1 | 1.02 |  |  |  |  |
| Top 5/1 | 1.02 | 1.11 | 1.30 | 1.67 | 1.64 |
| RSRP difference (dB) considering 4dB offset errors (only add in model input) |  |  |  |  |  |
| Top 1 | 4.28 |  |  |  |  |
| Top 5/1 | 4.28 | 4.32 | 4.34 | 4.44 | 4.47 |

From Table 10, it can be observed that the prediction accuracy is not impacted by the 4dB RF offset. The possible reason is that the trend of all the datasets remains unchanged when a 4dB offset is added to all the RSRP elements. From Table 11-12, the RSRP accuracy is impacted by the RF error, which is even worse than those without considering RF errors. For this case, a more relaxed accuracy requirement may be considered.

Besides, we also provide the simulation result based on the error modelled by uniform distribution in Table 13-15. Compared with the 4dB offset value as the errors, the uniform distribution error brings more degradation to prediction accuracy, while the performance of the RSRP difference between the two cases is similar.

Table 13 Evaluation results of prediction accuracy for BM-Case1 DL Tx beam prediction by considering RF errors as 4dB offset

| Simulation results | Prediction accuracy (%) |  |  |
| --- | --- | --- | --- |
|  | No error | 4dB offset in validation data | 4dB offset in training/validation data |
| Top 1 | 77.96 | 40.60 | 41.07 |
| Top 2/1 | 90.87 | 69.12 | 71.78 |
| Top 4/1 | 93.19 | 82.89 | 84.35 |
| Top 5/1 | 94.62 | 86.92 | 88.63 |
| Top 1/2 | 96.24 | 90.10 | 91.58 |
| Top 1/4 | 91.61 | 63.56 | 66.38 |
| Top 1/5 | 95.05 | 77.66 | 80.06 |

Table 14 90%-tile RSRP difference between the predicted L1-RSRP of the Top-1 and Top-5 predicted beams and the ideal L1-RSRP of the corresponding genie-aided beam

| Simulation results | RSRP difference (dB) without considering errors |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1st Beam | 2nd Beam | 3rd Beam | 4th Beam | 5th Beam |
| Top 1 | 0.83 |  |  |  |  |
| Top 5/1 | 0.83 | 0.84 | 0.90 | 1.10 | 1.30 |
| Top 1/5 | 0.83 | 1.50 | 2.95 | 5.19 | 10.44 |
| RSRP difference (dB) considering 4dB uniform distribution modelled errors (only add in model input) |  |  |  |  |  |
| Top 1 | 3.57 |  |  |  |  |
| Top 5/1 | 3.57 | 3.40 | 3.43 | 3.68 | 4.09 |
| Top 1/5 | 3.57 | 3.79 | 4.56 | 6.15 | 10.90 |

Table 15 90%-tile RSRP difference between the predicted L1-RSRP of the Top-1 and Top-5 predicted beams and the ideal L1-RSRP of the same beam

| Simulation results | RSRP difference (dB) without considering errors |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1st Beam | 2nd Beam | 3rd Beam | 4th Beam | 5th Beam |
| Top 1 | 1.02 |  |  |  |  |
| Top 5/1 | 1.02 | 1.11 | 1.30 | 1.67 | 1.64 |
| RSRP difference (dB) considering 4dB uniform distribution modelled errors (only add in model input) |  |  |  |  |  |
| Top 1 | 4.82 |  |  |  |  |
| Top 5/1 | 4.82 | 4.63 | 4.72 | 5.19 | 5.20 |

Based on the initial evaluation we provide above, there is some impact on the prediction of RSRP value due to RF error. From this perspective, we believe the modelling method for RF error also needs to be involved in the study.

Observation 8: There is some impact on the prediction accuracy of RSRP when RF error is considered

Proposal 7: For the study on the impact of measurement error, the method of RF error modelling needs to be involved in RAN4 discussion, e.g.,

- An offset value in the original dataset.

- Uniform distribution

- Truncated Gaussian distribution

- Summary

In this contribution, we further provided our views related issues on beam management. Based on the above analysis, the following proposals are presented.

Observation 1: There is a non-negligible performance degradation in beam prediction accuracy when considering measurement error modeling in training/validation data.

Observation 2: There is no large performance gap between the case that measurement error is only considered in validation data and the case that measurement error is considered both in validation data and training data

Proposal 1: RAN4 to define beam prediction accuracy requirements with consideration of the impact on measurement error

Observation 3: In addition to the rate of correct beam prediction, when additional 1dB RSRP margin is considered (e.g., For the success rate of the difference between the ideal RSRP of ideal Top-1 beam and the predicted RSRP of predicted Top-K beam are within the X=1dB range), it will bring a noticeable improvement in the rate of prediction accuracy.

Observation 4: The probability of predicted RSRP of predicted Top-K beam falling into the range of [the RSRP of Top-1 beam± XdB] has significant improvement with the increase of X value

Observation 5: When using predicted RSRP of predicted Top-1 beam and Ideal RSRP of ideal Top-1 beam as the benchmark, the probability that the difference between the RSRP of Top-1 beam and the predicted RSRP of Top-K beam is within the [X]dB range are similar

Proposal 2: RAN4 to use Top-K/1 as the metric of beam prediction accuracy for further performance evaluation

Observation 6: There is no large performance gap on the RSRP accuracy between the following two definitions of RSRP difference

- Alt1: RSRP difference only pertains to the case if AI/ML predicted beam ID is consistent with corresponding genie beam, or

- Alt2: RSRP difference between predicted Top-K beam with corresponding Top-K genie beam

Proposal 3: For metrics/KPIs for Beam Management requirements/tests, RAN4 to align the definition of RSRP accuracy as the difference between the predicted L1-RSRP of predicted beam and the ideal L1-RSRP of the same beam.

Proposal 4: For core requirements for beam management, RAN4 to define 1-sample based delay requirements for beam prediction of Spatial-domain in BM case-1. For Temporal beam prediction in BM case-2, the definition of delay requirements may take into account specific methods (e.g., caseA and caseB) and characteristics of time-domain.

Observation 7: From the simulated CDF curve of the RSRP difference for modelling errors from LLS and Gaussian distribution, the characteristics of baseband errors (e.g., variance, mean value) derived from LLS and Gaussian distribution are different.

Proposal 5: For the LLS-based measurement error modelling, RAN4 to discuss whether the simulation assumption is based on the TR 38.843 or the aligned assumption for L1-RSRP accuracy.

Proposal 6: RAN4 to consider the RSRP difference derived from link level simulation under specific SNR condition as the baseband error, and add the error into the ideal dataset derived from system level simulation so as to build the nonideal dataset for training and inference.

Observation 8: There is some impact on the prediction accuracy of RSRP when RF error is considered

Proposal 7: For the study on the impact of measurement error, the method of RF error modelling needs to be involved in RAN4 discussion, e.g.,

- An offset value in the original dataset.

- Uniform distribution

- Truncated Gaussian distribution

- References

- R4-2414323	WF on AI/ML, Qualcomm

- R4-2412250, Discussion on testability and interoperability issues for beam management, vivo

- RP-242399, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm

- TR 38.843 Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)