3GPP TSG RAN WG4 Meeting #112bis	R4-2415193

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:			6.17.5.2

Source:	vivo

Title:	Discussion on	testability and interoperability issues for positioning accuracy enhancement

Document for:	Discussion

- Introduction

RAN4 has studied many aspects about AI/ML use case aspects in Rel-18 SI stage, including CSI feedback enhancement, beam management enhancement and positioning accuracy enhancement. The outcomes of the study for positioning are captured in TR 38.843 [1]. In the RAN4#112 meeting, the WF [2] for AI/ML based positioning is captured as

Issue 3-2: Requirements for case 2a Agreement: RAN4 should not define any positioning accuracy requirements because positioning is LMF based Issue 3-3: Reported metrics for case 2a Agreement: - RAN4 will take existing requirements for existing metrics as baseline - FFS whether to revisit these requirements - RAN4 will study how to define requirement for any new reported metrics - FFS how to define requirements for LOS/NLOS Issue 3-5: Reported metrics for case 2b Agreement: postpone discussion on what and how to define requirements until RAN1 agrees to what metrics are reported. Issue 3-6: Reported metrics for case 3a/3b Agreement: postpone discussion until RAN1 agrees on any metric to be reported by gNBs.

In this contribution, we further provide our views on testability and interoperability issues for positioning accuracy enhancement.

- Discussion

For AI/ML positioning cases, 5 sub-use cases are identified and summarized in Table 1.

- A detailed list of various positioning cases

| Cases | Priority | Model deployment | Positioning methods | Measured by which entity | Model output | Position calculation |
| --- | --- | --- | --- | --- | --- | --- |
| Case 1 | 1st priority | UE-side | Direct AI/ML positioning | UE (Downlink positioning) | Position | UE-side |
| Case 2a | 2nd priority | UE-side | AI/ML assisted positioning | UE (Downlink positioning) | Intermediate feature | LMF |
| Case 2b | 2nd priority | LMF-side | Direct AI/ML positioning | UE (Downlink positioning) | Position | LMF |
| Case 3a | 1st priority | gNB-side | AI/ML assisted positioning | gNB (uplink positioning) | Intermediate feature | LMF |
| Case 3b | 1st priority | LMF-side | Direct AI/ML positioning | gNB (uplink positioning) | Position | LMF |

- 2.1	Testability aspects

Conformance and pre-deployment testing

For the conformance or the pre-deployment test, the issue of how to obtain the ground truth value needs to be first raised to considered. UE cannot derive its ground truth value of position and intermediate feature (except LOS/NLOS indicator) during the conformance testing by any methodology, if the legacy test method is used. The only way for DUT to derive the ground truth value is to deploy some PRUs in the test system.

For direct AI/ML positioning, it is not possible for UE to obtain its ground truth value for position, no matter which methodologies are used, e.g., GNSS, motion sensor, OTDOA, etc. All these methods would bring errors and cannot be treated as the ground truth value of target UE. However, the PRU (positioning reference unit), which is a type of use equipment with known location information, can perform positioning measurements, is introduced in Rel-18 positioning. With the known location and measurement information of nearby PRU, UE/NW can perform label-based performance validation by comparing the model output position with the known PRU location, while the model input is the channel measurement of PRU.

For AI/ML assisted positioning, PRU measurements together with the location information are still needed. Also, some other information, e.g. locations of TRPs stored in AMF, is also needed. Based on the assisted information, the ground truth value of intermediate feature can be calculated by geometry. Similar label-based validation procedure can be performed as the above contents for direct AI/ML positioning.

Based on the analysis, the test can also be feasible by performing label-based model output performance validation, and the test metrics should be KPIs defined in RAN4 accuracy requirement.

However, the label-based model input validation may be hard to conduct since the ground truth value of channel estimation is difficult to obtain. One potential feasible way is to transmit a package of additional reference dataset from LMF or server to the AI-deployed entity, and the dataset can be used as the ground truth for model input. In that case, the post-deployment and pre-deployment test can be feasible by comparing the provided ground truth value and the estimated channel measurements, and the test metrics should be KPIs defined in RAN4 accuracy requirement, if the model input KPI is identified by RAN4. Besides, if the reference dataset contains the ground truth value of model input and output, the test can be easily performed. The dataset can be generated by entity vendors or defined in the specifications.

Observation 1: PRUs can be deployed in the test scenario to provide measurements together with the location information.

Proposal 1: RAN4 to consider deploy PRUs in conformance or the pre-deployment test so as to derive the ground truth value of UE position and intermediate features.

Observation 2: At least LoS/NLoS indicator hard value can be tested since the LoS/NLoS environment is known in test system.

Generalization

AI/ML based positioning is a very area specific use case, especially for direct AI/ML positioning. According to RAN1 evaluation, assisted positioning with multi-TRPs is the methodology with better generalization performance than single-TRP. RAN1 also discussed other ways to improve the generalization performance, e.g., fine-tuning, which increases the performance in different drops. From RAN4 perspective, the testability for direct AI/ML positioning and assisted AI/ML positioning needs to be considered respectively. For the direct AI/ML positioning, RAN4 first needs to discuss whether to introduce the reference scenario/model/dataset. The test under reference scenario/model/dataset can provide a reference performance in certain conditions to guarantee the positioning performance would not degrade to an unexpected level in the field. For the assisted AI/ML positioning, since the generalization performance is better than AI/ML direct positioning, there may be no need to introduce the reference scenario/model/dataset, and the legacy test method can be reused. Further discussion is needed for the generalization aspects, and RAN4 needs to find out what kind of key factors would impact the performance of positioning.

Proposal 2: RAN4 to discuss the key factors that affect the performance and consider introduce reference scenario/model/dataset for AI/ML based positioning test for generalization.

- Summary

In this contribution, we provided our views on AI/ML based positioning. Based on above analysis, following observations and proposals are present.

Observation 1: PRUs can be deployed in the test scenario to provide measurements together with the location information.

Proposal 1: RAN4 to consider deploy PRUs in conformance or the pre-deployment test so as to derive the ground truth value of UE position and intermediate features.

Observation 2: At least LoS/NLoS indicator hard value can be tested since the LoS/NLoS environment is known in test system.

Proposal 2: RAN4 to discuss the key factors that affect the performance and consider introduce reference scenario/model/dataset for AI/ML based positioning test for generalization.

- References

- TR 38.842 Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)

- R4-2410570 WF on AI/ML Qualcomm Incorporated

- R4-2400617 WF on AI/ML Qualcomm Incorporated