3GPP TSG RAN WG4 Meeting #112bis	R4-2415196

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:			6.15.2.1

Source:	vivo

Title:	Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor

Document for:	Discussion

- Introduction

In the RAN#105 meeting, the revised WID on enhancements for NR and MR-DC RRM requirements was approved [1]. The objectives for RAN4 in the WID are as following

- FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - The CSSF requirement optimization is defined assuming a UE not operating with multiple-Rx based simultaneous L3 measurement: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2 - Note: Rel-19 CSSF optimization applies for CA/DC scenario independently of the UE support of multi-Rx capabilities. - Fast SCell activation for UE supporting Rel-18 EMR - Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting - Apply fast scell activation in FR1 and FR2-1 - Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024

In the last RAN4 meeting, the topic of FR2-1 SSB based L3 measurement delay reduction for connected model is discussed and the way forwards are captured in [2]. In this contribution, we provided our views on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor.

- Discussion

- 2.1 L3 measurement delay reduction by optimizing Rx BSF for UE supporting multiple-Rx simultaneous reception

In general, the FR2 L3 measurement delay is much longer than the delay in FR1 due to the Rx beam sweeping factor and the enhancement is needed. There are so many works have been already done in Rel-18 for reducing the FR2 measurement delay, e.g., introducing UE capability for FR2 L1 measurement delay reduction with reduced Rx beam sweeping factor (currently under discussion). For UE supporting multiple-Rx simultaneous reception on single carrier, RAN4 could consider to extend the capability to apply for L3 measurement for reducing the delay.

Proposal 1: RAN4 to consider introduce multi-Rx UE capability of reduced Rx beam sweeping factor for FR2 L3 measurement delay reduction.

In the last meeting, following issues are captured in the WF

FFS: RRM measurement with two panels activated, two searchers are occupied by this single carrier

For this issue, we support opt B and since 2 searchers are occupied by single carrier, it may have impacts to other CCs. For option A/1a that two searchers are occupied by single carrier, extend to CA scenario would be difficult.

Proposal 2: RAN4 only to consider the condition that two searchers are occupied by single carrier.

FFS: SSB processing delay/time for processing multiple beams received in a SMTC

SSB processing delay/time for processing multiple beams received in a SMTC. Since the interval between two SMTC is long enough for buffering and processing, there is no need to consider such issue for L3 measurement delay reduction in every measurement. However, an additional processing delay may be added in the last measurement sample for defining reporting delay requirement.

Proposal 3: No need to add additional processing time due to multiple SSBs within one SMTC, but to consider an additional processing delay in the last sample for defining reporting delay requirement.

FFS: Power consumption issue (including conditions to trigger UE using FBS for L3 measurement)

It is a very important issue for BSF optimization based RRM measurement since it is a long-term activity and there is no reason to active the multi-Rx all the time which will cause the overheating issues. In the last meeting, some companies support NW to indicate UE enabling/disabling fast beam sweeping through signalling or fast beam sweeping activated/deactived by following the Rel-18 mechanism. From our perspective, both options are not flexible to UE and more applicable way for UE is to define some additional triggering conditions, such as cell edge or centre, UE moving or keep stationary, or others, RAN4 could consider some triggering conditions to trigger UE using FBS for L3 measurement.

Proposal 4: RAN4 to consider additional conditions for triggering FBS for L3 measurement for power consumption issue, including active condition and de-active conditions.

Solutions to apply/specify L3 measurement delay reduction by optimizing Rx BSF

In the last meeting, following scenarios were at least agreed to use the L3 measurement delay reduction by optimizing Rx BSF.

- SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra

- SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra

- SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter

- SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter,  TSSB_time_index_inter and TSSB_measurement_period_inter

- Handover

- RRC Re-establishment/RRC Connection Release with Redirection

Take SSB based Intra-frequency measurement without MG as an example, in the current spec, beam sweeping factors related value M are corresponding to different power class in measurement delay requirement.

Mpss/sss_sync_w/o_gaps: For a UE supporting FR2-1 power class 1 or 5, Mpss/sss_sync_w/o_gaps =40. For a UE supporting power class 2, Mpss/sss_sync_w/o_gaps =24. For a UE supporting FR2-1 power class 3, Mpss/sss_sync_w/o_gaps =24. For a UE supporting FR2-1 power class 4, Mpss/sss_sync_w/o_gaps =24. For a UE supporting FR2-2 power class 1, Mpss/sss_sync_w/o_gaps = 60. For a UE supporting FR2-2 power class 2, Mpss/sss_sync_w/o_gaps = 36. For a UE supporting FR2-2 power class 3, Mpss/sss_sync_w/o_gaps = 36. Mmeas_period_w/o_gaps: For a UE supporting FR2-1 power class 1 or 5, Mmeas_period_w/o_gaps =40. For a UE supporting FR2-1 power class 2, Mmeas_period_w/o_gaps =24. For a UE supporting FR2-1 power class 3, Mmeas_period_w/o_gaps =24. For a UE supporting power class 4, Mmeas_period_w/o_gaps =24. For a UE supporting FR2-2 power class 1, Mmeas_period_w/o_gaps = 60. For a UE supporting FR2-2 power class 2, Mmeas_period_w/o_gaps = 36. For a UE supporting FR2-2 power class 3, Mmeas_period_w/o_gaps = 36. MSSB_index_intra: For a UE supporting FR2-2 power class 1, MSSB_index_intra = 72 samples. For a UE supporting FR2-2 power class 2, MSSB_index_intra = 48 samples. For a UE supporting FR2 power class 3, MSSB_index_intra = 48 samples.

It can be observed that the M value for FR2-1 in PC 3 is 24. In fact, the M value is a compromise value, which includes the BSF value and the measurement samples (e.g., M = BSF*sample). We can not derive the real value of BSF from the M since the sample value is unknow. If sample=4, the BSF value =6 and if sample=3, BSF value=8. Some company mentioned that Rx BSF can be reduced to 2, 4 or 6 or use Rel-18 BSF value as baseline

| 30. NR_FR2_multiRX_DL | [30-2] | Fast beam sweeping for layer-1 measurement | Supports beam sweeping factor reduction for SSB-based layer-1 measurement. | Per band | FR2-1 only | Candidate values for Component 2: {2,4,6} for FR2-1 Note: It is only supported for power class 3. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- |

Reuse {2,4,6} is a feasible way but may not applicable to L3 measurement. Based on the above analysis, we have the following approaches to consider this issue.

- Introduce the UE capability to indicate the reduced M value (M=sample*BSF), e.g., 12, 14, 16, etc.

- Introduce the UE capability to indicate the fraction value for reduction, e.g., 1/2, 1/3, etc.

- Introduce the UE capability to indicate the BSF value, the value needs to be discussed under a common assumption for the sample value, for example, sample = 4.

Proposal 5: RAN4 to consider the following approaches as solutions:

- Introduce the UE capability to indicate the reduced M value (M=sample*BSF), e.g., 12, 14, 16, etc.

- Introduce the UE capability to indicate the fraction value for reduction, e.g., 1/2, 1/3, etc.

- Introduce the UE capability to indicate the BSF value, the value needs to be discussed under a common assumption for the sample value, for example, sample = 4.

- Summary

In this contribution, we provided our views on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor. Based on above analysis, following proposals are present.

Proposal 1: RAN4 to consider introduce multi-Rx UE capability of reduced Rx beam sweeping factor for FR2 L3 measurement delay reduction.

Proposal 2: RAN4 only to consider the condition that two searchers are occupied by single carrier.

Proposal 3: No need to add additional processing time due to multiple SSBs within one SMTC, but to consider an additional processing delay in the last sample for defining reporting delay requirement.

Proposal 4: RAN4 to consider additional conditions for triggering FBS for L3 measurement for power consumption issue, including active condition and de-active conditions.

Proposal 5: RAN4 to consider the following approaches as solutions:

- Introduce the UE capability to indicate the reduced M value (M=sample*BSF), e.g., 12, 14, 16, etc.

- Introduce the UE capability to indicate the fraction value for reduction, e.g., 1/2, 1/3, etc.

- Introduce the UE capability to indicate the BSF value, the value needs to be discussed under a common assumption for the sample value, for example, sample = 4.

- References

- R4-242380 Revised WID: NR Radio Resource Management (RRM) Phase 5

- R4-2414026 WF for [112][228] NR_RRM_Ph5 Apple