3GPP TSG RAN WG4 Meeting #112bis	R4-2415198

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:			6.25.2

Source:	vivo

Title:	Discussion on RRM impacts for XR and reply LSs

Document for:	Discussion

# Introduction

A revised WID on XR (eXtended Reality) for NR [1] was approved in RAN#103 meeting. The objectives relevant to RAN4 are as follows.

- Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4] - Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4] - - Specify Core requirements related to the above objectives as necessary [RAN4]

In the RAN4#112 meeting, there were preliminary discussions on scope of the WI. Following agreements were made and captured in the WF [2].

## Issue 1-1: Workscope - As starting point, RAN4 to discuss the Tx/Rx in occasions of L3 measurement. - The progress in RAN1/2 should be taken into account. - FFS whether/when to start the discussion of Tx/Rx in occasions of scheduling restriction due to L1 operation. - RAN4 to discuss - The potential impact on measurement requirements due to measurement cancellation - The corresponding solution to address the impact if needed. ## Issue 1-2: Deployment scenarios - Discuss if there are aspects related to deployment scenarios to be considered. ## Issue 1-3: Types of measurement gaps to consider In this issue we discuss the types of measurement gaps to be considered in this WID. In order to help the discussion, we reuse the definition of Type-1 and Type-2 measurement gaps as agreed in Rel-18 NR_MG-enh2: - Type-1 MG: Gap(s) configured via GapConfig without suffix - Type-2 MG: Gap(s) configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17 - Rel-18 Concurrent Gap: gap with assigned priority which can be also NCSG or Pre-MG. discuss the priorities for RAN4 to work based on the following table: Cap types; First prio; Second prio; Don’t include. Type 1; Nokia, MTK, QC, Vivo, ZTE, Apple, CMCC, HW, Ericsson, OPPO, CATT, IDC. Type 2; QC, vivo, Apple, CMCC, HW, OPPO, CATT; Nokia; ZTE. NCSG; Apple, CMCC, HW, OPPO, MTK; QC, Nokia, CATT, IDC; vivo. MUSIM; HW; Vivo, Apple, Nokia, Ericsson, CATT. Positioning; QC, vivo, HW; ZTE, Apple, Nokia, Ericsson, CATT, IDC. Rel 18 concurrent gap; QC, Apple, CMCC, HW, CATT, MTK; Vivo, ZTE, Nokia, Ericsson. PreConfigured; Apple, QC, CATT, CMCC, HW, OPPO, MTK. ## Issue 1-4: Types of measurements - Discuss types of measurements that may potentially be impacted by the XR operation. - FFS: measurements without gaps

RAN1 had discussed UE assistance information related to measurement occasions for several meeting. In the RAN1#117 meeting, conclusions were made and an LS [3] was sent to RAN4 to trigger further discussions in RAN4. In addition, RAN1 concluded solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions as working assumption and an LS [4] was sent to RAN4 for confirmation.

In this contribution, we provide our views on RRM requirements impact for XR in Rel-19 and on questions raised in the two RAN1 LSs.

# RRM requirements impact

According to WID [1], one of the objectives of this WI aims to realize system capacity gains by enabling transmission/reception in gaps/restrictions that are caused by RRM measurements, while keeping impact to mobility performance limited. This was studied in TR 38.835 B.1.7. The solution to achieve this aim is to specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc).

The discussions in RAN1 had been continued for several meetings on triggering scheme to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements and whether/what UE assistance information is needed.

RAN1 made following working assumption.

- Alt. 1: Dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements.

  - Alt 1-1: Explicit indication by DCI to skip a particular gap/restriction;

Therefore, from RAN4 perspective, it needs to decide firstly what RRM measurements and corresponding scheduling restrictions are to be considered in the scope.

## 2.1 gaps/restrictions

RRM measurements can be categorized into L3 measurements and L1 measurements. L3 measurements can be further grouped as measurements within gaps and outside gaps. The scheduling restrictions are different for the three types of measurements.

L3 measurements within gaps

The L3 measurements within gaps are performed when measurement gaps are configured. During the measurement gaps, the UE is not required to conduct reception/transmission from/to the corresponding serving cells. The entire measurement gap is used for measurement only by receiving reference signals such as SSB, PRS.

L3 measurements outside gaps

The L3 measurements outside gaps are performed on SMTC occasions outside of measurement gaps if they are not fully overlapped. The scheduling restrictions are summarized as below.

- For UE which do not support simultaneousRxDataSSB-DiffNumerology for SSB with different subcarrier spacing than PDSCH/PDCCH on FR1 - The UE is not expected for transmission/reception on SSB symbols configured to be measured, and on 1 data symbol before and after each consecutive SSB symbols configured to be measured - intra-band carrier aggregation: apply to all other serving cells in the same band - intra-band non-contiguous carrier aggregation configured for a UE indicating intraBandNR-CA-non-collocated-r18: scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band if nonCollocatedTypeNR-CA-r18 is provided. - TDD bands on FR1 - The UE is not expected to transmit on SSB symbols configured to be measured, and on 1 data symbol before and after each consecutive SSB symbols configured to be measured - When TDD intra-band carrier aggregation is performed, the scheduling restrictions due to a given serving cell also apply to all other serving cells - When intra-band non-contiguous carrier aggregation is configured for a UE indicating intraBandNR-CA-non-collocated-r18, apply to all other serving cells in the same band if nonCollocatedTypeNR-CA-r18 is provided - When TDD inter-band carrier aggregation is performed, apply to another serving cell in a different band if UE does not have the capability of supporting simultaneousRxTxInterBandCA for this band pair. - FR2 - The UE is not expected for transmission/reception on restricted symbols, on SSB symbols configured to be measured, and on 1 data symbol before and after each consecutive SSB symbols configured to be measured - intra-band carrier aggregation: apply to all other serving cells in the same band - inter-band carrier aggregation: for UE supporting IBM, similar restrictions as TDD bands on FR1

L1 measurements

For L1 measurements, it has similar restrictions as L3 measurement outside gaps but the restrictions are only on reference signal symbols for L1 measurements. It is summarized as below.

- FR1 - For UE which do not support simultaneousRxDataSSB-DiffNumerology SSB with different subcarrier spacing than PDSCH/PDCCH on FR1, - The UE is not expected for transmission/reception on symbols corresponding to the SSB indexes configured for L1-RSRP measurement - intra-band carrier aggregation: apply to all other serving cells in the same band - intra-band non-contiguous carrier aggregation configured for a UE indicating intraBandNR-CA-non-collocated-r18: scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band if nonCollocatedTypeNR-CA-r18 is provided. - FR2 - The UE is not expected for transmission/reception symbols corresponding to the SSB indexes and/or periodic/semi-persistent/aperiodic CSI-RS resources which is NOT QCLed with active TCI state for PDCCH/PDSCH configured for L1 measurement - intra-band carrier aggregation: apply to all serving cells in the band on the symbols that fully or partially overlap with restricted symbols.

Moreover, L1 measurements including RLM/BFD/CDB/L1-RSRP/L1-SINR measurements. The reference signals for different L1 measurements may or may not be the same, or may or may not be overlapped.

It is observed that the UE cannot be scheduled during the measurement gaps. For L3 measurements outside gaps when certain conditions are met, the UE cannot be scheduled on some or all of the symbols within SMTC. For L1 measurements when certain conditions are met, the UE cannot be scheduled only on the reference signal symbols.

Observation 1: Scheduling restrictions are different in terms of conditions and restricted symbols for L3 measurements within gaps, L3 measurements outside gaps and L1 measurements.

## 2.2 Scope

It was agreed RAN4 in the last RAN4 meeting that as starting point, RAN4 is to discuss the Tx/Rx in occasions of L3 measurement. Regarding the types of measurements that may potentially be impacted by the XR operation, it is FFS for measurements without gaps.

L3 measurements within gaps

There are different measurement gaps being introduced in multiple releases, e.g., The per-UE gap and per-FR gap in Rel-15, pre-configured measurement gap, concurrent measurement gaps, NCSG and MUSIM gaps in Rel-17. Combination of concurrent gap and pre-MG or NCSG was further enhanced in Rel-18. RAN4 needs to decide what type of measurement gaps are considered in this WI. It would have a lot of requirements to be defined if all the gap types are considered.

In the last meeting, there were discussions on the type of measurement gaps to be considered, including Type 1 MG, Type 2 MG, NCSG, MUSIM gaps, positioning gaps, concurrent gaps and pre-configured gaps. Views from companies are captured in the WF [2].

It is noted that Type 1 MG and Type 2 MG are defined as below.

- Type-1 MG: Gap(s) configured via GapConfig without suffix - Type-2 MG: Gap(s) configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17

Type 1 MG is the baseline from Rel-15 and it should be supported. It includes both per-UE gap and per-FR gap configured via GapConfig.

The Type-2 MG, however, is not so clear. Whether it is a single gap configured via GapConfig-r17 or it can be multiple gaps. If it is the latter one, then what would be the difference between type-2 MGs and concurrent gaps. If it is the former one, then it should be supported as there is no difference compared to Type-1 MG in terms of gap skipping.

Concurrent gaps are quite complicated. Moreover, whether concurrent gaps with NCSG/pre-configured gaps are included or not should be clarified. It may not be necessary that NW configures concurrent gaps when XR service is enabled. It is preferred not to consider concurrent gaps in Rel-19.

For NCSG, in general there would be no difference compare to Type 1 MG or Type 2 MG from scheme of triggering gap skipping perspective. However, the interruption ratio due to VIL is relatively small, which is 2ms in FR1 and 1.5ms in FR2 within one visible interruption repetition period. In typical measurement gap configuration, 6ms MGL with 40/80ms MGRP would be configured for the UE. The interruption rate due to measurement gap is up to 15%. If 20ms MGRP is considered in FR2, the interruption rate is up to 30%. However, for NSCG the interruption rate can only be up to 7.5% for 20ms MGRP in FR2. Considering the possibility of collision between XR traffic and NCSG, the impact to XR service is very small. Thus, there is no need to consider NCSG when gaps are to be skipped.

Pre-configured measurement gaps are similar to Type-2 MG when it is activated which can be triggered by UE autonomous control or network configuration. When it is not activated, it is the same as no measurement gaps are configured. Thus, it can be considered.

Positioning measurement gaps are not clear by itself. In Rel-16, legacy per-UE gap is used for PRS measurements. When RRM measurements also need gaps, then gap sharing between RRM and PRS measurements is used. In Rel-17, per-FR gap can also be used for PRS measurements. Additionally, pre-configured MG by PosGapConfig-r17 is also supported for PRS measurement which can be activated via DL MAC CE or UL MAC CE depends on UE capability. If positioning measurements are based on legacy measurement gap, then it would be naturally be considered. For Rel-17 pre-configured MG for positioning, it is proposed not to consider it.

MUSIM gaps is for operations in a different network which can minimize impact under current network operation. If network believes XR service is more import it can reject to allocate MUSIM gaps when it is requested by UE. Similarly, if a device believes XR service is more import, it may not request MUSIM gaps at all. There is no need to specify complicated solutions to skip MUSIM gaps.

Proposal 1: Type 1 MG and Type 2 MG, including per-UE gap and per-FR gap, and pre-configured MG is considered in XR WI in Rel-19, where Type 2 MG needs further clarification.

L3 measurements outside gaps

L3 measurements outside gaps could also be intra-frequency measurements, inter-frequency measurements and inter-RAT measurements. As analyzed in section 3.2, there could be some SMTC occasions for a MO within a time period are not used for measurements depending on measurement configuration. These SMTC occasions can be used for data transmission/reception. Similar to gaps, the SMTC occasions not used for measurements in the first place may not be enough for XR traffic. Additional SMTC occasions may be indicated by NW for data transmission/reception.

On the other hand, in FR1 the scheduling restrictions for L3 measurements outside gaps is only for UE which do not support simultaneousRxDataSSB-DiffNumerology. In general, mixed numerology of data and SSB is not a typical deployment. Moreover, it should be no extra complexity for UE to support this UE capability. Thus, it is better choice for XR device to support the UE capability simultaneousRxDataSSB-DiffNumerology so that it can be scheduled without restriction under mixed numerology deployment. In FR2 the scheduling restrictions for L3 measurements outside gaps is due to Rx beam sweeping. It cannot be avoided during measurements. Intra-frequency measurement is very important for UE mobility and long measurement delay in FR2 has been a concern that there is dedicated WI to reduce beam sweeping factor. If the SMTC occasions are skipped then longer measurement delay is expected. Especially when any SMTC occasions overlapped with XR data is skipped then mobility performance could be highly degraded.

Proposal 2: Skipping SMTC occasions for L3 measurements outside gaps are deprioritized.

L1 measurements

L1 measurements include RLM, BFD, CBD or L1-RSRP/L1-SINR for beam reporting. L1 measurements are performed outside gaps. Similar to L3 measurements outside gaps, in FR1, the scheduling restriction is for UE which do not support simultaneousRxDataSSB-DiffNumerology. In FR2, it is due to Rx beam sweeping for SSB based measurements. However, for CSI-RS based L1 measurements there is no scheduling restriction when certain conditions are met.

The reference signals for L1 measurements includes SSB and/or CSI-RS, which may or may not be fully overlapped with SMTC occasions outside measurement gaps. If it is not fully overlapped with SMTC occasions, the measurement is performed on the reference signal symbols not overlapped with SMTC occasions. Otherwise, in FR2 it needs to share SMTC occasions for L3 measurements outside gaps. Since scheduling restriction for L1 measurements are only on L1-RS symbols which would not be many, the benefit of enabling data transmission/reception on the symbols is very small. Furthermore, it could have big impact on L1 measurement performance, especially for RLM/BFD measurements which is essential for link quality monitoring.

Therefore, it is not preferred to enable L1-RS symbols for data transmission/reception when there are scheduling restrictions.

Proposal 3: RAN4 does not consider enabling data transmission/reception on L1-RS symbols that need to be used for L1 measurements if there is scheduling restriction.

If Rx/Tx is enabled in gaps/restrictions with or without UE assistance information, it should be based on UE capability. On step further is that whether the UE capability has pre-requisite feature(s), e.g., whether it is only used for XR devices. In our view, the feature itself can be supported by any UE. If so, when defining requirements and determining solutions for enabling Rx/Tx in gaps/restrictions the trade-off between mobility performance degradation and data transmission delay should be considered.

Proposal 4: RAN4 to discuss and decide whether enabling Rx/Tx in gaps/restrictions can be supported by any UE.

## 2.3 Impact to requirements

L3 measurements within gaps

L3 measurements within gaps could be intra-frequency measurements, inter-frequency measurements and inter-RAT measurements. As analyzed in section 3.1, some of the gaps may not be used for measurements at all while measurement requirements being satisfied, depending on measurement configuration, e.g., gap, SMTC, MO etc. Then the UE can be scheduled in these gaps. In this case, there is no impact to RRM measurement requirements.

It would be also necessary, at least for XR use case, that some of the gaps, which are supposed to be used for measurements depending on measurement configuration, may also be used for data scheduling when NW indicates so. In such case, the UE cannot meet measurement requirements.

In general, gaps can be enabled for data transmission/reception. Depending on various conditions, measurement requirements may or may not be satisfied. RAN4 should study whether and how to define new core requirements when some of gaps that need to be used for measurements are enabled for data transmission/reception. It is depending on the complete solutions of how Tx/Rx is enabled during measurement gaps.

Proposal 5: RAN4 to study whether and how to define new core requirements when some of gaps that need to be used for measurements are enabled for data transmission/reception after solution for enabling Rx/Tx in gaps is clear.

As analyzed in section 3.1, there is case that UE may report enough gaps not used for measurement. If only these gaps are used for data scheduling, then existing measurement requirements can be fulfilled. Since the feature could be used for any UEs that data delivery delay is not critical, the case should be considered.

Proposal 6: When only those gaps not used for measurement based on UE assistance information / UE capability are scheduled for data transmission/reception, the UE should meet existing requirements.

When there are gaps that are supposed to be used for measurement are scheduled for data transmission/reception, the UE cannot meet existing requirements. Based on RAN1 working assumption, dynamic indication by DCI to enable Tx/Rx in particular gap/restriction is supported. If there is no restriction on the dynamic indication, then it would be not possible to define new requirements because it is unknown what percentage of gaps and/or which gaps are skipped. The RRM measurement requirements are based on gap periodicity and gap sharing scheme. In multi-frequency layer scenario, frequency layers would have different ‘priority’. For example, for a frequency layer configured with 160ms SMTC periodicity may have higher priority than other frequency layers with shorter SMTC periodicity when competing a measurement gap. If those gaps for the frequency layer are all indicated by network for data transmission/reception, there is no chance for the UE to measure the frequency layer. Thus, UE mobility performance will highly be degraded.

Thus, it is expected that some restrictions should be put on the dynamic indication. For example, the gaps to be skipped during a time period is indicated by RRC in advanced. The DCI indicated gaps are only selected from those configured by RRC. This will largely simplify UE measurement scheduler which only needs to be conducted after RRC reconfiguration and ensure mobility performance is not degraded too much. Other potential restriction(s) that can help to achieve good trade-off between mobility performance and data delivery delay can also be considered.

Proposal 7: RAN4 to study possible restrictions on dynamic indication by DCI, if it is confirmed, to achieve good trade-off between data deliver delay and mobility performance.

L3 measurements outside gaps

L3 measurements outside gaps could also be intra-frequency measurements, inter-frequency measurements and inter-RAT measurements. As analyzed in section 3.2, there could be some SMTC occasions for a MO within a time period are not used for measurements depending on measurement configuration. These SMTC occasions can be used for data transmission/reception. In this case, there is no impact to RRM measurement requirements for L3 measurements outside gaps.

Similar to gaps, the SMTC occasions not used for measurements in the first place may not be enough for XR traffic or there is no unused SMTC occasions. Additional SMTC occasions may be indicated by NW for data transmission/reception. Thus, it has similar situation as measurement within gap that RAN4 needs to study whether and how to define new core requirements when some of the SMTC occasions that need to be used for measurements are enabled for data transmission/reception when there are scheduling restrictions on the SMTC occasions.

Of course, this part depends on the agreements whether skipping SMTC occasions for L3 measurements without gaps is supported.

Proposal 8: If supported, RAN4 to study whether and how to define new core requirements when some of SMTC occasions that need to be used for measurements outside gaps are enabled for data transmission/reception when scheduling restrictions exists.

# UE assistance information

RAN1 had discussed UE assistance information related to measurement occasions for several meeting. In the RAN1#117 meeting, conclusions were made and an LS [3] were sent to RAN4 to trigger further discussions in RAN4.

Agreement RAN1 continues to discuss and decide whether or not to introduce new UE assistance information for solution(s) to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements. At least the following UE assistance information is considered for further study: - FFS: UE assistance information related to measurement occasions: - FFS: The number of needed measurement gaps/SMTC with restrictions within a time period; - FFS: The maximum number or ratio of MGs/SMTC with restrictions that can be skipped within a time period; - FFS: The number of required SSBs within a time period; - FFS: The number of consecutive RRM measurements that can be skipped; - FFS: The maximum interval between two consecutively reserved gap/restriction occasions for RRM measurements; - FFS: The patterns of gap(s)/restriction(s) where skipping is feasible or acceptable; - FFS: UE assistance information related to channel conditions: - FFS: RSRP is below/above search threshold (s-MeasureConfig); - FFS: UE assistance information related to traffic: - FFS: PSI (PDU set importance); - FFS: UE assistance information related to UE mobility: - FFS: L3 parameters related to mobility, e.g., static or not Companies are encouraged to provide additional details (e.g. how often the UE assistance info is provided, timing, applicable scenarios, performance gains, etc) on their preferred scheme. Note: From specification point of view, there is no mandated gNB behavior in response to any of the UE assistance information. RAN1 to make decision, from RAN1 perspective, in RAN1#117 on the support of UE assistance information. Conclusion RAN1 does not further discuss new UE assistance information related to channel conditions, traffic, UE mobility. Conclusion There is no consensus in RAN1 to support UE assistance information related to measurements occasions. The reason for this situation is lack of consensus on the need/feasibility for UAI and lack of technical understanding on issues outside of RAN1 expertise (e.g. impact of RRM measurement performance). It is up to other working groups to trigger further work in RAN1 on UE assistance information.

## 3.1 RRM measurements within gaps

For the L3 measurements performed within gaps, the cell identification delay and measurement period for a frequency layer i is depending on MGRP, SMTC period, CCSFwithin_gap,i. It is further depending on DRX cycle if C-DRX is configured.

For example, the measurement period requirements for inter-frequency measures with gaps in FR1 is specified as in Table 9.3.5-1.

Table 9.3.5-1: Measurement period for inter-frequency measurements with gaps (Frequency FR1)

| Condition NOTE1,2 | T SSB_measurement_period_inter |
| --- | --- |
| No DRX | Max(200ms, Ceil(8 * Kgap) Max(MGRP , SMTC period)) CSSFinter |
| DRX cycle ≤ 320ms | Max(200ms, Ceil(8 1.5 * Kgap) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | Ceil(8 * Kgap) DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: For a UE supporting concurrent measurement GAPs, the MGRP above is the MGRP of the activated Pre-MG or the measurement gap associated with the target frequency layer to be measured if concurrent GAPs are configured. |  |

CCSFinter is determined according to CSSFwithin_gap,i, which is derived based on following procedures.

If measGapSharingScheme is equal sharing, CSSFwithin_gap,i= max(ceil(Ri×Mtot,i,j)), where j=0…(160/MGRP)-1 If measGapSharingScheme is not equal sharing and - measurement object i is an intra-frequency measurement object, CSSFwithin_gap,i is the maximum among - ceil(Ri×Kintra×Mintra,i,j) in gaps where Minter,i,j≠0, where j=0…(160/MGRP)-1 - ceil(Ri×Mintra,i,j) in gaps where Minter,i,j=0, where j=0…(160/MGRP)-1 - measurement object i is an inter-frequency or inter-RAT measurement object or NR PRS measurement on any one positioning frequency layer, CSSFwithin_gap,i is the maximum among - ceil(Ri×Kinter×Minter,i,j) in gaps where Mintra,i,j ≠0, where j=0…(160/MGRP)-1 - ceil(Ri×Minter,i,j) in gaps where Mintra,i,j=0, where j=0…(160/MGRP)-1 Where Ri is the maximal ratio of the number of measurement gap where measurement object i is a candidate to be measured over the number of measurement gap where measurement object i is a candidate and not used for a long-periodicity measurement defined above. Where： - Mtot,i,j = Mintra,i,j + Minter,i,j : Total number of intra-frequency, inter-frequency and inter-RAT frequncy layers and up to one NR PRS measurement on any one positioning frequency layer, inter-frequency L1-RSRP measurement layers, which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise Mtot,i,j equals 0.

Based on the above procedure and depending on measurement configuration, the definition of CCSFwithin_gap,i could lead to a long measurement delay requirements where some the gaps may not be used for measurements.

For the example of measurement configuration depicted in Fig 1, measurement gap is configured together with three inter-frequency layers where different SMTC periodicity is used. For this configuration, CCSFwithin_gap,i is 3 for all the frequency layers.

Fig 1. Configuration of measurements with gaps

In this case, some of the gaps may not always be used to meet minimum measurement period requirements. The requirements are assumed that there are always 3 candidates of MOs to be measured within a gap. For MO#1 and MO#2, it is not the case. So, some of the gaps could be left unused as long as measurement period requirements are satisfied.

Suppose 2 samples are needed for one measurement period. one of the possible measurement implementations is depicted in Fig 2. Another possible measurement implementation is depicted in Fig 3.

Fig 2. Measurement implementation example 1

Fig 3. Measurement implementation example 2

Based on above analysis, it is observed that some of the gaps may not be used for measurements and it is up to implementation which gaps are not used for measurements.

Observation 2: Depending on measurement configuration, e.g., gap, SMTC, MO etc., some of the gaps may not be used for measurements while measurement requirements being satisfied.

Observation 3: It is up to UE implementation which gaps are not used for measurements under current measurement configuration.

In the case as depicted in Fig 1, Fig 2 and Fig 3, the unused gaps are about 37.5% (9/24). It is significant gain if these gaps can be used for data transmission/reception. Since it is up to UE implementation which gaps are not used, the information can only be provided by UE. Therefore, there is obvious benefit if UE can provide assistance information about which gaps are not used for measurements.

Observation 4: Significant gain on throughput is observed if UE can provide information about unused gaps for measurement without impact on mobility performance.

Moreover, for some advanced UE, it may not use all the gaps during the measurement period specified by minimum requirements. For example, measurement period is based on 8 samples for FR1 inter-frequency measurements. The advance UE may just use 5 of them. So, the UE either not to use some of the gaps or use all the gaps with shorter measurement period. If the former case, then the gaps not used for measurement can be reported to network to schedule the UE during the gaps.

Regardless of the gaps not used for measurements is due to measurement configuration or advanced UE implementation, it is more like UE capability rather than UE assistance information. It is better that RAN4 to discuss which scheme should be used. The difference is that NW will follow UE reporting if it is UE capability, but the NW may not follow UE reporting if it is UE assistance information depending on current specification.

Even for use case other than XR, these unused measurement gaps can be used for data transmission/reception, which means the gain comes at no cost.

Proposal 9: UE should provide information about gaps not used for measurement for enabling transmission/reception in these gaps.

Proposal 10: RAN4 to discuss the information about gaps not used for measurement are reported as UE capability or UE assistance information.

Then it comes the question that the reported gaps not used for measurement may not be enough for data transmission/reception considering QoS requirements. In some cases, there may be no gaps not used for measurement depending on measurement configuration. However, when enabling transmission/reception in gaps/restrictions that are caused by RRM measurements, it needs to keep impact to mobility performance limited. In order to achieve this goal, it needs UE to provide information of gaps not used for measurement. If it is totally up to NW indication, the mobility performance could highly be degraded as gaps may have different priority for measurements, e.g., the most loaded gap has highest priority and it may be used to measure sparse reference signals and should not be used for data transmission/reception, or if gap sharing scheme is not equal sharing, some gaps may have high priority to meet requirements.

One possible solution is that NW indicates the percentage of gaps not used for measurement during a period of time. It is up to UE to decide which gaps are not used for measurement during the period of time. This could be the best balance between QoS requirements of data transmission/reception and UE mobility performance.

Proposal 11: NW may indicate the percentage of measurement gaps not used for measurement for UE to provide information about gaps not used for measurement.

## 3.2 RRM measurements outside gaps

For the L3 measurements performed outside gaps, the cell identification delay and measurement period for a frequency layer i is depending on SMTC period, Klayer1_measurement and CCSFoutside_gap,i. It is further depending on DRX cycle if C-DRX is configured.

For example, the measurement period requirements for intra-frequency measurements outside gaps in FR2 are specified as in Table 9.2.5.2-2.

Table 9.2.5.2-2: Measurement period for intra-frequency measurements without gaps (FR2)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(400ms, ceil(Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(400ms, ceil(1.5x Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(Mmeas_period_w/o_gaps xKp x Klayer1_measurement ) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

CCSFintra is determined according to CSSFoutside_gap,i, which is based on configuration of number of CCs and inter-frequency/inter-RAT measurements outside gaps. The PCC has highest priority and would use a dedicated searcher. All the other CCs and inter-frequency/inter-RAT measurements will share another searcher, which means it will be measured sequentially.

Klayer1_measurement is sharing factor between L1 measurements and L3 measurements outside gaps when all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting are fully overlapped wither reference signal symbols, including 1 symbol before/after, for L3 measurements. The scaling factor is only needed in FR2 and it is 1 in FR1.

In general, when the SMTC for a MO, which can be measured outside gaps, are not fully overlapped with measurement gaps, the measurements are conducted on SMTCs not overlapped with measurement gaps. Otherwise, it becomes measurement within gaps. Furthermore, it the SMTCs not overlapped with measurement gaps are fully overlapped with reference signal symbols for L1 measurements, i.e., RLM, BFD, CBD or L1-RSRP for beam reporting, it will share the SMTC measurement occasions which only happens in FR2.

For the measurements on PCC, if sharing with L1 measurements are not considered, then all the SMTCs outside gaps would be used for measurements to meet minimum measurement requirements. It seems no strong motivation to provide UE assistance information about SMTC occasions not used for measurements. If there is sharing between L1 measurements and L3 measurements outside gaps, it could be useful to provide assistance information that some SMTC occasions during a period of time are used for L1 measurements since the scheduling restrictions are different between L3 measurements and L1 measurements.

For the measurements on SCCs, the CCSF is scaled by the number of CCs. It could leave to unused SMTC occasions depending on measurement configurations.

For the example of measurement configuration depicted in Fig 4, measurements on PCC and 3 SCCs are configured where different SMTC periodicity is used on SCCs. For this configuration, CCSFoutside_gap,i is 3 for all the 3 SCCs and is 1 for PCC.

Fig 4. Configuration of measurements outside gaps

In this case, one of the possible measurement implementations is depicted in Fig 5. the SMTC occasion filled with pattern is not used for measurements. All the SMTC occasions on PCC are used for L3 measurements. For SCC, UE just needs to measure 1 out of 3 SMTC occasions since CCSF is 3. It is obvious some of the SMTC occasions on SCCs are not used for measurement.

Fig 5. Configuration of measurements outside gaps

Observation 5: Depending on measurement configuration, e.g., SMTC, MO etc., some of the SMTC occasions may not be used for measurements while measurement requirements being satisfied.

Observation 6: It is up to UE implementation which SMTC occasions are not used for measurements under current measurement configuration.

Similar to UE assistance information about measurement gaps, it is beneficial to provide the SMTC occasions not used for measurements on each CC to NW so that NW can schedule the UE on these SMTC occasions by taking scheduling restriction requirements into consideration. Also similar to measurement with gaps, an advanced UE may use less SMTC occasions than in minimum requirements. Then, those SMTC occasions not used for measurement can be reported to NW.

Proposal 12: If supported, UE should provide information about SMTC occasions not used for L3 measurement for enabling transmission/reception on the SMTC occasions.

Proposal 13: If supported, RAN4 to discuss the information about SMTC occasions not used for measurement are reported as UE capability or UE assistance information.

Similar to UE information about measurement gaps, the reported SMTC occasions not used for measurement may not be enough for data transmission/reception considering QoS requirements. In some cases, there may be no SMTC occasions not used for measurement depending on measurement configuration. One possible solution is that NW indicates the percentage of SMTC occasions not used for measurement during a period of time. It is up to UE to decide which SMTC occasions are not used for measurement during the period of time. This could be the best balance between QoS requirements of data transmission/reception and UE mobility performance.

Proposal 14: If supported, NW may indicate the percentage about SMTC occasions not used for L3 measurement outside gaps for UE to provide information about SMTC occasions not used for measurement.

Proposal 15: If supported, the UE assistance information about SMTC occasions not used for measurement should be provided on each CC/MO.

## 3.3 UE information format

There serval aspects about UE assistance information had been discussed in RAN1.

- FFS: UE assistance information related to measurement occasions: - FFS: The number of needed measurement gaps/SMTC with restrictions within a time period; - FFS: The maximum number or ratio of MGs/SMTC with restrictions that can be skipped within a time period; - FFS: The number of required SSBs within a time period; - FFS: The number of consecutive RRM measurements that can be skipped; - FFS: The maximum interval between two consecutively reserved gap/restriction occasions for RRM measurements; - FFS: The patterns of gap(s)/restriction(s) where skipping is feasible or acceptable;

As discussed in section 3.1 and 3.2, the patterns of gaps/SMTC occasions not used for measurement within a time period should be provided to achieve best balance between QoS requirements and mobility performance.

Other options will have unknown and unexpected impact on UE mobility performance and it provides no information on which gaps have higher priority, e.g., most loaded gaps or gaps for sparse reference signals.

Proposal 16: The patterns of gaps/SMTC occasions not used for measurement within a time period should be provided as UE assistance information.

- FFS: UE assistance information related to channel conditions: - FFS: RSRP is below/above search threshold (s-MeasureConfig);

s-MeasureConfig is the threshold for NR SpCell RSRP measurement controlling when the UE is required to perform measurements on non-serving cells. When serving cell RSRP is above the threshold, UE may not perform measurements on neighbour cells. In this case, measurement gaps, if configured, may not be used for measurement at all.

However, UE still needs to perform measurements on SMTC occasions for serving cell, which are also the same for all neighbour cells on the serving cell frequency, if the measurements are performed outside gap. The channel condition of serving cell RSRP does not help UE to provide information about which SMTC occasions within a time period are not used for measurements.

Moreover, when serving cell RSRP is below the threshold, UE needs to measure neighbour cells where measurement gaps could be used. The information about gaps not used for measurement should still be provided.

Observation 7: Channel condition, e.g., RSRP, does not provide information about which SMTC occasions are not used for measurements. Moreover, additional information about gaps/SMTCs not used for measurement is still needed.

- FFS: UE assistance information related to traffic: - FFS: PSI (PDU set importance);

The UE assistance information related to traffic is not RAN4 expertise. It would be better to be discussed in RAN2.

- FFS: UE assistance information related to UE mobility: - FFS: L3 parameters related to mobility, e.g., static or not

There is no L3 measurement requirements related to UE mobility. In connected mode, requirements for UE fulfilling relaxed measurement criteria are specified for RLM and BFD measurements. The relaxed measurement criteria include good serving cell quality and low mobility. There is no L3 measurement requirements related to UE mobility. Thus, UE behaviour on L3 measurement is not changed based on different UE mobility status. If UE mobility is to be used as additional UE assistance information, relaxed L3 measurement requirements should be specified accordingly.

Observation 8: UE mobility information does not help how gaps/SMTC occasions are used for measurement unless relaxed L3 measurement requirements are specified.

Based on aforementioned discussions on RRM measurements within gaps and outside gaps, the UE assistance information about measurement gaps/SMTC occasions not used for measurement is dependent on measurement configurations related to number of measurement objects, SMTC, MGRP, DRX, number of CCs etc. These parameters are configured and updated via RRC signaling RRCReconfiguration. Thus, UE should provide the assistance information/UE capability about gaps/SMTC occasions not used for measurement in response to RRCReconfiguration.

Proposal 17: UE assistance information/UE capability regarding gaps/SMTC occasions (if supported) not used for measurement is reported in response to RRCReconfiguration.

When DRX is configured, the UE may measure once within gaps or on SMTC occasions in one DRX cycle. Only the gaps/SMTC occasions overlapped with DRX on duration can be used data transmission/reception. There could be different reporting scheme for UE assistance information, e.g., only reporting gaps/SMTC occasions overlapped with DRX on duration within a time period.

Proposal 18: RAN4 to further study how UE assistance information/UE capability is reported when DRX is configured.

# Solutions for enabling Tx/Rx in gaps/restrictions

In RAN1, the discussions had been continued for several meetings on solutions to enable transmission/reception in gaps/restrictions that are caused by RRM measurements. In the RAN1#118 meeting, conclusions were made and an LS [4] were sent to RAN4 to confirm the working assumptions.

Working Assumption For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select the following option: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements. - Alt 1-1: Explicit indication by DCI to skip a particular gap/restriction; - Indication is included as part of scheduling DCI: - Bit-field size is one bit; - The bit in the DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI. Send an LS to RAN4 to inform them of the above working assumption and ask them if there is any issue with it. Agreement If Alt. 1 from RAN1#117 agreement is supported, minimum time offset(s) X between indication to skip and skipped measurement occasion is up to RAN4 to discuss and decide on particular value(s).

## 4.1 Indication solution

In general, it would be okay to indicate skipping a particular gap by DCI. As discussed in section 2.3, the issue would be it could cause serious mobility performance degradation if there is no restriction on the indication. To achieve better trade-off between mobility performance and data delivery delay/throughput, RAN4 should discuss potential restrictions, e.g., configuring gaps to be skipped during a time period by RRC in advance and DCI indication is based on the RRC configuration. Other restrictions can also be considered.

Proposal 19: To guarantee mobility performance, one potential improvement on dynamic indication by DCI is configuring gaps to be skipped during a time period by RRC in advance and gaps indicated by DCI should be subset of gaps configured by RRC.

For Type-1 MG and Type-2 MG, per-FR gaps may be considered if a UE supports independent measurement gap configurations for FR1 and FR2. In this case, it is not clear which gap(s) are indicated by DCI, especially when the two gaps are overlapped in some occasions as depicted in Fig 6.

Fig 6. Per-FR Configuration of measurement gaps

Proposal 20: It needs to be clarified that when per-FR gaps are configured, the indication by DCI of the particular gap to be skipped is for which gap pattern(s).

## 4.2 Time offset

According to RAN1 LS [4], minimum time offset(s) X between indication to skip and skipped measurement occasion is up to RAN4 to discuss and decide on particular value(s).

In Rel-17, RAN4 specified requirements for pre-configured measurement gap where additional margin (time offset) is needed for gap activation/deactivation. If the gap activation is upon DCI-based BWP switch, the requirements are as follows.

8.19.2 Pre-configured measurement gap activation/deactivation upon DCI/timer-based BWP switch 8.19.2.1 Activation/deactivation upon DCI/timer-based BWP switch delay on a single CC The requirements in this clause only apply to the case that the DCI/timer-based BWP switch is performed on a single CC with more than one BWP configurations configured on the CC. When BWP switch occurs, which results in status change of pre-configured measurement gap according to clause 9.1.7, UE shall be able to finish pre-configured activation or deactivation within 5 ms after the completion of the active BWP switch. The active BWP switch delay for single CC is defined in clause 8.6.2. Activation/deactivation of Pre-MG takes effect from the first complete MG occasion after the activation and deactivation delay. If the end of activation/deactivation of Pre-MG is within a gap occasion, the Pre-MG status shall not be changed immediately. Instead, the Pre-MG status shall be changed prior to the next gap occasion.

The overall activation delay is depicted in Fig 7. According to the requirements, only the gaps after BWP switch delay + 5ms from starting of slot n will take effect. For example, the gap occasion filled with pattern cannot be used for measurement since the UE processing has not been finished. The 5ms is for UE processing. It is noted the time duration for DCI and DCI decoding is included in BWP switch delay.

Fig 7. Per-FR Configuration of measurement gaps

If network indicates a gap to be skipped by DCI, the overall timeline is depicted in Fig 8. If UE processing time is assumed the same for gap activation/deactivation of pre-configured measurement gap, i.e., 5ms, the overall time offset should include time for DCI and DCI decoding additionally. Thus, the gap filled with pattern cannot be skipped. Only the gap filled by blue can be skipped.

Fig 8. Per-FR Configuration of measurement gaps

The time duration for DCI and DCI decoding would be small. In our view, 5ms UE processing time would be enough to include this time duration. It is proposed the overall time offset between indication and gap occasion to be skipped is 5ms. It means if a DCI indicates skipping a gap in slot n, only the gap after 5ms from starting of slot n can be actually skipped.

Proposal 21: Minimum time offset(s) X between indication to skip and skipped measurement occasion is 5ms.

# Summary

In this contribution, we provided views on RRM requirements impact for XR in Rel-19 and on questions raised in the two RAN1 LSs. Following proposals and observations are present.

Proposal 1: Type 1 MG and Type 2 MG, including per-UE gap and per-FR gap, and pre-configured MG is considered in XR WI in Rel-19, where Type 2 MG needs further clarification.

Proposal 2: Skipping SMTC occasions for L3 measurements outside gaps are deprioritized.

Proposal 3: RAN4 does not consider enabling data transmission/reception on L1-RS symbols that need to be used for L1 measurements if there is scheduling restriction.

Proposal 4: RAN4 to discuss and decide whether enabling Rx/Tx in gaps/restrictions can be supported by any UE.

Proposal 5: RAN4 to study whether and how to define new core requirements when some of gaps that need to be used for measurements are enabled for data transmission/reception after solution for enabling Rx/Tx in gaps is clear.

Proposal 6: When only those gaps not used for measurement based on UE assistance information / UE capability are scheduled for data transmission/reception, the UE should meet existing requirements.

Proposal 7: RAN4 to study possible restrictions on dynamic indication by DCI, if it is confirmed, to achieve good trade-off between data deliver delay and mobility performance.

Proposal 8: If supported, RAN4 to study whether and how to define new core requirements when some of SMTC occasions that need to be used for measurements outside gaps are enabled for data transmission/reception when scheduling restrictions exists.

Proposal 9: UE should provide information about gaps not used for measurement for enabling transmission/reception in these gaps.

Proposal 10: RAN4 to discuss the information about gaps not used for measurement are reported as UE capability or UE assistance information.

Proposal 11: NW may indicate the percentage of measurement gaps not used for measurement for UE to provide information about gaps not used for measurement.

Proposal 12: If supported, UE should provide information about SMTC occasions not used for L3 measurement for enabling transmission/reception on the SMTC occasions.

Proposal 13: If supported, RAN4 to discuss the information about SMTC occasions not used for measurement are reported as UE capability or UE assistance information.

Proposal 14: If supported, NW may indicate the percentage about SMTC occasions not used for L3 measurement outside gaps for UE to provide information about SMTC occasions not used for measurement.

Proposal 15: If supported, the UE assistance information about SMTC occasions not used for measurement should be provided on each CC/MO.

Proposal 16: The patterns of gaps/SMTC occasions not used for measurement within a time period should be provided as UE assistance information.

Proposal 17: UE assistance information/UE capability regarding gaps/SMTC occasions (if supported) not used for measurement is reported in response to RRCReconfiguration.

Proposal 18: RAN4 to further study how UE assistance information/UE capability is reported when DRX is configured.

Proposal 19: To guarantee mobility performance, one potential improvement on dynamic indication by DCI is configuring gaps to be skipped during a time period by RRC in advance and gaps indicated by DCI should be subset of gaps configured by RRC.

Proposal 20: It needs to be clarified that when per-FR gaps are configured, the indication by DCI of the particular gap to be skipped is for which gap pattern(s).

Proposal 21: Minimum time offset(s) X between indication to skip and skipped measurement occasion is 5ms.

Observation 1: Scheduling restrictions are different in terms of conditions and restricted symbols for L3 measurements within gaps, L3 measurements outside gaps and L1 measurements.

Observation 2: Depending on measurement configuration, e.g., gap, SMTC, MO etc., some of the gaps may not be used for measurements while measurement requirements being satisfied.

Observation 3: It is up to UE implementation which gaps are not used for measurements under current measurement configuration.

Observation 4: Significant gain on throughput is observed if UE can provide information about unused gaps for measurement without impact on mobility performance.

Observation 5: Depending on measurement configuration, e.g., SMTC, MO etc., some of the SMTC occasions may not be used for measurements while measurement requirements being satisfied.

Observation 6: It is up to UE implementation which SMTC occasions are not used for measurements under current measurement configuration.

Observation 7: Channel condition, e.g., RSRP, does not provide information about which SMTC occasions are not used for measurements. Moreover, additional information about gaps/SMTCs not used for measurement is still needed.

Observation 8: UE mobility information does not help how gaps/SMTC occasions are used for measurement unless relaxed L3 measurement requirements are specified.

- References

- RP-240791	Revised WID on XR (eXtended Reality) for NR Phase 3, Nokia

- R4-2414043	WF on RRM requirements for XR_Ph3, Nokia

- R1-2405736	LS on UE assistance information, RAN1

- R1-2407561	LS on gaps/restrictions that are caused by RRM measurements, RAN1

- Appendix: Draft reply LS on UE assistance information

3GPP TSG RAN WG4 Meeting #112bis	R4-24x

Hefei, Anhui, China, 14th – 18th October, 2024

Title:	Reply LS on UE assistance information

Response to:	R1-2405736 (R4-2411007)

Release:	Rel-19

Work Item:	NR_XR_Ph3-Core

Source:	RAN4

To:	RAN1

Cc:	RAN2

Contact Person:

Name:	Qian Yang

E-mail Address: qian9.yang@vivo.com

1. Overall Description:

RAN4 thanks RAN1 for the LS on UE assistance information.

RAN4 discussed the UE assistance information for reporting not used measurement gas/SMTC occasions to network. Following conclusions are made.

From UE measurements and mobility performance perspective, RAN4 agreed that it is beneficial if UE reports [UE assistance information / UE capability] about not used measurement gaps/SMTC occasions to network and network indicates skipping gaps/SMTC occasions based on the information.

In order for NW and UE to have same understanding on which gaps should be not used for measurement, the patterns of gaps/SMTC occasions not used for measurement within a time period should be provided as [UE assistance information / UE capability].

RAN4 will further study the details and additional conditions of [UE assistance information / UE capability]. RAN1 will be informed once there are conclusions.

2. Actions:

To: RAN WG1

ACTION: 	RAN4 kindly asks RAN1 to take the above RAN4 agreements into account in the future work.

3. Reference:

- R1-2405736	LS on UE assistance information, RAN1

4. Date of Next TSG RAN WG4 Meetings:

TSG RAN WG4 Meeting #113	 	   November 18 – November 22, 2024			Orlando, US

- Appendix: Draft reply LS on gaps/restrictions

3GPP TSG RAN WG4 Meeting #112	R4-24x

Hefei, Anhui, China, 14th – 18th October, 2024

Title:	Reply LS on gaps/restrictions that are caused by RRM measurements

Response to:	R1-2407561

Release:	Rel-19

Work Item:	NR_XR_Ph3-Core

Source:	RAN4

To:	RAN1

Cc:	RAN2

Contact Person:

Name:	Qian Yang

E-mail Address: qian9.yang@vivo.com

1. Overall Description:

RAN4 thanks RAN1 for the LS on gaps/restrictions that are caused by RRM measurements.

RAN4 discussed the dynamic indication solution to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements. Following conclusions are made.

It is feasible to indicate skipping a particular gap by DCI. However, it may cause serious mobility performance degradation depending on network indication. RAN4 is studying solutions to achieve good trade-off between data delivery delay and mobility performance.

When UE is configured with two per-FR gaps, it is not clear whether all the gaps or which one are to be skipped if the two gaps are colliding on one occasion.

RAN4 agrees that the minimum time offset(s) X between indication to skip and skipped measurement occasion is 5ms.

2. Actions:

To: RAN WG1

ACTION: 	RAN4 kindly asks RAN1 to take the above RAN4 agreements into account in the future work.

3. Reference:

- R1-2407561	LS on gaps/restrictions that are caused by RRM measurements, RAN1

4. Date of Next TSG RAN WG4 Meetings:

TSG RAN WG4 Meeting #113	 	   November 18 – November 22, 2024			Orlando, US