3GPP TSG-RAN WG4 Meeting #112-bis                                        R4-2415204

Hefei, China, Oct 14th – 18th, 2024

Source: 	Huawei, HiSilicon

Title: 	Further consideration on BS RF for Rel-19 LP-WUS

Agenda Item:	6.23.3

Document for:	Approval

Introduction

BS RF requirements have been further discussed for LP-WUS and some aspects to be further analysed are captured in WF [1].

This contribution provides further analysis for the issues identified in the WF.

Discussion

Dynamic range/power boosting

The most outstanding issue for BS RF part is the requirement of dynamic range/power boosting for LP-WUS. Two options are listed in the WF:

Option 1: Introduce EPRE ratio to define LP-WUS power boosting, since it has the merit that power boosting does not vary with different CBWs.

Alt 1: The LP-WUS power boosting is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration and the average power NR REs (the NR carrier excluding the LP-WUS REs).

Alt 2: The LP-WUS RB power dynamic range (or LP-WUS power boosting) is the difference between the average power of an LP-WUS RE (averaged power per RE based on all LP-WUS REs within a NR transmission bandwidth configuration) and the average power of a non-LP-WUS RE (averaged power from all REs from the NR carrier excluding the LP-WUS REs).

Option 2: Adopt following definition rather than introduce EPRE ratio.

The LP-WUS RB power dynamic range (or LP-WUS power boosting) is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration) and the average power over all REs (from both LP-WUS and the NR carrier containing the LP-WUS REs).

As discussed in [2] that power boosting value in traditional definition would vary with different CBWs, though theoretically these two definitions can be converted to each, the EPRE based definition is simpler which is concept convenient and friendly to engineers. Thus, we prefer to consider dynamic range requirement for LP-WUS based on option 1.

Regarding the two alternatives, essentially, they are the same, while Alt 2 gives more detailed information. Either is fine with us, though Alt 2 is preferrable.

Proposal 1: EPRE ratio has the merit that power boosting declaration for LP-WUS does not vary with different CBWs, which can be considered as basis for definition of LP-WUS dynamic range/power boosting.

Once the option is determined, we could consider how to define the LP-WUS dynamic range/power boosting with appropriate wording. The following definition is proposed based on EPRE ratio:

  - The LP-WUS RB power dynamic range (or LP-WUS power boosting) is the difference between the average power of an LP-WUS RE (averaged power per RE based on all LP-WUS REs within a NR transmission bandwidth configuration) and the average power of a non-LP-WUS RE (averaged power from all REs from the NR carrier excluding the LP-WUS REs).

With settlement of the definition, naturally the other issue should be resolved as well. According to contributions in previous meetings, obviously companies identified that power boosting of LP-WUS will reduce the available output power for NR which share the same carrier with WUS, especially for lower CBWs. Several options are listed in the WF as follows to consider whether to have limitation for the CBWs to consider power boosting of LP-WUS.

- Option 1: Preclusion on small CBW respective to different power boosting level and/or power degradation on NR signal can be considered for LP-WUS power boosting.

- Option 1-1: Also introduce the maximum allowed power degradation as one of the core requirements for LP-WUS power boosting.

- Option 1-2: Operator inputs on the tolerable power degradation can be considered for the determination on the CBW to be precluded.

- Option 1-3: other option does not excluded.

- Option 2: No need to preclude any CBW, only if the LP-WUS power boosting can be defined as dynamic range where the level is up to manufacturer declaration and 0dB as minimum core requirement.

Based on our previous analysis and inputs from other companies, obviously it has less benefit to support power boosting for low channel BWs as the impact to NR signal is too large, which is not the purpose of boosting power of LP-WUS in sacrifice of coverage of NR.

Proposal 2: It is proposed to limit power boosting of LP-WUS, if supported, to larger channel BWs which has less impact on coverage of NR signal. Specific CBW can be determined with inputs from operators on the tolerable degradation for NR power in terms of single RB or EPRE.

The other issue which is yet determined is whether there is a limit for power boosting level. Though the specific level is up to manufacturer’s declaration, as an RF requirement, it should be reflected in the core specification. Similar to NB-IoT, a minimum boosting level needs to be considered. However, since 5MHz and 11RB are utilized by LP-WUS, it would have larger impact to NR available power, therefore, we propose to consider max 6dB power boosting for LP-WUS.

Proposal 3: It is proposed to specify 3dB as minimum level for LP-WUS power boosting but with 6dB as upper bound given the larger impact on NR available transmission power.

Applicable BS type

On applicable of BS type, the following options have been discussed in last meeting.

- FFS whether all available BS types can be considered for LP-WUS due to the support of LP-WUS is optional and the power boosting level could be up to manufacturer declaration.

- FFS whether only conductive based BS RF requirements should be defined for LP-WUS:

It is noted that the BS type from implementation perspective is also related to the operating bands. Usually, RRU is considered for less than 1GHz band, massive MIMO BS type is considered for above 2.5GHz bands, e.g., n41, n78, n79. While FDD 8T is also popular nowadays for the BS implementation. For different BS types, the appliable requirements could also be different, i.e. requirements type 1-C, 1-H and 1-O for FR1.

The definitions of BS type 1-C, 1-H and 1-O in the BS spec are cited as below:

BS type 1-C:	NR base station operating at FR1 with requirements set consisting only of conducted requirements defined at individual antenna connectors

BS type 1-H:	NR base station operating at FR1 with a requirement set consisting of conducted requirements defined at individual TAB connectors and OTA requirements defined at RIB

BS type 1-O:	NR base station operating at FR1 with a requirement set consisting only of OTA requirements defined at the RIB

To our understanding, these types are more like the applicable requirements to a certain BS as the same BS can support both type 1-C and type 1-H requirements, which is up to the manufacturer’s declaration.

Since the group has already reached consensus on the example bands for evaluation of LP-WUR requirements, in other words, the example bands could be considered as starting point of inputs from operators for the deployment consideration. We think that the BS type should consider take the example bands into account. However, due to complexity of the requirements for 1-O, we see no necessity to define 1-O requirements for LP-WUS. Only 1-C and 1-H could be considered.

Proposal 4: Only conductive based BS type 1-C and 1-H requirements are considered for LP-WUS.

Other requirements other than dynamic range

Besides the dynamic range requirement, other requirements have not been discussed in the SI and the first meeting of Rel-19 for LP-WUS. It is understandable that the introduction of LP-WUS should have minimal impact on BS implementation, as legacy BS must also accommodate this feature.

During the discussion, two alternatives were considered for requirements beyond dynamic range:

- Alt 1: All Tx requirements should be specified for LP-WUS, but FFS on the requirements to be tested.

- Alt 2: Requirements relevant to regulation and transmitter signal quality should be defined for LP-WUS.

We think that not all transmitter requirements need to be defined for LP-WUS. Some requirements, such as SEM and spurious emissions, are regulatory, while others pertain to signal quality, ensuring that the signal can be properly demodulated (e.g., EVM). Since LP-WUS is embedded in the NR signal and transmitted together, we think it is unnecessary to define all Tx requirements for LP-WUS. For unwanted emissions, ACLR is intended for coexistence, but specifically for LP-WUS, we believe that SEM and spurious emissions would suffice for the combined transmission of both LP-WUS and NR signals.

Regarding transmitted signal quality, due to the inherent characteristics of low-order modulation in LP-WUS OOK signals, EVM metrics are not expected to be a bottleneck. Furthermore, frequency error can be ensured through testing with NR signals only. However, preliminary simulation results indicate that the embedded LP-WUS signal may affect the transmitted quality of the NR signal. The combined signals exhibit a slightly increased PAPR compared to NR signals alone, which could cause some performance degradation for NR, especially with higher modulation orders.

Two options are considered for the transmitter signal quality requirement:

- Option 1: Legacy transmitted signal quality requirements apply, no impact when BS transmitting LP-WUS signal.

- Option 2: The OOK-based EVM should be defined for LP-WUS. FFS on how to specify this requirement with consideration of RAN1 waveform design.

The choice of which option to consider for the LP-WUS EVM requirement also depends on the testing methodology. Therefore, we think that further study is necessary, especially for Option 2.

Proposal 5: It is proposed to first determine whether a full set or simplified set of BS Tx requirements should be specified for LP-WUS.

Proposal 6: Unwanted emissions requirements of SEM and spurious emissions should be considered for transmitted signal with LP-WUS and NR in the same carrier.

Proposal 7: Transmitted signal quality requirements should be defined for LP-WUS, FFS how to define and test the requirement.

Conclusion

This contribution provides further consideration for BS RF of LP-WUS. With above analysis, we have the following proposals:

Proposal 1: EPRE ratio has the merit that power boosting declaration for LP-WUS does not vary with different CBWs, which can be considered as basis for definition of LP-WUS dynamic range/power boosting. The proposed definition could be:

The LP-WUS RB power dynamic range (or LP-WUS power boosting) is the difference between the average power of an LP-WUS RE (averaged power per RE based on all LP-WUS REs within a NR transmission bandwidth configuration) and the average power of a non-LP-WUS RE (averaged power from all REs from the NR carrier excluding the LP-WUS REs).

Proposal 2: It is proposed to limit power boosting of LP-WUS, if supported, to larger channel BWs which has less impact on coverage of NR signal. Specific CBW can be determined with inputs from operators on the tolerable degradation for NR power in terms of single RB or EPRE.

Proposal 3: It is proposed to specify 3dB as minimum level for LP-WUS power boosting but with 6dB as upper bound given the larger impact on NR available transmission power.

Proposal 4: Only conductive based BS type 1-C and 1-H requirements are considered for LP-WUS.

Proposal 5: It is proposed to first determine whether a full set or simplified set of BS Tx requirements should be specified for LP-WUS.

Proposal 6: Unwanted emissions requirements of SEM and spurious emissions should be considered for transmitted signal with LP-WUS and NR in the same carrier.

Proposal 7: Transmitted signal quality requirements should be defined for LP-WUS, FFS how to define and test the requirement.

- References

[1] R4-2413512, Way Forward for [112][309] NR_LPWUS, Huawei, HiSilicon