3GPP TSG-RAN WG4 Meeting # 112-bis	R4- 2415265

# Hefei, China, 14th – 18th October, 2024

Source: 	China Telecom

Title: 	Views on Power domain enhancements for single carrier

Agenda Item:	6.1.1.2.1	Power domain enhancements for single carrier

Document for:	Discussion

Introduction

In last RAN4 meeting, for scenario 2 of single carrier, the mainly unsettled discussion was captured in WF[1].

So in this contribution, we provide our views on it.

Discussion

ACLR/SEM and OOBE requirements relaxation

- Issue 1-2-5: Boundary to apply ACLR and SEM - Proposals - Proposal 1: ACLR and SEM should be applicable from the edge of extended UE CBW instead of the BS CBW. (Samsung, Sony, ZTE, vivo, LGE) - Proposal 2: ACLR, SEM and spurious emissions would be defined as of today but based on BS channel bandwidth. (Nokia, Ericsson) - WF - FFS in next meeting - Issue 1-2-7: Which CBW is utilized as the basis for the integral region of OOBE - Proposals - Option 1: The integral region and the boundary of OOBE should be based on UE CBW. (Samsung, vivo, ZTE, China Telecom) - Option 2: The integral region and boundary of OOBE is based on extended UE CBW. (Sony) - WF - FFS in next meeting

For boundary of ACLR/SEM and the integral region of OOBE, our point is unchanged. As a more analytical approach, ACLR/SEM should start from extended UE CBW and integral region should base on UE CBW.

Proposal1: The start point of ΔfOOB, also the applicable region of ACLR and SEM, should be extended UE CBW edge.

Proposal2: Integral region of OOBE should base on UE CBW as a start point.

Issue 1-2-4: IBE usage between UE CBW and extended UE CBW

- Issue 1-2-4: Whether and where to use IBE in the larger BS CBW - WF - Focus on how to extend IBE for usage between edges of UE CBW and extended UE CBW

Full RB allocation, when without CBW extension, means LCRB = NRB. So the IBE between UE CBW and extended UE CBW actually is the IBE in transmission bandwidth configuration NRB_new where UE transmission bandwidth is NRB.

For IQ Image and Carrier leakage, the in-band emissions limit depends on the carrier leakage frequency which is irrelevant to NRB_new. The UE CBW extension can only influence the general part of in-band-emissions.

Observation1: The UE CBW extension can only influence the general part of in-band-emissions.

And the NRB_new can influence the general parameter formula in TS 38.101-1, so NW and UE should have same understand of NRB_new. The information exchange between NW and UE should contain the required offset for UE to convert outer to inner. Besides, it also should have a UE capability, that indicates whether adjacent channel has enough space to extend, for NW to make configuration.

Proposal3: The information exchange between NW and UE should contain the required offset for UE to convert outer to inner.

Proposal4: It also should have a UE capability that indicates whether adjacent channel has enough space to extend which is for NW to make configuration.

Ratio size of extended CBW

- Issue 1-2-8: Ratio size of extended CBW between UE CBW and larger BS channel BW - Proposals - Proposal 1: (Apple) - If the goal is to convert all outer RB allocations into inner RB allocations, then the minimum BS channel needs to be at least twice the size of the UE channel. - If the sole goal is to obtain output power improvements, then it is possible to have a more convenient ratio between UE and BS channel such as two-third. - Proposal 2: (Qualcomm) - If minimum excess BW equal to half of the original UE BW can be placed on both sides of the original UE BW this extended UE BW can be treated as the new UE BW and all performance metrics can be based on this new BW. - If the minimum extended BW on both sides of original UE BW is less than half the original UE BW then the original UE BW should not be extended, and all performance metrics should be based on the original UE BW. - Proposal 3: The required extended bandwidth of the both sides could be set to 2/5 UE CBW as a starting point. (vivo) - Proposal 4: Consider extended UE CBW approach to improve MPR starting from 1.5*CBW (OPPO) - Proposal 5: (Huawei) - For the shifted start point of ΔfOOB, 1/2 UE CBW can be the default value for UE indication. - Consider to introduce shorter shifted frequency e.g. 1/3, 1/4 UE CBW in conjunction with the requirement that will be introduced to the shifted frequency. - For instance, the modified general part of IBE requirements which is further relaxed by replacing the EVM corresponding to the scheduling modulation order with the one for BPSK - WF - FFS with consideration of above proposals in next meeting

From our point, Rel-19 MPR reduction is based on converting outer RB allocation to inner RB allocation so we should make sure the offset at least half of original UE CBW at each side. Consequently, the minimum BS CBW should at least be two times of original UE CBW at the very beginning. We can continuously discuss the relaxation mechanism for UE cannot shift at least half of original UE CBW at each side.

Proposal5: The minimum BS CBW should at least be two times of original UE CBW at the very beginning.

Conclusion

Observation1: The UE CBW extension can only influence the general part of in-band-emissions.

Proposal1: The start point of ΔfOOB, also the applicable region of ACLR and SEM, should be extended UE CBW edge.

Proposal2: Integral region of OOBE should base on UE CBW as a start point.

Proposal3: The information exchange between NW and UE should contain the required offset for UE to convert outer to inner.

Proposal4: It also should have a UE capability that indicates whether adjacent channel has enough space to extend which is for NW to make configuration.

Proposal5: The minimum BS CBW should at least be two times of original UE CBW at the very beginning.

References

- R4-2414441, WF on power domain enhancement, RAN4#112