3GPP TSG RAN WG4 Meeting #112bis	R4-2415281

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	6.2.4.2

Source: 	Fujitsu

Title: 	Discussion on BS antenna array size for 14800 to 15350 MHz frequency range

Document for:	Approval

# 1. Introduction

In RAN4#112 meeting, the discussion for IMT parameters was conducted and LS for 7125 to 8400 frequency range was sent [1]. In this contribution, we share our thought on BS antenna array size for 14800 to 15350 MHz in the urban macro scenario.

# 2. Discussion

Firstly, we will summarize the relationship between the BS output power and the pathloss model for the coverage perspective. In May meeting, we already agreed on the WF that the pathloss model in TR 38.901 is used for co-existence study [2]. The following is the excerpt of pathloss model in TR 38.901.

Table 1: Pathloss model in TR 38.901 (Table 7.4.1-1)

| Scenario | LOS/NLOS | Pathloss [dB], fc is in GHz and d is in meters, see note 6 | Shadow fading std [dB] | Applicability range, antenna height default values |
| --- | --- | --- | --- | --- |
| UMa | LOS | PLUMa-LOS=PL110m≤d2D≤dBP'PL2dBP'≤d2D≤5km, see note 1 PL1=28.0+22log10(d3D)+20log10(fc) PL2=28.0+40log10(d3D)+20log10(fc) -9log10((dBP')2+(hBS-hUT)2) | σSF=4 | 1.5m≤hUT≤22.5m hBS=25m |
|  | NLOS | PLUMa-NLOS=max(PLUMa-LOS,PLUMa-NLOS') for 10m≤d2D≤5km | σSF=6 | 1.5m≤hUT≤22.5m hBS=25m Explanations: see note 3 |
|  |  | Optional PL=32.4+20log10fc+30log10d3D | σSF=7.8 |  |

From the Table 1, it is apparent that the frequency-dependent term is only 20 log10 (fc) where fc is the carrier frequency in GHz. In this study item, there are three frequency ranges, 4.4 – 4.8 GHz, 7.125 – 8.4 GHz and 14.8 – 15.35 GHz. When we compare 20 log10 (fc) with the upper frequency of each frequency range, the frequency dependent pathloss values are as follows.

Table 2: Frequency-dependent pathloss values in TR 38.901

| Frequency range [GHz] | Upper frequency [GHz] | Frequency dependant pathloss value 20 log10 (fc) at upper frequency [dB] |
| --- | --- | --- |
| 4.4 – 4.8 | 4.8 | 13.6 |
| 7.125 - 8.4 | 8.4 | 18.5 |
| 14.8 - 15.35 | 15.35 | 23.7 |

From the viewpoint of achieving the same coverage in each frequency range, the above analysis indicates that the base station output power/sector (e.i.r.p.) for 14.8 – 15.35 GHz needs 10.1 dB and 5.2 dB higher than that for 4.4 – 4.8 GHz and 7.125 – 8.4 GHz, respectively.

Observation 1: For achieving the same coverage in each frequency range, the base station output power/sector (e.i.r.p) for 14.8 – 15.35 GHz needs 10.1 dB and 5.2 dB higher than that for 4.4 – 4.8 GHz and 7.125 – 8.4 GHz, respectively.

The LSs already sent to ITU-R WP5D capture the maximum base station output power/sector (e.i.r.p.) [1][3]. The values for 4.4 – 4.8 GHz and 7.125 – 8.4 GHz are 72.2 dBm and 78.3 dBm, respectively. If we analyze the value of maximum base station output power/sector (e.i.r.p.) for 14.8 – 15.35 GHz based on the above result for 4.4 – 4.8 GHz, it is 82.3 dBm. The value 83.5 dBm can be calculated based on the above result for 7.125 – 8.4 GHz. Considering them, our view is as follows.

Proposal 1: The target value for maximum base station output power/sector (e.i.r.p.) for 14.8 – 15.35 GHz should be within 82.3 - 83.5 dBm.

Next, we will analyze the BS antenna array size for 14.8 – 15.35 GHz. Like other frequency range, 6.4 dBi element gain, 2.0 dB array ohmic loss and 46 dBm BS output power (43 dBm per polarization) are considered in the analysis. If the target value for maximum base station output power/sector (e.i.r.p.) is within 82.3 – 83.5 dBm, the BS antenna array gain can be calculated within 31.9 – 33.1 dB as follows.

If maximum base station output power/sector (e.i.r.p.) is 82.3 dBm:

BS antenna array gain = 82.3 – 6.4 – 46.0 + 2.0 = 31.9 dB

If maximum base station output power/sector (e.i.r.p.) is 83.5 dBm:

BS antenna array gain = 83.5 – 6.4 – 46.0 + 2.0 = 33.1 dB

From the above, BS antenna array size of 2048 (BS antenna array gain: 33.1 dB) is suitable for achieving the same coverage of other frequency ranges in this study item.

Observation 2: For achieving the same coverage in each frequency range, BS antenna array size of 2048 is suitable.

Additionally, as a BS vendor, we would not like to expect the excessive number of antenna array size from the feasibility perspective. Considering it and the above observations, the following is our proposal.

Proposal 2: RAN4 sets BS antenna array size for 15 GHz to 2048. If the additional margin needs to be considered, the BS antenna array size should be discussed based on the value of margin.

# 3. Conclusions

In this contribution, we shared our thoughts on BS antenna array size for urban macro scenario in 14800 to 15350 MHz frequency range as follows.

Observation 1: For achieving the same coverage in each frequency range, the base station output power/sector (e.i.r.p) for 14.8 – 15.35 GHz needs 10.1 dB and 5.2 dB higher than that for 4.4 – 4.8 GHz and 7.125 – 8.4 GHz, respectively.

Proposal 1: The target value for maximum base station output power/sector (e.i.r.p.) for 14.8 – 15.35 GHz should be within 82.3 - 83.5 dBm.

Observation 2: For achieving the same coverage in each frequency range, BS antenna array size of 2048 is suitable.

Proposal 2: RAN4 sets BS antenna array size for 15GHz to 2048. If the additional margin needs to be considered, the BS antenna array size should be discussed based on the value of margin.

# 4. Reference

- R4-2414449, LS Reply on Parameters for 7125 to 8400 MHz of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27, Ericsson

- R4-2410741, WF on IMT parameters study, Ericsson

- R4-2410576, LS on Parameters for 4400 to 4800 MHz of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27, Ericsson