3GPP TSG-RAN WG4 Meeting #112bis	R4-2415282

Hefei, Anhui, China, 14th – 18th October 2024

Source:	Nokia

Title:	Discussion on Spatial Channel Model for Demodulation Performance Requirements

Agenda item:	6.14.2

Document for:	Discussion

Introduction

Following the agreement of the study item on Spatial Channel Model (SCM) for demodulation performance requirements in RAN#104 [1], the first WG4 meeting concluded successfully in RAN4#112 with an agreed WF [2].

We want to highlight immediately that the WF introduces a distinction between results for alignment and for comparison:

- Alignment steps and results are intended to align the SCM candidate implementations between different contributors by aligning performance of configurations chosen for alignment.

- Comparison steps and results are intended to compare the SCM candidates and legacy channel models in comparison minimum performance requirements and configurations, to evaluate the need for SCMs and compare the SCM candidates.

Neither of the alignment and comparison configurations are intended to directly serve as minimum performance requirements in the future, albeit that they could serve as a starting point.

Nokia’s prior contributions cover the fundamentals of 38.827 CDL and legacy TDL channel models:

- [3]: Exhaustive analysis of legacy channel models and comparison to 38.827 CDL in terms of modelling expected MIMO effects.

- [4]: Implementation description of the 38.827 CDL model with conducted modification and implementation notes (“pitfalls”).

- [5]: Initial observations on the need for SCM in 2CW use cases.

This contribution addresses the open issues captured in the last WF [2], in particular the comparison setups and steps, and it fills in gaps around PMI and MU comparisons from [3].

We want to further highlight our companion simulation results contribution [6], as well as, two MNO driven contributions on measurement results [7] and on TDL and CDL comparison with respect to CSI feedback schemes [8].	
Please refer to our companion contribution [6] for an up-to-date TP-style description of the 827 CDL channel model, and implementation help (also refer to prior contribution for further pointers [4]).

Updated legacy issues and CDL comparisons

In this section we extend the results and observations presented in [3]. The observations will prove to be useful to advance open issues and add new topics to the study item.

SU TPUT

There are two issues that have previously been highlighted when using legacy TDL models in SU TPUT performance requirements:

- 2 CWs requirements performing equally for both CWs with same MCS at a shared SNR operating point.

- Functional differences in SDM receiver implementations not resulting in performance differences.

We will not repeat the widely recognized issues with 2CW requirements performing equally for both CWs with same MCS at a shared SNR operating point, and we refer to [3]/[5] for further information.	
For the sensitivity issues, we refine our prior results in section 2.1.1.

We also highlight an outcome of our TDL analysis in [3] that we did not sufficiently promote earlier:

TDL (low) channels severely overestimate the high rank support, as channel condition is forced to be statistically perfect. TDL (non-low) channels severely underestimate the high rank support, as channel condition is forced to be severely deficient.

Sensitivity to receiver implementations

In [3] we compared the TPUT sensitivity of CDL, TDL, and TDL+antCorr with respect to receiver implementation differences, i.e., we looked at whether the legacy TPUT demod requirements are fit to detect issues with SDM receiver implementations. We looked at covariance matrix estimation implementations specifically, and thus, we tested two versions of SU MMSE (“IRC” in RAN4 demod parlance) for TPUT performance. “MMSE CovD” and “MMSE CovDI”, denote MMSE receivers with covariance matrix averaged between REs, and re-use of instantaneous value between REs, respectively.	
Our previous results for TDLC300-100 MedA were skewing unexpectedly, which we attribute to the extremely strong single direction common spatial preference of TDL-medA, combined with the high SNR, leading to the instantaneous value being a better predictor than the average. Additionally, the saturation of the curves was attributed to the inherently ill-conditioned channels created by the legacy antenna correlation matrices. We have further aligned our baseline architecture with RAN4 needs after RAN4#112, which led to us re-running the simulation campaign.	
Updated results (see Figure 1) confirm the prior observations. For legacy TDL channel models, there is insufficient TPUT performance difference to distinguish between different SCM receiver implementations. Thus, we observe:

For SU TPUT use cases, legacy TDL channel model-based demodulation performance requirements are neither fit to test receiver functionality, nor fit to set a stable minimum performance requirement. Functionally differing SDM receiver implementation show same performance under legacy TDL channel requirements, but SCMs with spatially typical properties (in this case 827 CDL) can meaningfully differentiate between implementations, even under random precoding.

| TDLC300-100 low | TDLC300-100 MedA (MCS 7) |
| --- | --- |
| 827 CDLC UMa ULA |  |

Figure 1: 4T4R, 1CW (MCS17) full rank, random precoding, Differing SDM receiver implementation, relative TPUT vs. pre-EQ SNR.

SU PMI

Our prior contributions did not approach the SU PMI use case with simulations. We will rectify this in the following.
There are at least two issues, which are discussed in the following, when using legacy TDL models in SU PMI requirements:

- eTypeII PMI selection algorithm functionality and performance may have never been reliably tested, as TypeI and eTypeII perform similarly and TypeI is a subset of eTypeII.

- The PMI selection (esp. TypeI) is trivial. Random selection performs mostly equally well for TDL (low) as any other. In TDL+antCorr the choice is always broadside (with co-pol factor choice being mostly random), where principal beam direction steering was arguably never implemented in many TEs and many simulation campaigns.

Concerning the issues with eTypeI vs. eTypeII, we also recommend [8] for additional simulation results and observations.

Rules of thumb for (Rel-15) eTypeI and (Rel-16) eTypeII PMI functionality:	
It is our basic understanding that TypeI PMI is generally useful to focus energy in a single direction (plus limited additional fixed delta directions), while eTypeII allows for choosing “L” channel adapted spatial basis functions and offers useable phase control intended for interference control between spatial layers, which is most useful in MU scenarios, but also helps in high rank SU cases. However, this high-resolution potential of eTypeII comes with the risk of performing worse than TypeI, when choosing slightly mis-adapted PMI for the channel.

SU PMI profiles (4Rx)

In Figure 2 we present the histograms of TypeI PMI choices (max TPUT criterion) in a 4T4R 4 layer configuration under different channel models. “Randomization of the principal beam direction” is not applied as it was arguably never implemented in many TEs and many legacy simulation campaigns (as was discussed during the Demod maintenance sessions in RAN4#112 [9]). Additionally, it has implications on the RMS delay spread correctness during the test (see discussion later in this chapter).

The results are as expected in [3] with additional observations (see also notes further down):

- TDL low

  - The capacity optimal PMI choice is seemingly random, reflecting the outwardly random spatial properties of TDL low.

  - This explains the observation that PMI requirements with TDL low have a gamma ratio (follow PMI tput vs. random tput) of about 1, i.e., the precoding direction does not matter even for low 10Hz Doppler configurations.

- TDL+antCorr

  - The capacity optimal PMI choice is randomizing between ULA broadsight (PMI 0) and its co-phasing variant (PMI 2), and the ULA symmetric variants (PMI 8+9).

- 827 CDL

  - The capacity optimal PMI choice is switching between the defined and stable strong directions of the channel model, with variation introduced by the small scall fading component.

Thus, we reach the observation of:

The capacity optimal TypeI PMI choice is random in TDL low, fixed to broadsight in TDL+antCorr, and following defined and stable strong directions in CDL. Thus, only SCM model-based requirements can provide evidence for correct algorithmic function and set reasonable minimum performance expectations.

We additionally expected for the TDL+antCorr PMI choice to be fluctuating widely due to the ill conditioned channel. We did not observe this, however, follow-up studies with a more problematic correlation model (e.g., medA) are planned to see whether this is only due to the “not very correlated” XP High 4 Rx model.

| TDLC300-100 low SNR=15dB => CW Throughput = 96.24% | TDLC300-100 XP high SNR=25dB => CW Throughput = 90.54% |
| --- | --- |
| 827 CDLC UMa ULA SNR=14dB => CW Throughput = 92.57% |  |

Figure 2: Histograms of PMI feedback choices under different channel models (1000 CSI reports).
4T4R, 1CW, MCS13, full rank, PMI chosen based on (non-precoded) CSI-RS, N1=2, N2=1, O1=4, O2=1.

Notes:

- PMI indexes are translating to PMI bit fields as follows:

  - PMI 0: i1_1 = 0, i2=0, (i1_2 = 0, i1_3=n/a).

  - PMI 1: i1_1 = 0, i2=1, (i1_2 = 0, i1_3=n/a).

  - PMI 2: i1_1 = 1, i2=0, (i1_2 = 0, i1_3=n/a).

  - etc.

  - PMI 15: i1_1 = 7, i2=1, (i1_2 = 0, i1_3=n/a).

  - I.e., even and odd indices only differ w.r.t. co-phasing factors, but have the same direction.

- TypeI PMIs exhibit a symmetry or repeating patterns around the 0-7 vs. 8-15 indices.

  - This is caused by the one beam (a) for layers 1 and 3, switching roles with the one beam (b, c, or d) for layers 2 and 4.	
Beams b, c, and d, have a fixed relation to beam a:

SU PMI profiles (8Rx)

The same observations as from the previous 4Rx sections apply, but the WF requested the PMI profiles of the SU 8Rx case under TDL+antCorr, which we show here.

CDLC 365ns UMa: 22 dB - TPUT = CW0 65.8%, CW1 86.32% TDLC 300ns ULA Low: 16 dB - TPUT = CW0, 91.69% CW1 88.69% TDLC 300ns XP High: 40 dB - TPUT = CW0 6.75%, CW1 47.75%

Figure 3: Histograms of PMI feedback choices under different channel models (500 CSI reports).
8T8R, 1CW, MCS13, full rank, PMI chosen based on (non-precoded) CSI-RS, N1=4, N2=1, O1=4, O2=1.

Note that TDL+antCorr has large SNR issues and can’t reach 90% TPUT here. As such we expect some randomness in the PMI choice, which is seen in the “second half” of the codebook. The best PMI choice remains broadside, PMI0 (PMI 0: i1_1 = 0, i2=0, (i1_2 = 0, i1_3=n/a).

PMI choice: eTypeI vs. eTypeII

In earlier contributions on Rel-16 eTypeII demodulation requirement introduction [10] it was observed that TypeI and eTypeII PMI feedback implementations have the same performance for the medium and high correlation TDLA30 channel models.	
These results have been replicated in [8] and extended to evaluate TDL legacy channel models (with and without antenna correlation), as well as, the CDL class of channel models. These results also match our earlier results [11].

It is observed that when using TDL channel models, be it with or without legacy RAN4 antenna correlation modelling, the performance of Type-I and eType-II CBs is nearly identical. This raises serious concerns on whether performance requirements based on such channel models can set a reasonable minimum performance level, both from an implementation correctness and from the perspective of absolute values for the performance requirements of different MIMO techniques. Similarly, the performance difference between precoding performance bounds and derived requirements are seen to be unreasonably large, again casting doubt on the quality of the requirements.

Furthermore, it is observed that when using CDL channel models (at least the 38.901 version), the comparison between Type-I and eType-II CBs and PMI selection algorithms is as intended by 3GPP, when designing the features. Indeed, eType-II has higher spatial resolution, therefore it can exploit several spatial components at the same time and mitigate efficiently spatial layer interference at the expense of potentially worse performance when using it in inappropriate circumstances and with suboptimal PMI selection algorithms.

Combining these observations with our earlier observations from [12] and [11], leads us to the combined outcome of:

In legacy TDL channel models TypeI and eTypeII PMI feedback performs nearly identical, raising serious concerns whether such performance requirements are reasonable from an implementation correctness pov and from the perspective of absolute values for performance requirements. Performance requirements based on CDL channels behave as expected.

As SU-MIMO throughput with eTypeII PMI in TDL channels is generally not significantly larger than that achievable with TypeI-SP PMI, if one tested an eTypeII PMI report with a SU-MIMO set-up, it would be possible for a defective DUT to pass testing. This could happen, for example, by erroneously reporting a Type I PMI content in Type II format. The defective DUT could pass the test if the accuracy provided is enough to show gain over a random PMI, but it would fail an MU-MIMO test because the crosstalk with a co-scheduled UE would degrade the throughput significantly.

We also repeat that neither the Rel-16 NR_eMIMO WI [13], nor the Rel-17 NR_feMIMO WI [14][15] were able to agree on a suitable TDL based test setup for the MU use case for which the newly introduced codebooks were designed (in addition to increase SU accuracy).

MU TPUT

There are at least two issues that have previously been highlighted when using legacy TDL models in MU TPUT performance requirements:

- Advanced receivers not no behaving as expected in terms of relative performance.

  - Requirement assumption decisions taken based on compared receiver relative performances not reasonable and may lead to unused new features in deployment.

- Only orthogonal precoder requirements in Rel-18 (for DCI index 1-5, i.e., non-R-ML cases) as non-orthogonal did not reach feasible SNR levels.

In the following we will detail the concerns and show the impact.

In general, we see it useful to distinguish two forms of MU TPUT and MU PMI requirement setups:

- Single cell MU (SCMU) / Intra cell MU / Serving cell only.

  - In such a setup, it may often be sufficient to have one target DUT and one (or more) virtual receivers that are emulated solely via precoder choices and multi-user precoder application at the transmitter. However, SCMU does not allow to set receiver performance in all practically relevant interference profiles.

- Multi cell MU (MCMU)/ Inter cell MU / serving and interference cells.

  - In such a setup, it is not sufficient to only generate a single channel towards the target DUT. It is necessary to create spatially consistent channels towards all receivers, to capture correct interference profiles (as well as levels) and their impact on choices made at the target UE.

In the initial results presented in the following, a SCMU setup is used MU TPUT requirements, this is not sufficient to gauge the impact of MU deployments on receiver performance. Though we recognize that there are currently different expectations for MU TPUT requirements in RAN4:

Different expectations for MU TPUT requirements

There are two different focal points, when talking about MU TPUT requirements (the third one is common). One may consider whether the presence of (SC and/or MC) MU interference

(a) should impact the receiver algorithm implementation, or

(b) should impact the receiver performance, or

[(c) should highlight previously unseen strong issues with legacy implementations.]

If there are any signs of (c) the common expectation is that requirements are required.

If one focuses solely on (a) to define minimum performance requirements, then one can argue that

- Interference from an interference cell gives rise to implementation extensions, as one might want to chancel interference from blind-detected DM-RS ports that may or may not be matching the serving DM-RS ports, and may or may not be synchronized.

- Interference from signals intended for a different receiver in the serving cell gives rise to implementation extensions, as one might want to chancel interference from potentially known DM-RS ports are not the serving DM-RS ports, and is synchronized.

If one focuses solely on (b) to define minimum performance requirements, then one can argue that the same points from (a) also result in performance differences. However, the transmitter multi-user precoding choice needs to be modelled, either practical schemes, or versions of random/orthogonal MU precoding.

Hence in this one would answer (a) positively and (b) positively with the caveat that truly typical performance also needs multi-user precoding modelling.

It is our understanding that demodulation and CSI minimum performance requirements should cover both (a) and (b). As such, requirements shall be defined for MU TPUT and the setup shall model typical MIMO effects, from both single cell and multi-cell MU pov.

It is our understanding that demod minimum performance requirements should be defined, if there is either impact on the receiver algorithm implementation, or if there is impact on the resulting performance, or if there are previously unseen strong issues with legacy implementations.

Advanced receivers

This section is covering the behaviour of differing advanced receivers in MU TPUT setups, under the assumption that SCMU configurations (and corresponding precoder based interference modelling) are sufficient to model interference profiles and patterns in the presented channel models. This assumption should be revisited in the future.

In Figure 4 we compare the demodulation performance of two advanced IRC receivers (R-ML and E-IRC) to baseline SU MMSE (“IRC” in RAN4 terminology). We use a configuration that is based on the Rel-18 advRx requirements (2+2 layers, MCS13+MC4 for target and co-scheduled UE). We note several interesting points:

- In legacy TDL,

  - E-IRC (MU MMSE) and IRC (SU MMSE)

    - Have significantly different performance, for non-orthogonal precoding/non-negligible inter-user interference.

    - SNR requirements, for non-orthogonal precoding/non-negligible inter-user interference, are greater than 20dB.

  - R-ML and (E-)IRC

    - Have a huge performance gap.

- In 827 CDL,

  - E-IRC (MU MMSE) and IRC (SU MMSE)

    - Perform about equally well.

    - All SNR requirements

  - R-ML and (E-)IRC

    - Have a medium performance gap.

It is our understanding that only the observed scaling in 827 CDL is aligned with practical deployment expectations:

- SU and MU MMSE perform equally well in presence of MU interference

  - When interference is received from spatially consistent directions that are distinct from signal directions, i.e., S and I are received and transmitted on different and detectable spatial supports, receivers that focus on signal directions are already implicitly attenuating other (i.e., interference) directions. 	
Co-scheduled receivers do not share the same spatial subspaces.

  - SU interference whitening covers all interference, no matter what source.

  - Note that this expectation holds true for DL SU vs. MU, where the DUT and the SS precode and receive with the intent to improve the layers of the target UE only. This behaviour indirectly strongly attenuates signals/interference on different spatial supports. 	
In UL SU vs. MU the receiver goal is different. The reiver wants to detect all layers from all transmitters, thus the detection channel is an aggregation of all transmitter channels (MU). Just focusing on a single transmitter (SU) is expected to perform significantly worse.

- R-ML and MMSE perform within a reasonable margin

  - MMSE receivers perform worse than R-ML, but the gap is within 5dB for high interference scenarios and less for low interference.

  - As the complexity of ML increases with co-scheduled modulation order (MO), the performance gap between R-ML and liner receivers is largest for low co-scheduled MOs.

Note that SU and MU MMSE in the RAN4 DMD baseline implementation are statistically equal (only) for Gaussian interference [18]. We show result with QPSK for the co-scheduled users, which is the farthest MO from Gaussian signaling, but the implicit spatial rejection is independent of this assumption, as is seen from SU and MU MMSE still performing equally well at this MCS.

For SCMU TPUT is it expected that SU and MU MMSE perform about equally well in typical co-scheduling scenarios, and the R-ML to linear receiver gap is within a reasonable margin.

If a SCM was used in Rel-18 advanced receivers instead of TDL, RAN4 would not have needed to spend a lot of time unnecessarily analyzing and debating SU vs. MU MMSE in DL.

| TDLC300-100 low | TDLC300-100 XP Med |
| --- | --- |
| 827 CDLC UMa ULA |  |

Figure 4: Differing advanced receivers (R-ML, E-IRC, IRC) in presence of co-scheduled UE.
4T4R, 1CW (MCS13+MCS4), rank 2+2, MCS4 interference, orthogonal and random MU precoding.

SNR levels

The observation that “only orthogonal precoder requirements in Rel-18 (for DCI index 1-5, i.e., non-R-ML cases) as non-orthogonal did not reach feasible SNR levels” is a direct consequence of the discussion from Rel-18.	
I.e., the combination of [16]

For Rank 1+1 with 2T2R […] Nokia: Given the compromise from UE vendors on the high interference level, we could compromise towards using orthogonal precoding. We would like to minute that the reason is to reduce the operating SNR of the test. Ericsson: We are also ok with orthogonal as a compromise after offline discussion. Chair: The common understanding is the operating SNR is expected to be lower with orthogonal precoding and this is the reason it is selected.

and [17]:

Test setting for when UE is indicated Modulation order (DCI index 1-5 is indicated) - Agreement: - For Rank 1+1 with 2T2R - Case2: Orthogonal precoding, TDLC300-100, ULA medium, MCS 13 (Table 1) for Target UE, QPSK for co-UE. full FDRA for the co-UE - For Rank 2+2 with 4T4R: - Case7: Orthogonal precoding, TDLA30-10, ULA Low, MCS 17 (Table 1) for Target UE, 16QAM for co-UE. full FDRA for the co-UE - For Rank 1+1 with 2T4R: - Orthogonal precoding, TDLC300-100, ULA medium, MCS 13 (Table 1) for Target UE, QPSK for co-UE. full FDRA for the co-UE […] Whether to tests UE not support BD-MO with R-ML with DCI index 6 is indicated - Agreement: - Do not introduce such test for UE not support BD-MO with R-ML.

The results presented in section 2.3.1 (Figure 4) show that 827 CDL results in significantly reduced SNR requirements for all advanced receivers and allows for non-orthogonal precoder requirements, even for non-R-ML requirements.

Using 827 CDL in MU TPUT scenarios significantly reduces SNR requirements for all advanced receivers and allows for non-orthogonal precoder requirements, even for non-R-ML requirements

MU PMI

There are at least two issues that have previously been highlighted when using legacy TDL models in MU PMI performance requirements:

- eTypeII PMI selection algorithm functionality and performance may have never been reliably tested, as TypeI and eTypeII perform similarly and TypeI is a subset of eTypeII.

- Gains of MU MIMO are also hidden behind the similar TypeI and eTypeII PMI feedback performance, leading to unreasonable baseline decisions.

To evaluate these impacts, MCMU configurations are needed (see SCMU/MCMU distinction from section 2.3), to be able to setup SCMs with spatial consistency between near and far receivers, which provides both typical interference levels and detectable spatial profiles of interference sources.	
It will be instructive to determine, whether the PMI feedback of a receiver (should) change in the presence of a receiver, that interferes with and contributes to measurements for PMI derivation.

We will provide further treatment of this use case, after finishing the modification of 827 CDL to MCMU setups.

Different expectations for MU PMI choice requirements

There are two different focal points, when talking about MU PMI requirements (the third one is common). One may consider whether the presence of (SC and/or MC) MU interference

(a) should impact the PMI choice algorithm implementation, or

(b) should impact the PMI choice itself and the resulting performance, or

[(c) should highlight previously unseen strong issues with legacy implementations.]

If there are any signs of (c) the common expectation is that requirements are required.

If one focuses solely on (a) to define minimum performance requirements, then one can argue that

- Interference coming from an interference cell cannot be influenced via PMI choice in the serving cell.

- Interference received from an interference cell can be influenced via PMI choices of very advanced algorithms that influence receive directions via serving cell PMI choice.

- Interference received from signals intended for a different receiver in the serving cell, can be influenced via PMI choices of very advanced algorithms that derive used precoders for co-scheduled UEs or influence receive directions via serving cell PMI choice.

Hence, in this case one would answer (a) negatively, if non advanced receivers are considered, and positively in advanced algorithms.

If one focuses solely on (b) to define minimum performance requirements, then one can argue that

- The PMI choice itself and the resulting performance is obviously impacted by both interference types, inter- and intra-cell causes, as well as considerations, if the interference DM-RS ports are known, need to be blind detected, are synchronized, or are not synchronized.

Hence, in this case one would answer (b) positively.

It is our understanding that demodulation and CSI minimum performance requirements should cover both (a) and (b). As such, requirements shall be defined for MU PMI and the setup shall model typical MIMO effects to achieve valid results for (b), i.e., from both single cell and multi-cell MU pov.

It is our understanding that CSI minimum performance requirements should be defined, if there is either impact on the PMI choice algorithm implementation, or if there is impact on the PMI choice itself and the resulting performance, or if there are previously unseen strong issues with legacy implementations.

Other use cases

In addition to the issues with legacy demod use cases and requirements using non-SCM discussed until here, we highlight a few more. Though, we will likely not have enough time to investigate those in detail during FS_NR_demod_SCM:

CQI requirements

In the current specifications, we find multiple CQI requirements with fading channels (TDL) that have a gamma ratio of around 1. The issue with these requirements is as previously highlighted for PMI requirements with gamma=1, and the same solutions using SCMs apply.

FDD reciprocity CB requirements (Rel-17 feTypeII-PS)

As of now (end of Rel-18), no CSI requirements for feTypeII-PS codebooks exist, as it was judged untestable using current test setups.	
Following the SCM study insights this is not surprising, as the FDD reciprocity intent of this CB relies on UL to DL spatial consistency to compress/predict the DL spatial support from UL measurements (the transmitter needs to estimate the same spatial support from UL signals, as the receiver estimated from DL signals). TDL neither offers such a UL/DL spatial consistency, nor does it offer a diverse and stable enough spatial support. SCMs on the other hand are expected to solve both these problems.

Doppler compression CB (typeII-doppler-r18)

In Rel-18 is has been exceedingly difficult to find configurations that all to define reasonable CSI requirements for this CB.	
Following the SCM study insights this is not surprising, as the Doppler compression/prediction relies on a physical meaning and process behind the Doppler changes to be able to predict it. TDL based models only contain statistical Jakes distribution max Doppler effects (speed based), which cannot be use for prediction. An SCM with velocity and physical environment Doppler evolution is expected to be much more amenable for requirements for this CB.

Channel condition

The legacy channel models cannot be used to evaluate the rank, or rather condition, of a channel with corresponding antenna assumptions. TDL low is statistically always perfectly conditioned, i.e., overestimated the channel condition. TDL+antCorr is rank deficient/ill-conditioned by (antenna correlation matrix) design, which underestimates the channel condition. (827) CDL is deriving channel condition based on geometrical considerations that are typical for deployment (thus also resulting typical interference levels for physical distance/positioning).

Further channel model characteristics

In this section we quickly present additional results on channel model characteristics, that can guide the way forward of the study, but have not been treated as exhaustively as the previously presented characteristics.

Convergence speed/testing time

a) Convergence speed/testing time

To gauge potential testing-time increase issues with the presented channel models, we have run a trial simulation that uses a single draw/seed of 10000 slots and calculates the TPUT using 10/100/250/…/10000 slots.

To enable fair comparison, we tried to choose a 50%TPUT operating point for all channel models, however for TDLC (medA), the channel is very rank deficient, and we could not go above 30%; even with MCS reduced to 7:

CDLC @ 22dB (50% TPUT) / TDLClow @ 19dB (50% TPUT) / TDLCmedA @ 50dB/MCS7 (32% TPUT)

Figure 5: Simulation convergence trials: 8T8R 2CW (MCS17 both) full rank, SU-MMSE receiver, Random precoding vs. simulated slots (at 50% TPUT operating point target).

PDP and RMS DS

To confirm the RMS DS of channel model candidates, it is instructive to calculate the PDP of measured DM-RS symbols and calculate the RMS DS from the measured PDP. It is possible to mathematically transform perfect channel coefficient (in the RE domain) to the time domain, however we opted for measurements at very high SNR to reduce the number of sources for systematic errors.

Channel models with principal beam direction steering modifications, can also use such measurement to gauge the impact of this steering on the DS. It is understood that a fixed directive receive antennas setup in a channel model (e.g., array factors or antenna correlations of a fixed ULA setup), coupled with changing DoAs from directional steering, changes the powers received from different angles/clusters/delays. 	
Hence, in such a channel model, the RMS DS is changing over the course of sample generation, in accordance with the principal direction steering.

The CDL models from 38.901 and 38.827 anticipate this effect and have delay scaling procedures to be applied with changing DoAs and DoDs to keep the DS matched with the desired value (see, for example TR 38.827 section 7.2).

We show this exercise here for TDL low with and without DoD beamsteering. 	
Note that this case does not show impact from principal direction steering, since (1) only DoD steering is applied in (2) a channel model that does not have spatial selectivity in either DoD or DoA, i.e., the effective per tap/cluster powers do not change with changing directions:

| a) Without beam steering [22.7355 18.0173 18.9006 17.6107 17.1727 16.9964 20.2687 22.1507 ] * 16e-9 b) With DoD beam steering [22.7355 18.0173 18.9006 17.6107 17.1727 16.9964 20.2687 22.1507] * 16 e-9 Average RMS DS is 310ns. |  |
| --- | --- |

Figure 6: Calculated RMS DS based on PDP measurement for TDLC300-100 low.

827 CDL MU extension

To evaluate many MU related TPUT and PMI impacts, MCMU configurations are needed for the requirement setup (see SCMU/MCMU distinction from section 2.3). In legacy TDL configurations, such a setup is created by drawing independent realizations of the TDL channel model and applying them to the tx-rx links independently. In TDL low this creates links that are perfectly statistical independent, no matter if the receivers should be spatially correlated or not. In TDL+antCorr the small-scale fading is still independent, but the DoDs and DoAs are the precisely same for all links, i.e., signal and interference are sent and received from the same directions.

827 CDL is currently only described for SU setups, where the signals are transmitted and received in spatially separable directions. To extend this to the needs of MCMU, the created links need to have different spatial supports, but the differences between supports need to be spatially consistent with respect to the relative positions of target and interference receivers.

The easiest way to achieve spatially correlated channels is to modify the DoAs and DoDs to differ between receivers in line with intended correlation levels. For example, we can create CDL-A DoAs/DoDs as follows for a target receiver as well as a far and close interferer.

| Cluster | Target | Close | Far |
| --- | --- | --- | --- |
| 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 20 21 22 23 | AOD[°] AOA[°] ZOD[°] -63.592 70.175 89.2 -2.804 -154.23 96.575 -2.804 -154.23 96.575 -2.804 -154.23 96.575 30.194 92.166 101.51 30.194 92.166 101.51 30.194 92.166 101.51 41.136 -23.07 106.35 -29.895 -57.925 90.057 54.034 107.46 85.118 -30.349 70.697 102.27 45.785 -122.25 110.02 -54.818 48.706 106.56 -61.46 92.166 107.55 -46.744 -27.59 88.685 -48.876 -41.497 87.519 56.481 -62.531 88.016 50.539 121.54 108.34 45.051 151.18 82.442 -42.794 160.57 83.454 -55.203 114.24 110.04 42.883 -153.45 84.483 -20.981 73.565 105.44 | AOD[°] AOA[°] ZOD[°] -64.03 70.373 89.202 -3.9398 -153.74 96.691 -3.8466 -153.78 96.681 -3.4147 -153.96 96.637 29.897 93.001 101.73 29.954 92.84 101.69 30.014 92.673 101.65 41.076 -23.848 106.59 -30.465 -58.499 90.057 54.076 107.7 85.095 -30.56 70.789 102.28 45.787 -122.28 110.08 -55.008 48.718 106.55 -61.621 92.283 107.54 -46.902 -27.741 88.685 -49.005 -41.626 87.519 56.5 -62.623 88.013 50.548 121.63 108.37 45.051 151.27 82.431 -42.875 160.64 83.454 -55.284 114.32 110.03 42.881 -153.43 84.476 -21.019 73.584 105.45 | AOD[°] AOA[°] ZOD[°] -62.52 69.604 89.193 -10.939 -150.14 97.497 -0.45606 -155.22 96.339 8.8481 -159.71 95.353 31.274 89.14 100.72 49.72 37.962 87.153 30.773 90.577 101.09 41.533 -17.813 104.76 -36.103 -64.43 90.059 53.7 105.66 85.296 -34.136 72.276 102.53 45.82 -122.88 110.88 -57.045 48.83 106.5 -64.26 94.066 107.36 -44.419 -25.318 88.68 -43.61 -36.207 87.495 55.654 -58.981 88.157 50.171 118.4 107.24 45.044 145.06 83.377 -65.432 -179.59 83.471 -8.4316 70.437 113.8 42.637 -151.47 83.845 -22.074 74.119 105.56 |

Figure 7: Potential CDLA UMa DoA/DoE for target and close/far receivers. ZoA is 90deg as per 38.827.

Thus, whilst the initial focus of the FS_NR_demod_SCM SI might be based upon baseline CDL-C UMa, it is possible to modify the DoDs and DoAs to provide different levels of spatial correlations for different receivers in MU cases.

RAN4 to consider how SCM candidates can be modified to allow separately, but spatially consistently, correlated channels for different DoDs and DoAs corresponding to different multiple UEs.

Spatial Channel Modelling Methodology

This section follows the last WF [2] to advance open issues and add new topics, where found necessary.

Common for all Channel Modelling Methodologies

Test cases for SU-MIMO

The WF from RAN4#112 [2] sets the starting point of SU-MIMO test cases as follows:

As a starting point: - 8Rx: - 8 Layer (8T8R) - 8 Layer PDSCH TPUT (Options on precoder choice to be presented at next meeting) - FFS: 8 Layer PMI Reporting (CB Type 1, single panel) - FFS: 4 Layer (4T8R) - FFS: 4 Layer PDSCH TPUT - FFS: 4 Layer PMI Reporting (CB Type 1, single panel) - FFS: 4Rx: - FFS: 4 Layer (4T4R) - FFS: 4 Layer PDSCH TPUT Interested companies to bring views at the next meeting (for comparison and initial alignment).

From this we have several avenues to follow:

8T8R 8 Layer PDSCH TPUT

The precoder choices for such a requirement configuration have been analyzed in section 2.2.2 and [3].	
Both random and fixed precoding shall be studied to check whether differences in per CW throughput are observed.

From the prior PMI profiles, it is understood that PMI 4: i1_1 = 2, i2=0, (i1_2 = 0, i1_3=n/a) can be used as a valid choice in all channel models.

8T8R 8 Layer PDSCH TPUT: Use random and fixed (PMI 4: i1_1 = 2, i2=0, i1_2 = 0, i1_3=n/a) precoders for comparison and alignment of 827 CDLC UMa (+DMD modifications) and TDLC. Alternatively, allow usage of most common PMI choice for fixed PMI.

8T8R 8 Layer PMI

In section 2.2 we have demonstrated that legacy channel models have issues with unexpected PMI profiles/choices and when comparing TypeI vs. eTypeII CBs. As such we propose comparison and alignment for the following use cases:

8T8R 8 Layer PMI: Compare and align typeI CB PMI choice histograms in candidate channels.

8T8R 8 Layer PMI: Compare and align TypeI vs. eTypeII PMI choice performance in candidate channels.

4T8R 4 Layer PDSCH TPUT

In section 2.1 we have highlighted the potential sensitivity issues of legacy TDL channel with respect to receiver implementation. As such it seems appropriate to compare and align on this issue.

4T8R 4 Layer PDSCH TPUT: Compare receiver performance between different contributors in 827 CDLC UMa (+DMD modifications) and TDLC. Encourage receiver variation analysis. Use random and fixed precoding with fixed PMI chosen via PMI choice histograms.

4T8R 4 Layer PMI

In section 2.2 we have demonstrated that legacy channel models have issues with unexpected PMI profiles/choices and when comparing TypeI vs. eTypeII CBs. As such we propose comparison and alignment for the following use cases:

4T8R 4 Layer PMI: Compare and align typeI CB PMI choice histograms in candidate channels.

4T8R 4 Layer PMI: Compare and align TypeI vs. eTypeII PMI choice performance in candidate channels.

4T4R 4 Layer PDSCH TPUT

In section 2.1 we have highlighted the potential sensitivity issues of legacy TDL channel with respect to receiver implementation. As such it seems appropriate to compare and align on this issue.

4T4R 4 Layer PDSCH TPUT: Compare receiver performance between different contributors in 827 CDLC UMa (+DMD modifications) and TDLC. Encourage receiver variation analysis. Use random and fixed precoding with fixed PMI chosen via PMI choice histograms.

4T8R 4 Layer PMI

In section 2.2, we have demonstrated that legacy channel models have issues with unexpected PMI profiles/choices and when comparing TypeI vs. eTypeII CBs. As such we propose comparison and alignment for the following use cases:

4T8R 4 Layer PMI: Compare and align typeI CB PMI choice histograms in candidate channels.

4T8R 4 Layer PMI: Compare and align TypeI vs. eTypeII PMI choice performance in candidate channels.

Other

In section 2.5 we have mentioned other (SU and MU) use cases, where SCMs are necessary in RAN4 demod to achieve reasonable performance requirements. However, those are outside of the scope of the study, so we refrain from adding proposals at this point.

Further use cases, where SCMs are necessary in RAN4 demod to achieve reasonable performance requirements are, e.g., CQI, FDD reciprocity compression CBs, and Doppler compression CBs, those are however currently outside of the scope of this study.

Test cases for MU-MIMO

The WF from RAN4#112 [2] sets the starting point of MU-MIMO test cases as follows:

Test Cases for MU-MIMO Way Forward: Companies to bring proposals at the next meeting.

From this we have several avenues to follow:

4T4R (2+2) Layer PDSCH TPUT (in SCMU and MCMU)

In section 2.3 it was argued that current MU TPUT requirements may have questionable behaviour with respect to (a) relative performance between advanced receivers, casting doubt on algorithm test aspects, and (b) required SNR levels precluding high interference configurations (non-orthogonal precoders).	
As such we propose to compare and align on this issue:

4T4R (2+2) Layer PDSCH TPUT: Compare R-ML, IRC, and E-IRC implementation performance in candidate channels. Model interference via random and orthogonal MU precoders at transmitter side. Extend channel model candidates to multi-cell MU, compare both multi-cell and single-cell MU setups.

Note: It remains to be discussed, whether MCMU comparisons should include a serving cell and interfering cell receiver at the same time.

4T4R (2+2) Layer PMI (in SCMU and MCMU)

In section 2.4 we argue that eTypeII PMI selection algorithm functionality and performance may have never been reliably tested ad MU gains are hidden behind similar TypeI and eTypeII PMI performance, when legacy channels are used. Hence, we see it reasonable to compare our understanding in the comparison stage of the study:

4T4R (2+2) Layer PMI: Compare TypeI vs. eTypeII PMI choice performance in candidate channels in the presence of single cell MU interference.

4T4R (2+2) Layer PMI: With lower priority, compare TypeI vs. eTypeII PMI choice performance in candidate channels in the presence of multi cell MU interference.

TDL Based Methodologies

Tap Delay Line Parameters

The WF from RAN4#112 [2] sets the starting point as follows:

The following Legacy TDL models for comparison: - Option 1: TDLC 300-100 ULA Low - Option 2: TDLA 30-10 ULA Low - FFS: TDLC 300-100 medA - FFS: TDLC 300-100 medB - FFS: TDLB 100

As we did during all of chapter 2 and in our prior contribution [3], SCMs need to be compared to both TDL with and without antenna correlation, to see the issues with both setups. As such we propose:

Use at least TDLC 300-100 ULA Low, TDLC 300-10 ULA Low, and TDLC 300-100 medA for comparison studies.

Note: MedB was observed to have some numerical convergence issues in 8Rx, so we are not in favour of using this configuration.

TDL Model Extensions

The WF from RAN4#112 [2] sets the starting point as follows:

Way Forward: Open issue needs further discussion, 2 contributions provided during RAN4 #112, interested companies to review and provide views at next meeting.

During the whole chapter 2 and in our prior contribution [3], we have repeatedly argued that TDL based approaches, and most extensions amount to poorly understood hacks to align the output with expectations from spatial MIMO channels. Using such extensions would require extensive statistical and behavioral analysis to validate.	
Ultimately RAN4 needs to choose a reasonable version of SCM that is alignable, validated, implemented in test equipment, and agreeable, to define demodulation and CSI requirements.

Using newly conceived TDL extensions to model expectations from spatial MIMO channels, would require extensive statistical and behavioral analysis to validate the new model.

As such we propose:

RAN4 to not consider TDL based models to conceive new SCMs.

CDL Based Methodologies

The WF from RAN4#112 [2] sets the starting point as follows:

Cluster Delay Model Way Forward: Prioritise 38.827 UMa-CDL-C for initial comparison and alignment. FFS on parameters. Other options not precluded. UE Antenna Configuration Agreement: UE Antenna Configuration, for DL, for 38.827 : Cross Pol (1, n/2, 2), i.e. [ + + + +]. Where ‘n’ is number of Rx with half lambda spacing, with omni antenna element radiation pattern.

Cluster Delay Line Parameters

We agree with the previously agreed WF, and would even go so far as using 38.827 UMa-CDL-C (363ns DS, 30kph velocity) with UE antenna configurations as per agreement, as the baseline.

It is unclear to us, what other parameters are still open.

Use TR 38.827 UMa-CDL-C (363ns DS, 30kph velocity) with UE antenna configurations as per WF agreement as baseline.

Comparison of methodologies

The WF from RAN4#112 [2] sets the starting point as follows:

Comparison Approach In Priority Order: - SU-MIMO, with a single receiver implementation (as per test cases for SU-MIMO) – For comparison and alignment - MU-MIMO, based on the 4x4 existing intra-cell inter-user interference requirements – For comparison Other cases for comparison not precluded.

In line with our SU and MU test case proposals, we observe that MU MIMO should also be compared for inter-cell MU requirements, but we agree with the given priority order.

Testability and TE aspects

The WF from RAN4#112 [2] sets the starting point as follows:

Impact on Testing Duration: Way Forward: Open issue needs further discussion.

We approached the question of testing time and convergence in section 2.6.1, and our previous contribution [3]. It is observed that the convergence number of samples (~time) of 827 CDL is similar that of TL+antCorr. Furthermore, we note that 827 is already implemented in TEs from at least two vendors, and additionally used as part of TS 38.151. Thus, we feel confident in proposing that:

RAN4 to consider the 827 CDL model as feasible, implemented, and without additional impact to testing time above TDL+antCorr levels.

Other channel model candidates, will still need to be evaluated for statistical stability, correctness, implementability, testing time impact, reproducibility, etc.

Further TE considerations for 827 CDL

Cluster count:	
The proposed CDL model is based on TS 38.827, which is based on TR 38.901. The cluster-count is thus upper limited by TR 38.901, which is 24. Additionally, TS 38.827 based CDL do not consider midpaths, hence implementation should be less complex than TR 38.901.

Usage of TS 38.827 in conducted and virtual cable setups:	
This requires a slight modification of TR 38.827. By specifying receive side channel model antenna assumptions, like the transmit side channel model antenna assumptions already included in TR 38.827, the channel coefficients can be derived in a single step. This modification was agreed in RAN4#112 and it is now ready to be commonly used.

Requirements and Other

The WF from RAN4#112 [2] sets the starting point as follows:

Definition of New Requirements: Way Forward: Open issue needs further discussion.

The intend of this study is to first evaluate the possibility ad usefulness of introducing a SCM to demodulation requirement, with the goal of potentially following up the study with a work-item for requirement definition.

We should aim to keep the study setup aligned with this possibility and

Discuss introduction of requirements following the delivery of comparison results by contributors.

Conclusion

This contribution introduces a TS 38.827 based CDL spatial channel model and compares it with legacy TDL models to evaluate its usefulness and advantages in the context of minimum demodulation performance requirements for MIMO features. We also discuss why legacy channel models are not adapted to test MIMO and beam-based features.

The CDL model is found to be a reasonable demod (i.e., baseband branches referenced) channel model “black box”, that can be aligned between implementations of different contributors. It also exhibits MIMO effects and properties that we expect and observe in deployment, it is spatially consistent, predicable, and explainable. It can be directly applied to SU use cases and can be readily extended to MU.

TDL and TDL+antCorr channel models were found lacking in terms of spatial diversity, spatial stability, and modelling reasonable channel matrix condition. Which was shown to cause issues when setting both SU and MU requirements; both TPUT and PMI wise.

The following observations and proposals were made:

Chapter === Updated legacy issues and CDL comparisons

== SU TPUT

- TDL (low) channels severely overestimate the high rank support, as channel condition is forced to be statistically perfect. TDL (non-low) channels severely underestimate the high rank support, as channel condition is forced to be severely deficient.

= Sensitivity to receiver implementations

For SU TPUT use cases, legacy TDL channel model-based demodulation performance requirements are neither fit to test receiver functionality, nor fit to set a stable minimum performance requirement. Functionally differing SDM receiver implementation show same performance under legacy TDL channel requirements, but SCMs with spatially typical properties (in this case 827 CDL) can meaningfully differentiate between implementations, even under random precoding.

== SU PMI

= SU PMI profiles (4Rx)

The capacity optimal TypeI PMI choice is random in TDL low, fixed to broadsight in TDL+antCorr, and following defined and stable strong directions in CDL. Thus, only SCM model-based requirements can provide evidence for correct algorithmic function and set reasonable minimum performance expectations.

= PMI choice: eTypeI vs. eTypeII

In legacy TDL channel models TypeI and eTypeII PMI feedback performs nearly identical, raising serious concerns whether such performance requirements are reasonable from an implementation correctness pov and from the perspective of absolute values for performance requirements. Performance requirements based on CDL channels behave as expected.

== MU TPUT

Different expectations for MU TPUT requirements

It is our understanding that demod minimum performance requirements should be defined, if there is either impact on the receiver algorithm implementation, or if there is impact on the resulting performance, or if there are previously unseen strong issues with legacy implementations.

= Advanced receivers

For SCMU TPUT is it expected that SU and MU MMSE perform about equally well in typical co-scheduling scenarios, and the R-ML to linear receiver gap is within a reasonable margin.

If a SCM was used in Rel-18 advanced receivers instead of TDL, RAN4 would not have needed to spend a lot of time unnecessarily analyzing and debating SU vs. MU MMSE in DL.

= SNR levels

Using 827 CDL in MU TPUT scenarios significantly reduces SNR requirements for all advanced receivers and allows for non-orthogonal precoder requirements, even for non-R-ML requirements

== MU PMI

Different expectations for MU PMI choice requirements

It is our understanding that CSI minimum performance requirements should be defined, if there is either impact on the PMI choice algorithm implementation, or if there is impact on the PMI choice itself and the resulting performance, or if there are previously unseen strong issues with legacy implementations.

== Further channel model characteristics

= 827 CDL MU extension

- RAN4 to consider how SCM candidates can be modified to allow separately, but spatially consistently, correlated channels for different DoDs and DoAs corresponding to different multiple UEs.

Chapter === Spatial Channel Modelling Methodology

== Common for all Channel Modelling Methodologies

= Test cases for SU-MIMO

8T8R 8 Layer PDSCH TPUT

8T8R 8 Layer PDSCH TPUT: Use random and fixed (PMI 4: i1_1 = 2, i2=0, i1_2 = 0, i1_3=n/a) precoders for comparison and alignment of 827 CDLC UMa (+DMD modifications) and TDLC. Alternatively, allow usage of most common PMI choice for fixed PMI.

8T8R 8 Layer PMI

8T8R 8 Layer PMI: Compare and align typeI CB PMI choice histograms in candidate channels.

8T8R 8 Layer PMI: Compare and align TypeI vs. eTypeII PMI choice performance in candidate channels.

4T8R 4 Layer PDSCH TPUT

4T8R 4 Layer PDSCH TPUT: Compare receiver performance between different contributors in 827 CDLC UMa (+DMD modifications) and TDLC. Encourage receiver variation analysis. Use random and fixed precoding with fixed PMI chosen via PMI choice histograms.

4T8R 4 Layer PMI

4T8R 4 Layer PMI: Compare and align typeI CB PMI choice histograms in candidate channels.

4T8R 4 Layer PMI: Compare and align TypeI vs. eTypeII PMI choice performance in candidate channels.

4T4R 4 Layer PDSCH TPUT

4T4R 4 Layer PDSCH TPUT: Compare receiver performance between different contributors in 827 CDLC UMa (+DMD modifications) and TDLC. Encourage receiver variation analysis. Use random and fixed precoding with fixed PMI chosen via PMI choice histograms.

4T8R 4 Layer PMI

4T8R 4 Layer PMI: Compare and align typeI CB PMI choice histograms in candidate channels.

4T8R 4 Layer PMI: Compare and align TypeI vs. eTypeII PMI choice performance in candidate channels.

Other

Further use cases, where SCMs are necessary in RAN4 demod to achieve reasonable performance requirements are, e.g., CQI, FDD reciprocity compression CBs, and Doppler compression CBs, those are however currently outside of the scope of this study.

= Test cases for MU-MIMO

4T4R (2+2) Layer PDSCH TPUT (in SCMU and MCMU)

4T4R (2+2) Layer PDSCH TPUT: Compare R-ML, IRC, and E-IRC implementation performance in candidate channels. Model interference via random and orthogonal MU precoders at transmitter side. Extend channel model candidates to multi-cell MU, compare both multi-cell and single-cell MU setups.

4T4R (2+2) Layer PMI (in SCMU and MCMU)

4T4R (2+2) Layer PMI: Compare TypeI vs. eTypeII PMI choice performance in candidate channels in the presence of single cell MU interference.

4T4R (2+2) Layer PMI: With lower priority, compare TypeI vs. eTypeII PMI choice performance in candidate channels in the presence of multi cell MU interference.

== TDL Based Methodologies

= Tap Delay Line Parameters

Use at least TDLC 300-100 ULA Low, TDLC 300-10 ULA Low, and TDLC 300-100 medA for comparison studies.

= TDL Model Extensions

Using newly conceived TDL extensions to model expectations from spatial MIMO channels, would require extensive statistical and behavioral analysis to validate the new model.

RAN4 to not consider TDL based models to conceive new SCMs.

== CDL Based Methodologies

Cluster Delay Line Parameters

Use TR 38.827 UMa-CDL-C (363ns DS, 30kph velocity) with UE antenna configurations as per WF agreement as baseline.

== Testability and TE aspects

RAN4 to consider the 827 CDL model as feasible, implemented, and without additional impact to testing time above TDL+antCorr levels.

== Requirements and Other

Discuss introduction of requirements following the delivery of comparison results by contributors.

- References

- RP-241610; “New SID: Study on spatial channel model for demodulation performance requirements”, Nokia, BT Plc et al, RAN#104, Shanghai, June 2024.

- R4-2413606, “Way Forward for [112][327] FS_NR_demod_SCM”, Moderator (Nokia), RAN4#112.

- R4-2411300, Discussion on Spatial Channel Model for Demodulation Performance Requirements, Nokia, RAN4#112.

- R4-2411301, “Simulation Results and CDL Implementation for Spatial Channel Model for Demodulation Performance Requirements”, Nokia, RAN4#112.

- R4-2402277, “Discussion on 8Rx general demodulation aspects and spatial channel models”, Nokia, RAN4#110.

- R4-2415283, Simulation Results and CDL Implementation for Spatial Channel Model for Demodulation Performance Requirements, Nokia, RAN4#112bis.

- R4-2415673, Further observations from deployment and MU-MIMO use cases for SCM evaluation, BT plc, RAN4#112bis.

- R4-2415285, TDL and CDL comparison with respect to the CSI resolution feedback, Orange, BT, Telecom Italia, Vodafone, Bouygues Telecom, Fraunhofer, Deutsche Telekom, RAN4#112bis.

- R4-2413531, Ad-hoc meeting minutes for [112][317] Demod_Maintenance, Nokia, RAN4#112.

- R4-2016102, “Evaluations of Rel-16 Type II PMI testing”, Ericsson, 3GPP RAN4#97-e, Nov. 2020.

- R4-2009858, On PMI reporting test case for eType II codebooks, Nokia, Nokia Shanghai Bell, RAN4#96-e.

- R4-2014949, On PMI reporting requirements for enhanced Type II codebooks, Nokia, Nokia Shanghai Bell, RAN4#97-e.

- R4-2017678, “Way Forward on PMI Reporting Requirement for NR eMIMO”, RAN4#97-e.

- R4-2214397, “WF on UE demodulation and CSI requirements for feMIMO”, RAN4#104b-e.

- R4-2210669, “WF on CSI requirement for Rel-17 feMIMO”, RAN4#103-e.

- R4-2407000, RAN4#110-bis Meeting Report, ETSI MCC, RAN4#111.

- R4-2406114, Way Forward for [110bis][324] NR_demod_enh3_Part1, CTC, RAN4#110-bis.

- R4-2309351, On MU-MIMO advanced receiver, Qualcomm, Inc., RAN4#107.