3GPP TSG-RAN WG4 Meeting #112-bis                                   	R4-2415284

# Hefei, China, 14th – 18th Oct, 2024

Agenda item:			6.9.3

Source:	Eutelsat Group

Title:	Feasibility analysis of FR1 and FR2 Numerology

Document for:	Discussion

# Introduction

The Ku band WID [1] requires a choice between FR1 or FR2 numerology to be made by RAN#106 (December, 2024). During RAN4 #112 in Maastricht it was agreed [2] that the following technical criteria will be used to determine the feasibility of FR1 and FR2 numerologies to be used for Ku band:

- UL timing synchronization

- Phase noise

- Efficient channel utilization

- Beam hopping granularity

- Link budget UL/DL – as a function of SCS and channel bandwidth

This contribution briefly examines each of the above points and provides observations and proposals for the conditions under which the criteria should be evaluated in the context of NTN operation in the Ku band.

The following FR1 and FR2 options will be considered:

Table 1 FR1 and FR2 numerology / SCS combinations under consideration

| Sub-Carier Spacing | Numerology |  |
| --- | --- | --- |
|  | FR1 | FR2 |
| SCS of uplink signals | 15 kHz, 30 kHz, 60 kHz | 60 kHz, 120 kHz |
| SSB SCS | 15 kHz, 30 kHz | 120 kHz, 240 kHz |

# Discussion

## Uplink timing synchronization aspects

In this section the timing requirements specified in [3] are considered.

Section 7.1C requires that:

The UE initial transmission timing error shall be less than or equal to Te_NTN where the timing error limit value Te_NTN.

Te_NTN is specified in Table 7.1C.2-1 for FR1-NTN.

Te_NTN is specified in Table 7.1C.2-2 and Table 7.1C.2-3 for VSAT UE in FR2-NTN.

This requirement applies:

-	when it is the first transmission in a DRX cycle for PUCCH, PUSCH and SRS, or it is the PRACH transmission, or it is the msgA transmission.

The tables referred to above are reproduced below with additional information for the purpose of the present discussion.

FR2-NTN uplink timing synchronization

To date, the RRM for VSAT operation has been based on Ka band and thus only considered FR2 numerology. Reference: TS 38.133 v18.7.0 Section 7.1C UE transmit timing for Satellite Access, reproduced below with two columns added to show the absolute value of Te_NTN and the percentage of the CP this represents. Figures in red are outside the ±0.5TCP assumption:

Table 2: TS 38.133 v18.7.0 Table 7.1C.2-3: Te_NTN Timing Error Limit for mobile VSAT is served by GSO – with two additional columns

| #### Frequency Range | #### SCS of SSB signals (kHz) | #### SCS of uplink signals (kHz) | #### Uplink Signals CP Duration (ns) | #### Te_NTN #### [1] | #### Te_NTN (ns) | #### Te_NTN/CP (%) |
| --- | --- | --- | --- | --- | --- | --- |
| #### FR2-NTN | #### 120 | #### 60 | #### 1171.9 | #### 13*64*Tc | #### 423.2 | #### 36.11% |
|  |  | #### 120 | #### 585.9 | #### [10]*64*Tc | #### 325.5 | #### 55.56% |
|  | #### 240 | #### 60 | #### 1171.9 | #### 13*64*Tc | #### N/A |  |
|  |  | #### 120 | #### 585.9 | #### [10]*64*Tc | #### 325.5 | #### 55.56% |
| Note 1: Tc is the basic timing unit defined in TS 38.211 Tc is 0.509 ns for all numerologies. |  |  |  |  |  |  |

#### Observation 1: For Mobile VSAT served by GSO using 120 kHz SCS (with SSB SCS of 120 kHz or 240 kHz) the timing error formula contains a value indicated between square brackets. The presence of square brackets needs to be addressed as a maintenance action at the RAN4 #113 meeting.

For 120 kHz, Te = 325 ns which is more than 50% of the CP duration of 586 ns

For 60 kHz Te = 423 ns which is more than 50% of the CP duration of 1171 ns

#### Observation 2: The value of [10] in the timing error formula for 120 kHz SCS of uplink signals (for both SCS of SSB of 120 kHz and 240 kHz) results in a Timing Error Limit larger than 50% of the CP. while the timing error needs to be within 50% of the CP for reliable operation. This needs to be addressed as a maintenance action at the RAN4 #113 meeting.

The current figure of [10] for Mobile VSAT served by GSO is problematic. It is expected in the future (not in this WI) that support for Mobile VSAT to NGSO will be specified. Current assumptions are that NGSO timing will be more difficult than GSO, so this suggests the current figure of [10] which is problematic for Ka band, will be even less feasible for the future NGSO case.

FR1-NTN uplink timing synchronization

Uplink timing is specified for FR1-NTN assuming smartphone type terminals. As with Table 7.1C.2-3 for FR2-NTN, Table 7.1C.2-1: Te_NTN Timing Error Limit is the basis of this analysis for FR1-NTN.

Table 3: TS 38.133 v18.7.0 Table 7.1C.2-1: Te_NTN Timing Error Limit with two additional columns

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Uplink Signals CP Duration (ns) | Te_NTN | Te_NTN (ns) | Te_NTN/CP (%) |
| --- | --- | --- | --- | --- | --- | --- |
| FR1-NTN | 15 | 15 | 4687.5 | 29*64*Tc | 944.0 | 20.1% |
|  |  | 30 | 2343.75 | 24*64*Tc | 781.3 | 33.3% |
|  |  | 60 | 1171.875 | N/A | N/A | N/A |
|  | 30 | 15 | 4687.5 | 24*64*Tc | 781.3 | 16.7% |
|  |  | 30 | 2343.75 | 22*64*Tc | 716.1 | 30.6% |
|  |  | 60 | 1171.875 | N/A | N/A | N/A |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] Tc is 0.509 ns for all numerologies. |  |  |  |  |  |  |

It can be noted that, in all cases, where the timing error limit is specified, it is well within the ±0.5* TCP assumption.

However, note that there are presently no FR1-NTN specifications for SCS of uplink signals at 60 kHz. There are two possible reasons why FR1-NTN with 60 kHz uplink SCS is more challenging (and hence “N/A” in the table) than FR2-NTN with 60 kHz SCS. First the FR1-NTN assumption was a device using an omnidirectional antenna which will experience a much higher delay spread, eating into the delay budget, than a highly directional VSAT antenna and secondly, for FR1-NTN 15 or 30 kHz SSB SCS provides less precise downlink timing compared with 120 kHz SSB in FR2-NTN.

Considering Table 3, we note for 30 kHz (SSB and uplink signals) Te_NTN = 716 ns while the CP duration = 2343 ns, that is, Te_NTN is much less than half the CP).

A VSAT, with a directional antenna, will experience less multipath and so timing accuracy may not need to be as tight as implied by the present RRM specification.

Observation 3: For all FR1 SCS combinations, the related Timing Error Limit is significantly less than half the CP duration.

To sum-up the discussion of timing, the following observation and proposals are made:

Observation 4: FR1 numerology timing is less critical than FR2 numerology timing and thus more robust to GNSS location errors and other time/ position error sources and resolution limits.

Proposal 1: RAN4 should avoid selecting a numerology/ SCS combination that would disadvantage Mobile VSAT in the Ku band served by GSO.

Proposal 2:  RAN4 should consider the likely future specification of Mobile VSAT to NGSO (not in this WI) when considering the feasibility of different SCS.

## Phase noise [and residual Doppler]

Phase noise needs to be considered as a contributor to the overall performance. Specifications for 5G NR introduced enhancements to mitigate the effects of phase noise, especially in FR2 (24 GHz and above). Higher SCS and Phase Tracking Reference Signals (PT-RS) can be used to mitigate the effects of phase noise and are especially useful at higher frequencies where high order modulation is used. PT-RS are specified for FR1 and FR2 whereas the higher SCS, above 60 kHz, are only available in FR2. PT-RS increase the overhead so should only be used when a net benefit will result. The specifications include the provision for UE to indicate their preference for PT-RS to the network – UE with better (cleaner) oscillators can request the network uses sparser PT-RS than used for UE with less good phase noise characteristics.

The graphs below, from [4] illustrate the potential benefits of PT-RS.

Figure 1: Simulation results, at 30 GHz using 60 kHz SCS and 16-QAM code rate 3/4 and freq. density 2 [4, Figure 5]

Note that the higher edge of the Ku band Downlink frequency is less than half the RF carrier frequency (i.e., Ku band DL 12.7 GHz maximum vs. 30 GHz) and modelled 60 kHz SCS is double compared with FR1 30 kHz SCS; these factors will tend to cancel each other out and we may, therefore, expect results to be comparable.

The extent to which PT-RS might be an advantage in the NTN Ku band is uncertain since NTN operate under very different conditions to TN FR2 deployments (that is, typically, short range, high SNR, narrow beams supporting high order modulation schemes).

In contrast with FR2 TN, it is unlikely that NTN in Ku band will operate at very high SNR. For the purposes of this analysis, we could assume SNR much above 12 – 15 dB are difficult to achieve with modest terminal antenna sizes - typically SNR will be lower with 40 x 40 cm antenna apertures. Alternatively, in terms of spectral efficiency, a maximum of around 3 bits/s/Hz (e.g., 16-QAM 3/4 rate coded) is likely to be representative of the best achievable on the downlink (space to earth). Furthermore, for the purposes of analysis, we could assume the uplink (earth to space) is likely to be limited to around 2 bits/s/Hz due to practical constraints of transmitter power and antenna size. Terminals employing smaller antenna apertures, such as 20 x 20 cm which has been proposed for cars, will operate at significantly lower SNR (likely including negative SNR values).

Observation 5: NTN Ku band VSATs are expected to typically operate at around 12 SNR or below. Higher SNR are possible only with larger antennas, smaller antennas will operate at negative SNR.

Residual Doppler shift (i.e., an uplink frequency shift due to imperfectly corrected Doppler) will have a detrimental impact for multiple user operation it will also cause uplink orthogonality to be impaired or, in the limit, completely lost. For multi-user uplink different shifts cannot be compensated at the base station. Unlike TN, Doppler spread is likely to be minimal (very little multipath when directional VSAT antennas are used) typically two or more users will have different residual Doppler shift. The effect of any given amount of Doppler shift will be inversely proportional to SCS (e.g., a system using 15 kHz SCS will be more vulnerable to this effect than 120 kHz SCS).

Proposal 3: Phase noise and residual Doppler shift analysis should focus on SNR in the range 3 – 12 dB.

## Efficient channel utilization

The ability to efficiently utilize 250 MHz (downlink) and 125 MHz (uplink) transponders is critically important. Support of legacy transponder bandwidths (such as 36 MHz) and wider bandwidths (such as 500 MHz) is also important to some Ku band operators.

For the uplink we assume DFT-S-OFDM will be used to maximise power efficiency and note that multi-carrier operation may result in requiring transmitter power back-off.

In populating Table 4, for FR1 we assume 30 kHz SCS, resulting in a maximum bandwidth of 100 MHz (nominal) and for FR2 we assume 60 kHz or 120 kHz SCS are supported.

As Carrier Aggregation (CA) is not yet specified for any NTN band (and is outside the scope of the present Ku band WID) we only consider the ability to fully utilize the transponder bandwidths with one or more carriers.

Table 4: FR1/FR2 efficient transponder utilization examples.

| #### Transponder Bandwidth | #### FR1 Options | #### FR2 Options | #### Comments |
| --- | --- | --- | --- |
| #### 125 MHz (Uplink) | #### 100 MHz + [20 MHz or 25 MHz] | #### 100 MHz | #### Assuming FR1 30 kHz SCS. |
| #### 250 MHz (Downlink or Uplink) | #### 100 MHz x 2 + 50 MHz | #### 200 MHz + 50 MHz |  |
| #### 36 MHz (Bi-directional) | #### 35 MHz | #### ? | #### Example legacy GEO DVB transponder bandwidth. #### FR2 may use part of a 50 MHz FR2 carrier. |
| #### 500 MHz (Downlink) | #### 100 MHz x 5 | #### 400 MHz + 100 MHz |  |

For transponder bandwidths above 50 MHz selecting 15 kHz SCS would limit the maximum bandwidth per RF carrier to 50 MHz (nominal) and require a larger number of RF carriers to fully utilize the available bandwidth.

Once the FR1/FR2 down selection is made, RAN4 can then consider which CA combinations are of most practical use to operators.

Observation 6: FR1 numerology provides more flexibility in efficiently utilizing transponder bandwidths at the expense of requiring higher numbers of RF carriers per transponder for wider bandwidths.

Observation 7: If FR1 numerology is selected, 30 kHz SCS is preferred over 15 kHz SCS for transponder bandwidths above 50 MHz.

## Downlink beam hopping granularity

In this section we discuss the relative downlink (satellite to VSAT) coverage with FR1 or FR2 [5] numerology under the assumption of Rel-18, that is, without considering extended SSB periodicity under consideration in RAN1 for Rel-19.

Satellite resources are limited, especially power and, for higher frequencies, the number of RF analogue beamformers and thus, onboard resources need to be used efficiently. It is not practical to serve a large number of beam footprints simultaneously to cover all, or most of, the satellite coverage footprint. Downlink Beam Hopping (BH) enables the sharing of data transmission capacity (power) between different geographical areas ("beam footprints") in which UEs with active traffic are located. From the UE perspective, the downlink is not always present, but a maximum revisit time (when the beam footprint is illuminated) is preserved.

For the purpose of this simplified evaluation, we consider the coverage that can be provided from a single analogue beamformer (i.e., number of beam footprints that can be serviced per beamformer) assuming equal sharing of the capacity across the considered beam footprints.

Beam Hopping (BH) cycle represents the maximal scheduling latency expected for an active UE. Two options - 5ms and 10ms - are considered here, 5ms being required for low latency services. The BH cycle can be seen as the maximum "revisit time" for a given beam footprint.

How many ‘swaps’ can be performed during a BH cycle is evaluated, depending on the number of OFDM slots available during the BH cycle (assuming one hop per slot), which gives a maximum average capacity that can be served for a given beam footprint.

Table 5: Example Beam Hopping configuration for 5 ms and 10 ms revisit periodicity.

| BH cycle (ms) | SCS (kHz) | Numerology | Slot Duration (ms) | Beam footprints serviced per BH Cycle [Note 1] |
| --- | --- | --- | --- | --- |
| 5 | 15 | FR1 | 1 | 5 |
| 5 | 30 | FR1 | 0.5 | 10 |
| 5 | 60 | FR1, FR2 | 0.25 | 20 |
| 5 | 120 | FR2 | 0.125 | 40 |
| 10 | 15 | FR1 | 1 | 10 |
| 10 | 30 | FR1 | 0.5 | 20 |
| 10 | 60 | FR1, FR2 | 0.25 | 40 |
| 10 | 120 | FR2 | 0.125 | 80 |

Note 1: In practice one slot per cycle can be allocated to non-active cells. This "loss" of slot is ignored in the above table.

For Ku band, under the assumption that analogue beamforming is employed, the total number of beam footprints that can be served will be given by the number of beam footprints per cycle x number of beamformers.

Observation 8: Both FR1 and FR2 numerologies support the use of BH. Higher SCS provides more scheduling flexibility/ finer granularity than lower SCS. There is a trade-off between maximum scheduling latency and number of ’beam footprints’ that can be served from a single beamformer.

## Link budget

There should be no significant difference for bandwidths greater than the smallest number of RB for the highest SCS all else being equal (e.g., bandwidth allocated, modulation and coding scheme, maximum transmit power). That is, FR1 numerology only has advantages for uplink RB allocations of a few RB per UE; for example, two 15 kHz SCS RB can achieve a 3 dB higher power density (dBm/MHz) compared with allocating one RB using 60 kHz SCS. Although potentially a useful advantage for FR1 in ‘edge cases’ with low SNR, for typical VSAT applications the allocation of very small bandwidths is not considered a compelling reason to favour FR1 over FR2.

Observation 9: FR1 holds an advantage for very low uplink allocations; however, for most practical VSAT scenarios the link budgets for FR1 and FR2 numerologies are the same.

# Conclusions

The following observations and proposals are made:

#### Observation 1: For Mobile VSAT served by GSO using 120 kHz SCS (with SSB SCS of 120 kHz or 240 kHz) the timing error formula contains a value indicated between square brackets. The presence of square brackets needs to be addressed as a maintenance action at the RAN4 #113 meeting.

#### Observation 2: The value of [10] in the timing error formula for 120 kHz SCS of uplink signals (for both SCS of SSB of 120 kHz and 240 kHz) results in a Timing Error Limit larger than 50% of the CP. while the timing error needs to be within 50% of the CP for reliable operation. This needs to be addressed as a maintenance action at the RAN4 #113 meeting.

Observation 3: For all FR1 SCS combinations, the related Timing Error Limit is significantly less than half the CP duration.

Observation 4: FR1 numerology timing is less critical than FR2 numerology timing and thus more robust to GNSS location errors and other time/ position error sources and resolution limits.

Proposal 1: RAN4 should avoid selecting a numerology/ SCS combination that would disadvantage Mobile VSAT in the Ku band served by GSO.

Proposal 2:  RAN4 should consider the likely future specification of Mobile VSAT to NGSO (not in this WI) when considering the feasibility of different SCS.

Proposal 3: Phase noise and residual Doppler shift analysis should focus on SNR in the range 3 – 12 dB.

Observation 6: FR1 numerology provides more flexibility in efficiently utilizing transponder bandwidths at the expense of requiring higher numbers of RF carriers per transponder for wider bandwidths.

Observation 7: If FR1 numerology is selected, 30 kHz SCS is preferred over 15 kHz SCS for transponder bandwidths above 50 MHz.

Observation 8: Both FR1 and FR2 numerologies support the use of BH. Higher SCS provides more scheduling flexibility/ finer granularity than lower SCS. There is a trade-off between maximum scheduling latency and number of ’beam footprints’ that can be served from a single beamformer.

Observation 9: FR1 holds an advantage for very low uplink allocations; however, for most practical VSAT scenarios the link budgets for FR1 and FR2 numerologies are the same.

# References

[1] RP-241690, New WID on Introduction of Ku Band for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat

[2] R4-2413519, Way Forward for [112][314] NR_NTN_Ku_Band_General, Moderator (Eutelsat Group)

[3] TS 38.133 v18.7.0. Requirements for support of radio resource management (Release 18)

[4] R1-1701161, On DL and UL phase noise tracking RS (PTRS), Ericsson

[5] R1-2401470, An operator view on Downlink Coverage Enhancements for FR2-NTN, Eutelsat Group