3GPP TSG-RAN WG4 Meeting #112-bis	R4-2415298

Hefei, China, Oct 14 – Oct 18, 2024

Source:	Nokia

Title:	Discussion on MIMO layer evaluation for 6Rx UE

Agenda item:	6.1.1.3.2

Document for:	Discussion

Introduction

During RAN#104 the work item on UE RF enhancements in Rel-19 was revised [2].

The performance part of this work item has not commenced yet, however discussions during RAN4#110-bis and RAN4#111 on the number of MIMO layers to support 6Rx has led to an Agenda Item to be introduced during RAN4#112 and RAN4#112-bis to specifically discuss MIMO layers for this Work Item with regards to 6Rx.

Specifically looking at the following aspects of the WID:

6Rx for handheld and FWA UE [RAN4, RAN1] - Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n41, n77/n78, n79, n104 - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers - Specify the Rx requirements including reference sensitivity requirements for support 6Rx - Note: the specified requirements can be applicable to both handheld UE and FWA devices - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, depending on UE capability - RAN1 to study and specify, if necessary, SRS antenna switching to support 3T6R depending on UE capability. - Note: RAN1 discussion starts from RAN1#118bis (Oct. 2024) - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

In this document we present views on the open issues which were identified as part of the Way Forward from RAN4#112 [1].

Discussion

General considerations for MIMO layer evaluation for 6Rx UE

6-Layer Performance Evaluation Assumptions

In RAN4#112 the topic of 6-layer performance evaluation assumptions was discussed, with the following described in the Way Forward [1] :

Issue 3-1-2: 6-Layer Performance Evaluation Assumptions Way Forward: RAN4 to further discuss the 6-Layer MIMO performance evaluation assumptions considering realistic antenna correlation assumptions and deployment scenarios.

The proposed 6-layer MIMO configuration would utilize two codewords, a feasible approach that should be acknowledged. For performance evaluation, it is crucial to assess this configuration in a rank-deficient channel. This would allow for the observation of distinct performance differences among the MIMO layers, mirroring the conditions encountered in realistic channels.

In order to fully assess the performance of 6Rx a spatial channel model must be used.

Current most advanced demodulation performance channel models

The demodulation session of RAN4 typically uses the TDL channel model with a provision for modelling antenna correlation (in uni- and X-pol) to represent fading channels with multi-element antenna correlation. The currently used model for antenna correlation is a “continuous squared exponential antenna correlation decay” model which has been used to extend the initially 2x2 correlation matrix (2007, [4]) to 4x4 (in 2008, [5]) and later 8x8 (in 2017,[6]). 	
For 4Tx, such a (unipolar) antenna correlation model (medium=0.3) looks like:

10.31320.32320.3.1....1.0.3..1

Thus, the “continuous squared exponential correlation decay” name is self-explanatory. Both the initial 2x2 matrices [4] and squared exponential decay [5] were chosen adhoc and do not match measurements for >2 antenna systems.	
This antenna correlation model is applied to all taps of the TDL model equally.

The used method of extending the initial (pragmatically) chosen 2x2 correlation matrices has several deficiencies, with two important ones for this WI being:

- The continuous squared exponential correlation decay function, nearly 100% correlates antenna pair (1 and 0.3152for 6 antennas) and is based on adhoc decision and on measurements of correlation across antenna elements. It models a LOS environment with very low diversity in receive direction (small angular spread).

- The symmetric property of the correlation matrix determines a broadside only receive/transmit direction, which is applied to every reflection, and an ULA-like antenna setup.

Therefore, the RAN4 TDL + antenna correlation model severely overestimates the rank deficiency, or rather ill-conditioning, of the channel matrix and this model shall not be used to evaluate the feasibility of full rank transmission.

Note: The X-pol extension of the TDL + antenna correlation model for 6Rx would apply the antenna correlation coefficient scaling only on a 3-antenna model (but twice), where the overestimation of rank deficiency is less pronounced. However, there are too many adhoc choices in this correlation model for it to be used in good conscience.

The RAN4 TDL + antenna correlation model severely overestimates the rank deficiency, or rather ill-conditioning, of the channel matrix and this model shall not be used to evaluate the feasibility of full rank transmission

Feasibility evaluation

Since the current demodulation antenna correlation model is not directly applicable for 6 Rx as there is no correlation matrix in TS 38.101-4, we propose to check feasibility (but not performance at this stage) by using the TR 38.827 CDL channel model, which models the antenna array as ULA X-pol with lambda/4 antenna distance.

Following this evaluation, we see the TPUT to reach 100%, i.e., full rank 6 Rx is practically feasible, as can be seen in Figure 1 below.

Figure 1: Relative TPUT vs. pre-EQ SNR, 8T6R rank 6 MCS17, SU-MMSE receiver, random precoding, TS 38.827 based CDLC UMa channel model with 3x1 ULA X-pol receiver antenna assumption (AE distance = lambda/4).

Since feasibility is given even for lambda/4, but the considered channel model is not yet commonly used, RAN4 shall come back to the final performance evaluation methodology in the performance part of this work item.

Demodulation of 6 Rx full rank transmission is feasible for handheld devices.

Way forward

Since the performance aspect of this work item has not yet commenced, it is advisable to defer this discussion until the performance evaluation phase begins. The output of the 'Spatial Channel Model' study item can then provide valuable insights into establishing appropriate performance requirements.

The performance part of this work item has not yet commenced

There is an ongoing study item on introduction of a spatial channel model for RAN4 performance requirements.

RAN4 shall discuss the performance requirements for 6Rx once the performance part commences.

Whether to use same requirement for handheld UE and FWA

The decision to use the same requirements for handheld UE and FWA in RAN4#111 is currently deferred to after the evaluation of 6 MIMO layer performance, in accordance with the Way Forward [1]:

Issue 3-1-3: Whether to use same requirement for handheld UE and FWA Way Forward: RAN4 to defer decision after evaluation of 6 MIMO layer performance for handheld and FWA devices.

Initial evaluation of 6 MIMO layer performance is not enough to make a decision if the same requirements can be used for both handheld UE and FWA, hence the decision should be deferred for now and fully handled in the performance part.

Reuse of handheld for FWA requirements cannot be decided on initial evaluation of 6 MIMO layer performance as results might change during the performance part in-depth analysis.

Defer decision to use same requirements for handheld UE and FWA to the performance part.

We further would note that requirements would not be defined per FWA or handheld UEs, but link level parameters could vary depending on the UE deployment used.

RAN4 demodulation performance requirements are not defined per UE deployments.

6-layer Support

6-layer Support

In RAN4#112 it was discussed if 6-layer shall be supported, with the following described in the Way Forward [1] :

Issue 3-2-1: 6-layer Support Way Forward: RAN4 to further discuss the following options. - Option 1: 6-layer support is feasible for handheld and FWA based on some company evaluations considering realistic antenna correlation assumptions and deployment scenarios. - Option 2: 6-layer support is feasible for FWA devices. - Option 3: Companies to align on the feasibility criteria and simulation assumptions to evaluate 6-Layer feasibility using the following as a starting point. - Feasibility criteria - Consider 4-layer vs 6-layer throughput performance. - Simulation assumptions - Only consider link-level simulation for performance evaluations. - Simulation assumptions provided in Annex A as a starting point. Companies to clarify the settings used in the simulation assumptions for BS correlation matrix and Tx EVM. - Option 4: 6-layer support is not feasible for handheld UEs.

During RAN4#112 several offlines were held whereby delegates from different companies were unable to come to agreement on what ‘feasible’ means.

During RAN4#112 RAN4 was unable to agree what is determined by Feasibility.

Nokia noted that several companies, including Qualcomm [9], Nokia [11], Oppo [8] had identified that indeed 6-layer MIMO was feasible on devices that were able to accommodate 6Rx, and some companies such as Huawei [10], showed that it was feasible, even if in their simulations they showed it had no gain over 4-layers in SNR performance.

Several individual companies showed that in their opinion 6-layer MIMO was indeed feasible during RAN4#112.

Finally, during RAN4#112 a series of simulation assumptions were loosely agreed upon, for which Nokia was not currently able to provide simulations for during RAN4#112-bis but will aim to have these ready for RAN4#113, potentially there will be some available for discussion during RAN4#112-bis itself.

However, Nokia does wish to raise again that the ‘bar’ to assess feasibility in this case is unreasonable, and indeed if any one company assesses this as feasible and if there is a market driver (as indicated by several companies supporting this feasibility) then such a capability should be considered feasible for the purpose of progression of the work item.

RAN4 shall continue to discuss feasibility, with a view that several companies have already indicated feasibility based on their own simulations, aligned simulations to be provided during RAN4#113.

6-layer Support as optional feature

In RAN4#112 it was discussed if 6-layer shall be supported as an optional feature, with the following described in the Way Forward [1] :

Issue 3-2-2: 6-layer Support as optional feature Way Forward: RAN4 to further discuss if 6-Layer support should be considered an optional feature.

As 6-layer support would require a band that already supports more than 4 layers, as follows for the “maxNumberMIMO-LayersPDSCH” in TS 38.822, as well as requiring signaling between the BS and UE to indicate support we fully believe that it should be an optional feature

6Rx will require a band that supports deployments with “maxNumberMIMO-LayersPDSCH” above 4.

6Rx will require signaling to indicate UE support.

RAN4 shall consider 6-layer support as optional feature.

Conclusion

The paper discusses the topic of 6 Layer MIMO evaluation in RAN4, and presents some of the Nokia views on this topic for discussion during RAN4#112bis.

The following Observations and Proposals were made:

Observation 1: In order to fully assess the performance of 6Rx a spatial channel model must be used.

Observation 2: The RAN4 TDL + antenna correlation model severely overestimates the rank deficiency, or rather ill-conditioning, of the channel matrix and this model shall not be used to evaluate the feasibility of full rank transmission

Proposal 1: Demodulation of 6 Rx full rank transmission is feasible for handheld devices.

Observation 3: The performance part of this work item has not yet commenced

Observation 4: There is an ongoing study item on introduction of a spatial channel model for RAN4 performance requirements.

Proposal 2: RAN4 shall discuss the performance requirements for 6Rx once the performance part commences.

Observation 5: Reuse of handheld for FWA requirements cannot be decided on initial evaluation of 6 MIMO layer performance as results might change during the performance part in-depth analysis.

Proposal 3: Defer decision to use same requirements for handheld UE and FWA to the performance part.

Observation 6: RAN4 demodulation performance requirements are not defined per UE deployments.

Observation 7: During RAN4#112 RAN4 was unable to agree what is determined by Feasibility.

Observation 8: Several individual companies showed that in their opinion 6-layer MIMO was indeed feasible during RAN4#112.

Proposal 4: RAN4 shall continue to discuss feasibility, with a view that several companies have already indicated feasibility based on their own simulations, aligned simulations to be provided during RAN4#113.

Observation 9: 6Rx will require a band that supports deployments with “maxNumberMIMO-LayersPDSCH” above 4.

Observation 10: 6Rx will require signaling to indicate UE support.

Proposal 5: RAN4 shall consider 6-layer support as optional feature.

- References

- R4-2414285, “WF on requirements for 6Rx”, AT&T, RAN4#112, Maastricht, August2024

- RP-241656,  “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, Huawei, RAN#104 Shanghai, China, June 2024

- TS 38.822 v17.1.0 “NR; User Equipment (UE) feature list”, June 2023

- R4-072223, “TP for 36.803 for MIMO correlation matrices for LTE”, Motorola, Ericsson, Nokia, NEC, NXP, InterDigitial, Agilent, RAN4#45, Jeju, Korea, November 2007

- R4-080620, “High Dimension E-UTRA MIMO Correlation matrixes”, Ericsson, RAN4#46-bis, Shenzhen, China, April 2008

- R4-1708533, “Discussion on SI for 8Rx”, Huawei, RAN4#84, Berlin, Germany, Aug 2017

- R4-2311072, “Discussion on PDSCH Demodulation Requirements for 8Rx”, Nokia, Nokia Shanghai Bell, RAN4#108, Aug 2023

- R4-2412352, “R19 Simulation results of 6Layer performance”, OPPO, RAN4#112, Aug 2024

- R4-2411680, “Discussion on the demodulation performance requirements for 6Rx devices”, Qualcomm, RAN4#112, Aug 2024

- R4-2412572, “On MIMO layer evaluation for 6Rx UE”, Huawei, HiSilicon, RAN4#112, Aug 2024

- R4-2411525, “Discussion on MIMO layer evaluation for 6Rx UE, Nokia, RAN4#112, Aug 2024