3GPP TSG RAN WG4 Meeting #112-bis		R4-2415300

# Hefei, China, 14 - 18 September, 2024

Source:	EchoStar, DISH Network, Terrestar, Thales

Title:	Methodology for n252 UE to UE coexistence evaluation

Agenda item:			5.17.2, 5.18.2

Document for:	Approval

# 1. Introduction

This paper analyses prior studies on UE-to-UE coexistence and proposes a methodology to evaluate the appropriate requirement for the UE to UE coexistence between (band)n252 and (band)n2/n25.

# 2. Background and analysis of prior studies

## 2.1 Background – previous discussion

The WI to define (band)n252 was agreed to in RAN#104, with the following as one of the objectives:

- Specify a new NR-NTN FDD band with a UE transmitting at 2000 – 2020 MHz and SAN transmitting at 2180-2200 MHz; - Perform necessary co-existence analysis, for example UE-UE coexistence, for adjacent band per RAN4 scope; - Consider whether assumptions for coexistence should be re-evaluated for the NTN scenario

In the last RAN4#112 meeting, the current generic TN requirement of -50dBm/MHz for UE Coexistence was discussed and it was clarified that the requirement is based on a 1m UEs separation based on R4-080710 [1]. It was understood that it may difficult to meet this requirement without significant amount of A-MPR. It was also noted that there are some bands for which the value has been relaxed in the past. Therefore, the following way forward [2] was agreed:

- Further study the UE-to-UE coexistence between NTN S-band UL and B2/n2, B25/n25 DL, considering: o Realistic usage scenarios o consider UE antenna gain for smartphone of -5.5dBi o UE-to-UE separations distance o probability of the UE-to-UE coexistence scenarios occurring - Companies are encouraged to examine the previous work in TR 38.863 for UE-to-UE coexistence noting that the solution might need to be re-evaluated for this work. - Companies are encouraged to investigate the background scenario for applicability of the -50dBm/MHz requirement to assess its applicability to TN-NTN coexistence (e.g. reference R4-080710)

Observation 1: 	RAN4 agreed to further study the UE-UE coexistence aspects between n252 and n25/n2 considering realistic usage scenarios and probability of occurrence along such scenarios.

## 2.2 Regulatory background

The following table shows the FCC spectrum allocation table [3] for frequency 1930 – 2025 MHz, which includes n2/n25 DL and n252 UL.

Table 1: FCC spectrum allocation table for 1930 - 2025

As seen from the table, MOBILE service and MOBILE-SATELLITE service are both “primary” services. Refer to [4] for the definition of “primary” service.

Per FCC rules, there is no requirement of -50 dBm/MHz for UE-to-UE coexistence. However, for CBRS protection the FCC NPRM [5] paragraph 86 uses probability threshold of 99% for determining interference noting that “Establishing a probability threshold is important because worst-case conditions for highly transient and unlikely RF interference events would otherwise establish an excessive constraint on neighboring radio service operations”.

Observation 2: 	MOBILE service and MOBILE-SATELLITE service are both “primary” services in their respective band.

Observation 3: 	FCC does not specify a limit of -50 dBm/MHz for UE-to-UE coexistence requirement.

Observation 4:		There is precedence that FCC has used 99% probability threshold for determining interference amongst mobile services in adjacent bands/channels.

## 2.3 Prior NTN-TN coexistence study in TR 38.863

As part of the NR NTN specifications, several companies had performed co-existence studies across various scenarios. Focusing on scenario 1, where TN DL is the aggressor and NTN DL is the victim, the following 2 results (figures 1 and 2) were submitted in RAN4#101-e [6,7]. Both figures 1 and 2 showed excessive interference from TN DL to NTN DL even when the NTN UE are dropped at the edge of the TN cell cluster. The service would have been spotty at best when the traditional 3GPP UE receiver requirements (ACS=33dB) were applied to NTN UE along with varying elevation angle between the satellite and NTN UE. From discussions in RAN4#101 through RAN4#102, it was agreed that isolation distance of 1.5km as currently defined in the TR was agreed [8,9,10]. No study was undertaken to assess the impact of DL service to NTN UEs (with ACS of 33 dB) inside the TN cluster.

Given the observed degradation in the performance at the edge of TN cluster, one can infer that within the TN cell clusters, the NTN performance would degrade further as a result of higher levels of aggregate interference due to contributions from a larger number of TN cells and smaller distances from TN BS.

Fig. 1: Throughput Loss - TN DL to NTN DL from R4-2118145 [6]

Fig.2: Throughput Loss - TN Urban macro to NTN   LEO600 from R4-2120626 [7]

Observation 5:		3GPP concluded that DL service to NTN UE with ACS of 33 dB at the edge of TN cluster coverage will be degraded.

Observation 6: 	3GPP determined that an isolation distance of 1.5 km is required to provide DL service to NTN UE with throughput loss criterion of less than 5%.

Observation 7: 	Given the degradation at the edge of the TN cluster, it can be inferred that the NTN service inside the TN cluster is likely going to be even further degraded due to larger levels of ACI. No assessment of impact to NTN DL within the TN cluster has been performed within 3GPP.

## 2.4 UE to UE coexistence evaluation methodology based on TR 25.942

During Rel-8 specifications development, interference cases of different RF system scenarios were studied and documented in TR 25.942 [11].

The impact of MS to MS interference is described in the section 4.2.2 of [11], as excerpted below. From this specification we understand that 3GPP prefers the methodology indicated as “another/second approach” where the probability of the two UEs coming close to each other is taken into account to calculate probability of interference.

TR 25.942 ## 4.2 Mobile Station to Mobile Station … ### 4.2.2 Co-located MS and intermodulation … c) Methodology The first approach is to assume that the two mobile stations are collocated, and to derive the minimum coupling loss. It requires to assume that both mobiles are transmitting at maximum power. Another approach can take into account the probability that the two mobiles come close to each other, in a dense environment, and to calculate the probability that the intermodulation products interfere with the receiver. The second approach should be preferred. d) Inputs required Minimum separation distance: 5 m[ for outdoor, 1 m for indoor] Mobile station density: [TBD] Base station density: [TBD in relation with MS density] Power control algorithm: [TBD] Maximum acceptable probability of interference: 2 %

Observation 8: 	3GPP recommended Monte Carlo approach for UE-UE coexistence study.

Observation 9:		3GPP recommended a maximum acceptable probability of interference to be 2%.

## 2.5 The current -50 dBm/MHz generic requirement for UE Coexistence

One of the discussions that seems to have led to the adoption of the current UE to UE coexistence of -50dBm/MHz can be found in R4-080710 [1]. It proposed a UE to UE separation of 1m for dense indoor where the likelihood of two UEs in close proximity could potentially be high, for a terrestrial network case. 
In the background problem of the [1] it was mentioned that:

“

- As the number of operating bands increase it is becoming impossible to maintain this generic -60dBm/3.84MHz approach.

- As more operating bands are added i.e. IMT-ADV, the problem will become worse and will further increase for multi -band terminal when roaming into other regions.

”

The situation has significantly evolved since 2008; a lot more bands, technologies (SBFD) as well as services (traditional mobile, MSS) are being supported by 3GPP specifications. For 4G and 5G, operators are providing services using multiple bands spanning low and mid frequency bands. The simultaneous use of low and mid bands minimizes the probability of service disruption (e.g., out of coverage, call drop, etc.) due to transient issues (e.g., interference, etc.) on any of the frequency bands.

The document also acknowledged that the interference due to OOB is a probability function, depending on the following:

- UE to UE path loss - Geographical separation

- Channel bandwidth and channel plan

- UE antenna and body losses

- Transmit power taking into account power control

- Receiver noise floor allowed due to interfering emission

- Scheduler control of  RB allocation, RB location and RB configured power

- Location of victim Rx relative to its own base station

In spite of this acknowledgment, a fixed 1 m separation based on dense indoor deployment was considered as the basis determining the UE spurious coexistence limit. However, it is important to note that NTN service is for outdoor use as the power budget is inadequate to overcome indoor building penetration losses.

Observation 10:	In 2008, -60 dBm/3.85 MHz limit was replaced with a -50 dBm/MHz developed using 1m separation distance with maximum UE Tx power without considering the probability of such a scenario occurring in the real world as recommended in TR 25.942.

Observation 11:	3GPP continues to use -50 dBm/MHz based on consideration of dense indoor deployments even as more bands and services are getting added, without fully accounting for diverse usage scenarios and the probability of interference – this places undue burden on newer bands/services that have the same regulatory status as legacy bands/services.

Observation 12:	Mobile operators are simultaneously utilizing low, mid and high frequency bands to ensure service availability.

Observation 13:	NTN power budget limitations constrains the availability of NTN service to outdoor use only.

## 2.6 SBFD study in TR 38.858

In some of the SBFD (Sub-band Full Duplex) operation scenarios, there are cases where two UEs in close proximity would be configured such that UL allocation in a sub-band of the first UE becomes adjacent to DL allocation of the second UE. The study concluded that there is no issue related to existing UE RF requirements has been identified, and therefore the existing UE RF requirement is reused [12].

Fig. 3: SBFD coexistence scenarios

TR 38.858 ## 10.2 Impact on UE RF requirements RAN4 has analyzed inter-UE co-channel inter-subband CLI modeling and adjacent channel CLI modelling. In Table 10.2-1, in-channel RF assumptions (e.g., IBE and in-channel adjacent subband selectivity) and adjacent channel RF assumptions (e.g., ACLR and ACS) for the coexistence study are presented for UE Tx/Rx interference modelling. … Based on the study, reusing existing UE RF requirements is the conclusion of the study phase, since no issues related to existing UE RF requirements has been identified in the co-existence study.

Observation 14:		In Release 18, 3GPP concluded that there are no co-existence issues for inter-UE co-channel inter-subband CLI modeling and adjacent channel CLI modeling.

# 3. Proposal for methodology to evaluate n252 UE-to-UE coexistence

Based on prior studies as part of Scenario 1 in TR 38.863, it is unclear if NTN service is possible within a TN cluster due to lack of a reliable NTN DL. The NTN UE – TN UE coexistence study within the TN cluster only makes sense if it can be established that there will be reliable NTN service in the TN cluster in presence of ACI from the DL operation within the TN cluster.

To further progress the work and ensure appropriate protection for n2/n25 while avoiding undue burden of significant power back-off for n252, we propose the following methodology of evaluation steps for n252 UE coexistence.

Step 1: 	Evaluate whether reliable NTN service is possible within the coverage of a TN network operating on the adjacent band. (Scenario 1 in TR 38.863 with NTN UEs dropped within the coverage area of the TN cluster instead of at isolation distance of 1.5 km with varying elevation angle between the NTN UEs and the satellite). 
If reliable NTN service in the TN cluster is not possible, estimate the isolation distance at which NTN service is possible. This estimation will serve as input to the execution of procedure specified for Scenario 5 in TR 38.863 (Step 2a) or Step 2b.

Step 2a:  Re-run simulation of Scenario 5 in the TR 38.863, using preferred approach specified in TR 25.942, and using the parameters specified in TR 38.863, ITU-R M.2292 and R4-080710 (antenna gain+body loss parameters).

- Identify the outdoor TN UE densities using ITU-R M.2292 
(Urban cell: 3 TN UEs / 5 MHz / sq. km, outdoor % = 30%; Rural cell; 0.17 TN UEs / 5 MHz / sq. km, outdoor % = 50%)

- Identify NTN UE density per spot beam assuming a 20 MHz channel and PRB allocation per UE agreed to in TR 38.863. (NTN UEs/spot beam = 106/2 = 53)

- Identify the ACIR that would result in 5% loss to TN DL throughput

- Using the estimated ACIR, calculate the required NTN UE ACLR (TN UE ACS = 33 dB)

- Find the required SEM by adding the delta of the required NTN UE ACLR and 3GPP specified NTN UE ACLR.

- Determine the A-MPR based on the updated emission limit computed in the previous step utilizing the Tx duplexer filter rejection (if any)

Step 2b:	(As alternative of step 2a)
If RAN4 instead agrees to utilize the distance-based approach for UE–UE coexistence (instead of the method agreed to in TR 38.863 and as proposed in Step 2a), after the evaluation proposed in step 1, estimate an appropriate distance value based on probability of interference of 2% (as recommended by 3GPP TR 25.942) (or 1% as per FCC 99% probability rule).

- Identify the outdoor TN UE densities using ITU-R M.2292 
(Urban cell: 3 TN UEs / 5 MHz / sq. km, outdoor % = 30%; Rural cell: 0.17 TN UEs / 5 MHz / sq. km, outdoor % = 50%)

- Identify NTN UE density per spot beam assuming a 20 MHz channel and PRB allocation per UE agreed to in TR 38.863. (NTN UEs/spot beam = 106/2 = 53)

- Perform a Monte Carlo simulation to determine the histogram of UE-UE distance

- Identify the distance exceeded by 98% (or 99% per FCC) of the samples

- Use the identified distance along with appropriate antenna gain, body loss from R4-080710 to compute the UE-UE coexistence spurious emission limit

- Determine the A-MPR, based on the computed UE-UE coexistence spurious emission limit, utilizing Tx duplexer filter rejection (if any)

Proposal: 	RAN4 to agree on the proposed methodology of Monte Carlo simulation according to the above step 1 followed by step 2a or 2b.

# 4. Conclusion

Regulatory background and prior studies on UE-to-UE coexistence for NTN-TN and within TN were analysed. The following observations were made:

Observation 1: 	RAN4 agreed to further study the UE-UE coexistence aspects between n252 and n25/n2 considering realistic usage scenarios and probability of occurrence along such scenarios.

Observation 2: 	MOBILE service and MOBILE-SATELLITE service are both “primary” services in their respective band.

Observation 3: 	FCC does not specify a limit of -50 dBm/MHz for UE-to-UE coexistence requirement.

Observation 4:		There is precedence that FCC has used 99% probability threshold for determining interference amongst mobile services in adjacent bands/channels.

Observation 5:	3GPP concluded that DL service to NTN UE with ACS of 33 dB at the edge of TN cluster coverage will be degraded.

Observation 6:     3GPP determined that an isolation distance of 1.5 km is required to provide DL service to NTN UE with throughput loss criterion of less than 5%.

Observation 7:     Given the degradation at the edge of the TN cluster, it can be inferred that the NTN service inside the TN cluster is likely going to be even further degraded due to larger levels of ACI. No assessment of impact to NTN DL within the TN cluster has been performed within 3GPP.

Observation 8: 	3GPP recommended Monte Carlo approach for UE-UE coexistence study.

Observation 9:		3GPP recommended a maximum acceptable probability of interference to be 2%.

Observation 10:	In 2008, -60 dBm/3.85 MHz limit was replaced with a -50 dBm/MHz developed using 1m separation distance with maximum UE Tx power without considering the probability of such a scenario occurring in the real world as recommended in TR 25.942.

Observation 11:	3GPP continues to use -50 dBm/MHz based on consideration of dense indoor deployments even as more bands and services are getting added without considering usage scenarios and the probability of interference – this places undue burden on bands/services that have the same regulatory status as legacy bands/services.

Observation 12:	Mobile operators are simultaneously utilizing low, mid and high frequency bands to ensure service availability.

Observation 13:	NTN power budget limitations constrains the availability of NTN service to outdoor use only.

Observation 14:	In Release 18, 3GPP concluded that there are no co-existence issues for inter-UE co-channel inter-subband CLI modeling and adjacent channel CLI modeling.

To further progress the work and to ensure an appropriate amount of protection to n2/n25 while avoiding undue burden of too severe power back-off for n252, the following is proposed:

Proposal: 	RAN4 to agree on the proposed methodology of Monte Carlo simulation according to step 1 followed by step 2a or 2b.

Step 1: 	Evaluate whether reliable NTN service is possible within the coverage of a TN network operating on the adjacent band. (Scenario 1 in TR 38.863 with NTN UEs dropped within the coverage area of the TN cluster instead of at isolation distance of 1.5 km with varying elevation angle between the NTN UEs and the satellite). 
If reliable NTN service in the TN cluster is not possible, estimate the isolation distance at which NTN service is possible. This estimation will serve as input to the execution of procedure specified for Scenario 5 in TR 38.863 (Step 2a) or Step 2b.

Step 2a:  Re-run simulation of Scenario 5 in the TR 38.863, using preferred approach specified in TR 25.942, and using the parameters specified in TR 38.863, ITU-R M.2292 and R4-080710 (antenna gain+body loss parameters).

- Identify the outdoor TN UE densities using ITU-R M.2292 
(Urban cell: 3 TN UEs / 5 MHz / sq. km, outdoor % = 30%; Rural cell; 0.17 TN UEs / 5 MHz / sq. km, outdoor % = 50%)

- Identify NTN UE density per spot beam assuming a 20 MHz channel and PRB allocation per UE agreed to in TR 38.863. (NTN UEs/spot beam = 106/2 = 53)

- Identify the ACIR that would result in 5% loss to TN DL throughput

- Using the estimated ACIR, calculate the required NTN UE ACLR (TN UE ACS = 33 dB)

- Find the required SEM by adding the delta of the required NTN UE ACLR and 3GPP specified NTN UE ACLR.

- Determine the A-MPR based on the updated emission limit computed in the previous step utilizing the Tx duplexer filter rejection (if any)

Step 2b:	(As alternative of step 2a)
If RAN4 instead agrees to utilize the distance-based approach for UE–UE coexistence (instead of the method agreed to in TR 38.863 and as proposed in Step 2a), after the evaluation proposed in step 1, estimate an appropriate distance value based on probability of interference of 2% (as recommended by 3GPP TR 25.942) (or 1% as per FCC 99% probability rule).

- Identify the outdoor TN UE densities using ITU-R M.2292 
(Urban cell: 3 TN UEs / 5 MHz / sq. km, outdoor % = 30%; Rural cell: 0.17 TN UEs / 5 MHz / sq. km, outdoor % = 50%)

- Identify NTN UE density per spot beam assuming a 20 MHz channel and PRB allocation per UE agreed to in TR 38.863. (NTN UEs/spot beam = 106/2 = 53)

- Perform a Monte Carlo simulation to determine the histogram of UE-UE distance

- Identify the distance exceeded by 98% (or 99% per FCC) of the samples

- Use the identified distance along with appropriate antenna gain, body loss from R4-080710 to compute the UE-UE coexistence spurious emission limit

- Determine the A-MPR, based on the computed UE-UE coexistence spurious emission limit, utilizing Tx duplexer filter rejection (if any)

# Reference

- R4-080710, “TS36.101: TP for UE Spurious emission limits”, Motorola, RAN4#46bis

- R4-2414287, “WF on Rel-19 FR1 NTN bands”, Moderator (Inmarsat), RAN4#112

- https://transition.fcc.gov/oet/spectrum/table/fcctable.pdf

- https://www.ecfr.gov/current/title-47/part-2#p-2.104(d)

- https://docs.fcc.gov/public/attachments/FCC-14-49A1.pdf

- R4-2118145, “Coexistence simulation assumptions and results for NTN”, Qualcomm, RAN4#101-e

- R4-2120626, “NTN: Coexistence simulations results – satellite scenarios”, Ericsson, RAN4#101-e

- R4-2120741, “Email discussion summary for [101-e][310] NTN_Solutions_Part2”, Moderator (Samsung), RAN4#101-e

- R4-2207347, “WF on [309] NTN_Solutions_Part2”, Moderator (Samsung), RAN4#102-e

- R4- 2207351, “Draft text proposal to update TR 38.863 Chapter 6”, Samsung, RAN4#102-e

- TR 25.942, “Radio Frequency (RF) system scenarios”

- TR 38.858, “Study on Evolution of NR Duplex Operation”