3GPP TSG-RAN WG4 #112bis	R4-2415301

Hefei, China, October 14-18, 2024

Agenda item:	6.19.3.1		Enhancement for UE-initiated/Event-driven Beam Management

Source: 	Qualcomm Incorporated

Title: 	On RRM requirements for enhanced UE-initiated/event-driven beam management

Document for:	Discussion

# Introduction

In RAN4 #112, RAN4 work on Rel-19 MIMO Phase 5 was started [1]. The work plan and the way forward have been agreed in [2]. The following agreements and way forwards have been reached in RAN4 #112:

Issue 2-1-2: scope of the scenario of event-triggered L1 beam reporting

- Agreement: intra-cell and inter-cell for FR2

- Note: If specific requirements cannot be applicable, discuss it case by case

Issue 2-1-3: whether/how to define new delay requirements such as measurement reporting delay/measurement period

- Agreement: RAN4 will define event triggered measurement reporting delay requirement

- Further discuss:

  - Whether to support both mode A and mode B

  - Whether to differentiate between both mode A and mode B

  - Other related aspects

Issue 2-1-5: measurement metrics for beam report

- Agreement: Based on L1-RSRP

- Note: others can be revisited if RAN1 have further agreement

Issue 2-1-6: L1-RSRP for CSI-RS measurement

- Enhancement for UE-initiated/even-driven beam management

- Agreement: define CSI-RS based L1-RSRP measurement requirements. CSI-RS is periodic CSI-RS

Issue 2-1-7: whether to specify new unified TCI switching delay

- Way forward: FFS on whether to define new TCI state switching delay or existing requirements are sufficient

Issue 2-1-8: whether to filter L1-RSRP

- Way forward: FFS: RRM requirements for event-triggered L1 reporting without time-window-based filtering or other filtering configured

Issue 2-1-9: impact of beam on known/unknown conditions for TCI states

- Way forward: FFS. Depends on RAN1 further progress

Issue 2-1-10: whether to enhance beam sweeping in event triggered L1 beam reporting

- Way forward: FFS. The assumption of N = 8 fine beam sweeping can be enhanced to realize more efficient BM. For example, the combination of coarse Rx beam and fine Rx beam would facilitate faster Tx and Rx beam refinement.

Issue 2-1-11: whether to define new accuracy requirements by enhancement for UE-initiated/even-driven beam management

- Way forward:

  - Option 1: No new accuracy requirements for existing L1 measurements are required.

  - Option 2: RAN4 need to discuss whether to update legacy accuracy requirements.

In this paper, we address the issues above of the way forward as defined in [2].

# Discussion

In RAN4 #112, it was agreed in Issue 2-1-3 that new event triggered measurement reporting delay requirements will be defined by RAN4. The difference to existing event-triggered reporting delay requirements for mobility is that now L1 measurements instead of L3 measurements must be applied since the UE-initiated/event driven beam management is based on L1 measurements. Another open question in this context is whether reporting delay requirements will be defined for reporting mode A as well as for reporting mode B and whether both it is possible to differentiate in the requirements between both reporting modes. The time between the triggering event and the transmission of the 1st PUCCH is common in both mode A and mode B as can be seen in Figure 2-1.

Issue 2-1-3: Whether/how to define new delay requirements such as measurement reporting delay/measurement period?

Agreement:

- RAN4 will define Event triggered measurement reporting delay requirement

- Further discuss:

  - Whether to support both mode A and mode B

  - Whether to differentiate mode A and mode B

  - Other related aspects

Hence, we propose to use the time when the 1st PUCCH can be sent as a starting point for initial analysis how to define the requirements. In a second step it can be analyzed whether it is possible to differentiate further between mode A and mode B.

Figure 2-1: Illustration of Beam Report Mode A and Mode B

Observation 1: The time between the triggering event and the transmission of the 1st PUCCH is common in both mode A and mode B.

Proposal 1: RAN4 should use the time when the 1st PUCCH can be sent as the reference point for the definition of the delay requirements (Issue 2-1-3).

Additional delays after the 1st PUCCH both in mode A as well as in mode B do not depend on the measurement itself but on gNB behavior how quickly resources are allocated in mode A or on the configuration of the configured grant resources in mode B and are outside control of the UE. Hence, these delays should not be part of the delay requirement.

Figure 2-2: Illustration of Contributions to Measurement Reporting Delay

In Figure 2-1, the first step of the reporting mode A and mode B is shown. The PUCCH is sent to the gNB to indicate to the gNB that the trigger event has been identified by the UE. This PUCCH can only be sent after the event has been detected. Since measurements happen on periodic CSI-RS or SSB resources, there can be a certain gap till the measurement can start. Figure 2-2 illustrates the contributions to the measurement reporting delay.

Because of the periodicity of resources, a certain delay Tgap can occur till the UE can start the L1 measurements for the triggered event. Depending how long the L1 measurements take to identify the event, there is also a certain gap from the end of the measurements till the next PUCCH opportunity, called TRS in Figure 2-2, to inform the network. Both delays Tgap and TRS need to be taken into account for defining the measurement reporting delay.

Observation 2: Because of the periodicity of resources used for measurements, a certain delay Tgap can occur till the UE can start the L1 measurements for the triggered event.

Observation 3: Depending how long the L1 measurements take to identify the event, there is also a certain gap called TRS from the end of the measurements till the next PUCCH opportunity to inform the network.

Observation 4: Both delays Tgap and TRS contribute to the overall measurement reporting delay. Their magnitude depend on the periodicity of the CSI-RS / SSB resources and the PUCCH resources.

Proposal 2: Both delays Tgap (time from occurrence of the event till first available resource for measurement) and TRS (time from detection of event in the UE till first available PUCCH resource for event reporting) need to be taken into account for defining the measurement reporting delay (Issue 2-1-3).

The third component that contributes to the overall measurement reporting delay is the required time to identify the event, called Tmeas in Figure 2-2. Tmeas is analyzed further in the next step. So far, the agreement in RAN1 is that L1-RSRP is supported as a measurement quantity. Defining L1-SINR as a second measurement is under discussion in RAN1 but has not yet been agreed. Requirements for L1-RSRP reporting based on SSB and CSI-RS measurement are already defined in [3], section 9.5 fulfilling the absolute and relative measurement accuracy requirements defined [3], section 10.1. The measurement periods TL1-RSRP_Measurement_Period_SSB and TL1-RSRP_Measurement_Period_CSI-RS are defined for FR2. RAN4 should investigate whether those measurement periods can be reused for defining measurement reporting delay requirement. No additional accuracy requirements are required for L1-RSRP.

Observation 5: Requirements for L1-RSRP reporting based on SSB and CSI-RS measurement are already defined in [3], section 9.5 fulfilling the absolute and relative measurement accuracy requirements defined [3], section 10.1.

Proposal 3: RAN4 should investigate whether those measurement periods can be reused for defining measurement reporting delay requirements for UE-initiated/event-driven beam management (Issue 2-1-3).

One of the open issues in the way forward [3] is whether any new accuracy requirements need to be defined. Currently, the following events are supported in RAN1.

- Event-2: quality of at least one new beam becomes a threshold value better than the current beam

- Event-1: quality of the current beam is worse than a certain threshold

- Event-7: quality of at least one new beam becomes a threshold value better than the RS derived from the activated TCI state with the M-th best quality

All these events allow to measure current and new beam and derive the decision whether the event occurs from L1-RSRP measurement. In our opinion it is not needed to tighten the measurement accuracy since the existing L1-RSRP accuracy requirements automatically apply if the existing measurement periods for L1-RSRP can be reused.

Issue 2-1-11: Whether to define new accuracy requirements by Enhancement for UE-initiated/event-driven beam management?

<Way forward>

- Option 1: No new accuracy requirements for existing L1 measurements are required.

Option 2: RAN4 need to discuss whether to update legacy accuracy requirements.

Proposal 4: No additional accuracy requirements are needed for L1-RSRP measurements (Issue 2-1-11).

One of the open issues in the way forward [3] is whether any kind of filtering should be applied to the L1 measurements. Since measurement period for L1-RSRP can be reused, no additional filtering can be applied.

Issue 2-1-8: Whether to filter L1-RSRP?

<Way forward>

FFS: RRM requirements for event-triggered L1 reporting without time-window-based filtering or other filtering configured.

Proposal 5: No additional time-window-based filtering or other filtering should be applied for the L1-RSRP measurements. (Issue 2-1-8).

Another discussion in RAN4 #112 was whether CSI-RS should be used for the inter-cell scenario and whether TRS shall be used for L1-RSRP measurements.

Issue 2-1-6: L1-RSRP for CSI-RS measurement?

Enhancement for UE-initiated/event-driven beam management

Agreement: Define CSI-RS based L1-RSRP measurement requirements. CSI-RS is periodic CSI-RS.

For the question whether to use TRS for L1-RSRP measurement, RAN4 should wait for RAN1 to make further progress. There is no need for RAN4 to spend time on this point as of now. In Rel-17, measurement requirements for inter-cell beam management have only been introduced for SSB based L1-RSRP. We propose not to introduce in Rel-19 CSI-RS based L1-RSRP measurements for inter-cell beam management as long as RAN1 has not identified a clear use case.

Observation 6: RAN4 should wait for RAN1 to make further progress whether TRS can be used for L1-RSRP measurement. There is no need for RAN4 to spend time on this point as of now.

Observation 7: In Rel-17, measurement requirements for inter-cell beam management have only been introduced for SSB based L1-RSRP but not for CSI-RS based L1-RSRP.

Proposal 6: RAN4 should not introduce in Rel-19 CSI-RS based L1-RSRP measurements for inter-cell beam management as long as RAN1 has not identified a clear use case (Issue 2-1-6).

One goal in the work item is to reduce the latency of the beam switch.

Issue 2-1-9: Impact of beam on known/unknown conditions for TCI states?

<Way forward>

FFS. Depends on RAN1 further progress.

RAN1 has agreed that the current beam is a beam corresponding to the indicated TCI state in the Rel-17 unified TCI framework, i.e., the current serving beam of the PDCCH/PDSCH. For the new beam(s) to be measured it has been agreed that they will be explicitly configured but details are still to be down selected in RAN1 in the upcoming meeting. The action to be taken by the network after the beam report is left open, however it can be expected that the network triggers a TCI state switch to a new target TCI. Figure 2-3 illustrates the definition of TCI state switch delay in [3]. It is the delay between the reception of the TCI switch command and the time when PDCCH with the new target TCI is receivable.

Figure 2-3: Illustration of TCI State Switch Delay

RAN1 allows to configure the reference signals of the new beam(s) in such a way that also TCI states that are not yet activated can be measured. If such a new beam corresponding to a TCI state that is not yet activated leads to an event trigger, the network has to switch the TCI state with a MAC CE.

Depending on whether the target TCI state is known, or unknown, different TCI state switch delays are defined in [3]. The condition of a known TCI state includes that the TCI switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement and that the UE has sent a valid L1-RSRP report for the target TCI state. The existing delay requirement already covers the case that the target TCI state is known but not yet in the active TCI state list. This adds additional switch delay. If there is no L1-RSRP report before the TCI state switch command, the TCI state is unknown which adds additional delay in the TCI state switch. The existing conditions for known and unknown TCI state can be reused.

Observation 8: The existing conditions for known and unknown TCI state can be reused for UE-initiated/event-triggered beam management.

It should be further studied in RAN4 whether any new required TCI state switch delays need to be defined or existing requirements are sufficient.

Proposal 7: RAN4 should investigate whether any new required TCI state switch delays need to be defined or existing requirements are sufficient (Issue 2-1-9).

The final discussion in RAN4 #112 was whether to enhance the Rx beam sweeping. Currently, the assumption is that N = 8 beams are swept for the L1-RSRP report. This discussion should be postponed till the baseline requirement for delay and measurement period have taken place.

Issue 2-1-10: Whether to enhance beam sweeping in event triggered L1 beam reporting?

<Way forward>

FFS: The assumption of N = 8 fine beam sweeping can be enhanced to realize more efficient BM. For example, the combination of coarse Rx beam and fine Rx beam would facilitate faster Tx and Rx beam refinement.

Proposal 8: Discussions on beam sweeping enhancements should be postponed till the baseline requirement for delay and measurement period have taken place (Issue 2-1-10).

# Conclusions

In this paper, we provided our views on several of the open issues identified in RAN4 #112. Our observations and proposals are summarized as follows:

Observation 1: The time between the triggering event and the transmission of the 1st PUCCH is common in both mode A and mode B.

Proposal 1: RAN4 should use the time when the 1st PUCCH can be sent as the reference point for the definition of the delay requirements (Issue 2-1-3).

Observation 2: Because of the periodicity of resources used for measurements, a certain delay Tgap can occur till the UE can start the L1 measurements for the triggered event.

Observation 3: Depending how long the L1 measurements take to identify the event, there is also a certain gap called TRS from the end of the measurements till the next PUCCH opportunity to inform the network.

Observation 4: Both delays Tgap and TRS contribute to the overall measurement reporting delay. Their magnitude depend on the periodicity of the CSI-RS / SSB resources and the PUCCH resources.

Proposal 2: Both delays Tgap (time from occurrence of the event till first available resource for measurement) and TRS (time from detection of event in the UE till first available PUCCH resource for event reporting) need to be taken into account for defining the measurement reporting delay (Issue 2-1-3).

Observation 5: Requirements for L1-RSRP reporting based on SSB and CSI-RS measurement are already defined in [3], section 9.5 fulfilling the absolute and relative measurement accuracy requirements defined [3], section 10.1.

Proposal 3: RAN4 should investigate whether those measurement periods can be reused for defining measurement reporting delay requirements for UE-initiated/event-driven beam management (Issue 2-1-3).

Proposal 4: No additional accuracy requirements are needed for L1-RSRP measurements (Issue 2-1-11).

Proposal 5: No additional time-window-based filtering or other filtering should be applied for the L1-RSRP measurements. (Issue 2-1-8).

Observation 6: RAN4 should wait for RAN1 to make further progress whether TRS can be used for L1-RSRP measurement. There is no need for RAN4 to spend time on this point as of now.

Observation 7: In Rel-17, measurement requirements for inter-cell beam management have only been introduced for SSB based L1-RSRP but not for CSI-RS based L1-RSRP.

Proposal 6: RAN4 should not introduce in Rel-19 CSI-RS based L1-RSRP measurements for inter-cell beam management as long as RAN1 has not identified a clear use case (Issue 2-1-6).

Observation 8: The existing conditions for known and unknown TCI state can be reused for UE-initiated/event-triggered beam management.

Proposal 7: RAN4 should investigate whether any new required TCI state switch delays need to be defined or existing requirements are sufficient (Issue 2-1-9).

Proposal 8: Discussions on beam sweeping enhancements should be postponed till the baseline requirement for delay and measurement period have taken place (Issue 2-1-10).

# References

- R4-2411815, Topic summary for [112][220] NR_MIMO_Ph5, RAN4 #112, August 2024

- R4-2414035, WF on RRM requirements for NR_MIMO_Ph5, RAN4 #112, August 2024

- TS 38.133, Requirements for support of radio resource management, v18.6.0, June 2024