3GPP TSG-RAN WG4 #112bis	R4-2415302

Hefei, China, October 14-18, 2024

Agenda item:	6.19.3.2		Other RRM Requirements

Source: 	Qualcomm Incorporated

Title: 	Views on additional RRM requirements for NR MIMO Phase 5

Document for:	Discussion

# Introduction

In RAN4 #112, the following way forward for the definition of the RRM core requirements has been agreed [2]. It was agreed that RRM core requirements need to be defined for UE-initiated/event-driven beam management and that no RRM core requirements need to be defined for Type-I & Type-II codebook refinements and CRI-based CSI refinements.

For aperiodic standalone coherent joint-transmission reporting, 3-antennaport codebook-based UL transmission and asymmetric DL sTRP/UL mTRP it is FFS whether RRM core requirements need to be defined

| Topic | Description | RRM impact |
| --- | --- | --- |
| UE-initiated/event-driven beam management | UE-initiated/event-driven beam management | YES |
| CSI enhancement | Type-I codebook refinement supporting up to a total of 128 CSI-RS ports | NO |
|  | Type-II codebook refinement supporting up to a total of 128 CSI-RS ports | NO |
|  | CRI-based CSI refinement for up to 128 CSI-RS ports | NO |
|  | Aperiodic standalone CJT calibration reporting | FFS |
| 3TX | 3-antenna-port codebook-based UL transmission | FFS |
| Asymmetric DL sTRP/UL mTRP |  | FFS |

In this paper, we share our view whether core requirements should be defined for the topics above that are marked as FFS.

# Discussion

## 3-Antenna-port Codebook-based UL Transmission

As we already mentioned in our paper in RAN4 #112, we do not see that the non-coherent uplink codebook has RRM impact. Hence, no requirements need to be defined in TS 38.133. It should be discussed further in RAN4 whether base station performance requirements should be defined in TS 38.104.

Observation 1: Non-coherent uplink codebook for three antenna ports does not have RRM impact.

Proposal 1: No RRM core or performance requirements need to be defined for non-coherent uplink codebook

Proposal 2: It should be discussed further in RAN4 whether base station performance requirements should be defined in TS 38.104.

## Aperiodic Standalone CJT Calibration Reporting

One of the open points in the way forward in RAN4 #112 is whether RRM requirements for reporting enhancements for CJT deployments should be defined at all. The second question is whether RRM core (Issue 2-3-1) or performance (Issue 2-3-2) requirements should be defined if it is agreed that RRM requirements will be introduced.

Reporting enhancements for coherent joint transmission deployments under non-ideal synchronization and backhaul deal with reporting of additional information by the UE for delay offset compensation, per-TRP frequency offset compensation and per TRP DL/UL Rx-Tx phase compensation to enhance reciprocity.

Issue 2-3-1: Whether RRM core requirements impacts exist by UE reporting enhancement for CJT calibration?

<Way forward>

- Option 1: No RRM core requirements impact.

- Option 2: RAN4 to define the measurement delay or measurement behaviour for the aperiodic standalone CJT reporting.

Issue 2-3-2: Whether RRM performance requirements impacts exist by UE reporting enhancement for CJT calibration?

<Way forward>

- Option 1: No RRM performance requirements impact.

- Option 2: No accuracy requirements. Potential mapping tables are needed if any.

- Option 3: Define accuracy requirements.

- Option 4: Define accuracy requirements and report mapping tables.

In our view these reporting enhancements do not impact the core requirements of MIMO Phase 5 but rather impact the performance part of this work item. Since the reporting enhancements impact performance, it needs to be discussed whether these performance aspects should be treated under RRM performance or under demodulation performance.

In our view, these reporting enhancements are similar to the time domain channel properties (TDCP) in Rel-18. We think that the main specification impact is do define accuracy requirements for this additional reporting but how to apply this information in the base station to enhance performance is in our opinion out of the scope of the 3GPP specification. Hence, no PDSCH throughput tests should be defined for these reporting enhancements. Since RRM performance specification deals with accuracy requirements for measurement reports, specification impact of reporting enhancement for CJT deployments should be covered in TS 38.133 and not in TS 38.101-4.

Observation 2: Reporting enhancements for CJT deployments impact performance requirements but not the RRM core requirements.

Observation 3: Performance impact by reporting enhancements for CJT deployments can be appropriately covered by defining accuracy requirements. Defining PDSCH throughput requirements for CJT reporting enhancements is beyond the scope of 3GPP.

Proposal 3: Specification impact of reporting enhancement for CJT deployments should be covered in TS 38.133 under RRM performance and not in 38.101-4 under demodulation performance.

According to the TU allocation of the work item, work on performance aspects for MIMO Phase 5 starts in RAN4 #115 in May ’25. Hence, we propose to postpone working on defining requirements for reporting enhancements for CJT till the RAN4 #115 meeting.

Proposal 4: The work on defining requirements for reporting enhancements for CJT deployments under non-ideal synchronization and backhaul should start in RAN4 #115 in May ’25.

## Asymmetric DL sTRP/UL mTRP

In Issue 2-5-3 the RRM impact of supporting two Tas was left open. The reason for that was that RAN1 had not yet decided whether two TA is supported or not. Meanwhile, RAN#105 has decided that Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources are supported [5]. Hence, RAN4 can wait for further progress in RAN1 on two TA support before discussion time is spent on this topic.

Issue 2-5-3: Whether RRM impacts exist by 2TA enhancement?

<Way forward>

FFS on whether there are RRM impacts by 2TA enhancement.

Proposal 5: RAN4 to wait for further progress in RAN1 on two TA support before discussion time is spent on this topic.

The next open topic was whether there is RRM core impact of active uplink TCI switching.

Issue 2-5-4: RRM core impacts of Active uplink TCI state switching delay for unified TCI?

<Way forward>

- Option 1: The legacy requirements can be reused.

- Option 2: The legacy requirements cannot be reused.

- Option 3: RAN4 to discussion how to update R17/R18 requirement for asymmetric DL sTRP/UL mTRP scenarios

In RAN1 #117, the following conclusion was drawn that Rel-17 unified TCI/IBCM and Rel-18 unified TCI framework is reused. Since the UL mTRP do not send any DL signals, the TCI states of the UL mTRP do not have any source RS that is identical to the PL-RS or do not have any source RS that is QCLed Type D to the PL-RS. Hence, the existing requirements for active uplink TCI state switching in [3] cannot be directly applied to DL sTRP / UL mTRP.

Observation 4: The existing requirements for active uplink state switching in [3] cannot be directly applied to DL sTRP / UL mTRP if the uplink TCI switches involve the UL mTRPs.

Since the uplink TCI switching delays cannot be applied, there are no requirements how long a beam switch for and to the UL mTRP would take. RAN4 should discuss whether new requirements for uplink TCI switching should be introduced.

Proposal 6: RAN4 should discuss whether new requirements for uplink TCI switching should be introduced involving the UL mTRP.

Issue 2-5-5: Whether to define RRM core requirements of pathloss offset update?

Agreement:

Whether to define RRM core requirements of pathloss offset update?

- Not define RRM requirement for MAC CE decoding delay.

The agreement can be revisited depending on further RAN1 progress.

Since the PL-RS is transmitted from the DL sTRP and also UL signals can be transmitted to the DL sTRP without applying the pathloss offset the requirements for the pathloss reference signal switch delay can be directly applied for the DL sTRP.

Observation 5: Since the PL-RS is transmitted from the DL sTRP and also UL signals can be transmitted to the DL sTRP without applying the pathloss offset the requirements for the pathloss reference signal switch delay can be directly applied for the DL sTRP.

This also allows to reuse the uplink TCI switching delay requirements for switches contained to UL transmissions to the DL sTRP and do not involve the UL mTRP.

Observation 6: The uplink TCI switching delay requirements can be applied for switches contained to UL transmission to the DL sTRP that do not involve the UL mTRP.

# Conclusions

In this paper, we provided our views on several of the open issues identified in RAN4 #112. Our observations and proposals are summarized as follows:

3-Antenna-port Codebook-based UL Transmission

Observation 1: Non-coherent uplink codebook for three antenna ports does not have RRM impact.

Proposal 1: No RRM core or performance requirements need to be defined for non-coherent uplink codebook

Proposal 2: It should be discussed further in RAN4 whether base station performance requirements should be defined in TS 38.104.

Aperiodic Standalone CJT Calibration Reporting

Observation 2: Reporting enhancements for CJT deployments impact performance requirements but not the RRM core requirements.

Observation 3: Performance impact by reporting enhancements for CJT deployments can be appropriately covered by defining accuracy requirements. Defining PDSCH throughput requirements for CJT reporting enhancements is beyond the scope of 3GPP.

Proposal 3: Specification impact of reporting enhancement for CJT deployments should be covered in TS 38.133 under RRM performance and not in 38.101-4 under demodulation performance.

Proposal 4: The work on defining requirements for reporting enhancements for CJT deployments under non-ideal synchronization and backhaul should start in RAN4 #115 in May ’25.

Asymmetric DL sTRP/UL mTRP

Proposal 5: RAN4 to wait for further progress in RAN1 on two TA support before discussion time is spent on this topic.

Observation 4: The existing requirements for active uplink state switching in [3] cannot be directly applied to DL sTRP / UL mTRP if the uplink TCI switches involve the UL mTRPs.

Proposal 6: RAN4 should discuss whether new requirements for uplink TCI switching should be introduced involving the UL mTRP.

Observation 5: Since the PL-RS is transmitted from the DL sTRP and also UL signals can be transmitted to the DL sTRP without applying the pathloss offset the requirements for the pathloss reference signal switch delay can be directly applied for the DL sTRP.

Observation 6: The uplink TCI switching delay requirements can be applied for switches contained to UL transmission to the DL sTRP that do not involve the UL mTRP.

# References

- R4-2411815, Topic summary for [112][220] NR_MIMO_Ph5, RAN4 #112, August 2024

- R4-2414035, WF on RRM requirements for NR_MIMO_Ph5, RAN4 #112, August 2024

- TS 38.133, Requirements for support of radio resource management, v18.6.0, June 2024

- R4-2411784, Views on the RRM impact of NR MIMO Phase 5, Qualcomm, RAN4 #112, August 2024

- RP-242394, WID revision: NR MIMO Phase 5, RAN #105, September 2024