3GPP TSG-RAN WG4 #112bis	R4-2415303

Hefei, China, October 14-18, 2024

Agenda item:	6.22.2.1		On-demand SSB SCell Operation

Source: 	Qualcomm Incorporated

Title: 	On RRM requirements for on-demand SSB SCell operation

Document for:	Discussion

# Introduction

In RAN4 #112, RAN4 work on Rel-19 NES was started. The work plan and the way forward have been agreed in [1] and [3], respectively. According to the work plan, the first focus for on-deman SSB is on MAC CE based single SCell activation. In a second step, RAN4 may discuss other SCell activation scenarios if needed. According to the work plan, the focus in RAN4 #112bis and RAN4 #113 is MAC CE based single SCell activation (Issue 1-1-1).

The following additional agreements have been made for on-demand SSB:

- Sub-topic 1-1: General on-demand SSB requirements

  - Issue 1-1-2: OD-SSB scenarios

    - Agreement: RAN4 to discuss OD-SSB based SCell activation requirements based on following RAN1 agreed scenarios.

      - Case #1 and Scenario #2

      - Case #1 and Scenario #2A

      - Case #2 and Scenario #2

      - Case #2 and Scenario #2A

      - Note 1: Follow RAN1 definition on the scenarios and cases.

      - Note 2: This list can be updated pending on RAN1 further agreement.

  - Issue 1-1-2 Deactivated SCell measurement requirements

    - Agreement: RAN4 to discuss OD-SSB based deactivated SCell measurement SCell activation requirements

      - Includes both FR1 and FR2

- Sub-topic 1-2: OD-SSB requirements (Case 1)

  - Issues 2-2-1: Case 1 OD-SSB based deactivated SCell measurements

    - Proposals

      - Option 1: For deactivated SCell measurement requirements, follow the periodicity of configured OD-SSB (not the measurement cycle).

      - Option 2: For deactivated SCell measurement requirements, follow the measCycleSCell.

      - Other option is not precluded

  - Issue 2-2-2: Case 1 OD-SSB based deactivated SCell measurement before OD-SSB indication

    - Agreement: In (Case #1 Scenario #2) and (Case #1 Scenario #2A), UE is not expected to perform SSB based deactivated SCell L3 measurements before OD-SSB is triggered

      - Note: How to differentiate OD-SSB SCell and normal SCell depends on other WG’s discussion

  - Issue 2-2-10: Case #1 and Scenario #2A – unknown SCell activation

    - Way forward

      - Option 1: RAN4 to only define unknown SCell activation provided that the OD-SSB is the first OD-SSB triggered by NW for (Case #1, Scenario #2A).

      - Option 2: RAN4 only specifies requirements assuming the target SCell is unknown in (Case #1, Scenario #2A).

      - Option 3: RAN4 needs to further consider the scenario (Case #1, Scenario #2A) together with (Case #1, Scenario #2) for deactivated SCell OD-SSB measurement.

      - Other option is not precluded

In this paper, we address the issues above of the way forward as defined in [3] and several of the issues list in the topic summary of RAN4 #112 as defined in [5].

# Discussion

In RAN1 #118, it was agreed that on-demand SSB is at least supported for NCD-SSB. Hence, RAN4 should start defining requirements for NCD-SSB.

Proposal 1: RAN4 should start defining requirements for NCD-SSB (Issue 2-1-5).

RAN1 has agreed so far that on-demand SSB can be triggered at least when the SCell is deactivated and when the UE receives SCell activation command. Both cases are supported with and without always-on SSB and are illustrated in Figure 2-1 and Figure 2-2, respectively.

Figure 2-1: On-demand SSB without always-on SSB

Figure 2-2: On-demand SSB with always-on SSB

We first discuss the conditions for known and unknown SCell

## Condition for Known/Unknown SCell with on-demand SSB

### Case #1 – No always-on SSB

Proposal 2: The condition for SCell to be known is based L3 measurements as in existing SCell activation scenarios.

Proposal 3: L1 measurements are not supported for deactivated SCell.

Observation 1: The SCell in case #1, scenario #2 can be known or unknown. Potentially, a new condition for known SCell can be derived.

One of the conditions for a SCell to be known is that during the period of 5*max{measCycleSCell, DRX cycles} the UE has sent a valid measurement report before the SCell activation command is received. This is illustrated in Figure 2-3. The figure shows periodic SSBs, typically with a periodicity of 20 ms. The known SCell condition does not require the UE to measure each SSB for deactivated SCell but only to measure each nth SSB, depending on the DRX cycle and configured value for measCycleSCell (n = measCycleSCell [ms] / SSB periodicity [ms]). Only at most one SSB needs to be considered by the UE during the period max{measCycleSCell, DRX cycles}. If the DRX cycle is larger than measCycleSCell, the SSBs need to be measured less frequently. This allows the UE to skip additional SSBs and to save power by doing less frequent measurements.

Figure 2-3: SSB Measurement Periodicity for Known SCell (FR1)

In addition, the known SCell condition in FR1 requires the UE to send one valid measurement report only once in the period 5*max{measCycleSCell, DRX cycles} (assuming Kp = CSSFintra = 1) for a DRX cycle > 320 ms as seen in Table 2-1. This allows the UE to combine up to five samples of SSB occurrences to meet the measurement accuracy requirements. That means for a cell to be known it is required that the UE is able to send a valid measurement report which again implies that the accuracy requirements for measuring RSRP are met. For meeting the accuracy requirements, the UE is allowed to observe up to five SSB occurrences.

Table 2-1: Measurement Period for Intra-frequency Measurements without Gaps (Deactivated SCell) (FR1) [4]

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | Ceil(5 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(5 x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(5 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: The requirements also apply to deactivated SCG SCell |  |

Also, for on-demand SSB, the UE should be allowed to observe up to five SSB occurrences. If this is the case also no new accuracy requirements need to be defined.

Observation 2: No new accuracy requirements need to be defined for L3 measurements for a deactivated SCell to be known in case #2, scenario #2A, if the same number of SSB occurrences as for always-on SSB is allowed for on-demand SSB.

Proposal 4: No new measurement accuracy requirements should be defined for on-demand SSB.

Figure 2-4 and Figure 2-5 show two options, how five on-demand SSBs can be positioned within the measurement periodicity before SCell activation. In the first figure, the five SSBs follow a periodicity of measCycleSCell while in the second figure, the periodicity is equal to 20 ms. In the first figure the measurement period is maintained while in the second figure it would also be possible to reduce the measurement periodicity.

Figure 2-4: on-demand SSB with enlarged Periodicity

Figure 2-5: on-demand SSB with legacy Periodicity and shortened Timespan

Both cases allow to exploit a NES gain due to the finite number of on-demand SSBs while the second case allows also to reduce the measurement period. It needs to be discussed further in RAN4, which of both cases allow a larger NES gain while not impacting UE power. Both scenarios are feasible.

Observation 3: A NES gain can be exploited by only sending the required number of on-demand SSBs during the measurement period.

Proposal 5: It should be further discussed in RAN4 whether reducing the periodicity of the on-demand SSB allows additional NES gain while not impacting UE power (Issue 2-2-1).

Proposal 6: It should be investigated further in RAN4 whether a new condition for known SCell can be defined for OD-SSB following the measurement periodicity (Issue 2-2-1).

In case #1, scenario #2A there are no SSBs before SCell activation has been triggered. Hence, also no L3 measurements for the deactivated SCell are possible in this scenario. Since no L3 measurements for this case are possible, the cell is always unknown.

Observation 4: Following the existing condition for a SCell being known, the SCell in case #1, scenario #2A is always unknown.

Proposal 7: For case #1, scenario #2A, the SCell activation requirements should only be defined for the condition of unknown SCell (Issue 2-2-10).

### Case #2 – Always-on SSB

In case #2 there is always-on SSB present. That means that both scenarios #2 and #2A can be known or unknown. In case of scenario #2A, there is only always-on SSB before the SCell activation happens. That implies that the condition for known SCell can be reused.

Observation 5: The SCell in case #2, scenario #2 can be known or unknown. Potentially, a new condition for known SCell can be derived.

Observation 6: The condition of known SCell for case #2, scenario #2A can be reused from legacy scenario with always-on only SSB.

Proposal 8: For case #2, scenario 2A, the condition for a known SCell should be the same as in legacy since the measurements are the same as the legacy L3 measurements.

Another question to be addressed is how to handle always-on SSB and on-demand SSB if both have different periodicities and potentially have even different frequencies. The additional gains are not obvious, and it is not clear how much the requirements can be enhanced if measurements based on always-on SSB and on-demand SSB are combined. Also, there might be additional implications at the UE both in implementation as well as in processing timeline.

To simplify standardization impact and implementation in the UE, we propose not to define requirements based both on always-on SSB and on-demand SSB if no clear gains can be shown.

Observation 7: The additional gains are not obvious, and it is not clear how much the requirements can be enhanced if measurements based on always-on SSB and on-demand SSB are combined.

Proposal 9: No requirements based both on always-on SSB and on-demand SSB should be defined if no clear gains can be shown.

## SCell Activation with on-demand SSB

SCell activation timeline with on-demand SSBs depends how quickly the MAC CE triggering of the on-demand SSB can be decoded, like the legacy SCell activation timeline, when the first on-demand SSB arrives. In the following, we first address the question how quickly the UE can be ready to receive on-demand SSB.

### MACE CE Timeline

In [5], an LS was received from RAN1 with the following question

RAN1 would like to ask RAN4 if the working assumption of T_min=. can be confirmed to agreement.

The MAC CE timeline mentioned above applies primarily for A-TRS and aperiodic CSI-RS. However, the RAN1 LS extends the MAC CE triggering to SSB. Since A-TRS and aperiodic CSI-RS on the one side and SSB on the other side have usually different purposes, it cannot easily be assumed that the same timeline can be applied for these signals.

Observation 8: Since A-TRS and aperiodic CSI-RS on the one side and SSB on the other side have usually different purposes, it cannot easily be assumed that the same timeline can be applied for these signals.

Hence, we propose that an additional margin  is applied to the timeline. It should be discussed further in RAN4 what margin is required and whether it is needed to define different margins for different scenarios.

Proposal 10: An additional margin  in slots should be added to T_min=m+3Nslotsubframe,μ+1, i.e., T_min=m+3Nslotsubframe,μ+∆. ∆ >1 is FFS in RAN4.

RAN4 should draft the LS reply to RAN1 accordingly that the RAN1 working assumption cannot be confirmed.

Proposal 11: RAN4 should agree that the RAN1 working assumption of Tmin=m+3Nslotsunframe,μ+1 cannot be confirmed.

# Conclusions

In this paper, we provided our views on several of the open issues identified in RAN4 #112. Our observations and proposals are summarized as follows:

Proposal 1: RAN4 should start defining requirements for NCD-SSB (Issue 2-1-5).

Proposal 2: The condition for SCell to be known is based L3 measurements as in existing SCell activation scenarios.

Proposal 3: L1 measurements are not supported for deactivated SCell

Observation 1: The SCell in case #1, scenario #2 can be known or unknown. Potentially, a new condition for known SCell can be derived.

Observation 2: No new accuracy requirements need to be defined for L3 measurements for a deactivated SCell to be known in case #2, scenario #2A, if the same number of SSB occurrences as for always-on SSB is allowed for on-demand SSB.

Proposal 4: No new measurement accuracy requirements should be defined for on-demand SSB.

Observation 3: A NES gain can be exploited by only sending the required number of on-demand SSBs during the measurement period.

Proposal 5: It should be further discussed in RAN4 whether reducing the periodicity of the on-demand SSB allows additional NES gain (Issue 2-2-1).

Proposal 6: It should be investigated further in RAN4 whether a new condition for known SCell can be defined for OD-SSB following the measurement periodicity (Issue 2-2-1).

Observation 4: Following the existing condition for a SCell being known, the SCell in case #1, scenario #2A is always unknown.

Proposal 7: For case #1, scenario #2A, the SCell activation requirements should only be defined for the condition of unknown SCell (Issue 2-2-10).

Observation 5: The SCell in case #2, scenario #2 can be known or unknown. Potentially, a new condition for known SCell can be derived.

Observation 6: The condition of known SCell for case #2, scenario #2A can be reused from legacy scenario with always-on only SSB.

Proposal 8: For case #2, scenario 2A, the condition for a known SCell should be the same as in legacy since the measurements are the same as the legacy L3 measurements.

Observation 7: The additional gains are not obvious, and it is not clear how much the requirements can be enhanced if measurements based on always-on SSB and on-demand SSB are combined.

Proposal 9: No requirements based both on always-on SSB and on-demand SSB should be defined if no clear gains can be shown.

Observation 8: Since A-TRS and aperiodic CSI-RS on the one side and SSB on the other side have usually different purposes, it cannot easily be assumed that the same timeline can be applied for these signals.

Proposal 10: An additional margin  in slots should be added to T_min=m+3Nslotsubframe,μ+1, i.e., T_min=m+3Nslotsubframe,μ+∆. ∆ >1 is FFS in RAN4.

Proposal 11: RAN4 should agree that the RAN1 working assumption of Tmin=m+3Nslotsunframe,μ+1 cannot be confirmed.

# References

- R4-2411817, Topic summary for [112][222] Netw_Energy_enh

- R4-2412508, Work plan for R19 NES, RAN4 #112, August 2024

- R4-2413885, WF on Netw_Energy_NR_enh, RAN4 #112, August 2024

- TS 38.133, v18.6.0

- R4-2414911, LS on timeline for on-demand SSB operation in SCell, RAN4 #112bis, October 2024

The numbering in [2] named this issue as issue 1-2-1, but we believe the correct numbering is issue 2-2-1. Hence, we use this numbering.