# 3GPP TSG-RAN WG4 Meeting #112bis		  R4-2415306

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	6.21.2.2

Source:		Qualcomm Incorporated

Title:			Discussion on Ambient IoT Coexistence Evaluations

Document for:	Approval

# 1  Introduction

In RAN4 #112 meeting, the way forward in [1] captured the agreements on ambient IoT coexistence study. 10 evaluation cases were agreed for the Ambient IoT coexistence study including CW inside and outside topology. In this meeting, we provided the simulation results for CW inside topology in [4]. This paper provides our views on the simulation results and issues identified during the coexistence evaluations.

# 2  Evaluation parameters

## 2.1 Adjacent RB Tx and Rx configurations in evaluations

Adjacent RB Tx and Rx configurations are key parameters in the Ambient IoT coexistence evaluations. The below table summarizes the Tx and Rx configurations in the evaluations for CW inside topology.

Table 1: Tx and Rx configurations in the evaluations

| No. | deployment scenario and topology | spectrum | (Aggressor -> Victim) | note | Tx and Rx config. |
| --- | --- | --- | --- | --- | --- |
| 1-1 | D1T1-A2- legacy UE only outdoor | R2D: DL CW2D and D2R: DL | Device->NR DL | D2R | Device Tx: 25dBc NR UE Rx: perfect orthogonality |
|  |  |  | NR DL->reader | D2R | NR BS Tx: ACLR = 30dB Reader Rx: ACS = 46dB |
|  |  |  | Reader->NR DL | R2D | Reader Tx: ACLR (i.e. ACLR1:40dBc, ACLR2:50dBc) NR UE Rx: perfect orthogonality |
|  |  |  | NR DL->device | R2D | NR BS Tx: ACLR = 30dB Device: no selectivity |
| 1-2 | D1T1-A2-legacy UE indoor | R2D: DL CW2D and D2R: DL | Device->NR DL | D2R | Device Tx: ACLR = 25dBc NR UE Rx: perfect orthogonality |
|  |  |  | NR DL->reader | D2R | NR BS Tx: ACLR = 30dB Reader Rx: ACS = 46dB |
|  |  |  | Reader->NR DL | R2D | Reader Tx: ACLR (i.e. ACLR1:40dBc, ACLR2:50dBc) |
|  |  |  | NR DL->device | R2D | NR BS Tx: ACLR = 30dB Device Rx: no selectivity |
| 3 (O) | D1T1-A2-legacy UE indoor | R2D: UL CW2D and D2R: DL | Device->NR DL | D2R | Device Tx: ACLR = 25dBc NR UE Rx: ACS=30dB |
|  |  |  | NR DL->reader | D2R | NR BS Tx: ACLR = 30dB Reader Rx: ACS = 46dB |
|  |  |  | Reader->NR UL | R2D | Reader Tx: ACLR (i.e. ACLR1:40dBc, ACLR2:50dBc) NR BS Rx: perfect orthogonality |
|  |  |  | NR UL->device | R2D | UE Tx: IBE Device: no selectivity |
| 5-1 | D2T2-A2-legacy UE only outdoor | R2D: UL CW2D and D2R: UL | Device->NR UL | D2R | Device Tx: ACLR = 25dBc NR BS Rx: perfect orthogonality |
|  |  |  | NR UL->reader | D2R | UE Tx: IBE Intermediate UE Rx: perfect orthogonality |
|  |  |  | Reader->NR UL | R2D | Intermediate UE Tx: IBE NR BS Rx: perfect orthogonality |
|  |  |  | NR UL->device | R2D | UE Tx: IBE Device: no selectivity |
| 5-2 | D2T2-A2-legacy UE indoor | R2D: UL CW2D and D2R: UL | Device->NR UL | D2R | Device Tx: ACLR = 25dBc NR BS Rx: perfect orthogonality |
|  |  |  | NR UL->reader | D2R | UE ACLR = IBE Intermediate UE Rx: perfect orthogonality |
|  |  |  | Reader->NR UL | R2D | Intermediate UE Tx: IBE NR BS Rx: perfect orthogonality |
|  |  |  | NR UL->device | R2D | UE Tx: IBE Device: no selectivity |

## 2.2 IBE for NR UE and Intermeiate UE

In [1], legacy UE IBE was agreed as the Tx leakage model for NR UE and Intermediate UE, but the details of parameters were not provided.

IBE requirements are specified in Table 6.4.2.3-1 of TS38.101-1 [2]. For the general part, the limits are related to several parameters including NRB which is the transmission bandwidth configuration, EVM,  is an average of the transmitted power, i.e., 23dBm in AIoT simulations.

Table 6.4.2.3-1: Requirements for in-band emissions [2]

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB |  |  | Any non-allocated (NOTE 2) |
| IQ Image | dB | -28 | Image frequencies when output power > 10 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -25 | Image frequencies when output power ≤ 10 dBm |  |
| Carrier leakage | dBc | -28 | Output power > 10 dBm | Carrier leakage frequency (NOTES 4, 5) |
|  |  | -25 | 0 dBm ≤ Output power ≤ 10 dBm |  |
|  |  | -20 | -30 dBm ≤ Output power < 0 dBm |  |
|  |  | -10 | -40 dBm ≤ Output power < -30 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of PRB - 30 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. PRB is defined in NOTE 10. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. For pi/2 BPSK with Spectrum Shaping, the limit is expressed as a ratio of measured power in one non-allocated RB to the measured power in the allocated RB with highest PSD. NOTE 3: The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated bandwidth, based on symmetry with respect to the carrier leakage frequency, but excluding any allocated RBs. NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 5: The applicable frequencies for this limit depend on the parameter txDirectCurrentLocation in UplinkTxDirectCurrent IE, and are those that are enclosed either in the RB containing the carrier leakage frequency, or in the two RBs immediately adjacent to the carrier leakage frequency but excluding any allocated RB. NOTE 6: LCRB is the Transmission Bandwidth (see clause 5.3). NOTE 7: NRB is the Transmission Bandwidth Configuration (see clause 5.3). NOTE 8: EVM is the limit specified in Table 6.4.2.1-1 for the modulation format used in the allocated RBs. NOTE 9: is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. ∆RB = 1 or ∆RB = -1 for the first adjacent RB outside of the allocated bandwidth. NOTE 10: is an average of the transmitted power over 10 sub-frames normalized by the number of allocated RBs, measured in dBm. NOTE 11: For almost contiguous allocations defined in clause 6.2.2, LCRB = NRB_alloc + NRB_gap with no in-gap emission requirement. |  |  |  |  |

In the simulation, 10MHz with 52RBs is assumed. As shown in Figure 1, 1RB is configured for AIoT, and 3UEs with 17RBs for each are assumed.

For the case that NR UE is the aggressor, NRB is 51 and EVM should follow UE SINR in the simulation which can be mapped to moderation.

For the case that Intermediate UE is the aggressor, NRB can be configured either 51 or 1 depending on whether IoT Tx is separate from NR Tx or not. If NRB =1, IBE is not appropriate to be used. Instead, ACLR for NB-IoT UE can be considered. In addition, OOK is assumed as the waveform of R2D link, while RAN4 has not defined the EVM for OOK. Therefore, EVM for OOK is needed for the simulations.

Observation 1: NRB and EVM for IBE need to be provided for the coexistence simulation.

Figure 1: Illustration of BW for NR UE and AIoT

For the simulation results presented in this paper, we use NRB = 51 and EVM = 17.5% (QPSK) for the evaluation cases of NR UE or Intermediate UE as the aggressor. As shown in Figure 2, the leakage can be calculated as follows:

- NR UL to D2R: IBE is 18dB

- R2D to NR UL: IBE is 18dB for UE2 and UE3, IBE is 42dB for UE1

Figure 2: IBE with QPSK, NRB=51, 1RB transmission

## 2.3 CW cancellation capability

In [1], it was agreed that companies report the CW cancellation capability in the simulation.

Agreement in RAN4#112: - Companies to report the CW cancellation capability used for co-existence evaluation, CW cancellation capability for inside topology and outside topology can be different. - total interference cancellation capability = spatial isolation + RF cancellation + digital cancellation - CW transmission impact on the received SINR, e.g. degradation

From system performance perspective, we believe the CW cancellation capability should guarantee the coverage reader in D1T1 and D2T2. The below figures provide the curve of reader coverage and SINR for D1T1 and D2T2.

Figure 3: Distance between reader and device vs. SINR for D1T1

Figure 4: Distance between reader and device vs. SINR for D2T2

When assuming the required SINR = 10dB and target coverage is 15m, the required CW cancellation capacity is 140dB for D1T1 and 130dB for D2T2 respectively. Note that, 140dB and 130dB are not representing CW cancellation capability from feasibility perspective. The analysis for the feasibility of CW cancellation capability can be found in [3].

Observation 2: 140dB and 130dB CW cancellation capability are assumed for device 1 in D1T1-A2 and D2T2-A2 respectively in the simulation assuming the coverage of reader is 15m and the required SINR for D2R is 10dB. The analysis for the feasibility of CW cancellation capability can be found in [3].

# 3  Evaluation results

With the evaluation parameters in section 2, our simulation results for CW inside topology are captured in [4]. Note that for legacy UE indoor cases, 100% indoor UE is assumed.

Table 2: Summary for coexistence evaluation results

| No. | Deployment scenario and topology | Spectrum | (Aggressor -> Victim) | Note | Coexistence summary |
| --- | --- | --- | --- | --- | --- |
| 1-1 | D1T1-A2- legacy UE only outdoor | R2D: DL CW2D and D2R: DL | Device->NR DL | D2R | Marginal |
|  |  |  | NR DL->reader | D2R | SINR degradation @5-ile is 2dB, 50-ile is 0.8dB with |
|  |  |  | Reader->NR DL | R2D | Marginal |
|  |  |  | NR DL->device | R2D | SINR degradation @50-ile is 0.4dB |
| 1-2 | D1T1-A2-legacy UE indoor | R2D: DL CW2D and D2R: DL | Device->NR DL | D2R | Marginal |
|  |  |  | NR DL->reader | D2R | SINR degradation @5-ile is 2dB, 50-ile is 0.8dB with 140dB CW cancellation capability |
|  |  |  | Reader->NR DL | R2D | Interference is too high. Need to check the NR UE Blocking issue |
|  |  |  | NR DL->device | R2D | SINR degradation @50-ile is 0.4dB |
| 3 (O) | D1T1-A2-legacy UE indoor | R2D: UL CW2D and D2R: DL | Device->NR DL | D2R | Same with scenario 1-2, Marginal |
|  |  |  | NR DL->reader | D2R | Same with scenario 1-2, SINR degradation @5-ile is 2dB, 50-ile is 0.8dB with 140dB CW cancellation capability |
|  |  |  | Reader->NR UL | R2D | 8.73% mean T-put loss, cell edge T-put is N/A due to outage |
|  |  |  | NR UL->device | R2D | 1.78dB for 5-ile SINR degradation, 1.22dB for 50-ile SINR degradation |
| 5-1 | D2T2-A2-legacy UE only outdoor | R2D: UL CW2D and D2R: UL | Device->NR UL | D2R | Marginal |
|  |  |  | NR UL->reader | D2R | SINR degradation is marginal with 130dB CW cancellation capability. |
|  |  |  | Reader->NR UL | R2D | T-put loss 48%, edge T-put loss 78%; |
|  |  |  | NR UL->device | R2D | Marginal |
| 5-2 | D2T2-A2-legacy UE indoor | R2D: UL CW2D and D2R: UL | Device->NR UL | D2R | Marginal |
|  |  |  | NR UL->reader | D2R | SINR degradation @5-ile is 13dB, @50-ile is 4.9 dB with 130dB CW cancellation capability. |
|  |  |  | Reader->NR UL | R2D | Mean T-put loss 65%, cell edge T-put is N/A due to outage |
|  |  |  | NR UL->device | R2D | 1.9dB for 5-ile SINR degradation, 2.8dB for 50-ile SINR degradation |

# 4  Remaining issues for evaluation parameters

  - IBE and leakage model for NR UE and Intermediate UE

As discussed in section 2.2 and presented results in section 3, RAN4 needs to further discuss and confirm the IBE and leakage model for NR UE and Intermediate UE. We have the following proposals:

Proposal 1: For R2D to NR UL, the following IBE and leakage model is considered:

- EVM: follow SINR or fixed modulation, e.g., QPSK

- NRB = 51

- No Guard band: IBE is 18dB for UE2 and UE3, IBE is 42dB for UE1 (see Figure 1)

- With Guard band: IBE is IBE with xRB offset, for UE2 and UE3, IBE is 43 for UE1 (see Figure 1)

Proposal 2: For NR UL to D2R, the following IBE and leakage model is considered:

- EVM: need a reference for OOK

- NRB = 51

  - No Guard band: IBE is 18dB

  - With Guard band: IBE is IBE with xRB offset

## 4.2 NR UE Blocking

As shown in Table 2, the ACIR values vary with different bandwidth settings and device types. In some cases, ACIR is limited by ACLR/Tx leakage, while in others, it is limited by ACS/Rx selectivity.

The scenario of R2D interfering with NR DL UE reception on DL spectrum can serve as an example of an ACS-limited case. When the aggressor with smaller bandwidth and therefore ACLR keep the same value with symmetrical bandwidth. The victim UE ACS is 33dB. Figure 10 illustrates the UE received power distribution, the intra-system interference distribution and the inter-system interference distribution from different active BS number.

Figure 5. Distribution of signal power and interference power for the R2D interfering to the NR DL (100% indoor)

As shown in Figure 5, the interference from reader to NR DL would be very high which might result in the UE being blocked with a certain probability.

During the online and offline discussion in RAN4#112, companies raised that in-band blocking requirement that is specified in TS 38.101-1 can be referred to analyze the blocking issues from R2D interfering to NR DL. But the BW of the interferer for in-band blocking is 3MHz that is not consistent with R2D BW of 180kHz and CW waveform (single tone).

Proposal 3: The UE blocking issue and corresponding maximum interference power need further study in RAN4.

  - Phase Noise model for CW cancellation

In [1], the agreements for CW cancellation were captured. In the agreements, it was mentioned that the capability can be different for inside and outside topology.

Agreement in RAN4#112: - Companies to report the CW cancellation capability used for co-existence evaluation, CW cancellation capability for inside topology and outside topology can be different. - total interference cancellation capability = spatial isolation + RF cancellation + digital cancellation - CW transmission impact on the received SINR, e.g. degradation

Regarding the CW cancellation, one of key differences between inside and outside topology is that we can assume the timing and frequency offset between CW interference and D2R are known for the reader for inside topology. While for outside topology, the perfect synchronization for timing and frequency cannot be assumed since CW node and readers are not co-located so the down conversion will be done with a different signal source [3]. Therefore, phase noiseof CW interference would have a big impact on the performance.

Observation 3: For outside topology, CW phase noise plays a role in the range since the down conversion will be done with a different signal source Therefore, phase noise would be introduced at the receiver of the reader for outside topolgoy.

The phase noise is usually modeled as the phase noise power level (Y dBm/Hz, Y is related to received CW power at the reader) with different frequency offsets. To simplify the phase noise model in the coexistence simulation, we have the following proposal for phase noise modelling for CW outside topology.

Proposal 4: RAN4 should account for phase noise in CW outside topology and baseline SINR can be calculated by:

baseline SINR for CW outside=received wanted signal powernoise + intrasysteminterference+phase noisewithin total receiver bandwidth+residual CW interference after cancellationin linear scale

Phase noise is YdBm/Hz, and Y is related to the received CW power level at the reader.

# 3  Conclusion

In summary, we have the following observations and proposals.

Observation 1: NRB and EVM for IBE need to be provided for the coexistence simulation.

Observation 2: 140dB and 130dB CW cancellation capability are assumed for device 1 in D1T1-A2 and D2T2-A2 respectively in the simulation assuming the coverage of reader is 15m and the required SINR for D2R is 10dB. The analysis for the feasibility of CW cancellation capability can be found in [3].

Proposal 1: For R2D to NR UL, the following IBE and leakage model is considered:

- EVM: follow SINR or fixed modulation, e.g., QPSK

- NRB = 51

- No Guard band: IBE is 18dB for UE2 and UE3, IBE is 42dB for UE1 (see Figure 1)

- With Guard band: IBE is IBE with xRB offset, for UE2 and UE3, IBE is 43 for UE1 (see Figure 1)

Proposal 2: For NR UL to D2R, the following IBE and leakage model is considered:

- EVM: need a reference for OOK

- NRB = 51

  - No Guard band: IBE is 18dB

  - With Guard band: IBE is IBE with xRB offset

Proposal 3: The UE blocking issue and corresponding maximum interference power need further study in RAN4.

Proposal 4: RAN4 should account for phase noise in CW outside topology and baseline SINR can be calculated by:

baseline SINR for CW outside=received wanted signal powernoise + intrasysteminterference+phase noisewithin total receiver bandwidth+residual CW interference after cancellationin linear scale

Phase noise is YdBm/Hz, and Y is related to the received CW power level at the reader.

# 4  References

[1] R4-2414304, WF on Co-existence study for ambient IoT, CMCC

[2] TS 38.101-1

[3] R4-2415529, CW cancellation feasibility for the intermediate node, Qualcomm

[4] R4-2416355, Collection of simulation results for Ambient-IoT co-existence study, Huawei, HiSilicon