# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2415309

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda Item:	6.12.2.2

Source: 		Qualcomm Incorporated

Title: 	On FR1 NTN devices OTA

Document for:	Approval

# Introduction

In RAN4#112, WF for [111][338] TRP_TRS_MIMO_OTA was approved in [2] capturing agreements and open issues for FR1 NTN OTA testing.  In this paper, we provide our views on OTA test method and performance metric for FR1 NR NTN and IoT NTN devices.

# 2. Discussion

## 2.1 UE type and usage scenarios

For UE type and usage scenarios for NR-NTN and IoT-NTN, the following agreements were made in [1].

Issue 3-1-1: Usage scenarios for NR-NTN mobile handheld UE Agreements: - Test method development consider hand only and head+hand, RAN4 will discuss requirement applicability rules based on device type, NR-NTN/IoT-NTN support, or other criteria Prioritize hand only and head+hand - FFS down-selection of usage scenarios for requirements - Other usage scenarios are not precluded Issue 3-1-4: UE type for IoT-NTN Agreements: - For IoT-NTN test method development, RAN4 can consider handheld UE type as 1st priority. Issue 3-1-5: Usage scenarios for IoT-NTN handheld UE Agreements: - Same prioritized usage scenarios as NR-NTN handheld UE

Specifically, NR-NTN and IoT-NTN handheld UE have two main usage scenarios.

- Talk mode

  - Talk mode with hand+head

  - Talk mode with Hand only (via speaker)

- Browsing mode

  - Browsing mode with hand only

Compared to the hand-only, using talk mode with hand+hand will need better coverage due to additional penetration loss across the head and hand. As 3GPP is currently working on the coverage enhancement of NTN including specifying higher power of NTN UE, etc., hand+head scenario should not be precluded in the scope of Rel-19 TRP/TRS. RAN4 can consider different UE performance metrics for hand+head scenario and hand-only scenario.

Proposal 1: The test method for both hand-only and head+hand should be discussed in Rel-19 TRP/TRS WI. Different UE performance metrics for hand+head scenario and hand-only scenario can be considered.

## 2.2 UE performance metric

For talk and browsing modes with hand-only scenario, omni-radiated performance such as TRP and TRS are not suitable for NTN device OTA testing any more since usually device is pointing to the satellite with the help of assistant software before/during the communication with the satellite. Hence, both UE antenna radiated performance for partial hemisphere and spherical coverage performance should be verified in this scenario. Therefore, Peak EIRP/EIS+ partial Spherical coverage CDF (e.g., within selected 30, 60, 90, 180 degree range of angles) can be considered as the starting point for the NR-NTN and IoT-NTN handheld UE. FFS on the range of angles.

Proposal 2: For talk and browsing modes with hand-only scenario, Peak EIRP/EIS+ partial Spherical coverage CDF (e.g., within selected 30, 60, 90, 180 degree range of angles) can be considered as the starting point for the NR-NTN and IoT-NTN handheld UE. FFS on the range of angles.

For talk mode with hand+head scenario, it is quite similar to the traditional talk mode since the assistant software of pointing to the satellite can’t used. Therefore, the metric of traditional TRP/TRS or partial TRP/TRS (e.g., upper hemisphere TRP/TRS) can be considered as the baseline.

Proposal 3: For talk mode with hand+head scenario, the legacy metric of TRP/TRS or partial TRP/TRS (e.g., upper hemisphere TRP/TRS) can be considered as the starting point for the NR-NTN and IoT-NTN handheld UE.

# 3.	Conclusion

In this paper, we provide our views on OTA test method for FR1 NTN devices. The following proposals were made:

Proposal 1: The test method for both hand-only and head+hand should be discussed in Rel-19 TRP/TRS WI. Different UE performance metrics for hand+head scenario and hand-only scenario can be considered.

Proposal 2: For talk and browsing modes with hand-only scenario, Peak EIRP/EIS+ partial Spherical coverage CDF (e.g., within selected 30, 60, 90, 180 degree range of angles) can be considered as the starting point for the NR-NTN and IoT-NTN handheld UE. FFS on the range of angles.

Proposal 3: For talk mode with hand+head scenario, the metric of legacy TRP/TRS or partial TRP/TRS (e.g., upper hemisphere TRP/TRS) can be considered as the starting point for the NR-NTN and IoT-NTN handheld UE.

# 4.	References

- R4-2413603, WF for [112][333] TRP_TRS_MIMO_OTA, vivo

- TR 38.863