# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2415310

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda Item:	6.12.2.3

Source: 		Qualcomm Incorporated

Title: 	On FR1 dynamic MIMO OTA test method

Document for:	Approval

# Introduction

In RAN4#112, FR1 dynamic MIMO OTA test methods were discussed and the WF was agreed in [1]. In this paper, we provide our further views on FR1 dynamic MIMO OTA test method.

# 2. Discussion

## 2.1 Channel model

To develop a dynamic channel model that accurately reflects real-world conditions, we should initially investigate the approach commonly employed by operators for evaluating their networks using actual devices. This approach, often referred to as ‘drive testing,’ involves driving along a predefined route with one or more reference devices to assess factors such as throughput performance and dropped calls. Before OTA testing for transmitter and receiver performance metrics including TRP/TRS and MIMO OTA, drive testing was the standard practice to assess new UE performance before deployment on the network. Dynamic OTA testing is to facilitate the realistic test in the chamber which has better repeatability and could save the test cost.

To create the dynamic channel parameters, large scale channel characteristics, such as angular power distribution, power delay, Doppler shift, and LOS/NLOS condition, are varying over the emulation time. Thus, the channel model parameters including the path loss, power spread, delay spread, cluster number, AoA/AoD, UE speed, etc., need to be defined in FR1 dynamic OTA MIMO OTA test methodology.

In RAN4#112, [2] was approved in which the outcome of CTIA regarding the channel model was reused. For example, the following equation defines the linearly interpolated between two successive waypoints for smooth/continuous channel modelling.

Parameters are interpolated between way points 𝑎 and 𝑏 as εt=εaTab-t+εbtTab,; (1). where 𝜀(𝑡), 𝜀(𝑎), and 𝜀(𝑏) are the parameter values in the time instant 𝑡, way point 𝑎, and 𝑏, respectively, and 𝑇_𝑎𝑏 is the total time from way point 𝑎 to 𝑏 in seconds, such that 𝑡=0 at way point 𝑎 and 𝑡=𝑇_𝑎𝑏 at way point 𝑏.

Then for the channel model of each way point, the CDL channel model defined in [3] is applied. Note that some aspects of randomness in channel models of Clause 7.2 in [3] are removed to simiplfy the test setup. For example, fixed 2×2 initial phases for each ray, fixed coupling of ray angles within one cluster, and fixed gNB Tx beam directions are used in TR 38827. However, it is not clear how to handle above fixed parameters in the dynamic MIMO OTA channel model and make chanel model continuous.

Proposal 1: RAN4 needs to clarify how to handle the fixed channel parameters in dynamic MIMO OTA channel model including initial phases for each ray, the coupling of ray angles, and gNB Tx beam directions in dynamic MIMO OTA channel model for continuous channel modelling.

## 2.2 Test parameters

In RAN4#112, the following agreements were made for test parameters [1].

Issue 4-2-1: Link adaption configuration for Dynamic FR1 MIMO OTA Agreements: - Adopt the mapping table between CQI and MCS used for demodulation in Table A.4-2 and Table A.4-3 in the annex of TS 38.101-4 for dynamic MCS adaption. Issue 4-2-3: Test parameters for FR1 dynamic MIMO OTA Agreements: - Set the maximum number of HARQ transmissions to [x] for FR1 dynamic MIMO OTA with x>1.

In [2], the maximum number of HARQ transmission is set as 1, and the number of HARQ processes for TDD and FDD bands are set as 8 and 4 respectively.

In [4], the maximum number of HARQ transmission is set as 4, and the number of HARQ processes for TDD and FDD bands are set as 8 and 4 respectively.

As agreed in [1], the maximum number of HARQ transmissions is [x] for FR1 dynamic MIMO OTA with x>1. To verify UE’s performance in the realistic scenario, RAN4 can take the following test parameters as the starting point and further discuss the feasibility and impact on the UE’s performance.

- Maximum number of HARQ transmission: [4]

- Number of HARQ processes for TDD bands: [8]

- Number of HARQ processes for FDD bands: [4]

Proposal 2: RAN4 can take the following test parameters as the starting point and further discuss the feasibility and impact on the UE’s performance.

- Maximum number of HARQ transmission: [4]

- Number of HARQ processes for TDD bands: [8]

- Number of HARQ processes for FDD bands: [4]

# 3.	Conclusion

In this paper, we provide our views on OTA test method for FR1 dynamic MIMO OTA testing. The following proposals were made:

Proposal 1: RAN4 needs to clarify how to handle the fixed channel parameters in dynamic MIMO OTA channel model including initial phases for each ray, the coupling of ray angles, and gNB Tx beam directions in dynamic MIMO OTA channel model for continuous channel modelling.

Proposal 2: RAN4 can take the following test parameters as the starting point and further discuss the feasibility and impact on the UE’s performance.

- Maximum number of HARQ transmission: [4]

- Number of HARQ processes for TDD bands: [8]

- Number of HARQ processes for FDD bands: [4]

# 4.	References

- R4-2413603, WF for [112][333] TRP_TRS_MIMO_OTA, vivo

- R4-2413536, TP on Dynamic MIMO Aspects, Keysight Technologies, Spirent Communications

- TR 38827

- TS 38101-4