# 3GPP TSG-RAN WG4 Meeting #112bis		  R4-2415312

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	6.17.4.2

Source:		Qualcomm Incorporated

Title:			Discussion on testability issues for AI/ML beam management

Document for:	Approval

# 1  Introduction

In RAN4 #112 meeting, testability and interoperability issues for beam management were summarized in [1]. The following agreements regarding testability are captured in [2].

Issue 2-3: Channel models Agreement: Further discuss and down-select the following options considering TE feasibility - Option 2: TDL - Option 5: simplified CDL - CDL channel model with a small number of clusters. CDL simplification needs to be further discussed Issue 2-4: Test setup needs Issues listed below to be further investigated in order to determine the capabilities of the test equipment: - set B Tx beams; set A Tx beams - number of beams transmitted simultaneously - overall number of beams transmitted during the test - AoA - AoD - UE rotation during the test Other aspects can also be considered

In this paper, we provide our considerations on the testability issues for AI/ML beam management.

# 2  Discussion

## 2.1 Channel model in existing FR2 test systems

FR2 beam management performance is heavily influenced by the spatial properties of the multipath propagation environment.

Base station Single path channel DUT Chamber TE emulation to determine probe power Beam 1: arrives at UE with higher gain probe Base station Single path channel DUT Chamber TE emulation to determine probe power Beam 2: arrives at UE with lower gain probe

Figure 1: Predictability of strongest beam in a TDL single path-based channel

There are two cases captured in 38.843 [2] for beam prediction of DL Tx beam use cases:

- BM-Case1: Spatial-domain Downlink beam prediction for Set A of beams based on measurement results of Set B of beams

- BM-Case2: Temporal Downlink beam prediction for Set A of beams based on the historic measurement results of Set B of beams

The prediction for set A includes:

- Alt.1: Tx and/or Rx Beam ID(s) and/or the predicted L1-RSRP of the N predicted DL Tx and/or Rx beams

-	e.g., N predicted beams can be the Top-N predicted beams

- Alt.2: Tx and/or Rx Beam ID(s) of the N predicted DL Tx and/or Rx beams and other information

-	e.g., N predicted beams can be the Top-N predicted beams

- Alt.3: Tx and/or Rx Beam angle(s) and/or the predicted L1-RSRP of the N predicted DL Tx and/or Rx beams

-	e.g., N predicted beams can be the Top-N predicted beams

To verify the accuracy of the L1-RSRP prediction, sufficient randomness and variation in time and spatial domain of L1-RSRP of Set B and Set A beams must be configured in the test and emulated by the test equipment. In addition, spatial domain prediction relies on sufficient number of observed beams (in Set B) from different (peak direction) AoAs on UE’s  spherical coverage to learn the spatial domain characteristics and statistics to infer correlation between Set B and Set A in order to predict Set A from Set B.

Observation 1: An effective test case for beam prediction use case BM-Case1 requires the following:

- Sufficient randomness and variation in time and spatial domain of L1-RSRP

- DL Tx beam sweeping with enough number of Tx beams in Set B and Set A

Figure 1 illustrates the ineffectiveness of single path based TDL model in beam prediction. In the setup of Figure 1, UE can easily predict that beam 1 is stronger than beam 2 even without using any AI-ML model. UE will deterministically know that it will receive beam 1 with higher power than beam 2. No neural network is needed. Moreover, when TE sweeps more beams, many beams will look similar from UE perspective, since in the entire beam pattern of each beam (as plotted above), we only select one AoD angle to observe, and therefore many of the beams look alike even though their beam pattern may look different. Hence, verifying the AI/ML beam management performance in a TDL channel without spatial characteristics makes the test too simple and inconsistent with the actual field environment.

Observation 2: Verifying the AI/ML beam management performance (e.g., prediction of top K beams) in a single AoA based TDL channel without spatial characteristics makes the test too simple and inconsistent with the actual field environment.

Some companies suggested to emulate multiple beams with single AoA by using different power. The power levels will have to be defined in test configuration to enable this method. That will allow UE to predict the strongest beam through simple interpolation.

Observation 3: Some companies suggested to emulate multiple beams with single AoA by using different power. The power levels will have to be defined in test configuration to enable this method. That will allow UE to predict the strongest beam through simple interpolation.

In the CDL channel model, the angle of arrival (AoA) values, delay, and power levels received by the UE vary. To verify the FR2 AI/ML beam management performance, it is essential to emulate a more realistic environment that accommodates the gNB and UE’s antenna pattern impact in the CDL model with spatial properties.

Observation 4: CDL with spatial properties is a more realistic and proper channel model for AI/ML beam management testing.

The applicability of the channel model in the existing FR2 test system were analysed in [3][4][5]. The CDL channel model is supported by FR2 MIMO OTA test system with simplifications. The NLOS channel coefficient for time instant t, Rx antenna/beam u, Tx beam s, and cluster n is defined by following equation in TR 38.827 with comments.

Figure 2: CDL channel model in TR 38.827 with simplifications

As shown in Figure 2, the following simplifications for CDL channel model are considered in FR2 MIMO OTA test setup

- Instead of random procedure, the coupling of ZoD/ZoA/AoD/AoA rays within one cluster is performed using the fixed coupling pattern

- Intra-cluster delay spread is fixed, thus not following Table 7.5-5 of TR38.901.

- The gNB antenna configuration and beamforming codebook are pre-defined with gNB Tx beam always pointing to the strongest cluster

In FR2 MIMO OTA test setup discussion, PAS similarity percentage (PSP) was defined as the metric, e.g., >[83%] to characterize the FR2 channel model within the quiet zone. PSP calculation and measurement were initially discussed in [6] and then concluded in clause 7.4.1.6 of TR 38.827.

With simplified CDL channel model and target PSP metric, the existing FR2 MIMO OTA test system layout is limited to 6 probes, within a 55º by 10º sector, applying for two simplified CDL channel modes (i.e. CDL-A InO and CDL-C UMi).

## 2.2 AI/ML BM Testability - What do we need in test environment?

First of all, the test steps for AI/ML BM-case 1 can be described below:

- Progress A: UE BM performance with AI/ML for BM-case 1

  - Step 1: Data training step - Channel parameters set #1

    - Option 1: Offline data including field data and/or specified channel model data in the spec

    - Option 2: Real measurement in the chamber

  - Step 2: Measurement step - Channel parameter set #2

    - TE transmits Set B beams

  - Step 3: Prediction step

    - UE reports top K predicted beams of set A based on measurement of set B beams.

- Progress B: Ground truth of UE BM performance without AI/ML for BM-case 1

  - Step 1: Measurement step – Channel parameters set #2

    - TE transmits set A beams.

    - UE reports measured RSRP of all beams of set A.

  - Setp 2:   TE compares the measured RSRP of the strongest beam and the maximum RSRP of top K predicted beam.

Figure 3: Test steps for AI/ML BM-case 1

Based on the illustration of the test steps, we have the following observation and proposal.

Observation 5: The key requirements for AI/ML BM test setup are listed following:

- Requirement#1: Channel model in the test

  - CDL or simplified CDL is needed for multiple AoAs to properly verify beam management performance.

- Requirement#2: Avoid the cheating

  - Progress A: Data training for Set A’s is generated with channel parameters #1. Set B in UE measurement should have enough randomness compared to Set A’.

  - Progress B: Set A is generated to make sure to use the same parameters (initial phase, angular spread, delay spread) as the Set B in the Progress A measurement step. NW antenna/beam configurations for Set A and Set B should be deterministic.

- Requirement#3: Support the beam number of Set A

  - TE needs to support Set A for ground truth verification, i.e., upto 128.

Proposal 1: Take FR2 MIMO OTA as the baseline test setup for AI/ML BM testing and further discuss how to enhance the FR2 MIMO OTA test setup to support the multiple AoA/AoD (e.g., beam number of Set A) under simplified CDL channel models with enough randomness.

## 2.3 Enhancements on FR2 MIMO OTA test setup

The number of probes supporting multiple Tx beams from the gNB was studied in [4][5]. In [5], as shown in Figure 4, when the Tx beam number from the gNB is increased and points to other clusters, the number of probes in the chamber will increase significantly to achieve the PSP metric.

Observation 6: There will be plenty of additional probes if BS Tx beams point to other clusters even though some of the probes could be reused.

Figure 4: Probe number of supporting multiple Tx beam from gNB

It is obvious that the existing system can not be upgraded to support so many probes. To keep the number of probes within a reasonable range, we can consider reducing the number of clusters, i,e., removing the weak clusters in which the power is less than a threshold.

Table 1: Channel model parameters for UMi CDL-C at 28 GH

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -4.4215 | -30.4353 | -134.4434 | 98.9242 | 83.3318 |
| 2 | 12.594 | -1.25 | -20.9269 | 129.1633 | 99.1915 | 72.5229 |
| 3 | 13.314 | -3.4684 | -20.9269 | 129.1633 | 99.1915 | 72.5229 |
| 4 | 13.974 | -5.2294 | -20.9269 | 129.1633 | 99.1915 | 72.5229 |
| 5 | 13.056 | -2.5215 | -28.0782 | -152.8206 | 99.5732 | 71.1282 |
| 6 | 38.196 | 0 | -11.6982 | 164.1145 | 99.306 | 74.7544 |
| 7 | 38.688 | -2.2185 | -11.6982 | 164.1145 | 99.306 | 74.7544 |
| 8 | 39.36 | -3.9794 | -11.6982 | 164.1145 | 99.306 | 74.7544 |
| 9 | 39.504 | -7.4215 | 17.3861 | 84.3647 | 100.4513 | 69.2454 |
| 10 | 47.61 | -7.1215 | -37.5865 | 92.0623 | 98.5616 | 66.7349 |
| 11 | 49.278 | -10.7215 | 20.2226 | -97.7585 | 100.6231 | 72.0348 |
| 12 | 56.016 | -11.1215 | -50.6106 | 78.4702 | 98.218 | 64.4337 |
| 13 | 73.71 | -5.1215 | -33.911 | 93.1719 | 100.165 | 85.4238 |
| 14 | 78.498 | -6.8215 | -37.5066 | -112.0441 | 100.2604 | 64.1548 |
| 15 | 130.224 | -8.7215 | -43.1797 | 102.4645 | 98.1225 | 64.7824 |
| 16 | 162.63 | -13.2215 | 29.2116 | 67.2359 | 100.2604 | 92.467 |
| 17 | 255.534 | -13.9215 | 27.8133 | 34.5731 | 98.4852 | 65.6889 |
| 18 | 276.018 | -13.9215 | 23.6584 | 48.5813 | 98.1416 | 68.7572 |
| 19 | 329.412 | -15.8215 | -52.5282 | 36.4455 | 97.9698 | 59.1339 |
| 20 | 336.462 | -17.1215 | 25.0168 | 52.6729 | 100.7376 | 65.3402 |
| 21 | 378.39 | -16.0215 | 25.4562 | 49.8296 | 98.1225 | 58.4365 |
| 22 | 398.244 | -15.7215 | 30.7697 | 46.4316 | 98.1034 | 65.2705 |
| 23 | 422.562 | -21.6215 | 35.9234 | 30.759 | 100.4513 | 62.6903 |
| 24 | 519.138 | -22.8215 | -61.2775 | 69.2469 | 100.9476 | 61.993 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 0.799 | 10.4021 | 0.5726 | 4.8814 | 7 |  |

The principle of removing weak clusters is two-fold.

- First, the weak clusters  have less impact on the UE beam management performance due to the narrow UE beam width, especially if the clusters fall outside UE’s spherical coverage. For example, an over 10dB gap between peak and 50%-tile spherical coverage is defined for FR2 PC3 UE. And the clusters highlighted in yellow, which have power less than [-10dB], could be removed as they might have a limited impact on beam management.

- Second, the weak clusters are unlikely to contribute to the strongest gNB Tx beam selection. TE will create its own beams based on the predefined multi-beam codebook of setA and setB. The beams created by the TE may not be exactly aligned with the AoDs/ZoDs of different clusters of a CDL model. Hence, the strongest beam may not be adjacent to the AoD and ZoD of the strongest cluster. However, the AoDs/ZoDs of very weak clusters are unlikely to influence the selection of the strongest beam in any reasonably designed codebook of setA and setB.

Observation 7: The UE’s antenna gain gap between peak and 50%-tile spherical coverage is over 10dB for PC3. The very weak clusters would not influence the UE beam management even if they reach the UE within the UE’s spherical coverage.

Observation 8: The AoDs/ZoDs of very weak clusters are unlikely to influence the selection of the strongest beam in any reasonably designed codebook of setA and setB.

Observation 9:The weak clusters that have a limited impact on the beam management could be removed to reduce the number of clusters in the CDL channel model.

In addition, the clusters that have the same/similar AoAs could be merged (e.g., highlighted by green in Table 1). And the angle spread intra cluster, e.g., CASD, CASA, CZSD, CZSA can be reduced to simplify the test setup.

Observation 10: The clusters that have the same/similar AoAs could be merged. And the intra-cluster angle spread, e.g., CASD, CASA, CZSD, CZSA can be reduced to simplify the test setup.

Proposal 2: RAN4 considers reducing the cluster number with weak power level, merging the cluster with the same/similar AoA, and decreasing intra-cluster angle spread in CDL channel model to simplify the test setup based on the evaluation of the impact on beam management performance considering the UE beam width.

Another potential solution, i.e., CDL channel models presented sequentially using positioner rotation is raised in [3] [4]. As illustrated in Figure 5, Set B -1 and Set B-2 are sequentially transmitted by TE and coordinated by UE positioner to present Set B by TDM manner. But as mentioned in [4], the positioning time for UE rotation is in the order of ~1s which will be too excessive to properly test the AI/ML beam management performance. For example, even a long SSB transmitted periodicity, e.g., 160ms cannot accommodate UE positioning time in AI/ML beam management testing.

Figure 5: CDL channel models presented sequentially using positioner rotation

Observation 11. TE vendors need to check UE positioning time to mimic CDL channel model by sequentially rotating the positioner. Order of ~1s dwell time will lead to an unrealistic test environment for AI/ML beam management testing.

## 2.4 Randomization of beams between SetB and SetA

It is essential to randomize the transmission of setA beams during step1 of progress B of Figure 3 to properly test UE’s AI-ML model. Let us look at a particular prediction metric to understand the importance of this.

3GPP studied different performance metrics for AI/ML based beam management during study item discussion. These metrics are captured in [9]:

Both spatial-domain DL beam prediction and temporal DL beam prediction are considered. For metrics for beam management requirements/tests, the following test metrics/KPIs are identified and could be considered - Option 1: RSRP accuracy - Option 2: Beam prediction accuracy - Top-1 (%) : the percentage of "the Top-1 strongest beam is Top-1 predicted beam" - Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams" - Top-1/K (%) : the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams" - Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB, - Related measurement accuracy can be considered to determine x - Option 4: combinations of above options The overhead/latency reduction should be considered for the requirements as the side condition.

Assume that TE wants to test option 2’s Top-1(%) metric. If the transmission of setA beams is not randomized during measurement step of progressB, UE can simply inform TE that its top-1 strongest beam is same as its top-1 predicted beam of progressA. UE does not even need to measure the beams to predict it. However, UE won’t be able to cheat the test like thisif the transmission of setA beams is randomized during measurement step of progressB.

Observation 12: For some prediction metrics (e.g., option 2 and 3), UE can pass the test without actual measurement if TE does not randomize the transmission of its beams during measurement step of progress of Figure 3.

Proposal 3: During measurement step of progressB, TE randomizes the transmission of setA beams, e.g., during time t1, TE does not transmit with beam shape A1 and TE transmits using the shape of a beam whose ID is randomly selected from set A).

# 3. Conclusion

In summary, we have following observations and proposals.

Observation 1: An effective test case for beam prediction use case BM-Case1 requires the following:

- Sufficient randomness and variation in time and spatial domain of L1-RSRP has to be emulated in the test

- Support emulation of DL Tx beam sweeping with enough number of Tx beams in Set B and Set A

Observation 2: Verifying the AI/ML beam management performance (e.g., prediction of top K beams) in a TDL channel without spatial characteristics makes the test too simple and inconsistent with the actual field environment.

Observation 3: Some companies suggested to emulate multiple beams with single AoA by using different powers. The power levels will have to be defined in test configuration to enable this method. That will allow UE to predict the strongest beam through simple interpolation.

Observation 4: CDL with spatial properties is a realistic and proper channel model for AI/ML beam management testing.

Observation 5: The key requirements for AI/ML BM test setup are listed following:

- Requirement#1: Channel model in the test

  - CDL or simplified CDL is needed for multiple AoAs to properly verify beam management performance.

- Requirement#2: Avoid the cheating

  - Progress A: Data training for Set A’s is generated with channel parameters #1. Set B in UE measurement should have enough randomness compared to Set A’.

  - Progress B: Set A is generated to make sure to use the same parameters (initial phase, angular spread, delay spread) as the Set B in the Progress A measurement step. NW antenna/beam configurations for Set A and Set B should be deterministic.

- Requirement#3: Support the beam number of Set A

  - TE needs to support Set A for ground truth verification, i.e., upto 128.

Observation 6: There will be plenty of additional probes if BS Tx beams point to other clusters even though some of the probes could be reused.

Observation 7: The UE’s antenna gain gap between peak and 50%-tile spherical coverage is over 10dB for PC3. The very weak clusters would not influence the UE beam manamgent even if they reach the UE within the UE’s spherical coverage.

Observation 8: The AoDs/ZoDs of very weak clusters are unlikely to influence the selection of the strongest beam in any reasonably designed codebook of setA and setB.

Observation 9: The weak clusters that have a limited impact on the beam management could be removed to reduce the number of clusters in the CDL channel model.

Observation 10: The clusters that have the same/similar AoAs could be merged. And the intra-cluster angle spread, e.g., CASD, CASA, CZSD, CZSA can be reduced to simplify the test setup.

Observation 11: TE vendors need to check UE positioning time in order to mimic CDL channel model by sequentially rotating the positioner. Order of ~1s dwell time will lead to an unrealistic test environment for AI/ML beam management testing.

Observation 12: For some prediction metrics (e.g., option 2 and 3), UE can pass the test without actual measurement if TE does not randomize the transmission of its beams during measurement step of progress of Figure 3.

Proposal 1: Take FR2 MIMO OTA as the baseline test setup for AI/ML BM testing and further discuss how to enhance the FR2 MIMO OTA test setup to support the multiple AoA/AoD (e.g., beam number of Set A) under simplified CDL channel models with enough randomness.

Proposal 2: RAN4 considers reducing the cluster number with weak power level, merging the cluster with the same/similar AoA, and decreasing intra-cluster angle spread in CDL channel model to simplify the test setup based on the evaluation of the impact on beam management performance considering the UE beam width.

Proposal 3: During measurement step of progressB, TE randomizes the transmission of setA beams, (e.g., during time t1, TE does not transmit with beam shape A1, and TE transmits using the shape of a beam whose ID is randomly selected from set A).

# 4  References

- R4-2412831, ‘Topic summary for [112][129] NR_AIML_air’, Moderator (Qualcomm)

- R4-2414323, ‘WF on AI/ML’, Qualcomm

- R4-2413168, ‘On AI/ML RAN4 Use Cases: Beam Management’, Qualcomm

- R4-2411254, ‘OTA Test System/Testability Considerations for FR2 AI/ML Beam Management Use Cases’, Keysight

- R4-2411587, ‘Discussion on test setup for AI/ML based beam management’, R&S

- R4-1705831, ‘Metrics for evaluating RRM/Demodulation Measurement Setup’, Keysight

- TR38.827

- TR 38.901

[9] 3GPP TR 38.843 V18.0.0, Technical Report (TR): Study on artificial Intelligence (AI)/Machine Learning (ML) for NR air interface.