3GPP TSG-RAN WG4 Meeting #112-bis									                    R4-2415313

Hefei, China, October 14 – 18, 2024

Agenda item:			6.1.1.3.4

Source:	Ericsson

Title:	On the SRS insertion loss imbalance reporting

Document for:	Approval

# Introduction

In RAN#104, a revised WID has been approved [1] for “UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4” WI. One of the three main parts of the WI is to specify 6Rx UE RF requirements for both handheld and FWA UEs, and in this contribution we focus only on the last bullet point from the objectives:

- Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario

  - …

  - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

This topic has been discussed in depth already in Rel-18 under “Enable 8Rx for CPE/FWA/vehicle/industrial devices” topic of “Further RF requirements enhancement for NR and EN-DC in frequency range 1 (FR1)” WI. There, the discussion was mostly focused on the concrete solution rather than the justification to introduce the SRS IL imbalance reporting.

# Discussion

Due to divergent views from different companies, this topic has made very little progress so far. According to the approved WF [2], the companies are still invited to bring analysis on the existing UE behavior and the expected power imbalance for SRS transmissions based on current specifications and the UE implementation. Also, they are encouraged to analyze the impact of SRS IL imbalance on NW performance degradation.

## General considerations for SRS IL imbalance issue

### Existing UE behavior

Based on the limited feedback so far from the companies and the online discussion, it seems that there is no common understanding in the group when it comes to the UE behavior in practice regarding the IL compensation. In our understanding, there is no unified behavior in the field, even though according to clause 7.3.1 in 3GPP TS38.213 specifications [3], the UE shall compensate for the IL below the maximum configured output power for a SRS resource, as additional Tx  insertion loss is not accounted for in the uplink power control equation for SRS transmissions. For a detailed analysis of this issue, one can refer to our RAN4#112 contribution [4], as we will not repeat the same arguments in the present one. Instead, we will reiterate the observations we have made.

Observation 1	According to the current specifications, the UE shall compensate for the IL below the maximum configured output power for a resource, as additional Tx  insertion loss is not accounted for in the SRS power control equations.

Observation 2	It is recognized that in practice the insertion loss may not be fully accounted for (and thus compensated) for all UE implementations. That is a reason to introduce the SRS IL imbalance reporting since the behavior of different UE implementations is impossible to predict for the network.

### Impact of SRS IL imbalance on NW performance degradation

The main issue discussed in this agenda item is that, depending on the UE RF architecture, the power sounded from each receive antenna port/connector may vary due to non-reciprocal insertion losses on given paths and the implementation of the SRS power control on diversity connectors. RAN4 has only specified a maximum allowed relaxation ΔTRxSRS for such ports at the maximum output power. For an accurate DL CSI estimation and good reciprocity-based DL-MIMO performance based on SRS antenna switching, the gNB would ideally be aware of any non-reciprocal IL for each sounded antenna connector/port.

Given how large the specified ΔTRxSRS values are, up to more than 10dB for certain SRS-AS capabilities for higher frequency bands when the device is capable of power class 2, the introduction of some kind of IL (insertion loss) reporting mechanism for SRS Antenna Switching is necessary.

To further justify the need for introducing an IL reporting mechanism for AS we have performed non-full-buffer system-level simulations in [5]. Some of the main conclusions which can be drawn from that document were presented in our RAN4#112 contribution [4] and will not be repeated here. We only reiterate the observation:

Observation 3	Based on simulation results, the SRS IL imbalance without reporting/compensation results in a non-negligible DL MIMO performance loss.

### Lab measurements of Tx/Rx non-reciprocal imbalance

To investigate the actual Tx and Rx imbalances in the field and the extent to which they are reciprocal, we have put a commercial UE under laboratory testing.

The testing has been performed under the following set-up:

| Operating band | n78 |
| --- | --- |
| DL channel model | 1x4 AWGN static |
| UL channel model | 4x16 AWGN static |
| Cell bandwidth | 20 MHz |
| UL MIMO support | Dual Tx |
| AS capability | 2T4R |
| UE Power Class | PC2 |

First, to study the Rx imbalance, the same SSB signal was applied to all four antenna connectors/ports of the UE, where a power level calibration has been performed to obtain a balanced Rx power delivered by the RF cables connected to each antenna connector, i.e. no impact on different cables. In Figure 1, we show the measured SS-RSRP (dBm) over time per antenna connector/port where RSRP0, …, RSRP3 correspond to Port0, …, Port3 respectively. Port0 and Port2 are the primary ports (used for PUSCH/PUCCH transmissions) while Port1 and Port3 are the diversity ports.

Figure 1: SS-RSRP (dBm) per RX port/connector over time

We observe that for this particular UE implementation, ports 0 and 3 are balanced and have around 1.5dB larger SS-RSRP in average than ports 1 and 2 which are also quite balanced. Ideally, the Rx imbalance across all four ports should be minimal, but these measurements show a “small” Rx imbalance, which can only add to the many already identified uncertainties. Another aspect which should be further investigated is whether the Rx gain depend on the input power level, i.e. does the 1.5dB difference between ports 0/3 and 1/2 remain roughly the same for different input power levels.

In Figure 2, we show the received SRS power from four UE antenna ports at the same RX port by the gNB.

Figure 2: SRS power (dBm) per UE antenna port/connector received by the gNB

We observe that the Tx imbalance is considerably larger than the Rx imbalance, the differences between port 0 and port 1, port 0 and port 2, as well as port 0 and port 3 being approximately 6dB, 2dB and 3.5dB, respectively. The Tx imbalance we observe is non-reciprocal to the Rx imbalance (we see no correlation). Primary ports 0 and 2 have around 3.5 – 4.0 dB larger powers compared with the corresponding diversity ports 3 and 1, respectively, which is not unexpected.

Observation 4	Based on the lab testing results of a commercial UE, the Tx imbalance is much larger than and non-reciprocal to the Rx imbalance.

Since the SRS resources are two-ports (2T4R AS), according to TS38.213 and TS38.215 the power of the ports of the given SRS resource shall be the same, i.e. ideally, the total power at the antenna connectors should be split equally for 2T4R. However, according to Figure 2 that is not the case, which also adds to the overall uncertainty. In the current specifications, there is no defined test which verifies the expected behavior.

Observation 5	Based on the lab testing results of a commercial UE, contrary to TS38.213 and TS38.215, the total power at the antenna connectors is not split equally for multi-port SRS resources (e.g. 2T4R), and there is no specified test which verifies the expected behavior.

In our view, it would be very beneficial to introduce testing (e.g. as a part of TEI-17 for the existing ΔTRxSRS requirements) of the SRS output power at least at the maximum power for multi-port SRS resources. For example, for 2T4R AS, setting up a test with 2T4R in mid-bands with the SRS in two adjacent UL slots and with 6 symbols per SRS resource to reduce uncertainty. For the first SRS resource then the total power does not allow any ΔTRxSRS at the maximum power setting, which would be tested. In RAN4, to introduce such a test, new RMCs would be needed in 38.101-1, otherwise the rest could be handled by RAN5.

Observation 6	It would be most beneficial to introduce testing (e.g. as a part of TEI-17) of the SRS output power at least at the maximum power for multi-port SRS resources, which would verify the existing ΔTRxSRS Rel-17 requirements and whether the power is equally split between the SRS ports.

In conclusion, several aspects are very important for the performance of the reciprocity-based DL MIMO (the list is not exhaustive):

- that the SRS output power is tested at least for multi-port SRS resources at the maximum power,

- that the Rx chains are properly calibrated/balanced at the UE antenna connector (could also be tested),

- that the information related to Tx IL imbalance is reported to the network (see clause 2.2 for our proposal).

The last bullet point is probably the most important one given how large the Tx IL imbalance is and hence its contribution to the overall uncertainty, but RAN4 should also work on other mentioned points as part of maintenance/future work.

- Introduce reporting for the SRS insertion loss imbalance issue in Rel-19.

- The introduced solution for the SRS insertion loss imbalance issue should be applicable to all 2Rx/4Rx/6Rx/8Rx cases.

## Proposed SRS IL reporting solution

According to the approved WF [2], the companies are encouraged to provide compromise solutions for consideration with minimal impact to the specification and to indicate the specific impacts to RAN1, RAN2, and RAN4 specifications and performance gain.

Based on the discussion in clause 2.1.1 and the conclusion that the UE behavior when it comes to the IL compensation is not clearly defined (unpredictable), reporting of the actual insertion loss (or the actual IL imbalance) per branch is in general insufficient for the accurate DL CSI estimation. In many scenarios, the UE Tx power is below the maximum configured transmission power, and thus it would be unclear how the network would use this information (since it doesn’t know if the UE performed the IL compensation or not). The actual insertion loss should be implicitly included in the SRS output power.

Observation 7	Since the UE behavior when it comes to the IL compensation is unpredictable, reporting of the actual insertion loss (or the actual IL imbalance) per branch is in general insufficient for the accurate DL CSI estimation (it is unclear how the network would use this information).

Our proposed solution is that reporting should include the configured maximum output power per SRS resource and the power headroom (PH) per SRS resource as well as the information on the power class fallback or the return to the supported power class. In that way the network can have a relatively precise information on each SRS output power at the connector, which is in line with the reasoning in previous clauses.

The information on the configured maximum output power per SRS resource allows. The configured maximum output power includes the actual insertion loss (TX/RX) imbalance (for resources for which this is allowed) on top of the MPR used for the SRS resource. If the MPR is the same between the different ports (which is expected), by inspecting the differences between the configured maximum output power per SRS resource the information on the IL imbalance can be obtained. On the other hand, by transmitting in addition the power headroom (PH) report per SRS resource, the network can be aware of whether a UE operates at its maximum power and it can obtain the information on the actual output power per SRS resource (and observe whether the IL compensation has been performed). So, both cases when UEs perform or do not perform a compensation for the ILs are covered by the proposed solution.

One of the motivations to include ΔPPowerClass in the reporting is the discussion under the Rel-18 maintenance agenda for “Further RF requirements enhancement for NR frequency range 1 (FR1)” WI, where the ΔPPowerClass requirements for AS for 4Tx are currently still incomplete. To see a detailed argumentation from our side, one could check e.g. [6]. Here, we will only reiterate that the solution similar to the one specified for 2Tx would be sub-optimal (4Tx adds a lot of complexity compared with 2Tx) and that the issue could be optimally solved only by introducing the reporting of ΔPPowerClass.

- IL imbalance reporting mechanism for SRS AS should include the configured maximum output power per SRS resource, the power headroom per SRS resource and ΔPPowerClass.

The PH used for the SRS resource can be a Type 3 but used for a new MAC-CE “SRS resource power report” (instead of PHR MAC-CE) and can be used also for a carrier configured for PUSCH transmission. This would be a RAN1 change but very limited (i.e. apply the Type 3 for the new MAC-CE and also on a PUSCH carrier). A new MAC-CE specified for SRS power reporting should be dynamic since SRS AS power levels can change.

- The PH used for the SRS resource can be a Type 3 but used for a new MAC-CE “SRS resource power report” and can be used also for a carrier configured for PUSCH transmission.

As a potential compromise, one way to resolve the SRS IL imbalance reporting issue (especially if companies could not find an agreement on one solution) would be to introduce two types of reporting for a UE: a “baseline” and an “enhanced” reporting. The UE could indicate which one it is using. The baseline reporting would not require any calibration at the UE and it could include e.g. the PHR and ΔPPowerClass information. This would also be useful for UEs not compensating for the IL. That information alone is not enough for the accurate knowledge of the SRS transmission power but it would give at least the information on whether the UE operates at its maximum output power, as well as which power class is being used. The advanced reporting could in addition include the information on e.g. the configured maximum output power which would substantially increase the knowledge of the SRS transmission output power by the network.

- Another way to resolve the SRS IL imbalance reporting issue would be to introduce two types of reporting for a UE: a “baseline” and an “enhanced” reporting. The baseline reporting would not require any calibration at the UE and it could include e.g. the PHR and ΔPPowerClass information. The advanced reporting could in addition include the information on e.g. the configured maximum output power.

# Conclusion

In the previous sections, we made the following observations:

Observation 1	According to the current specifications, the UE shall compensate for the IL below the maximum configured output power for a resource, as additional Tx  insertion loss is not accounted for in the SRS power control equations.

Observation 2	It is recognized that in practice the insertion loss may not be fully accounted for (and thus compensated) for all UE implementations. That is a reason to introduce the SRS IL imbalance reporting since the behavior of different UE implementations is impossible to predict for the network.

Observation 3	Based on simulation results, the SRS IL imbalance without reporting/compensation results in a non-negligible DL MIMO performance loss.

Observation 4	Based on the lab testing results of a commercial UE, the Tx imbalance is much larger than and non-reciprocal to the Rx imbalance.

Observation 5	Based on the lab testing results of a commercial UE, contrary to TS38.213 and TS38.215, the total power at the antenna connectors is not split equally for multi-port SRS resources (e.g. 2T4R), and there is no specified test which verifies the expected behavior.

Observation 6	It would be most beneficial to introduce testing (e.g. as a part of TEI-17) of the SRS output power at least at the maximum power for multi-port SRS resources, which would verify the existing ΔTRxSRS Rel-17 requirements and whether the power is equally split between the SRS ports.

Observation 7	Since the UE behavior when it comes to the IL compensation is unpredictable, reporting of the actual insertion loss (or the actual IL imbalance) per branch is in general insufficient for the accurate DL CSI estimation (it is unclear how the network would use this information).

Based on the discussion in the previous sections we propose the following:

Proposal 1	Introduce reporting for the SRS insertion loss imbalance issue in Rel-19.

Proposal 2	The introduced solution for the SRS insertion loss imbalance issue should be applicable to all 2Rx/4Rx/6Rx/8Rx cases.

Proposal 3	IL imbalance reporting mechanism for SRS AS should include the configured maximum output power per SRS resource, the power headroom per SRS resource and ΔPPowerClass.

Proposal 4	The PH used for the SRS resource can be a Type 3 but used for a new MAC-CE “SRS resource power report” and can be used also for a carrier configured for PUSCH transmission.

Proposal 5	Another way to resolve the SRS IL imbalance reporting issue would be to introduce two types of reporting for a UE: a “baseline” and an “enhanced” reporting. The baseline reporting would not require any calibration at the UE and it could include e.g. the PHR and ΔPPowerClass information. The advanced reporting could in addition include the information on e.g. the configured maximum output power.

# References

- RP-241656, “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, RAN#104.

- R4-2414285, “WF on 6Rx UE requirements”, AT&T, RAN4#112.

- 3GPP TS38.213 V18.4.0, “NR; Physical layer procedures for control”.

- R4-2413360, “On need for SRS insertion loss imbalance reporting”, Ericsson, RAN4#112.

- R1-2305911, “Discussion on RAN4 LS on the UE SRS imbalance issue”, Ericsson, RAN1#113.

- R4-2413357, “(NR_ENDC_RF_FR1_enh2-Core) On DeltaP_PowerClass for SRS AS for 4Tx”, Ericsson, RAN4#112