3GPP RAN WG4 Meeting #112bis																R4-2415318

Hefei, China, 14th – 18th October 2024

Agenda item:	6.8.3.2

Source:	Apple

Title:	On UE RF requirements for the 3MHz channel

WI/SI:	NR_IoT_NTN_req_test_enh

Release:	Rel-19

Document for:	Decision

# 1	Introduction

After the RAN#86 meeting, the Rel-17 WI was approved that aimed at enabling the 5G/NR radio access technology for non-terrestrial satellite deployments [1]. One of the WI objectives was to define and introduce the corresponding bands into the 3GPP specifications. The outcome of these discussions was two NTN bands – L-band and S-band – which were added as bands n255 and n256, respectively, to the Rel-17 core specifications. However, already at that time it was noted that there exist other satellite deployments. To limit the overall workload in Rel-17, it was decided by RAN WG4 to focus on L- and S-bands with the common understanding that other potential NTN bands should follow the "normal" process of submitting a new spectrum WI. For instance, after the RAN#98 meeting a new Rel-18 NTN spectrum WI was agreed with the objective of adding support for a new NTN band (band n254) where the UL is on the L-band and the DL is on the S-band [2]. So-called extended L-band was also completed in Rel-18. And after the RAN#104 meeting another spectrum WI was agreed for a satellite band operating in the S-band frequency range [3].

After the RAN#104 meeting a new WI was agreed [4], one of the aims of which is to enable the 3MHz channel for the NTN bands. Based on the WF document from the previous RAN4#112 meeting [5] there are several open issues for the UE Rx and Tx requirements, for which this discussion paper presents more technical considerations.

# 2	UE RF requirements for the 3MHz NTN channel

## 2.1	Rx requirements

One of the open Rx issues is which REFSENS values to adopt for the 3MHz channel in the existing NTN bands: n254, n255 and n256.  As summarised in Table 2.1-1 below, the NTN bands n256 and n254 belong to the the same frequency range and thus can have the same REFSENS, as in fact was already agreed for the 5MHz channel in these NTN bands. And the value of -101.2dBm is chosen so that it is aligned with existing LTE band 65 as defined in TS 36.101. As for the NTN band n255, since its DL resides in the lower frequency ranges, it may have better REFSENS as it is already the case for the 5MHz channel.

Table 2.1-1: Proposed REFSENS values for the existing NTN bands.

| NTN band | REFSENS value | Comment |
| --- | --- | --- |
| n256 | -101.2dBm | Aligned with LTE band 65 from TS 36.101 |
| n254 | -101.2dBm | Aligned with the NTN band n256 because of the same DL frequency range |
| n255 | -101.7dBm |  |

Proposal 1a:	REFSENS values for the 3MHz channel in existing NTN bands n254, n255 and n256 are -101.2dBm, -101.7dBm and -101.2dBm, respectively.

It is worth noting that in Rel-19 new NTN bands will be standardised, where the 3MHz channel can be also deployed. If so, the REFSENS values for those bands can be as the ones presented in Table 2.1-2 below.

Table 2.1-2: Proposed REFSENS values for the new NTN bands.

| NTN band | REFSENS value | Comment |
| --- | --- | --- |
| n252 | -101.2dBm | Aligned with the NTN band n256 |
| n251 | -101.7dBm | Aligned with the NTN band n255 as these bands share the same DL frequency range |
| n250 |  |  |

Proposal 1b:	REFSENS values for the 3MHz channel in new NTN bands n252, n250/n251 are -101.2dBm and -101.2dBm, respectively.

## 2.2	Tx requirements

One of the action points from the previous meeting was to check basic Tx requirements, such as maximum output power, emission mask and ACLR. As for the maximum output power, current NTN core specifications already support PC3, 23dBm, which can be naturally adopted for the 3MHz channel. It should be also noted that once 3GPP progresses further on 26dBm, it can be considered as well. As for ACLR and emission mask, our view is that existing 3MHz requirements from the terrestrial bands can be applied also to the NTN core requirements.

Proposal 2a:	The maximum output power is 23dBm (and once 26dBm is standardised it can be also considered).

Proposal 2b:	ACLR and the emission mask can be re-used from the existing terrestrial 3MHz Tx requirements.

Another open issue for the UE Tx requirements was the potential UE power back-off and A-MPR values. In general, there should be band-by-band analysis on whether A-MPR is needed for the 3MHz channel in the given NTN band. At least for the NTN band n254 our preliminary analysis in [6] indicates that some A-MPR is needed for at the lower and upper edges of the band.

Proposal 2c:	Whether A-MPR is needed for the 3MHz channel depends on the given NTN band and should be analysed accordingly.

Proposal 2d:	The 3MHz channel needs A-MPR at least in case of band n254.

# 3	Conclusions

In this discussion paper we have provided our input of the UE RF requirements for the 3MHz NTN channel. For the Rx requirements, we propose REFSENS values for the existing NTN bands – n254, n255 and n256 – whereupon the proposed values are aligned with the existing requirements. And since new NTN bands, such as n250/n251 and n252, share the same frequency range as the existing bands, the corresponding values can be also considered for new bands. As for the Tx requirements, we can already use PC3 with the 3MHz NTN channel, for which we suggest re-using existing core Tx requirements for ACLR and emission mask. Nevertheless, A-MPR needs to be checked further; and at least for band n254 our initial results indicate that some A-MPR is needed also for the 3MHz channel.

Proposal 1a:	REFSENS values for the 3MHz channel in existing NTN bands n254, n255 and n256 are -101.2dBm, -101.7dBm and -101.2dBm, respectively.

Proposal 1b:	REFSENS values for the 3MHz channel in new NTN bands n252, n250/n251 are -101.2dBm and -101.2dBm, respectively.

Proposal 2a:	The maximum output power is 23dBm (and once 26dBm is standardised it can be also considered).

Proposal 2b:	ACLR and the emission mask can be re-used from the existing terrestrial 3MHz Tx requirements.

Proposal 2c:	Whether A-MPR is needed for the 3MHz channel depends on the given NTN band and should be analysed accordingly.

Proposal 2d:	The 3MHz channel needs A-MPR at least in case of band n254.

# 4	References

- RP-193234, "Solutions for NR to support non-terrestrial networks (NTN)", Thales

- RP-223431, "New WID on Introduction of the satellite L-/S-band", Globalstar, Apple

RP-241661, "New WID on Introduction of NR-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL)", Dish Network, EchoStar, TerreStar, OmniSpace, Telus

- RP-240857, "New WID: Enhanced requirements and test methodology for NR and IoT NTN"

- R4-2413528, "WF on for [112][312] NR_IoT_NTN_less_than_5MHz_UERF", Xiaomi

- R4-2415319, "Initial results for the power back-off values for the 3MHz channel in band n254", Apple, Globalstar