3GPP RAN4 Meeting #112bis		R4-2415321

Hefei, China, 14th – 18th October, 2024

Agenda item:	6.12.2.1

Source:	Apple

Title:	Discussion on XR test method and Configurations

WI/SI:	TRP_TRS_MIMO_OTA_Ph3-Core

Release:	Rel-19

Document for:	Discussion

# 1	Introduction

During the RAN4 #112 meeting, the following agreements have been achieved in WF [1].

This contribution provides our further views related to this objective.

# 2	Discussion

As discussed during the RAN4 #112 meeting, there are many proposals discussing how to reduce test time for XR testing. This contribution makes the following observations and proposals.

## 2.1 TRP test time reduction

According to TS 38.161, The measurement is performed with a constant sampling step, e.g., 15 degrees or 30 degrees, in both theta (θ) and phi (ϕ) axes for TRP measurement.

Observation 1: 		Based on 15 degrees sampling step, there are 266 measurement points. Our observation is test time for one channel takes around 10-15 minutes in anechoic chamber, and there is diminishing value in reducing this test time any further with a coarse TRP grid.

Proposal 1: 			For TRP testing, use the same procedure as defined in TS38.161, and RAN4 should no longer discuss coarse grid for TRP.

## 2.1 TRS test time reduction

According to TS 38.161, The measurement is performed with a constant sampling step, e.g., 30 degrees or 45 degrees, in both theta (θ) and phi (ϕ) axes for TRP measurement. Receiver sensitivity measurements is defined as the minimum downlink signal power received at the UE antenna input required to provide a data throughput rate greater than or equal to 95% of the maximum throughput of the reference measurement channel (RMC).

Observation 2: 		Based on 30 degrees sampling step, there are 62 measurement points. Our observation is test time for one channel takes around 60 minutes in anechoic chamber.

This test procedure is to allow the radiated Rx sensitivity three-dimensional scan as defined in TS 38.161 to be performed at low Tx power levels at 10 dBm. This will extend the DUT battery life during testing, enabling more testing to be done without interrupting the DUT and test measurement setup.

The entire TRS test procedure shall be performed using same low Tx power. Following the completion of the TRS test, the position and polarization needed to be identified for the maximum radiated sensitivity measurement for one channel. Perform the TRS measurement at the identified single point at the DUT’s maximum output power, with as few changes to the DUT and test setup as possible. The delta between these two measurements at peak radiated sensitivity position can be used as an offset to calculate the TRS at maximum transmit power. These steps shall be repeated for each specified channels and bands.

TRS maximum transmit power = TRS Tested with low power + EIS maximum Tx power - EIS low Tx power

The UL output power of LTE and NR carrier should be set as a constant power of 10dBm, i.e., with fixed p-MaxEUTRA-r15=10 dBm and p-NR-FR1=10dBm.

Observation 3: 	Conducting TRS testing at low transmit power allows for more tests to be performed without requiring the DUT to be recharged.

Proposal 2:  		To reduce TRS test time, enable TRS testing at a low transmit power of 10 dBm. This can be achieved by offsetting the TRS from the delta at Peak EIS, using both maximum output power and low transmit power.

# 3	Conclusions

This contribution provides our views on the topic of headworn XR test scenarios and configurations for XR headworn devices. This contribution makes the following proposals.

Proposal 1: 			For TRP testing, use the same procedure as defined in TS38.161, and RAN4 should no longer discuss coarse grid for TRP.

Proposal 2:  			To reduce TRS test time, enable TRS testing at a low transmit power of 10 dBm. This can be achieved by offsetting the TRS from the delta at Peak EIS, using both maximum output power and low transmit power.

# 4	References

R4-2413603, “Way Forward for [112][333] TRP_TRS_MIMO_OTA” vivo, 3GPP RAN4 #112