3GPP RAN TSG-RAN4 Meeting # 112-Bis		R4-2415322

Hefei, China, October 14 – 18, 2024

Agenda item:	5.20.2

Source:	Apple

Title:	On UE RF Requirements for n28 with 40MHz UL CBW

WI/SI:	NR_n28_PC2_40MHz_CBW-Core

Release:	Rel-19

Document for:	Discussion

# 1	Introduction and Scope

In the previous RAN4 meeting, (Meeting # 112, Maastricht, Netherlands, August 19-23, 2024), discussions were held on several proposals summited for the recently introduced Rel-19 WID (NR_n28_PC2_40MHz ) which defines the requirements for band n28 with 40MHz uplink channel bandwidth with full-band duplexer considered[1]. As captured in the topic WF [2], several aspects of the WID were considered and a summary of the main open issues are shown in the diagram below. In this discussion paper, we will focus on the following aspects:

- A-NPR for PC2

- n28 REFSENS for PC3 and RSD for PC2

### Sub-topic 2-3 40MHz requirements for PC3 and PC2

Issue 2-3-1 Uplink configuration for n28 REFSENS

Agreement:

- Adopt the following UL configuration for 40MHz

| Operating band / SCS (kHz) / Channel bandwidth (MHz) / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 | Duplex Mode |
| n28 | 15 | 15 | 25 | 251 | 251 | 251 | 251 | 251 |  | 251 |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 101 | 101 | 101 | 101 | 101 |  | 101 |  |  |  |  |  |  |  |  |
| Note 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Issue 2-3-2 n28 REFSENS for PC3

Agreement:

- FFS on PC3 REFSENS based on more companies’ input.

- Following values are proposed in this meeting as starting point.

| Source | 40 MHz (dBm) |
| --- | --- |
| Skyworks(R4-2413062) | -66.3 |
| Qualcomm (R4-2413149) | -65.9 |
| Muruta (R4-2411476) | -67.1 for 15KHz -67.2 for 30KHz |
| Average | -66.4 |

Issue 2-3-3 PC2 RSD for 1Tx and 2Tx for 40MHz

Agreement:

- FFS on PC2 RSD for 40MHz

Issue 2-3-4 ∆MPR for 40MHz

Agreement:

- Further check if ΔMPR= 0.5dB is sufficient

Using the 40MHz UL configuration shown above, we will present the results of our Tx simulations for n28 A-NPR and receiver desensitization due to Tx impairments. The simulated Tx noise included the PA non-linearity effects as well as other Tx impairments such as LO leakage, IQ imbalance, and Tx intermodulation effects.

# Discussion

##### 2.1 On PC2 NS_17

The situation for NS_17 for a 10MHz channel with lower edge at 718MHz is depicted below. The in-band emission limit is applicable up to frequency 710MHz since no exception for FOOB applies.

The simulation results for emission compliance and A-MPR were provided in [2]. The WF [3] from last meeting captures the starting point for the on-going A-MPR discussion. Table 1 is copied from the WF and highlights a single change which has been previously proposed in [2].

Table 1: A-MPR regions for NS_17 for PC2

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 10 MHz | 723 ≤ Fc ≤ 728 | ≤ 0.18 | ≤ 1.44 | A1 |
|  |  | ≥ 0 | >= 5.4 | A2 |

Table 2: A-MPR for NS_17 for PC2

| Modulation/Waveform |  | A1 | A2 |
| --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | ≤ [3] | ≤ [4] |
|  | QPSK | ≤ [3] | ≤ [4] |
|  | 16 QAM | ≤ [3.5] | ≤ [4] |
|  | 64 QAM | ≤ [4] | ≤ [4.5] |
|  | 256 QAM |  | ≤ [5.5] |
| CP-OFDM | QPSK | ≤ [5] | ≤ [5.5] |
|  | 16 QAM | ≤ [5] | ≤ [5.5] |
|  | 64 QAM | ≤ [5] | ≤ [5.5] |
|  | 256 QAM |  |  |

Proposal 1: Consider Table 1 and Table 2 when specifying NS_17.

##### 2.2 On PC2 NS_18

This section discusses the introduction of 40MHz channel for NS_18. Simulations results for smaller channel with PC2 have been provided in an earlier paper [4]. The Table 3 and Table 4 provide simulation results for PC3 and PC2. Additionally, the tables provide a visualization for the proposed RB region layout. It can be observed that PC2 has an additional region which is needed as there exist several allocations which exceed MPR. A new region is required to cover their power back-off need.

Table 3: PC3 simulation results for NS_18 with 40MHz CBW and QPSK

| DFT-s-OFDM | CP-OFDM | Proposed RB region layout |
| --- | --- | --- |

Table 4: PC2 simulation results for NS_18 with 40MHz CBW and QPSK

| DFT-s-OFDM | CP-OFDM | Proposed RB region layout |
| --- | --- | --- |

Table 5: PC3 region definition for 25MHz, 30MHz and 40MHz channel bandwidth

| Channel Bandwidth, MHz | Frequency range of UL transmission bandwidth configuration, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 25 | 703~733 | >(LCRB*12*SCS)/2+3.6 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+3.6 | ≥5.4 | A4 |
|  |  | ≤6.3 | <5.4 | A5 |
| 30 | 703~733 | >(LCRB*12*SCS)/2+5.22 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+5.22 | ≥5.4 | A4 |
|  |  | ≤7.92 | <5.4 | A5 |
| 40 | 703~743 | >(LCRB*12*SCS)/2+8.46 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+8.46 | ≥5.4 | A4 |
|  |  | ≤11.16 | <5.4 | A5 |

Table 6: PC2 region definition for 25MHz, 30MHz and 40MHz channel bandwidth

| Channel Bandwidth, MHz | Frequency range of UL transmission bandwidth configuration, MHz | Regions (PC2) |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 25 | 703~733 | >(LCRB*12*SCS)/2+3.6 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+3.6 | ≥5.4 | A4 |
|  |  | ≤6.3 | <5.4 | A5 |
|  |  | >(LCRB*12*SCS)/2+3.6 ≤(LCRB*12*SCS)/2+5.76 | <Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) ≥5.4 | A6 |
| 30 | 703~733 | >(LCRB*12*SCS)/2+5.22 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+5.22 | ≥5.4 | A4 |
|  |  | ≤7.92 | <5.4 | A5 |
|  |  | >(LCRB*12*SCS)/2+5.22 ≤(LCRB*12*SCS)/2+7.38 | <Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) ≥5.4 | A6 |
| 40 | 703~743 | >(LCRB*12*SCS)/2+8.46 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+8.46 | ≥5.4 | A4 |
|  |  | ≤11.16 | <5.4 | A5 |
|  |  | >(LCRB*12*SCS)/2+8.46 ≤(LCRB*12*SCS)/2+11.7 | <Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) ≥5.4 | A6 |

Table 7: PC2 A-MPR for NS_18

| Modulation/Waveform |  | A3 (dB) | A4 (dB) | A5 (dB) | A6 (dB) |
| --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | [3+0.5] | [8+2] | [3+1.5] | [0+2.0] |
|  | QPSK | [3+0.5] | [8+2] | [3+1.5] | [0+2.0] |
|  | 16 QAM | [3+0.5] | [8+3] | [3+2] | [1.0+1.0] |
|  | 64 QAM | [3+0.5] | [8+3] | [4.5+1] | [2.5] |
|  | 256 QAM | [3+0.5] | [8+3] | [5.5] | [4.5] |
| CP-OFDM | QPSK | [4.5+0.5] | [9.5+2] | [5+1] | [1.5+2.5] |
|  | 16 QAM | [4.5+0.5] | [9.5+2] | [5+1] | [2.0+2.0] |
|  | 64 QAM | [4.5+0.5] | [9.5+2] | [5.5+0.5] | [3.5+0.5] |
|  | 256 QAM | [4.5+0.5] | [9.5+2] | [7.5] | [6.5] |

Proposal 2: Consider Table 5 to 7 when specifying NS_18.

## REFSENS and RSD analysis for n28 Rx with 40MHz UL CBW

When 40MHz of uplink channel bandwidth is considered for Band n28, significant receiver desensitization can be observed because the first UL adjacent channel overlaps with the Rx band. As captured in Figure 2.1 below, pronounced noise can be seen when the transmitter noise is simulated with the several UL impairments considered. This Tx noise will contribute to n28 RX desensitization, and therefore requirements for REFSENS has to be derived.

For the analysis, we used the UL CBW configuration shown below:

| Operating band / SCS (kHz) / Channel bandwidth (MHz) / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 | Dpx Mode |
| n28 | 15 | 15 | 25 | 251 | 251 | 251 | 251 | 251 |  | 251 |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 101 | 101 | 101 | 101 | 101 |  | 101 |  |  |  |  |  |  |  |  |
| Note 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 2.1: UL configuration for 40MHz UL CBW

For the REFSENS and RSD analysis, we used the following worst-case assumptions for the RF front-end performance parameters:

- Antenna isolation: 10dB

- Front-end insertion loss: 4.0dB

- Duplexer isolation at Tx/Rx: 50 dB

- Duplexer attenuation at Rx band: 45 dB

- UL CBW: 40MHz

- SNR: -1dB

Given the assumptions above, the REFSENS analysis is summarized in the table below.

| Parameter | Unit | PC3 |  | PC2 (1Tx) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Main Path | Div. path | Main Path | Div. path |
| TX noise at PA output | dBm/CBW | -11.2 |  | -9.3 |  |
| Tx noise @ ANT | dBm/CBW | -57.2 | -66.7 | -55.3 | -65.3 |
| 3GPP REFSENS (40MHz CBW) | dBm | -89 | -89 | -89 | -89 |
| Thermal noise | dBm | -88 | -88 | -88 | -88 |
| Composite noise | dBm | -57.2 | -67.2 | -55.3 | -65.3 |
| REFSENS after MRC | dBm | -65.9 |  | -63.7 |  |

Table 2.2: REFSENS analysis for n28 with 40MHz UL CBW

Observation 1: Based on the analysis above, the PC3 40MHz REFSENS for n28  estimated at -65.9dBm, resulting into a self-desensitization of 23.1dB.

Observation 2: For PC2 1Tx, the amount of self-desensitization increases to 25.3dB, which translates into an RSD value of 2.2dB.

Proposal 3: For band n28 with 40MHz CBW, we propose the following requirements for REFSENS and RSD:

- REFSENS for PC3 = -65.9dBm

- PC2 RSD for 1Tx = 2.2dB

##### 2.4 On 40MHz CBW and ∆MPR

Special relaxation for the output power is defined for FDD bands where the relative bandwidth is larger than 3%. For 30MHz with a relative bandwidth is 4.1% there exists a delta value of 0.5dB.  The RAN4#112 WF [2] encourages to further discuss ∆MPR as the relative bandwidth is around 5.5% for a 40MHz channel on band n28. The situation shares similarity with n71 where 35MHz channel has a relative bandwidth of around 5.1%. Therefore, we think that 0.5dB should be sufficient for 40MHz on band n28.

Proposal 4: It is proposed to set the ∆MPR for 40MHz channel to 0.5dB.

# 3	Conclusion

In this contribution, we shared the UE RF requirements for band n28 with 40MHz of CBW. Below is a summary of our proposals:

Proposal 1: Consider Table 1 and Table 2 when specifying NS_17.

Proposal 2: Consider Table 5 to 7 when specifying NS_18.

Observation 1: Based on the analysis above, the PC3 40MHz REFSENS for n28  estimated at -65.9dBm, resulting into a self-desensitization of 23.1dB.

Observation 2: For PC2 1Tx, the amount of self-desensitization increases to 25.3dB, which translates into an RSD value of 2.2dB.

Proposal 3: For band n28 with 40MHz CBW, we propose the following requirements for REFSENS and RSD:

- REFSENS for PC3 = -65.9dBm

- PC2 RSD for 1Tx = 2.2dB

Proposal 4: It is proposed to set the ∆MPR for 40MHz channel to 0.5dB.

# 4	References

RP-241660, “New WID: Introduction of Power Class 2 and UE 40MHz Channel Bandwidth in NR band n28”, CMCC, CBN, 3GPP TSG RAN Meeting #104, Shanghai, June 17 – 20, 2024

R4-2414274, “WF on introduction of PC2 and 40MHz CBW in NR band n28”, CMCC, 3GPP TSG RAN Meeting #112, Maastricht, Netherlands, August 19 – August 23, 2024