3GPP RAN TSG-RAN4 Meeting # 112-Bis		R4-2415323

Hefei, China, October 14 – 18, 2024

Agenda item:	5.10.2

Source:	Apple

Title:	On DL interruption for Tx switching across 4 bands

WI/SI:	DL_intrpt_combos_TxSW_R19-Core

Release:	Rel-19

Document for:	Discussion

# 1	Introduction and Scope

During the previous RAN4 meeting (3GPP TSG-RAN Meeting #112, Maastricht, Netherlands, 19th – 23rd August 2024), discussions were held on proposals submitted for the recently introduced Rel-19 WID on DL interruption due to UL Tx switching across 3 and 4 bands (DL_intrpt_combos_TxSW_R19-Core) [1]. The way forward [2], as summarized below, highlights the decisions made on the main issues that were debated. As can be seen from the WF, the DL interruption applicability was applied for the three CA combinations included in the original WID, based on the criterias agreed in the meeting.

# Way forward Issue 2-1-1: Applicability criteria Agreement: Considering the following criterias when deciding to mandate no DL interruption applicability for TX switching among 3 or 4 bands - Frequency separation between the DL band and the bands involved in the Tx switching for FDD+TDD band combinations and the corresponding higher order combinations - Whether the same UL-DL duplex mode is used for TDD+TDD band combinations and the corresponding higher order combinations - Whether simultaneous RX/TX is mandated between the DL band(s) and the UL band(s) - Whether same antenna group is used for band combinations and the corresponding higher order combinations - Number of TX chains to be switched Issue 2-2-1: Definition format for indicating no DL interruption requirement Way forward: Considering the following format as starting point for indicating no DL interruption requirement for TX switching among 3 or 4 bands NR CA Band; NR Band (Table 5.2-1); DL interruption allowed (Note 4). CA_n1-n3-n783; n1, n3, n78; No for CA_n1-n78, CA_n3-n78. CA_n3-n5-n78; n3, n5, n78; No for CA_n3-n78, CA_n5-n78. CA_n1-n3-n5-n78; n1, n3, n5, n78; No for CA_n1-n78, CA_n3-n78, CA_n5-n78.

In this discussion proposal, we applied the DL interruption applicability rules highlighted in the WF as shown above to the 4-band CA combinations found in Table 5.2A.2.3 of the TS 38.101-1.

# 2	Discussions

In the current version of the specifications (TS 38.101-1), for the 2-band CA combinations, there is a column in Table 5.2A.2.1-1 that indicates the applicability of DL interruption: For example, DL interruption is allowed for the CA_n1-n7 CA combinations while no DL interruption is allowed for the CA_n1-n77 combination.

Similarly, Table 5.2A.2.2-1 contains a column that indicates the applicability of DL interruption for the 3-band CA combinations: As an example, DL interruption is allowed for any pair configured for UL Tx switching within the CA_n1-n3-n5 CA combination, while for CA_n1-n3-n78, there is no DL interruption for CA_n1-n78 and CA_n3-n78.

Observation 1: In the current Release 18 specifications, DL interruption applicability for UL Tx switching across 2 bands and 3 bands is clearly defined for every band NR CA band combination.

However, in Table 5.2A.2.3-1 where the band configurations for 4-band CA combinations are defined, there is no DL applicability defined for all the combinations. There is no indication on whether DL interruption should or shouldn’t be allowed for none of the UL Tx band combo that could be configured for UL TX switching.

Observation 2: There is no DL interruption applicability for UL Tx switching across 4 bands in the current specifications.

Using the rules highlighted in the previous meeting as shown above, we modified Table 5.2A.2.3-1 to include the column for DL applicability for every 4-band CA combination as shown in Table 3-1 below.

# 3	Conclusions

Observation 1: In the current Release 18 specifications, DL interruption applicability for UL Tx switching across 2 bands and 3 bands is clearly defined for every band NR CA band combination.

Observation 2: There is no DL interruption applicability for UL Tx switching across 4 bands in the current specifications.

Proposal 1: Using the DL interruption rules adopted in the previous meeting, we propose Table 3-1 below to highlight the DL interruption applicability for UL Tx switching involving 4 NR bands.

| NR CA Band | NR Band (Table 5.2-1) | DL interruption allowed |
| --- | --- | --- |
| CA_n1-n3-n5-n7 | n1, n3, n5, n7 |  |
| CA_n1-n3-n5-n28 | n1, n3, n5, n28 |  |
| CA_n1-n3-n5-n78 | n1, n3, n5, n78 | No for CA_n3-n78, CA_n5-n78 |
| CA_n1-n3-n7-n8 | n1, n3, n7, n8 |  |
| CA_n1-n3-n7-n26 | n1, n3, n7, n26 |  |
| CA_n1-n3-n7-n28 | n1, n3, n7, n28 |  |
| CA_n1-n3-n7-n38 | n1, n3, n7, n38 |  |
| CA_n1-n3-n7-n67 | n1, n3, n7, n67 |  |
| CA_n1-n3-n7-n75 | n1, n3, n7, n75 |  |
| CA_n1-n3-n7-n79 | n1, n3, n7, n79 | No for CA_n3-n79 |
| CA_n1-n3-n7-n781 | n1, n3, n7, n78 | No for CA_n3-n78 |
| CA_n1-n3-n7-n105 | n1, n3, n7, n105 |  |
| CA_n1-n3-n8-n77 | n1, n3, n8, n77 | No for CA_n3-n77, CA_n8-n77 |
| CA_n1-n3-n8-n781 | n1, n3, n8, n78 | No for CA_n3-n78, CA_n8-n78 |
| CA_n1-n3-n18-n28 | n1, n3, n18, n28 |  |
| CA_n1-n3-n18-n41 | n1, n3, n18, n41 |  |
| CA_n1-n3-n18-n77 | n1, n3, n18, n77 | No for CA_n3-n77, CA_n18-n77 |
| CA_n1-n3-n20-n67 | n1, n3, n20, n67 |  |
| CA_n1-n3-n26-n78 | n1, n3, n26, n78 | No for CA_n3-n78, CA_n26-n78 |
| CA_n1-n3-n28-n38 | n1, n3, n28, n38 |  |
| CA_n1-n3-n28-n41 | n1, n3, n28, n41 |  |
| CA_n1-n3-n28-n771 | n1, n3, n28, n77 | No for CA_n3-n77, CA_n28-n77 |
| CA_n1-n3-n28-n78 | n1, n3, n28, n78 | No for CA_n3-n78, CA_n28-n78 |
| CA_n1-n3-n28-n791 | n1, n3, n28, n79 | No for CA_n5-n79, CA_n3-n79 |
| CA_n1-n3-n40-n77 | n1, n3, n40, n77 | No for CA_n3-n77 |
| CA_n1-n3-n40-n105 | n1, n3, n40, n105 |  |
| CA_n1-n3-n41-n77 | n1, n3, n41, n77 | No for CA_n3-n77 |
| CA_n1-n3-n41-n79 | n1, n3, n41, n79 | No for CA_n3-n79 |
| CA_n1-n3-n67-n78 | n1, n3, n67, n78 | No for CA_n3-n78 |
| CA_n1-n3-n75-n78 | n1, n3, n75, n78 | No for CA_n3-n78 |
| CA_n1-n3-n77-n79 | n1, n3, n77, n79 | No for CA_n3-n77, CA_n3-n79 |
| CA_n1-n5-n7-n40 | n1, n5, n7, n40 |  |
| CA_n1-n5-n7-n78 | n1, n5, n7, n78 | No for CA_n5-n78 |
| CA_n1-n5-n7-n105 | n1, n5, n7, n105 |  |
| CA_n1-n5-n28-n78 | n1, n5, n28, n78 | No for CA_n5-n78, CA_n28-n78 |
| CA_n1-n5-n28-n79 | n1, n5, n28, n79 | No for CA_n5-n79, CA_n28-n79 |
| CA_n1-n5-n40-n78 | n1, n5, n40, n78 | No for CA_n5-n78 |
| CA_n1-n5-n40-n105 | n1, n5, n40, n105 |  |
| CA_n1-n5-n78-n79 | n1, n5, n78, n79 | No for CA_n5-n78, CA_n5-n79 |
| CA_n1-n5-n78-n105 | n1, n5, n78, n105 | No for CA_n5-n78 |
| CA_n1-n7-n8-n40 | n1, n7, n8, n40 |  |
| CA_n1-n7-n8-n781 | n1, n7, n8, n78 | No for CA_n8-n78 |
| CA_n1-n7-n26-n78 | n1, n7, n26, n78 | No for CA_n26-n78 |
| CA_n1-n7-n28-n38 | n1, n7, n28, n38 | No for CA_n28-n78 |
| CA_n1-n7-n28-n78 | n1, n7, n28, n78 | No for CA_n28-n78 |
| CA_n1-n7-n40-n78 | n1, n7, n40, n78 |  |
| CA_n1-n7-n40-n105 | n1, n7, n40, n105 |  |
| CA_n1-n7-n67-n78 | n1, n7, n67, n78 |  |
| CA_n1-n7-n75-n78 | n1, n7, n75, n78 | No for CA_n5-n78 |
| CA_n1-n7-n78-n105 | n1, n7, n78, n105 |  |
| CA_n1-n8-n40-n78 | n1, n8, n40, n78 | No for CA_n8-n78 |
| CA_n1-n8-n78-n79 | n1, n8, n78, n79 | No for CA_n8-n78, CA_n8-n79 |
| CA_n1-n18-n28-n41 | n1, n18, n28, n41 |  |
| CA_n1-n18-n28-n77 | n1, n18, n28, n77 | No for CA_n18-n77, CA_n28-n77 |
| CA_n1-n18-n41-n77 | n1, n18, n41, n77 | No for CA_n18-n77 |
| CA_n1-n28-n38-n78 | n1, n28, n38, n78 | No for CA_n28-n78 |
| CA_n1-n28-n40-n77 | n1, n28, n40, n77 | No for CA_n28-n77 |
| CA_n1-n28-n40-n78 | n1, n28, n40, n78 | No for CA_n28-n78 |
| CA_n1-n28-n41-n77 | n1, n28, n41, n77 | No for CA_n28-n77 |
| CA_n1-n28-n41-n79 | n1, n28, n41, n79 | No for CA_n28-n79 |
| CA_n1-n28-n75-n78 | n1, n28, n75, n78 | No for CA_n28-n78 |
| CA_n1-n28-n77-n79 | n1, n28, n77, n79 | No for CA_n28-n77, CA_n28-n79 |
| CA_n1-n28-n78-n79 | n1, n28, n78, n79 | No for CA_n28-n78, CA_n28-n79 |
| CA_n1-n41-n77-n79 | n1, n41, n77, n79 |  |
| CA_n2-n5-n30-n66 | n2, n5, n30, n66 |  |
| CA_n2-n5-n30-n77 | n2, n5, n30, n77 | No for CA_n5-n77 |
| CA_n2-n5-n48-n66 | n2, n5, n48, n66 |  |
| CA_n2-n5-n48-n77 | n2, n5, n48, n77 | No for CA_n5-n77 |
| CA_n2-n5-n66-n77 | n2, n5, n66, n77 | No for CA_n5-n77 |
| CA_n2-n12-n30-n66 | n2, n12, n30, n66 |  |
| CA_n2-n12-n30-n77 | n2, n12, n30, n77 | No for CA_n12-n77 |
| CA_n2-n12-n66-n77 | n2, n12, n66, n77 | No for CA_n12-n77 |
| CA_n2-n14-n30-n66 | n2, n14, n30, n66 |  |
| CA_n2-n14-n30-n77 | n2, n14, n30, n77 | No for CA_n14-n77 |
| CA_n2-n14-n66-n77 | n2, n14, n66, n77 | No for CA_n14-n77 |
| CA_n2-n29-n30-n66 | n2, n29, n30, n66 |  |
| CA_n2-n29-n30-n77 | n2, n29, n30, n77 |  |
| CA_n2-n29-n66-n77 | n2, n29, n66, n77 |  |
| CA_n2-n30-n66-n77 | n2, n30, n66, n77 |  |
| CA_n2-n41-n66-n71 | n2, n41, n66, n71 |  |
| CA_n2-n48-n66-n77 | n2, n48, n66, n77 |  |
| CA_n2-n66-n71-n77 | n2, n66, n71, n77 | No for CA_n71-n77 |
| CA_n2-n66-n71-n78 | n2, n66, n71, n78 | No for CA_n71-n78 |
| CA_n3-n5-n7-n78 | n3, n5, n7, n78 | No for CA_n3-n78, CA_n5-n78 |
| CA_n3-n5-n28-n78 | n3, n5, n28, n78 | No for CA_n3-n78, CA_n5-n78, CA_n28-n78 |
| CA_n3-n5-n28-n79 | n3, n5, n28, n79 | No for CA_n3-n79, CA_n5-n79, CA_n28-n79 |
| CA_n3-n7-n8-n781 | n3, n7, n8, n78 | No for CA_n3-n78, CA_n8-n78 |
| CA_n3-n7-n20-n67 | n3, n7, n20, n67 |  |
| CA_n3-n7-n20-n78 | n3, n7, n20, n78 | No for CA_n3-n78, CA_n20-n78 |
| CA_n3-n7-n26-n78 | n3, n7, n26, n78 | No for CA_n3-n78, CA_n26-n78 |
| CA_n3-n7-n28-n38 | n3, n7, n28, n38 |  |
| CA_n3-n7-n28-n78 | n3, n7, n28, n78 | No for CA_n3-n28, CA_n28-n78 |
| CA_n3-n7-n40-n105 | n3, n7, n40, n105 |  |
| CA_n3-n7-n67-n78 | n3, n7, n67, n78 | No for CA_n3-n78 |
| CA_n3-n7-n75-n78 | n3, n7, n75, n78 | No for CA_n3-n78 |
| CA_n3-n7-n78-n105 | n3, n7, n78, n105 | No for CA_n3-n78 |
| CA_n3-n8-n41-n79 | n3, n8, n41, n79 | No for CA_n3-n79 |
| CA_n3-n18-n28-n41 | n3, n18, n28, n41 |  |
| CA_n3-n18-n28-n77 | n3, n18, n28, n77 | No for CA_n3-n77, CA_n18-n77, CA_n28-n77 |
| CA_n3-n20-n67-n78 | n3, n20, n67, n78 | No for CA_n3-n78 |
| CA_n3-n28-n40-n77 | n3, n28, n40, n77 | No for CA_n3-n77, CA_n28-n77 |
| CA_n3-n18-n41-n77 | n3, n18, n41, n77 | No for CA_n3-n77, CA_n18-n77 |
| CA_n3-n28-n41-n77 | n3, n28, n41, n77 | No for CA_n3-n77, CA_n28-n77 |
| CA_n3-n28-n41-n79 | n3, n28, n41, n79 | No for CA_n3-n79, CA_n28-n79 |
| CA_n3-n28-n77-n79 | n3, n28, n77, n79 | No for CA_n3-n77, CA_n28-n77, CA_n28-n77, CA_n28-n79 |
| CA_n3-n28-n41-n78 | n3, n28, n41, n78 | No for CA_n3-n78, CA_n28-n78 |
| CA_n3-n41-n77-n79 | n3, n41, n77, n79 | No for CA_n3-n77, CA_n3-n79 |
| CA_n5-n7-n40-n78 | n5, n7, n40, n78 | No for CA_n5-n789 |
| CA_n5-n7-n40-n105 | n5, n7, n40, n105 |  |
| CA_n5-n7-n66-n77 | n5, n7, n66, n77 | No for CA_n5-n77 |
| CA_n5-n7-n78-n105 | n5, n7, n78, n105 |  |
| CA_n5-n25-n29-n66 | n5, n25, n29, n66 |  |
| CA_n5-n25-n66-n77 | n5, n25, n66, n77 | No for CA_n5-n77 |
| CA_n5-n25-n66-n78 | n5, n25, n66, n78 | No for CA_n5-n78 |
| CA_n5-n28-n78-n79 | n5, n28, n78, n79 | No for CA_n5-n78, CA_n5-n79, CA_n28-78, CA_n28-79 |
| CA_n5-n30-n66-n77 | n5, n30, n66, n77 | No for CA_n5-n77 |
| CA_n5-n40-n78-n105 | n5, n40, n78, n105 | No for CA_n14-n78 |
| CA_n5-n48-n66-n77 | n5, n48, n66, n77 | No for CA_n5-n77 |
| CA_n7-n8-n40-n78 | n7, n8, n40, n78 | No for CA_n8-n78 |
| CA_n7-n12-n25-n66 | n7, n12, n25, n66 |  |
| CA_n7-n20-n67-n78 | n7, n20, n67, n78 | No for CA_n20-n78 |
| CA_n7-n25-n66-n71 | n7, n25, n66, n71 |  |
| CA_n7-n25-n66-n77 | n7, n25, n66, n77 |  |
| CA_n7-n25-n66-n78 | n7, n25, n66, n78 |  |
| CA_n7-n40-n78-n105 | n7, n40, n78, n105 |  |
| CA_n7-n66-n71-n77 | n7, n66, n71, n77 | No for CA_n71-n77 |
| CA_n8-n20-n28-n75 | n8, n20, n28, n75 |  |
| CA_n8-n39-n41-n79 | n8, n39, n41, n79 | No for CA_n8-n79 |
| CA_n12-n30-n66-n77 | n12, n30, n66, n77 | No for CA_n12-n77 |
| CA_n13-n25-n66-n77 | n13, n25, n66, n77 | No for CA_n13-n77 |
| CA_n14-n30-n66-n77 | n14, n30, n66, n77 | No for CA_n14-n77 |
| CA_n18-n28-n41-n77 | n18, n28, n41, n77 | No for CA_n18-n77, CA_n28-79 |
| CA_n25-n38-n66-n78 | n25, n38, n66, n78 |  |
| CA_n25-n41-n66-n71 | n25, n41, n66, n71 |  |
| CA_n25-n41-n66-n77 | n25, n41, n66, n77 |  |
| CA_n25-n41-n66-n78 | n25, n41, n66, n78 |  |
| CA_n25-n41-n66-n85 | n25, n41, n66, n85 |  |
| CA_n25-n41-n71-n77 | n25, n41, n71, n77 |  |
| CA_n25-n41-n71-n78 | n25, n41, n71, n78 |  |
| CA_n25-n41-n71-n85 | n25, n41, n71, n85 |  |
| CA_n25-n41-n77-n85 | n25, n41, n77, n85 |  |
| CA_n25-n66-n71-n77 | n25, n66, n71, n77 |  |
| CA_n25-n66-n71-n78 | n25, n66, n71, n78 |  |
| CA_n25-n66-n71-n85 | n25, n66, n71, n85 |  |
| CA_n25-n66-n77-n85 | n25, n66, n77, n85 |  |
| CA_n28-n41-n77-n79 | n28, n41, n77, n79 | No for CA_n28-n77, CA_n28-79 |
| CA_n29-n30-n66-n77 | n29, n30, n66, n77 |  |
| CA_n29-n66-n70-n71 | n29, n66, n70, n71 |  |
| CA_n41-n66-n70-n78 | n41, n66, n70, n78 |  |
| CA_n41-n66-n71-n77 | n41, n66, n71, n77 | No for CA_n71-n77 |
| CA_n41-n66-n71-n78 | n41, n66, n71, n78 | No for CA_n71-n77, CA_n71-78 |
| CA_n41-n66-n71-n85 | n41, n66, n71, n85 |  |
| CA_n41-n66-n77-n85 | n41, n66, n77, n85 |  |
| CA_n48-n66-n70-n71 | n48, n66, n70, n71 |  |
| CA_n48-n66-n70-n77 | n48, n66, n70, n77 |  |
| CA_n48-n66-n71-n77 | n48, n66, n71, n77 | No for CA_n71-n77 |
| CA_n48-n70-n71-n77 | n48, n70, n71, n77 | No for CA_n71-n77 |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability. |  |  |

Table 3-1: Proposed DL applicability for UL Tx switching involving 4 bands

# 4	References

RP-24xxxx, “ New WID: Rel-19 downlink interruption for NR and EN-DC band combinations at dynamic Tx Switching in Uplink”, by China Telecom, 3GPP TSG-RAN Meeting #104,  Shanghai, China, June 17-20, 2024

R4-2004895, “WF on downlink interruption for UL Tx switching”, by China Telecom, 3GPP TSG-RAN Meeting #112, Maastricht, Netherlands, 19th – 23rd August 2024