3GPP RAN TSG-RAN4 Meeting # 112-Bis		R4-2415325

Hefei, China, October 14 – 18, 2024

Agenda item:	6.1.1.3.4

Source:	Apple

Title:	Views On 6Rx REFSENS evaluation and SRS IL imbalance solution

WI/SI:	NR_ENDC_RF_Ph4-Core

Release:	Rel-19

Document for:	Discussion

# 1	Introduction and Scope

In the previous RAN4 meeting (Meeting #112, Maastricht, Netherlands, August 19-23, 2024), discussions were held on the requirements for 6Rx for handheld and FWA, which are parts of the new  Release 19 WID on RF enhancements for NR FR1/FR2 and EN-DC (NR_ENDC_RF_Ph4). The WF captures the decisions made on several aspects debated during the meeting [1]. Below, we highlighted two of the remaining open issues taken from the meeting WF:

- Verification of 6Rx REFSENS requirements

- Whether a solution is needed to address the SRS IL imbalance issue

# Topic 1: REFSENS (delta RIB,6R) ## Sub-topic 1-1: General considerations for specifying ΔRIB,6R value Issue 1-1-3: Verification of 6Rx receiver requirements Way Forward: Companies encouraged to provide their views on the ZTE proposal in Proposal 5 in R4-2411883 and any further positions on verification of 6Rx receiver requirements at future meetings given that ΔRIB,6R values are now defined. Topic 4: SRS IL Imbalance Issue ## Sub-topic 4-1: General considerations for SRS IL imbalance issue Issue 4-1-1: Whether to solve SRS IL imbalance issue in Rel-19 Way Forward: RAN4 to further discuss the following options. - Option 1: RAN4 should not continue the discussion on how to solve the SRS IL imbalance issue. - Option 2: Continue to pursue a solution to the SRS IL imbalance issue dependent on the outcome of Issue 4-1-2 in the approved WF in R4-2410751.

In this discussion paper, we will provide our views on the two issues mentioned above.

# Discussion

  - SRS Insertion Loss Imbalance Solution

The SRS insertion loss imbalance issue has been highly debated in RAN4 and RAN1 meetings since the introduction of the 8Rx WID in Rel-18. Based on simulation results from several companies, it was recognized that, if this issue is not addressed, there could be a potential performance degradation in user throughput [3-8]. In RAN4#106 meeting (Athens, Greece, February 27 – March 3, 2023), an LS [2] from RAN4 was sent to RAN1 describing the SRS IL imbalance issue for 8Rx and its potential impact on inaccurate channel estimation at the gNB receiver, which could lead to an incorrect DL CSI estimation. In response to the RAN4 LS, a lot of companies provided simulation results to highlight the potential impacts of IL imbalance on the degradation of overall system performance. Some of those simulation results taken from topic summary [3] are shown in the figures below. It can be observed from the figures that SRS IL imbalance, without any reporting or compensation, can result in a non-negligible performance loss for both SU-MIMO and MU-MIMO.

Observation #1: During 8Rx discussions, based on simulation results from several companies, it was recognized that, if the SRS IL imbalance issue is not addressed, there could be a potential performance degradation in user throughput.

Figure 2.1-1:	Mean and cell-edge MU-MIMO user throughput with/without SRS IL imbalance [3]

Figure 2.1-2:	Mean and cell-edge MU-MIMO user throughput with/without SRS IL imbalance [3]

Although the majority of the companies saw the need for a solution to address the SRS IL imbalance issue, there was no agreement on a candidate solution during the 8Rx discussions. One of the objectives of the Rel-19 6Rx WID was then to continue the discussions on this issue and specify, if possible, a candidate solution to alleviate its negative impacts on the UE system performance. Over the past RAN4 meetings, several discussions have been held on the issue with no clear solution identified yet. Most of the companies have proposed so far solutions centered around UE power pre-compensation and IL imbalance reporting. Backgrounds

For UE power pre-compensation, RAN4 identified in Meeting # 110-Bis three cases that determine the effectiveness of applying compensation [9]:

- Case 1(non near max Tx power): In this scenario, the UE is not power-limited and should be able to compensate for the IL loss imbalance across the SRS ports if the PA headroom is higher than the maximum IL imbalance.

- Case 2 (near max Tx power): For this case, the UE is close to be power-limited so performing a reliable compensation for this case could be a challenge because there could be not enough PA headroom remaining. However, this would depend on UE implementation.

- Case 3 (max Tx power): The UE is power-limited in these conditions and would probably not be do power compensation.  the UE is close to be power-limited so performing a reliable compensation for this case could be a challenge because there could be not enough PA headroom remaining.

SRS IL imbalance reporting has also been considered as a solution. In this case, the UE reports to the NW the values of IL imbalance across the SRS ports because the UE cannot always perform self-compensation.

In the previous meeting (Meeting #112, Maastricht, Netherlands, August 19-23, 2024), we proposed a hybrid solution based on both UE self-compensation and reporting where the UE always compensates when it is capable to do so in a non-power limited situations and does some reporting to the network when the power at an SRS port is above certain threshold [10].

Based on the multiple contributions and the extensive discussions made so far on the SRS IL imbalance solution we can make the following proposal on whether to continue to pursue a solution to address the issue.

Proposal #1: On whether to continue to pursue a solution to address the SRS IL imbalance issue, we prefer Option 2 because of the potential UE performance benefits when the issue is addressed as it’s been shown so far in several company’s contributions. However, even if a solution is ultimately found, we recommend that this feature remains optional and should depend on UE implementation, given its potential implementation challenges.

  - Verification of 6Rx receiver requirements

In the previous meeting, a discussion paper [11] proposed how, for 6Rx-capable UEs, the REFSENS and other RX performance requirements could be evaluated. Table 2.2-1 provides a summary of how the different RX performance metrics could be tested according to the proposal.

|  | Single carrier REFSENS requirement | Other single carrier Rx requirements |
| --- | --- | --- |
| UE is equipped with 2Rx | 2Rx | 2Rx |
| UE is equipped with 4Rx | 2Rx, 4Rx | 4Rx, skip 2Rx |
| UE is equipped with 6Rx | 2Rx, 4Rx, 6Rx | 6Rx, skip both 2Rx and 4Rx |

Table 2.2-1: Test proposals for 6Rx-capable UEs [11]

It seems that the evaluation approach shown above is an extension of the approach already used for 4Rx testing wherein today we have the following note for conformance testing of all Rx tests except REFSENS (Taken from the TS38.521 UE performance testing requirements):

NOTE 2:   In a band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (Table 7.3.2.5-2).

In addition, the REFSENS evaluation shown above is similar to what is being done now for 4Rx according to Note 5.

NOTE 5:  In a band where UE supports 4Rx, the test needs to be repeated with only 2Rx antennas connected and the other antennas terminated.

We see no issue adopting the same evaluation methodology currently being used for 4Rx to test 6Rx. Therefore, we can make the following proposal:

Proposal #2: For 6Rx evaluation, we support the proposal highlighted in [R4-2411883] as it is an extension what is currently in the UE performance evaluation for 4Rx.

# 3	Conclusions

Observation #1: During 8Rx discussions, based on simulation results from several companies, it was recognized that, if the SRS IL imbalance issue is not addressed, there could be a potential performance degradation in user throughput.

Proposal #1: On whether to continue to pursue a solution to address the SRS IL imbalance issue, we prefer Option 2 because of the potential UE performance benefits when the issue is addressed as it’s been shown so far in several company’s contributions. However, even if a solution is ultimately found, we recommend that this feature remains optional and should depend on UE implementation, given its potential implementation challenges.

Proposal #2: For 6Rx evaluation, we support the proposal highlighted in [R4-2411883] as it is an extension what is currently in the UE performance evaluation for 4Rx.

# 4	References

R4-2414285, “WF on requirements for 6Rx”, by AT&T, 3GPP TSG-RAN WG4 Meeting #112, Maastricht, Netherlands, August 19 – August 23, 2024

R4-2303519, “LS on the UE SRS IL imbalance issue”, 3GPP TSG-RAN WG4 Meeting #106, Athens, Greece, February 27 – March 3, 2023

R1-2306200, “FL summary on UE SRS IL imbalance issue”, 3GPP TSG RAN WG1 #112bis-e, Incheon, Korea, May 22nd – May 26th, 2023

R1-2304643, “Discussion on the UE SRS IL imbalance issue”, by Huawei, HiSilicon, 3GPP TSG-RAN WG1 Meeting #113,  Incheon, Korea, 22 – 26 May 2023

R1-2304433, “On SRS Imbalance”, by InterDigital, Inc., 3GPP TSG-RAN WG1 Meeting #113,  Incheon, Korea, 22 – 26 May 2023

R1-2305646, “Evaluation of SRS antenna switching with insertion loss”, by MediaTek, 3GPP TSG-RAN WG1 Meeting #113,  Incheon, Korea, 22 – 26 May 2023

R4-2300696, “8RX UE RF requirements”, by Qualcomm Inc., 3GPP TSG-RAN WG4#106 Athens, Greece, February 27th ‒ March 3rd, 2023

R1-2305838, “Discussion the UE SRS IL imbalance issue”, by vivo, 3GPP TSG-RAN WG1 Meeting #113,  Incheon, Korea, 22 – 26 May 2023

R4-2406585, “WF on requirements for 6Rx”, by AT&T, 3GPP TSG-RAN WG4 Meeting #110-bis,  Changsha, China, 15th Apr 2024 - 19th Apr 2024

R4-2411151, “On SRS IL imbalance issue for 6Rx”, by Apple, 3GPP RAN TSG-RAN4 Meeting # 112, Maastricht, NL, August 19 – 23, 2024

R4-2411883, “Further discussion on 6Rx Reference sensitivity requirements”, by ZTE Corporation, Sanechips,  3GPP TSG-RAN WG4 Meeting #110-bis,  Changsha, China, 15th Apr 2024 - 19th Apr 2024