3GPP RAN WG4 Meeting #112bis																R4-2415327

Hefei, China, 14th – 18th October 2024

Agenda item:	6.1.1.2.3

Source:	Apple

Title:	On Rel-19 power domain enhancement for FR2

WI/SI:	NR_ENDC_RF_Ph4-Core

Release:	Rel-19

Document for:	Discussion

# 1	Introduction

Power domain enhancement is a re-occurring topic and enters the 4th phase when starting the Rel-19 work. The objectives are covered in the WID [1]. This contribution focuses on the high-level aspects of MPR reduction. The WID lists those goals as:

# 2  Discussion

##### 2.1 Applicable MPR for FR2 UL CA and single CC with DL intra-band CA

#### Issue 3-2-1: Applicable MPR for FR2 single carrier UL with DL intra band CA - Proposals - Proposal 1: For the case of DL intra band CA with single UL CC (activated or configured), the reference MPR (originally being Table 6.2A.2.4-1) is changed as the MPR requirement for single carrier. (Huawei) #### Issue 3-2-2: Applicable MPR for FR2 UL CA with DL intra band CA - WF - In general, MPR based on UL BWchannel_CA applies instead that based on cumulative aggregated channel BW (CABW) - If only 1 UL CC is activated, the MPR requirements of single carrier could be reused - No need to specify new column for <400MHz CABW in the MPR table. FFS specific spec change to accommodate 200MHz BW granularity - FFS whether to take the minimum values of MPR between the intra-band CA case and the single carrier for FR2-2

It has been proposed that in case of DL intra-band CA with single UL CC the reference MPR is modified. Similar proposal is made for the case where UL CA is configured but transmission only occurs inside a single CC.

Currently, a UE is allowed to apply the UL CA table 6.2A.2.4-1 for intra-band MPR. This table is designed to provide enough back-off for the cumulative aggregated channel bandwidth (CABW). Per definition this is the frequency range from the lowest edge of the lowest CC to the upper edge of the highest CC of all UL and DL configured CCs. The UL CCs can be anywhere inside this frequency range. Most probably the UL CCs are not centred at the LO and the CA MPR guarantees that a UE can apply enough back-off to comply to emission limits if the CCs are off-centre.

PC1 and PC3 feature an exception for CABW sizes up to 400MHz where the RB allocations are contiguous inside a single CC and modulation is not higher than QPSK. The exception improves MPR for CABW and BWchannel_CA for up to 400MHz.

Enhancements could be made by targeting MPR for CABW larger than 400MHz with BWchannel_CA up to 400MHz. For CABW above 400MHz a UE might need to implement two separate PLLs. One for downlink and one for uplink operation. Aside from the additional area consumption this would also lead to larger power consumption as a second PLL needs to be operated. Another option would be to switch the LO location to the centre of downlink and uplink when a DL or UL slot occurs. However, this solution would result into high power consumption as fast PLL settling times are typically paid with considerable current consumption. When modifying the requirements from UL CA MPR to single CC MPR then RAN4 might need to discuss the implications on architecture. It might be the case that this kind of MPR improvement is not suitable for PC3 due to its limitations in form factor and battery. Targeting only specific power classes such as PC1 might be the best option to introduce meaningful enhancements.

Observation: Enhancements might be introduced by improving MPR for CABW larger than 400MHz with BWchannel_CA up to 400MHz. For CABW above 400MHz a UE might need to implement two separate PLLs. One for downlink and one for uplink operation. Aside from the additional area consumption this would also lead to larger power consumption as a second PLL needs to be operated. Another option would be to switch the LO location to the centre of downlink and uplink when a DL or UL slot occurs. However, this solution would result into high power consumption as fast PLL settling times are typically paid with considerable current consumption.

Proposal 1: By modifying the requirements from UL CA MPR to single CC MPR it is proposed that RAN4 discusses the implications on architecture. It might be the case that this kind of MPR improvement is not suitable for PC3 due to its limitations in form factor and battery. Targeting only specific power classes such as PC1 might be the best option to introduce meaningful enhancements.

##### 2.2 On UE capability signalling

The targeted improvements for FR2 require changes to the legacy architecture assumptions. The modifications should be part of a new UE capability to efficiently separate those from legacy behaviour. As some of the improvements might not be feasible for all UE power classes and form factors the support of the new capabilities shall be optional and per band combination.

Proposal 2: The modifications should be part of a new UE capability to efficiently separate those from legacy behaviour. As some of the improvements might not be feasible for all UE power classes and form factors the support of the new capabilities shall be optional and per band combination.

# 4	Conclusions

This contribution provides certain considerations to the discussion on the MPR reduction which his part of the power domain enhancement WID. The following observations are made:

Observation: Enhancements might be introduced by improving MPR for CABW larger than 400MHz with BWchannel_CA up to 400MHz. For CABW above 400MHz a UE might need to implement two separate PLLs. One for downlink and one for uplink operation. Aside from the additional area consumption this would also lead to larger power consumption as a second PLL needs to be operated. Another option would be to switch the LO location to the centre of downlink and uplink when a DL or UL slot occurs. However, this solution would result into high power consumption as fast PLL settling times are typically paid with considerable current consumption.

Proposal 1: By modifying the requirements from UL CA MPR to single CC MPR it is proposed that RAN4 discusses the implications on architecture. It might be the case that this kind of MPR improvement is not suitable for PC3 due to its limitations in form factor and battery. Targeting only specific power classes such as PC1 might be the best option to introduce meaningful enhancements.

Proposal 2: The modifications should be part of a new UE capability to efficiently separate those from legacy behaviour. As some of the improvements might not be feasible for all UE power classes and form factors the support of the new capabilities shall be optional and per band combination.

# 4	References

- RP-240828, “UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, Huawei, 3GPP TSG RAN Meeting #103

- R4-2410749, “WF on power domain enhancements”, Huawei, HiSilicon, 3GPP TSG-RAN WG4 Meeting #111