3GPP RAN WG4 Meeting #112bis		R4-2415331

Hefei, China, October 14th – 18th, 2024

Agenda item:	6.1.1.1.2

Source:	Apple

Title:	On PSD imbalance for PC1.5 intra-band UL CA MPR evaluations

WI/SI:	NR_ENDC_RF_Ph4-Core

Release:	Rel-19

Document for:	Approval

# 1	Introduction

The Rel-19 RAN4 non-spectrum WID/SID package has been approved in RAN #103 meeting. In consideration that the uplink coverage is crucial for 5G and 5G-adavanced roll-out and the continued strong interests from operators in higher UE maximum output power such as PC1.5 for their band combinations, one of the key objectives under the new Rel-19 “UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4” work item [1] is to specify the generic requirements of HPUE for NR uplink CA in FR1 and EN-DC with NR FR1 bands where the features include,

    - PC1.5 for intra-band contiguous UL CA with and without UL MIMO

    - PC1.5 Intra-band non-contiguous UL CA without UL MIMO

    - PC1.5 2-band inter-band UL CA with 2Tx or 3Tx for handheld UE and FWA

    - PC2 and PC1.5 2-band inter-band EN-DC with 2Tx or 3Tx for handheld UE and FWA UE

    - Increasing UE transmission power limit up to the sum of maximum output power per band for NR inter-band UL CA and EN-DC HPUE

In last RAN4 meeting, companies had provided their further views on the respective features above where the additional agreements were captured in the approved “WF on HPUE for CA in TN” [2]. For PC1.5 intra-band UL CA, RAN4 has agreed to evaluate MPR only based on TxD architecture for contiguous UL CA, while for non-contiguous UL CA, only dualPA architecture would be considered. Additionally, the MPR evaluations may account for PSD imbalance with up to 6 dB between the two carriers for both contiguous and non-contiguous configurations and whether to define requirements based on equal PSD or with PSD imbalance is subject to further studies.

In this contribution, we share our views on PSD imbalance for PC1.5 intra-band UL CA MPR evaluations and propose that for contiguous UL CA, MPR requirements are only evaluated and specified based on equal PSD between the two carriers.

# Discussion

## 2.1	PSD imbalance for PC1.5 intra-band contiguous UL CA

For PC1.5 intra-band contiguous UL CA based on TxD architecture as shown in Figure 2.1-1, each individual analog Tx signal path is shared by the two aggregated carriers which are expected to experience the same analog gain. Provided the PSD is set equal in digital domain between the two carriers, there should be no unintentional PSD imbalance caused by any transceiver design impairment. Since cell collocation has always been assumed for intra-band contiguous CA, there would be no compelling reason to intentionally set unequal PSD between the two UL carriers as they would observe different SNR when reaching the gNB. In our view, the consideration of PSD imbalance up to 6dB could be a misconception inherited from the DL PSD imbalance assumption for intra-band contiguous DL CA which is meant to accommodate the unintentional RF gain mismatch or different channel properties between the two cells despite they are physically collocated. On the other hand, PSD imbalance has never been considered for PC3 and PC2 intra-band contiguous UL CA for MPR evaluations and requirements. Therefore, we do not see the need to consider PSD imbalance for PC1.5 intra-band contiguous UL CA as well.

Figure 2.1-1 UE architecture to support PC1.5 intra-band contiguous UL CA

Observation 1: For PC1.5 intra-band contiguous UL CA based on TxD architecture, provided the PSD is set equal in digital domain between the two carriers, there should be no unintentional PSD imbalance caused by any transceiver design impairment.

Observation 2: Cell collocation has always been assumed for intra-band contiguous CA.

Observation 3: For intra-band contiguous UL CA, there would be no compelling reason to intentionally set unequal PSD between the two UL carriers as they would observe different SNR when reaching the gNB.

Observation 4: The consideration of PSD imbalance up to 6dB could be a misconception inherited from the DL PSD imbalance assumption for intra-band contiguous DL CA which is meant to accommodate the unintentional RF gain mismatch or different channel properties between the two cells.

Observation 5: PSD imbalance has never been considered for PC3 and PC2 intra-band contiguous UL CA for MPR evaluations and requirements.

Based on the above observations, we propose that for PC1.5 intra-band contiguous UL CA, MPR requirements are only evaluated and specified based on equal PSD between the two carriers.

Proposal 1: For PC1.5 intra-band contiguous UL CA, MPR requirements are only evaluated and specified based on equal PSD between the two carriers.

## 2.2	PSD imbalance for PC1.5 intra-band non-contiguous UL CA

For PC1.5 intra-band non-contiguous UL CA based on dualPA architecture as shown in Figure 2.2-1, each individual analog signal path only transports one UL carrier. Unlike TxD architecture where either one of the UL carriers may reach maximum output power at 29 dBm, the dualPA architecture only permits maximum output power of 26 dBm for each UL carrier. To reach the total output power of 29 dBm as advertised by PC1.5, each carrier would have to be transmitted at 26 dBm simultaneously. In this case, PSD imbalance between the two carriers would be expected when there is unequal transmission bandwidth between them. Under the extreme configuration where one carrier could be at 100 MHz with full allocation (273RB at 30kHz SCS) while the other carrier is only allocated with 1RB as shown in Figure 2.2-2, the PSD imbalance could be as high as 10*log10(273/1) dB = 24.4 dB.

26 dBm

26 dBm

Figure 2.2-1 UE architecture to support PC1.5 intra-band non-contiguous UL CA

Figure 2.2-2 Maximum PSD imbalance under extreme UL configuration for PC1.5 NC CA

Observation 6: For PC1.5 intra-band non-contiguous UL CA based on dualPA architecture, each UL carrier can only be transmitted with up to 26dBm output power.

Observation 7: For PC1.5 intra-band non-contiguous UL CA based on dualPA architecture, to reach the maximum total output power at 29 dBm, the PSD imbalance could be up to 24.4 dB between the two carriers.

Under the dualPA architecture, to fully utilize the maximum total output power capability at 29 dBm, PSD imbalance would be desired when there is unequal transmission bandwidth between the two carriers even the cell deployment is collocated. The narrower carrier may leverage the higher PSD with better SNR at gNB to exploit higher order modulation for better UL throughput, provided the emission requirements can be fulfilled.

Observation 8: For PC1.5 intra-band non-contiguous UL CA based on dualPA architecture, to fully utilize the maximum total output power capability at 29 dBm, PSD imbalance would be desired when there is unequal transmission bandwidth between the two carriers even the cell deployment is collocated.

On the other hand, MPR requirement is also expected to be referenced to PC1.5 maximum output power at 29 dBm, therefore, PSD imbalance would be unavoidable for MPR evaluations, and the imbalance could be up to 24.4 dB instead of only up to 6 dB. Compared to PC3 and PC2 MPR evaluations where equal PSD has been assumed and the power back-off is always evenly applied between the two carriers, the PC1.5 MPR evaluations with unequal PSD could be more complicated as the power back-off may not be evenly applied between the two carriers, or the UL performance could be compromised. Using the above extreme PC1.5 UL configuration as an example, the IMD3 products on either side of the carriers would also manifest unequal PSD due to that the same IMD3 power (assuming same OIP3 for the two Tx paths) is distributed over the unequal bandwidth of the IMD3 products which is a result of frequency convolution as conceptually illustrated in Figure 2.2-3.

Figure 2.2-3 Conceptual illustration of IMD3 products from unequal PSD carriers

Observation 9: For PC1.5 intra-band non-contiguous UL CA based on dualPA architecture, MPR requirement is expected to be referenced to PC1.5 maximum output power at 29 dBm, therefore, PSD imbalance would be unavoidable for MPR evaluations.

Observation 10: PC1.5 MPR evaluations with unequal PSD could be more complicated as the power back-off may not be evenly applied between the two carriers, or the UL performance could be compromised.

As the IMD3 close to the narrower carrier is failing the emission limit with higher PSD but not the IMD3 close to the wider carrier with lower PSD, intuition tells that MPR would be more efficient when applying more power back-off on the narrower carrier than the wider carrier. However, MPR evaluations based on uneven power back-off could be rather complicated and time consuming.

Observation 11: MPR evaluations based on uneven power back-off could be rather complicated and time consuming.

On the other hand, for PC3 and PC2 intra-band non-contiguous UL CA, there is already relatively high MPR, as summarized in Table 2.2-1, to meet the general spurious emission requirement at -30dBm/MHz which would have substantial impact on UL coverage.

| PC3 |  | PC2 |  |
| --- | --- | --- | --- |
| B (MHz) | MPR (dB) | B (MHz) | MPR (dB) |
| 0 ≤ B < 1.08 | 15.0 | 0 ≤ B < 1.44 | 15.5 |
| 1.08 ≤ B < 2.16 | 14.5 | 1.44 ≤ B < 2.88 | 15.0 |
| 2.16 ≤ B < 3.24 | 13.5 | 2.88 ≤ B < 5.76 | 14.0 |
| 3.24 ≤ B < 5.04 | 12.5 | 5.76 ≤ B < 10.8 | 12.0 |
| 5.04≤ B < 10.08 | 11.5 | 10.8 ≤ B < 23.04 | 10.5 |
| 10.08 ≤ B < 16.38 | 10.5 | 23.04 ≤ B | 9.0 |
| 16.38 ≤ B < 21.78 | 10.0 |  |  |
| 21.78 ≤ B | 9.0 |  |  |
| NOTE: B is the effective aggregated bandwidth in MHz |  |  |  |

Table 2.2-1 Intra-band NC UL CA MPR requirements with dualPA for -30dBm/MHz spurious emission

PC1.5 intra-band non-contiguous UL CA likely would be subject to further higher MPR as compared to PC2 and PC3 since both half-rated PAs are operating at maximum power where linearity is expected to be worse than PC2 and PC3. As a result, supporting PC1.5 could be a diminishing return as there may not be any performance gain over PC2 or even PC3 and the onerous efforts to specify the requirements may not be well justified. Alternatively, if UL Tx switching can be supported for intra-band non-contiguous UL CA as proposed in [3], the use case limitation and MPR/A-MPR performance concern for simultaneous UL transmission can both be avoided, and yet without compromising the UL throughput performance as each switched UL carrier can leverage the 2 Tx paths to support UL MIMO up to PC1.5.

Observation 12: Supporting PC1.5 intra-band non-contiguous UL CA could be a diminishing return as there may not be any performance gain over PC2 or even PC3 and the onerous efforts to specify the requirements may not be well justified.

Proposal 2: Introduce UL Tx switching for intra-band non-contiguous UL CA with UL MIMO as an enhanced feature to PC1.5 intra-band non-contiguous UL CA without UL MIMO.

# 3	Conclusion

In this contribution, we share our views on PSD imbalance for PC1.5 intra-band UL CA MPR evaluations and propose that for contiguous UL CA, MPR requirements are only evaluated and specified based on equal PSD between the two carriers.

Observation 1: For PC1.5 intra-band contiguous UL CA based on TxD architecture, provided the PSD is set equal in digital domain between the two carriers, there should be no unintentional PSD imbalance caused by any transceiver design impairment.

Observation 2: Cell collocation has always been assumed for intra-band contiguous CA.

Observation 3: For intra-band contiguous UL CA, there would be no compelling reason to intentionally set unequal PSD between the two UL carriers as they would observe different SNR when reaching the gNB.

Observation 4: The consideration of PSD imbalance up to 6dB could be a misconception inherited from the DL PSD imbalance assumption for intra-band contiguous DL CA which is meant to accommodate the unintentional RF gain mismatch or different channel properties between the two cells.

Observation 5: PSD imbalance has never been considered for PC3 and PC2 intra-band contiguous UL CA for MPR evaluations and requirements.

Proposal 1: For PC1.5 intra-band contiguous UL CA, MPR requirements are only evaluated and specified based on equal PSD between the two carriers.

Observation 6: For PC1.5 intra-band non-contiguous UL CA based on dualPA architecture, each UL carrier can only be transmitted with up to 26dBm output power.

Observation 7: For PC1.5 intra-band non-contiguous UL CA based on dualPA architecture, to reach the maximum total output power at 29 dBm, the PSD imbalance could be up to 24.4 dB between the two carriers.

Observation 8: For PC1.5 intra-band non-contiguous UL CA based on dualPA architecture, to fully utilize the maximum total output power capability at 29 dBm, PSD imbalance would be desired when there is unequal transmission bandwidth between the two carriers even the cell deployment is collocated.

Observation 9: For PC1.5 intra-band non-contiguous UL CA based on dualPA architecture, MPR requirement is expected to be referenced to PC1.5 maximum output power at 29 dBm, therefore, PSD imbalance would be unavoidable for MPR evaluations.

Observation 10: PC1.5 MPR evaluations with unequal PSD could be more complicated as the power back-off may not be evenly applied between the two carriers, or the UL performance could be compromised.

Observation 11: MPR evaluations based on uneven power back-off could be rather complicated and time consuming.

Observation 12: Supporting PC1.5 intra-band non-contiguous UL CA could be a diminishing return as there may not be any performance gain over PC2 or even PC3 and the onerous efforts to specify the requirements may not be well justified.

Proposal 2: Introduce UL Tx switching for intra-band non-contiguous UL CA with UL MIMO as an enhanced feature to PC1.5 intra-band non-contiguous UL CA without UL MIMO.

# 4	References

RP-240828 “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, RAN4 Chair (Huawei), 3GPP TSG-RAN Meeting #103, Maastricht, Netherlands, March 18th – 21st, 2024

R4-2414277 “WF on HPUE for CA in TN”, Samsung, 3GPP TSG RAN WG4 Meeting #112, Maastricht, Netherlands, August 19th – 23rd, 2024

R4-2415338 “Motivation for UL Tx switching for FR1 intra-band non-contiguous UL CA in Rel-19”, Hefei, China, October 14th – 18th, 2024