3GPP RAN WG4 Meeting #112bis		R4-2415335

Hefei, China, October 14th – 18th, 2024

Agenda item:	7.1

Source:	Apple

Title:	Views on RAN1 LS on FR2-NTN inclusion to specifications

WI/SI:	NR_NTN_enh-Core

Release:	Rel-18

Document for:	Approval

# 1	Introduction

RAN4 received an LS from RAN1 on “FR2-NTN inclusion to specifications” [1] to seek for RAN4’s clarifications on the following two questions:

- RAN1 identified that some RAN1 specifications are still unclear regarding whether FR2-1 or FR2-2 procedure should apply to the FR2-NTN, as the current RAN1 specifications define different behaviour between FR2-1 and FR2-2.

- Furthermore, RAN1 understands that NOTE 2 in Table 5.1-1 of 38.101-5 states that FR2-NTN are regarded as FR2 bands, likely to minimize the impact when referencing RAN1 and RAN2 specifications. RAN1 would like to ask RAN4 to consider whether NOTE 2 may need to be updated.

In this contribution, we provide our views on how to address RAN1’s questions above.

# Discussion

In response to RAN1’s first question on whether FR2-1 or FR2-2 procedure should apply to the FR2-NTN, in our view, it should be FR2-1 procedure and that can be confirmed by the following two observations.

First, based on the agreed RAN1 CR on “FR2-NTN inclusion to TS 38.213” [2] as attached in the LS, we can recognize that the UE procedure for determining PDCCH assignment is differentiated by whether the operation is without or with “shared spectrum channel access”. Since FR2-NTN access is only within licensed spectrum where the operation is without “shared spectrum channel access” which is also the procedure for FR2-1, the FR2-NTN shall follow the FR2-1 procedure.

Observation 1: From the operation without or with “shared spectrum channel access” perspective, FR2-NTN shall follow the FR2-1 procedure.

Second, from the observation that the FR2-NTN frequency range only overlaps with FR2-1 and the FR2-NTN numerologies as well as channel bandwidths also align with FR2-1 specifications, it further ensures that the FR2-NTN procedure shall follow the FR2-1 procedure.

Observation 2: From the operation frequency range and the supported numerologies and channel bandwidths perspective, FR2-NTN shall follow the FR2-1 procedure.

Proposal 1: RAN4 to confirm with RAN1 that FR2-NTN shall follow the FR2-1 procedure.

In response RAN1’s second question on whether NOTE 2 in Table 5.1-1 of 38.101-5 may need to be updated, since the note is currently still in square bracket as shown in Table 2-1 (recaptured from TS 38.101-5 V18.6.0 Table 5.1-1) which means RAN4 is not yet certain on whether the note is necessitated or needs to be updated. In our view, though FR1-NTN frequency range and system parameters are well aligned with the terrestrial network FR1 specifications, the FR2-NTN frequency range however does not fully correlate with the terrestrial network FR2 nor FR2-1 range, therefore, having NOTE 2 referencing to either FR2 or FR2-1 band may cause some confusion. If FR2-NTN frequency range can be confirmed to follow the FR2-1 procedure to RAN1 and RAN2, then NOTE 2 may be removed to avoid the potential confusion with FR2 or FR2-1. Or another alternative is to update the note to as “NTN bands within this frequency range are to follow FR2-1 band procedure when referenced from other specifications.”

| Frequency range designation | Corresponding frequency range |
| --- | --- |
| FR1-NTN1 | 410 MHz – 7125 MHz |
| FR2-NTN2 | 17300 MHz – 30000 MHz |
| NOTE 1: [NTN bands within this frequency range are regarded as a FR1 band when references from other specifications.] NOTE 2: [NTN bands within this frequency range are regarded as a FR2 band when references from other specifications.] |  |

Table 2-1 Definition of NTN frequency ranges

Observation 3: FR2-NTN frequency range does not fully correlate with the terrestrial network FR2 nor FR2-1 range, having NOTE 2 referencing to either FR2 or FR2-1 band may cause some confusion.

Proposal 2: If FR2-NTN frequency range can be confirmed to follow the FR2-1 procedure to RAN1 and RAN2, RAN4 to consider the following two options on the handling of NOTE 2:

Option 1: Remove NOTE 2 to avoid the potential confusion with FR2 or FR2-1.

Option 2: Update NOTE 2 to as “NTN bands within this frequency range are to follow FR2-1 band procedure when referenced from other specifications.”

# 3	Conclusion

In this contribution, we provide our views on how to address RAN1’s questions in the LS on FR2-NTN inclusion to specifications.

Observation 1: From the operation without or with “shared spectrum channel access” perspective, FR2-NTN shall follow the FR2-1 procedure.

Observation 2: From the operation frequency range and the supported numerologies and channel bandwidths perspective, FR2-NTN shall follow the FR2-1 procedure.

Proposal 1: RAN4 to confirm with RAN1 that FR2-NTN shall follow the FR2-1 procedure.

Observation 3: FR2-NTN frequency range does not fully correlate with the terrestrial network FR2 nor FR2-1 range, having NOTE 2 referencing to either FR2 or FR2-1 band may cause some confusion.

Proposal 2: If FR2-NTN frequency range can be confirmed to follow the FR2-1 procedure to RAN1 and RAN2, RAN4 to consider the following two options on the handling of NOTE 2:

Option 1: Remove NOTE 2 to avoid the potential confusion with FR2 or FR2-1.

Option 2: Update NOTE 2 to as “NTN bands within this frequency range are to follow FR2-1 band procedure when referenced from other specifications.”

# 4	References

R1-2407406 “LS on FR2-NTN inclusion to specifications”, RAN1, 3GPP TSG-RAN WG1 Meeting #118, Maastricht, Netherlands, August 19th – 23rd, 2024

R1-2407407 “CR on FR2-NTN inclusion to TS 38.213”, Vivo et. al., 3GPP TSG-RAN WG1 Meeting #118, Maastricht, Netherlands, August 19th – 23rd, 2024