3GPP RAN WG4 Meeting #112bis		R4-2415339

Hefei, China, October 14th – 18th, 2024

Agenda item:	9

Source:	Apple

Title:	Motivation for MSD reporting enhancement in Rel-19

WI/SI:

Release:	Rel-19

Document for:	Information

# 1	Introduction

Despite the Rel-18 lower MSD capability reporting feature has been successfully introduced [1,2], it was left with considerable speculation as whether such database-intensive lower MSD signaling can be effectively and efficiently utilized by gNB to improve the band combination scheduling. On the other hand, since MSD is a substantial part of the UE RF requirements which will have different performance impacts on different carrier configurations and operating conditions, there has been increasing interest in more effective and efficient real-time MSD/SIR measurement and reporting scheme [3,4] as a superior alternative to the Rel-18 lower MSD capability signaling. In this contribution, we summarize the drawbacks of the Rel-18 lower MSD capability signaling and propose a more effective and efficient MSD/SIR measurement and reporting scheme based on real carrier configurations as part of Rel-19 RAN4 objectives for consideration.

# Discussion

## 2.1	Drawbacks of the Rel-18 lower MSD capability signaling

One of the most often criticized drawbacks of the Rel-18 lower MSD capability signaling is that the MSD reported by UE only occurs at a particular worst-case carrier configuration defined in RAN4 specifications as the minimum RF requirement, but not necessarily and likely not the MSD for the configuration scheduled by the network in real field operations which could potentially impact the network scheduling efficiency.

Observation 1: The Rel-18 lower MSD capability reported by UE only occurs at a particular worst-case carrier configuration defined in RAN4 specifications, but not necessarily and likely not the MSD for the configuration scheduled by the network in real field operations which could potentially impact the network scheduling efficiency.

Secondly, a band combination may be subject to impacts from different MSD mechanisms under different carrier configurations, such as due to UL harmonics, harmonic mixing, 2UL IMD, and cross-band interference which may not occur simultaneously under the scheduled carrier configuration. Reporting all MSD mechanisms and the associated parameters could increase signaling overhead substantially.

Observation 2: A band combination may be subject to impacts from different MSD mechanisms which may not occur simultaneously under the scheduled carrier configuration. Reporting all MSD mechanisms and the associated parameters could increase signaling overhead substantially.

On the other hand, without real-time MSD measurement, UE would have to store all the pre-measured MSD values for all the supported band combinations subject to MSD impact where a huge UE memory loading is anticipated. And last but not least, MSD measurement is a rather time-consuming process which could substantially increase the factory test cost per UE.

Observation 3: Without real-time MSD measurement, UE would have to store all the pre-measured MSD values for all the supported band combinations subject to MSD impact where a huge UE memory loading is anticipated.

Observation 4: MSD measurement is a rather time-consuming process which could substantially increase the factory test cost per UE.

By accounting all the aforementioned drawbacks, the Rel-18 lower MSD capability has seemingly lost steam in attracting both UE and network sides to support the feature. If this feature would never be implemented on either side or both sides of UE and network, then we are essentially back to the origin that the concern with MSD uncertainty in all potentially impacted band combinations would still remain. Therefore, it is imperative to have a more effective and efficient alternative in Rel-19 to resolve the long-standing MSD uncertainty issue.

## 2.2	Motivation for real-time MSD/SIR measurement and reporting in Rel-19

In contrast to the Rel-18 static lower MSD reporting, the real-time MSD/SIR measurement and reporting is based on the carrier configuration to be scheduled by network in real field operation, but not the worst-case carrier configuration artificially defined in RAN4 specifications. It is with the following characteristics and completely avoids all the drawbacks in Rel-18 lower MSD capability signaling.

- The process is triggered by network where network can first perform UE self-interference analysis based on the scheduled carrier configuration and MSD mechanism(s) to determine whether the MSD measurement and reporting is needed or not.

- It does not require UE to pre-measure and store all the MSD values for all MSD impacted band combinations.

- The measurement is RSSI-based instead of throughput-based as in REFSENS test where the latter is more time consuming.

- UE does not need to report multiple MSD mechanisms as they are transparent to UE.

- The same measurement and reporting procedure is applicable for both semi-static MSD reporting and dynamic SIR (Signal to Interference Ratio) reporting [5].

- Semi-static reporting is only performed once after RRC configuration or reconfiguration.

One other important aspect in the real-time MSD/SIR measurement is that it is an Over-The-Air (OTA) based measurement as opposed to the conductive based MSD measurement where MSD is often underestimated due to that the true antenna isolation was not reflected or emulated in conductive test environment. As a result, the real-time MSD measurement shall possess better MSD fidelity than the conductive measurement.

Observation 5: MSD is often underestimated in conductive based measurement due to that the true antenna isolation was not reflected or emulated in conductive test environment.

Observation 6: Real-time MSD measurement is OTA based measurement which shall possess better MSD fidelity than the conductive measurement.

Furthermore, the current MSD analysis has been known for consuming the most time and efforts in the new band combination introduction process. The artificially defined MSD requirements not only occupy a substantial portion of the RAN4 technical specifications with poor readability, but also create humongous workloads for UE conformance tests which do not really serve the purpose in gauging the UE RF performance but to further augment the device cost. If real time MSD measurement and reporting would become available, the existing MSD requirements may no longer be needed and can potentially be removed from the technical specifications as proposed in [6] which could substantively simplify the UE RF specifications and streamline the new band combination introducing process. The workloads for UE conformance tests could also be greatly reduced.

Observation 7: The current MSD analysis has been known for consuming the most time and efforts in the new band combination introduction process.

Observation 8: The artificially defined MSD requirements not only occupy a substantial portion of the RAN4 technical specifications with poor readability, but also create humongous workloads for UE conformance tests.

Observation 9: If real time MSD measurement and reporting would become available, the existing MSD requirements may no longer be needed and can potentially be removed from the technical specifications which could substantively simplify the UE RF specifications and streamline the new band combination introducing process. The workloads for UE conformance tests could also be greatly reduced.

## 2.3	Rel-19 RAN4 objectives for MSD reporting enhancement

The following Rel-19 RAN4 objectives for MSD reporting enhancement are proposed for consideration:

- Specify the process flow for real-time RSSI-based MSD/SIR measurements and reporting and the corresponding RRM requirements.

- Define a throughput-based OTA MSD measurement to correlate the UE RSSI-based MSD reporting.

- Introduce new feature/capability signaling if necessary.

# 3	Conclusion

In this contribution, we summarize the drawbacks of the Rel-18 lower MSD capability signaling and propose a more effective and efficient MSD/SIR measurement and reporting scheme based on real carrier configurations as part of Rel-19 RAN4 objectives for consideration.

Observation 1: The Rel-18 lower MSD capability reported by UE only occurs at a particular worst-case carrier configuration defined in RAN4 specifications, but not necessarily and likely not the MSD for the configuration scheduled by the network in real field operations which could potentially impact the network scheduling efficiency.

Observation 2: A band combination may be subject to impacts from different MSD mechanisms which may not occur simultaneously under the scheduled carrier configuration. Reporting all MSD mechanisms and the associated parameters could increase signaling overhead substantially.

Observation 3: Without real-time MSD measurement, UE would have to store all the pre-measured MSD values for all the supported band combinations subject to MSD impact where a huge UE memory loading is anticipated.

Observation 4: MSD measurement is a rather time-consuming process which could substantially increase the factory test cost per UE.

Observation 5: MSD is often underestimated in conductive based measurement due to that the true antenna isolation was not reflected or emulated in conductive test environment.

Observation 6: Real-time MSD measurement is OTA based measurement which shall possess better MSD fidelity than the conductive measurement.

Observation 7: The current MSD analysis has been known for consuming the most time and efforts in the new band combination introduction process.

Observation 8: The artificially defined MSD requirements not only occupy a substantial portion of the RAN4 technical specifications with poor readability, but also create humongous workloads for UE conformance tests.

Observation 9: If real time MSD measurement and reporting would become available, the existing MSD requirements may no longer be needed and can potentially be removed from the technical specifications which could substantively simplify the UE RF specifications and streamline the new band combination introducing process. The workloads for UE conformance tests could also be greatly reduced.

The following Rel-19 RAN4 objectives for MSD reporting enhancement are proposed for consideration:

- Specify the process flow for real-time RSSI-based MSD/SIR measurements and reporting and the corresponding RRM requirements.

- Define a throughput-based OTA MSD measurement to correlate the UE RSSI-based MSD reporting.

- Introduce new feature/capability signaling if necessary.

# 4	References

R4-2321785 “DraftCR for introduction of lower-MSD requirements for inter-band CA”, Huawei, HiSilicon et al, 3GPP TSG-RAN WG4 Meeting #109, Chicago, USA, November 13th – 17th, 2023

R4-2321786 “DraftCR for introduction of lower-MSD requirements for inter-band EN-DC”, Huawei, HiSilicon et al, 3GPP TSG-RAN WG4 Meeting #109, Chicago, USA, November 13th – 17th, 2023

RP-231998 “T-Mobile USA’s RAN4 Priorities for Release-19”, T-Mobile USA, TSG-RAN Meeting #101, Bangalore, India, September 11th – 15th, 2023

RP-233498 “On RAN4 led topics with RF, RRM and Demod in Rel-19”, Apple, TSG-RAN Meeting #102, Edinburgh, UK, December 11th – 15th, 2023

R4-2117986 “UE SIR measurement for network scheduling assistance”, Apple, 3GPP RAN WG4 Meeting #101-e, Online, November 1st – 12th, 2021

R4-2411147 “Simplifying or removing MSD tables”, Apple, 3GPP TSG-RAN WG4 Meeting #112, Maastricht, Netherlands, August 19th – 23rd, 2024

# 5	Appendix

- Figure 5-1 is a conceptual illustration of UE RSSI-based MSD estimation.

- Figure 5-1 A conceptual illustration of UE RSSI-based MSD estimation