3GPP TSG-RAN WG4 Meeting # 112bis	R4-2415357

Hefei, China, 14th - 18th Oct 2024

Title:	TP to TR 36.719-01-01 Addition of ULCA_8-11 to CA_8-11

Source:	Softbank Corp.

Agenda item:	5.4.2

Document for:	Approval

1	Introduction

This contribution is a text proposal for TR 36.719-01-01[1] to ad ULCA_8-11 to CA_8-11.

2. Reference

[1] TR36.719-01-01, Rel-19 LTE Advanced CA for x bands DL (1<=x<=6) with y bands UL (y=1, 2)  V0.0.0

Text Proposal

-- Start of TP –

-- Unaffected parts omitted –

## 5.3	LTE-A inter-band CA for 2 bands DL with 2 bands UL

### 5.3.x	CA_8-11

#### 5.3.x.1	Channel bandwidths per operating band for CA

Table 5.3.x.1-1: Inter-band CA operating bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
| 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |

Table 5.3.x.1-2: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_8A-11A | CA_8A-11A | 8 |  |  | Yes | Yes |  |  | 20 | 0 |
|  |  | 11 |  |  | Yes | Yes |  |  |  |  |

#### 5.3.x.2	Co-existence studies

Coexistence for CA_8-11 has already been specified in TS 36.101.

#### 5.3.x.3	∆TIB and ∆RIB values

Relaxation values for CA_8-11 have already been specified in TS 36.101.

#### 5.3.x.4	REFSENS Requirements

No additional REFSENS relaxation required.

-- Unaffected parts omitted –

-- End of TP --