3GPP TSG-RAN WG4 Meeting #112-bis	                               R4-2415359

Hefei, China, October 14 – October 18, 2024

Source: 	Murata Manufacturing Co Ltd.

Title: 	Further discussion on simultaneous Rx-Tx of CA_n40-n41

Agenda item:	5. 11. 2

Document for:	Discussion

Introduction

In the last RAN4#112 meeting, the remaining issues on the simultaneous Rx-Tx in Rel-19 was discussed and the WF [1] was approved. The method on how to relax the 4 Rx requirement of simultaneous Rx-Tx of CA_n40-n41 was especially controversial.

In this contribution we discuss the issue related to simultaneous Rx-Tx of CA_n40-n41 further and provide our view.

Discussion

Below is the way forward from [1]. The opinions from companies were still diverse so no concrete agreement has been made and one operator shared the view to keep 4Rx and not to relax 4Rx to 2Rx.

<Way forward>

The way to handle the simultaneous Rx-Tx for CA_n40-n41 in Rel-19.

- Option 1: Keep the requirements with n41 using 4Rx in simultaneous Rx-Tx for CA_n40-n41

- Option 2: Analyse the UE architecture with n41 using 2Rx in simultaneous Rx-Tx for CA_n40-n41, and try to define the separate requirement including MSD optionally from that in Rel-18, taking the implementation difficulties into consideration.

  - However, the UE should support 4Rx for n41 when the UE does not indicate simultaneous Rx-Tx for CA_n40-n41, i.e single n41 or other CA combo.

- Option 3: Study whether new UE signalling(s) could be introduced for a certain band combination to indicate that:

  - concurrent support of 4x4 MIMO and simultaneous Rx/Tx is not supported

  - concurrent support of UL MIMO, and/or TxD and/or SRS antenna switching and simultaneous Rx/Tx is not supported

- Other options are not precluded

We will discuss the MSD for different UE operations, i.e. 4Rx and relaxed to 2Rx first in the following session.

After that we will propose the new MSD test point which is additional to the current existing MSD test point.

Discussion on MSD analysis for simultaneous Rx-Tx of CA_n40-n41

Existing MSD

Table 1 and Table 2 show PC3 and PC2 MSD respectively which were agreed previously.

Table 1 PC3 MSD due to cross band isolation for simultaneous Rx-Tx CA_n40A-n41A

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n41 | 2350 | 100 | 30 | 270 (RBstart=3) | 2501 | 10 | 28.1 | ACLR2 |
| n41 | n40 | 2546 | 100 | 30 | 270 (RBstart=0) | 2397.5 | 5 | 31.4 | ACLR2 |

Table 2 PC2 MSD due to cross band isolation for simultaneous Rx-Tx CA_n40A-n41A

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n41 | 2350 | 100 | 30 | 270 (RBstart=3) | 2501 | 10 | 31.1 | ACLR2 |
| n41 | n40 | 2546 | 100 | 30 | 270 (RBstart=0) | 2397.5 | 5 | 34.4 | ACLR2 |

It should be noted that the above MSD calculation is based on 2Rx receiver as shown in Figure 1.

Figure 1. UE RFFE architecture for MSD calculation of CA_n40-n41 (n40 UL, n41 DL)

There is the interferer noise power (ACLR2 for this case) imbalance between PRX and DRX. It can be 9~12dB depending on the UE RF front end architecture implementation and antenna isolation. In other words, DRX of n41 is separated from the PTX of n40 through antenna isolation and the interferer noise power at DRX is lower than at PRX. Therefore, DRX diversity gain can be achieved.

MSD calculation of n41 when relaxed to 2Rx

As mentioned in the sub bullet in option 2 in the above WF, the UE should support 4Rx mandatory for single n41 band operation or other CA combination. Therefore, if we relax the requirement to 2Rx, we can turn off two receiver chains among four receiver chains of n41. Figure 2 shows the RFFE architecture for relaxed 2Rx MSD calculation to achieve higher isolation between n40 UL and n41 DL.

Figure 2. UE RFFE architecture for relaxed 2Rx MSD calculation of CA_n40-n41 (n40 UL, n41 DL)

We analysed the MSD of n41 with relaxed 2Rx for this case as Table 3. The difference from the MSD analysis in Table 1 and Table 2 is that both PRX and DRX of n41 have same interferer noise power (i.e. 0dB imbalance) with the help of antenna isolation for both receiving paths. We found that MSD is reduced by only 1.4dB (26.7dB for PC3 and 29.7dB for PC2). In conclusion, the MSD is still very high even though we relax 4 Rx requirement to 2Rx for n41, so it is not applicable to high throughput data communication, either.

Table 3 PC3 and PC2 MSD of n41 with relaxed 2Rx

|  | PC3 |  | PC2 |  |
| --- | --- | --- | --- | --- |
|  | PRX | DRX | PRX | DRX |
| Interferer noise at PRX Ant | -55.1 |  | -52.1 |  |
| Interferer imbalance | 12 |  | 12 |  |
| REFSENS | -94.8 |  | -94.8 |  |
| Thermal | -90.8 | -90.8 | -90.8 | -90.8 |
| Interferer | -67.1 | -67.1 | -64.1 | -64.1 |
| Composite | -67.1 | -67.1 | -64.1 | -64.1 |
| MRC SENS (correlated) | -68.1 |  | -65.1 |  |
| MSD after MRC (correlated) | 26.7 |  | 29.7 |  |

Observation 1: Large MSD is expected even though 4Rx requirement is relaxed to 2Rx for n41

Additional MSD test point for simultaneous Rx-Tx of CA_n40-n41

Another point we need to pay attention to is the test point of MSD in Table 1 and Table 2. Generally, the cross band isolation test point was agreed to use max UL channel bandwidth and min DL channel bandwidth for the worst MSD calculation.

Therefore, if we follow this agreed MSD test point then the channel bandwidth combination of the CA is either 100MHz+10MHz or 100MHz+5MHz. From the data throughput point of view, this kind of combination is not efficient because not only the total CA channel bandwidth (105MHz or 110MHz) is not much larger than single band operation channel bandwidth (100MHz) but we have huge MSD leading to unavailable higher order modulation communication. In other words, there is no benefit of adopting CA since the main purpose of CA is to improve the data rate.

Observation 2: There is no benefit of using 100MHz+10(5)MHz bandwidth combination because DL with 10(5)MHz bandwidth cannot support high data rate due to large MSD

Instead, we can think of another CA channel bandwidth combination to achieve high data rate with reasonable sensitivity degradation or MSD as shown in Figure 3 and Figure 4. Both UL and DL bandwidth are set to 50MHz. The interfere noise source is >ACLR2 since the gap between n40 upper and n41 lower is 96MHz. So, we can expect lower interferer noise would be falling into DL compared to interferer noise source from 100MHz UL (ACLR2) and lead to lower MSD compared to Table 1 and Table 2. Our initial estimates shows that the MSD for PC3 ranges from 8~12dB but needs further verification with measurement.

Figure 3. Landscape when n40 is aggressor and n41 is victim when both channel bandwidth is 50MHz

Figure 4. Landscape when n41 is aggressor and n40 is victim when both channel bandwidth is 50MHz

Table 4 below is the proposal of the additional MSD test point which is one optional candidate.

Table 4 Additional MSD test point due to cross band isolation for simultaneous Rx-Tx CA_n40A-n41A

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n41 | 2375 | 50 | 15 | 270 (RBstart=0) | 2521 | 50 | TBD | >ACLR2 |
| n41 | n40 | 2521 | 50 | 15 | 270 (RBstart=0) | 2375 | 50 | TBD | >ACLR2 |

Observation 3: Both 50MHz channel bandwidth of n40 and n41 might lead to lower MSD with total aggregated 100MHz channel bandwidth

Proposal 1: Consider Table 4 for the additional MSD test point for simultaneous Rx-Tx CA_n40-n41

Conclusion

In this contribution, we discussed the remaining issue regarding the simultaneous Rx-Tx of CA_n40-n41 and provided our view as below.

Observation 1: Large MSD is expected even though 4Rx requirement is relaxed to 2Rx for n41

Observation 2: There is no benefit of using 100MHz+10(5)MHz bandwidth combination because DL with 10(5)MHz bandwidth cannot support high data rate due to large MSD

Observation 3: Both 50MHz channel bandwidth of n40 and n41 might lead to lower MSD with total aggregated 100MHz channel bandwidth

Proposal 1: Consider Table 4 for the additional MSD test point for simultaneous Rx-Tx CA_n40-n41

Table 4 Additional MSD test point due to cross band isolation for simultaneous Rx-Tx CA_n40A-n41A

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n41 | 2375 | 50 | 15 | 270 (RBstart=0) | 2521 | 50 | TBD | >ACLR2 |
| n41 | n40 | 2521 | 50 | 15 | 270 (RBstart=0) | 2375 | 50 | TBD | >ACLR2 |

References

[1]	R4-2412606, “WF on simultaneous Rx-Tx for CA_n40-n41”, Huawei, Hisilicon, 3GPP TSG RAN WG4 Meeting #112, Maastricht, Netherlands, August 19th – 23rd, 2024