3GPP TSG-RAN WG4 Meeting #112-bis	                               R4-2415360

Hefei, China, October 14 – October 18, 2024

Source: 	Murata Manufacturing Co Ltd.

Title: 	Discussion on UE RF requirement impact for DL fragmented carriers

Agenda item:	6. 6. 3

Document for:	Discussion

Introduction

In the last RAN4#112 meeting, the topic on DL fragmented carriers was discussed [1] and the WF [2] was approved. However there has not been detailed discussion since it was the first meeting for this study item and the companies’ views are diverse. In this contribution we discus the RF requirement impact especially for reference sensitivity.

Discussion

DL fragmented carriers study item was introduced in the previous RANP. The purpose of this SI is to reduce the number of Rx signal paths from two to one supporting non-contiguous DL CA and to utilize the remaining Rx signal path for another CC to increase the data throughput. This is because some operators have non-contiguous or fragmented carriers within their own licensed band.

The reference UE RF architecture is agreed within the companies but to go further forward the UE RF requirement impacts should be carefully reviewed. One of the important RF requirements is reference sensitivity so we only focus on how much reference sensitivity would be impacted for DL fragmented carriers in this contribution. In other words, the in-gap interferer requirements such as ACS, IBB, NBB or other operator’s DL CC from same BS location are not scope of this paper.

From LTE non-contiguous DL CA discussion, the ΔRIBNC concept has been introduced for relaxation of DL SCC due to UL PCC Rx band noise. The requirement is highly dependent on the BW of PCC and SCC, Wgap and UL PCC RB allocation. It should be noted that this requirement is derived based on the RF front end block diagram which divides the path for each CCs after external LNA or before down conversion mixer of RFIC receiver as shown in Figure 1. This kind of block diagram can be referred as partially shared architecture since duplexer and external LNA share the PCC and SCC DL path.

On the other hand, Figure 2 shows the fully shared architecture since all DL receiver components share PCC and SDD DL path for supporting DL fragmented carriers.

We think that there are four factors that can affect both PCC and SCC DL sensitivity performance as listed below and each will be examined in the following sections.

- Rx band noise due to PCC Tx nonlinearity (RFIC impairment, PA noise and duplexer isolation)

- DL image falling into another DL CC (image rejection ratio of down conversion mixer)

- IMD2 noise due to Tx leakage (duplexer isolation and IIP2 of receiver)

- Decreased ADC SNR due to Tx leakage (LPF attenuation)

Figure 1. UE RFFE block diagram of partially shared architecture

Figure 2. UE RFFE block diagram of fully shared architecture

Rx band noise due to PCC Tx nonlinearity (RFIC impairment, PA noise and duplexer isolation)

The Rx band noise due to PCC Tx nonlinearity is the main interferer noise source for deriving ΔRIBNC of non-contiguous DL CA with partially shared RF architecture. This Rx band noise includes >ACLR or Tx impairments such as CIMD3/5, IMD between CIMD3 and UL Tx signal, etc. Also, duplexer isolation plays a major role to determine the Rx band noise generated by PCC Tx nonlinearity which injects into receiver chain since SCC DL frequency is close to PCC UL frequency.

When we consider fully shared RF architecture for DL fragmented carriers, we can expect that the same Rx band noise as partially shared RF architecture would fall into PCC DL and SCC DL (at the LNA input) because this interferer noise falling into DL frequency would not be changed regardless of RF architecture as shown in Figure 1 and Figure 2.

Therefore, no additional sensitivity degradation would be observed due to Rx band noise.

Observation 1: No additional sensitivity degradation would be observed between architectures due to Rx band noise since the noise level would be same.

DL image falling into another DL CC (image rejection ratio of down conversion mixer)

In the previous meeting, some companies shared concern that down converted image signal of one DL CC might be overlapped with another DL CC because RX LO is between two DL CCs.

Figure 3 shows how much the sensitivity would be degraded for the worst case where the power imbalance between two CCs is 6dB. We can see almost no sensitivity degradation if the image rejection ratio of the Rx path is better than 25dB which is easily achieved in the modern RFIC receiver design.

Figure 3. Desense vs image rejection ratio of down conversion mixer

Observation 2: No additional sensitivity degradation would be observed between architectures due to DL image since the modern RFIC receiver has IRR performance better than 25dB.

IMD2 noise due to Tx leakage (duplexer isolation and IIP2 of receiver)

For FDD band, IMD2 noise due to Tx leakage also should be considered for the sensitivity derivation. This IMD2 noise down converted to DC increases the total noise in receiving bandwidth degrading sensitivity performance. It is well known that this is because down conversion mixer has finite IIP2. And it is reasonable to assume that Tx leakage power going into down conversion mixer and the IIP2 of the receiver would be same for both fully shared and partially shared architecture. Therefore, no additional sensitivity degradation would be also expected from Tx leakage IMD2 noise for fully shared architecture.

Observation 3: No additional sensitivity degradation would be observed between architectures due to IMD2 noise from Tx leakage since the IIP2 would be same

Decreased ADC SNR due to Tx leakage (LPF attenuation)

The final factor which might impact on the reference sensitivity is the decrease of ADC SNR. The cutoff frequency of the LPF should be much higher which leads to poor attenuation of Tx leakage power at the input of ADC. It should be noted that the LPF attenuation is determined by the filter order and ADC SNR is generally referred to as ENOB of ADC. So, it is highly RFIC design implementation dependent.

In this contribution, we briefly calculated the LPF attenuation requirement for Tx leakage. In our view, we can refer the current existing ACS requirement referenced in 38.101-1 Table 7.5-3. It is assumed that the LPF cannot attenuate ACS interferer which is 31.5dB higher than the wanted DL signal. That means that the Tx leakage going into ADC should be less high than 31.5dB compared to the wanted signal going into ADC to meet the required SNR for demodulation. If we assume 4dB FE loss and 50dB Tx-Rx isolation with -100dBm reference sensitivity for single CC, the required LPF attenuation for Tx leakage can be calculated as follows.

Tx leakage at LNA input: PA out - duplexer Iso = 27 – 50 = -23dBm

Wanted signal at LNA input: -100 - 4 = -104dBm

If we assume the down conversion gain for Tx leakage and wanted signal is the same, then LPF attenuation for Tx leakage calculated below is required.

LPF attenuation: Tx leakage - Wanted signal - 31.5 = -23 - (-104) - 31.5 = 49.5dB

Observation 4: Using the fully shared/partially shared architecture for NC DLCA, [50]dB LPF attenuation is required to reduce the impact of the Tx leakage and satisfy the required SNR for demodulation

Based on the above discussion, we would like to propose below.

Proposal 1: Interested companies to provide the feasibility and performance of wider low pass filter attenuation

Proposal 2: Consider increasing ΔRIBNC for cases where the low pass filter attenuation is worse than [50]dB and evaluate the ΔRIBNC increase on a case by case basis

Conclusion

In this contribution, we discussed the UE RF requirement impact for DL fragmented carriers especially for reference sensitivity and provided our view as follows:

Observation 1: No additional sensitivity degradation would be observed between architectures due to Rx band noise since the noise level would be same.

Observation 2: No additional sensitivity degradation would be observed between architectures due to DL image since the modern RFIC receiver has IRR performance better than 25dB.

Observation 3: No additional sensitivity degradation would be observed between architectures due to IMD2 noise from Tx leakage since the IIP2 would be same

Observation 4: Using the fully shared/partially shared architecture for NC DLCA, [50]dB LPF attenuation is required to reduce the impact of the Tx leakage and satisfy the required SNR for demodulation

Proposal 1: Interested companies to provide the feasibility and performance of wider low pass filter attenuation

Proposal 2: Consider increasing ΔRIBNC for cases where the low pass filter attenuation is worse than [50]dB and evaluate the ΔRIBNC increase on a case by case basis

References

[1]	R4-2412827, “Topic summary for [112][122] FS_NR_FR1_DL_Frag_Carrier”, MediaTek Inc, 3GPP TSG RAN WG4 Meeting #112, Maastricht, Netherlands, August 19th – 23rd, 2024

[2]	R4-2414311, “WF on fragmented DL carriers study”, MediaTek Inc, 3GPP TSG RAN WG4 Meeting #112, Maastricht, Netherlands, August 19th – 23rd, 2024