3GPP TSG-RAN WG4 Meeting # 112bis	R4-2415361

# Hefei, Anhui, China, 14th – 18th October, 2024

Source:	LG Electronics

Title:	Discussion on RRM requirements for Fast SCell activation for UE supporting R18 EMR

Agenda item:	6.15.3

Document for:	Discussion

Introduction

In the last RAN4#112 meeting, the fast SCell activation for UE supporting R18 eEMR discussion was started. But there are only three meetings for the item. So, most companies are trying to reduce and simplify the issues. We also think RAN4 should not expand the issues to meet the target schedule. Based on this thinking, we show our views on the remain issues.

Discussion

  - Sub-topic 1-1 General

- Issue 1-1-2: Applicability of fast SCell activation delay requirements Agreement: - The fast SCell activation delay requirements are defined for the case when - the UE supports Rel-18 eEMR: - Including both EMR and cell reselection measurement, - Including the case when measIdleValidityDuration-r18 and / or measReselectionValidityDuration-r18 are configured - FFS Including the case when measIdleValidityDuration-r18 and / or measReselectionValidityDuration-r18 are not configured - and the UE has reported valid results on the SCell to be activated before SCell activation command.

The intention of the fast SCell activation for UE supporting R18 EMR is to reduce the SCell activation delay when the UE report valid results on the SCell. But if the measIdleValidityDuration-r18 and/or measReselectionValidiryDuration-r18 are not configured, the UE can’t know whether the measured results are valid or not. So, the case when measIdleValidityDuration-r18 and /or measReselectionValidityDuration-r18 are not configured should be removed from the applicability. If network want to reduce the SCell activation delay for supporting R18 EMR UE, it is recommended for network to configure the measIdleValidityDuration-r18 and /or measReselectionValidityDuration-r18.

Proposal 1: RAN4 to remove the case when measIdleValidityDuration-r18 and /or measReselectionValidityDuration-r18 are not configured from the applicability of fast SCell activation delay requirements for supporting Rel-18 eEMR UE.

Sub-topic 1-2 SCell activation delay requirements

- Issue 1-2-1: How to define the fast SCell activation delay requirements with valid eEMR reporting Agreement: - RAN4 to update the known condition for SCell activation to include the case when UE has valid EMR reporting before SCell activation command. - FFS how to update the known condition (issue 1-2-1a). - FFS the applicability of valid EMR reporting for the Rel-19 fast SCell activation - Issue 1-2-1a: How to update the known condition with consideration of valid eEMR reporting FFS on proposals listed in [1] for issue 1-2-1a.

There are some ambiguity in the known cell definition whether the valid measurement report include valid eEMR report or not. And many companies have different views on that. We think the legacy valid measurement report covers valid eEMR report also. So, we would like not to change current known cell definition.

Proposal 2: RAN4 not to change current known cell condition with consideration of valid eEMR reporting.

Conclusion

In this contribution, we provide our views on fast SCell activation for UE supporting R18 EMR item.

Proposal 1: RAN4 to remove the case when measIdleValidityDuration-r18 and /or measReselectionValidityDuration-r18 are not configured from the applicability of fast SCell activation delay requirements for supporting Rel-18 eEMR UE.

Proposal 2: RAN4 not to change current known cell condition with consideration of valid eEMR reporting.

Reference

[1] R4-2414041, “WF on NR_RRM_Ph5_Part2 – fast SCell activation”, CATT.