3GPP TSG-RAN WG4 Meeting #112bis	R4-2415367

Hefei, China, 14th-18th, October, 2024

Title: 	MSD requirement for new PC2/PC1.5 band combinations

Agenda Item:	6.1.1.1.1

Source: 	Nokia

Document for:	Approval

# Introduction

In previous RAN4 meeting #112 the discussion on HPUE MSD continued like it has continued for quite some time now. From moderator summary [2] we can see the company proposals for RAN4#111.

#### Issue 1.2.4-1: MSD rules

Proposals：

  - Proposal 1: (Samsung)

- RAN4 to clarify that MSD general rule discussion is within the WI scope, just the specific MSD values are left for corresponding basket WIDs to evaluate.

- For all types of MSD, RAN4 defines only the default power class inter-band CA and intra-band CA MSD requirements, and the new HPUE MSD requirements are no longer specified in the TS. The legacy agreed HPUE MSD test points are not impacted and are kept in the TS.

  - Proposal 2: (Xiaomi)

- If the supported maximum power class of the constituted band are not changed in the higher power class case for the same band combination, no new MSD case is needed compared to the existing MSD requirement in the spec when defining the harmonic, harmonic mixing and cross isolation requirements.

- A discussion is needed on whether we can agree that for a given band combination, not to reconsider additional IMD MSD requirements for higher total power class on the basis that the MSD requirement has already been defined for the same power class configuration in lower total power class.

  - Proposal 3: (Nokia)

- WID objectives preclude band combination specific MSD discussions but allow generic MSD discussion applicable to all band combinations.

- When new PC1.5 NR TDD intra-band UL contiguous and non-contiguous CA with 2Tx configuration is introduced no additional MSD test points are specified.

- When new PC1.5 UE for two band NR inter-band uplink CA with 2Tx and/or 3Tx for handheld and FWA, and PC1.5 and PC2 for two band EN-DC with 2Tx and/or 3Tx for handheld and FWA is introduced no additional MSD test points are specified.

- Framework for those new configurations which are not covered by proposals in this paper can be discussed in relevant basket WI separately unless there is a consensus to have common WF in this meeting for all HPUE power classes no matter if it is single band, CA or DC.

  - Proposal 4: (vivo)

There are some basic options for MSD derivation for HPUE BC:

    - Re-Calculate the MSD requirements.

    - Using High Power + Default power class requirements

    - Using Default Power + Default power class requirements

  - Proposal 5: (Qualcomm)

- Instead of going directly into “PC3 MSD only” approach, companies should first consider if some simplifications e.g. by specifying just single set of requirements for HPUE FDD band combinations and/or by using some equations to derive PC2/PC1.5 MSD from respective PC3 MSD would help to bring the MSD specification framework back on track.

  - Proposal 6: (NTT docomo)

Introduce new guidelines for MSD requirements due to IM. Below are the key points.

- The default is the MSD requirement for PC3 CA/DC.

- Regarding test points for HPUE CA/DC;

    - if there are in the default MSD requirements, they will be reused.

    - if there are not in the default MSD requirements, new test points are added to other tables.

- Regarding MSD values for HPUE CA/DC, it can be calculated using default MSD value for PC3 and new parameter.

- Regarding power configuration for test;

    - for PC2 CA/DC, assuming both of the transmitters shall be set min(+23 dBm, PCMAX_L,f,c).

    - for PC1.5 CA/DC, assuming both of the transmitters shall be set min(+26 dBm, PCMAX_L,f,c).

  - Proposal 7: (CHTTL)

- Regarding the HPUE for inter-band CA/EN-DC in Rel.19, suggest to focus on the SAR solutions according to the WID at this stage.

# Discussion

WF [2] in RAN4#111 had following agreement for MSD topic

Way forward:

- RAN4 can discuss the MSD general rules, and strive to down-select among the following options

- PC2/PC1.5 MSD requirements are not specified. UE shall meet the Rx requirements of default power class, with default power class transmission

- Using some equations or look-up tables or other means to derive PC2/PC1.5 MSD from respective PC3 MSD

- Other solutions are not precluded

NOTE: RAN4 continues normal procedure specifying MSD for all PC’s band combination supports until down-selection is done

So basically nothing is agreed yet. We have been advocation the first option i.e. no new MSD is specified and naturally not tested either. Reason for this is that as a network vendor we do not see any usage for additional MSD for PC2/PC1.5 band combinations. Using MSD value dynamically in scheduling is difficult if not almost impossible. So the default power class MSD would be enough as a flag that this band combination may have issues due to self-interference, nothing more than that is not useful in our opinion. We do not think that new additional MSD value is helpful in network planning either. We can understand that from component and UE vendor point of view there may be benefits in customer communications and customer expectations point of view to have MSD also for HPUE operation. In order to progress this topic, we can also consider some simple way of generation of additional MSD for PC2/PC1.5 band combinations. Unified table method as described in [3] or equation-based method can be considered.

As an example, what we mean with unified table method we present three tables from [3]. With unified table method there is no need to discuss MSD per band combination, instead MSD requirement is looked up from tables like below. Some companies have used term look up table method instead of unified table method.

Table 4: Harmonic mixing requirements PC2/PC1.5. UHB = UHB + VHB

| PC2/PC1.5 |  |  |  |
| --- | --- | --- | --- |
| Order | UHB/LB | UHB/MB | UHB/HB |
| UL1/DL2 | N/A | 9.2dB/11.9dB | N/A |
| UL1/DL4 | 8.1dB/10.7dB | N/A | N/A |
| UL1/DL5 | 34dB/37dB | N/A | N/A |
| UL2/DL3 | N/A | N/A | 13.2dB/16.1dB |

Table 7: IMD requirements for PC2.

| 2 CA IMD |  |  |  | 3 CA IMD |  |
| --- | --- | --- | --- | --- | --- |
| Order | MSD | 2CA (NC_ULCA) | MSD | Order | MSD |
| IMD2 | 36dB | IMD4 | 18.6dB | IMD2 | 36.5dB |
| IMD3 same b.gr | 25dB | IMD5 same b.gr | 32.5dB | IMD3 same b.gr | 26.5dB |
| IMD3 diff. b.gr | 15dB | IMD5 diff. b.gr | 22dB | IMD3 diff b.gr. | 22dB |
| IMD4 | 19dB | IMD7 same b.gr | 13.5dB | IMD4 | 22dB |
| IMD5 | 14dB | IMD7 diff. b.gr | 9dB | IMD5 | 21dB |
|  |  | IMD9 | 3dB | IMD7 |  |

Table 8: IMD requirements for PC1.5.

| 2 CA IMD |  |  |  | 3 CA IMD |  |
| --- | --- | --- | --- | --- | --- |
| Order | MSD | 2CA (NC_ULCA) | MSD | Order | MSD |
| IMD2 | 37.5dB | IMD4 | N/A | IMD2 | N/A |
| IMD3 same b.gr | N/A | IMD5 same b.gr | N/A | IMD3 same b.gr | N/A |
| IMD3 diff. b.gr | N/A | IMD5 diff. b.gr | N/A | IMD3 diff b.gr. | N/A |
| IMD4 | 26.5dB | IMD7 same b.gr | N/A | IMD4 | N/A |
| IMD5 | 30dB | IMD7 diff. b.gr | N/A | IMD5 | N/A |
|  |  | IMD9 | N/A | IMD7 | N/A |

Observation 1: Unified table method as described in [3][4][5] or equation-based method can be considered for MSD for PC2/PC1.5 new band combinations.

These methods could be specified as informative in which case those are not tested or normative in which case they are tested.

Observation 2: MSD for new PC2/PC1.5 band combinations could be specified as informative in which case those are not tested or normative in which case they are tested

Unified table method or equation-based method would similarly as no new MSD method speed up the specification work for introduction of new PC2/PC1.5 band combinations which is very beneficial for all parties in industry.

Observation 3: Speedy introduction of new PC2/PC1.5 band combinations is very beneficial for all parties in industry.

We hope that in this meeting RAN4 can agree the method for defining the additional MSD for new PC2/PC1.5 band combinations and then continue on details on subsequent meetings.

Proposal: RAN4 to decide MSD method for new PC2/PC1.5 band combinations in RAN4#112bis. Candidates are unified table/equation-based method and no new MSD based method.

# 3	Conclusion

In this contribution we have discussed HPUE MSD and made following observation and proposal

Observation 1: Unified table method as described in [3][4][5] or equation-based method can be considered for MSD for PC2/PC1.5 new band combinations.

Observation 2: MSD for new PC2/PC1.5 band combinations could be specified as informative in which case those are not tested or normative in which case they are tested

Observation 3: Speedy introduction of new PC2/PC1.5 band combinations is very beneficial for all parties in industry.

Proposal: RAN4 to decide MSD method for new PC2/PC1.5 band combinations in RAN4#112bis. Candidates are unified table/equation-based method and no new MSD based method.

# 4	References

[1] R4-2412007, UE RF Enh 4: On necessity of additional MSD requirement for HPUE, Nokia, RAN4#112

[2] R4-2412819, Topic summary for [112][117] NR_ENDC_RF_Ph4_part2, Samsung, RAN4#112

[3] R4-2409190, On UE RF specifications table improvement, Nokia, RAN4#111

[4] R4-2413338, On UE RF specifications table improvement, Nokia, RAN4#112

[5] R4-2415392, On UE RF specifications table improvement, Nokia, RAN4#112bis