3GPP TSG-RAN WG4 Meeting #112bis	R4-2415373

Hefei, China, 14th-18th, October, 2024

Title: 	On Rel-19 non-collocated deployment UE Capability/UE behavior and network signaling

Agenda Item:	6.5.2.2

Source: 	Nokia, NSB

Document for:	Approval

# Introduction

In this contribution we express our views on open items listed in previous meeting WF [1] to make further progress.

# Discussion

In WF one open and urgent issue to be agreed was whether separate UE capabilities are required for UE Type 4a and 4b or is it enough that new UE capability is defined just for type 4 including both 4a and 4 b.

### < Issue 2-3-1:  New UE Capability for Type 4a(6Rx/UE) and 4b(8Rx/UE) for EN-DC/NR-CA >

Way forward:

- Define one capability for NR CA and one UE capability for EN-DC

  - FFS on how to differentiate type 4a and 4b.

In our contribution [2] submitted to previous meeting we proposed that it is enough UE to just indicate support for Type 4 as number of LTE receivers is indicated with fourLayer-TM3-TM4 signalling. However as most of companies were inclined to define separate UE capability for 4a and 4b we can accept that to allow RAN4 to progress the work

Proposal 1: Define signalling to distinguish UE types 4a and 4b

Furthermore, a new BS signalling to command UE to switch between type 4 and type 1 4L/CC and type 4 and type 1 6L/8L/CC were discussed in [1]. This new potential BS signalling is exactly same as current R18 IE nonCollocatedTypeMRDC-r18.

### < Issue 2-3-4: New BS Signaling to switch between Type 4a/4b and Type 1 4L/CC(collocated)>

Way Forward:

- First, conclude discussions of whether to add new UE capability(s) for Type 4.

- And then, continue further discussion on BS signalling in the next meeting.

### < Issue 2-3-5:  New BS Signaling to switch between Type 4a/4b and Type 1 6L/8L/CC(collocated) >

Way Forward:

- First, conclude discussions of whether to add new UE capability(s) for Type 4.

- And then, continue further discussion on BS signalling in the next meeting.

We believe that in order to ensure proper network handling of different UE types on R19 the best approach is to follow R18 way and define new network indications to command UE to switch from type 4 and type 1 4L/CC and type 4 and type 1 6L/8L/CC. RAN4 should not proposed to treat R19 network indication matter to be done differently compared to R18 as this would create further confusion on this already now too complicated mesh of capabilities and network indications.

Proposal 2: Define new r19 network indication IEs to command UE to switch from type 4 to type 1 4L/CC and from type 4 to type 1 6L/8L/CC

Lastly we discuss UE behavior during initial connection between Type 1, Type 2 and Type 4a/4b

< Issue 2-3-7:  UE behavior during initial connection between Type 1, Type 2 and Type 4a/4b >

Way Forward:

- First, conclude discussions of whether to add new UE capability(s) for Type 4 and BS signaling.

- And then, continue further discussion on UE behavior in the next meeting.

In R18, a Type 2 capable UE is assumed operating Type 2 as a default if nonCollocatedTypeNR-CA-r18 is not provided. As Type 4 UE has the same requirements as Type 2 UE, we could follow the same principle in R19 hence Type 2 should be the default UE type during initial connection.

Proposal 3: Type 2 should be the default UE type during initial connection also for Type 4 capable UE.

# Conclusion

In this contribution we have made our proposals for open issues.

Proposal 1: Define signalling to distinguish UE types 4a and 4b

Proposal 2: Define new Rel-19 network indication IEs to command UE to switch from type 4 to type 1 4L/CC and from type 4 to type 1 6L/8L/CC

Proposal 3: Type 2 should be the default UE type during initial connection also for Type 4 capable UE.

# References

[1] WF on Rel-19 non-collocated UE RF requirements, RAN4#112

[2] R4-2412013, Discussion intra-band non-collocated deployment Phase2 UE Capability/UE behavior and network signaling, RAN4#112, Nokia, NSB