3GPP TSG-RAN WG4 Meeting #112bis	R4-2415374

Hefei, China, 14th-18th, October, 2024

Title: 	Rel-19 clarifications of RF requirements for different power imbalance UE types

Agenda Item:	6.5.2.3

Source: 	Nokia

Document for:	Approval

# 1		Introduction

In previous RAN4 meeting a WF was approved on Rel-18 clarifications of RF requirements for different UE types. This contribution discusses how to extend the content of that WF to cover also potential Rel-19 UE types.

# 2		Discussion

Compared to WF [1] we have changed the naming of UE type into power imbalance UE type. Name is open to further discussion. But in general, we believe that introduction of this kind of tables into specification allows to use terms type 1, type 2 and type 3 in main body of text and that would make specification more readable and easier to understand. Currently as there are no UE type descriptions there is no short way of referencing to them instead it is done by referencing UE capabilities and network indications.

So table 1 would go to Rel19 of 38.101-1 and a version without type 4 would go to Rel18 version. Furthermore table 2 would go to Rel19 of 38.101-3 and a version without type 4 would go to Rel18 version.

Table 1: NR CA power imbalance UE capability types

| NR CA power imbalance UE capability types | Type description | dedicated UE capability |
| --- | --- | --- |
| Type 1 | It supports intra-band RF requirements (with a maximum power spectral density imbalance between downlink carriers, within 6 dB) and MRTD requirements according to clause 7.6.4-1 in 38.133 | N/A |
| Type 2 | The UE supports the inter-band RF requirements (with DL power imbalance as specified in clause 7.10A.2) and MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133 | intraBandNR-CA-non-collocated-r18 |
| Type 4 | The UE supports the inter-band RF requirements (with DL power imbalance as specified in clause 7.10A.2) and MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133 | TBD |

Table 2: EN-DC power imbalance UE capability types

| Power imbalance UE types | Type description | Dedicated UE capability |
| --- | --- | --- |
| Type 1 | It supports intra-band RF requirements (with a maximum power spectral density imbalance between downlink carriers, within 6 dB) and an (NG)EN-DC/NE-DC MTTD/MRTD according to clause 7.5.3/7.6.3 in TS 38.133. | N/A |
| Type 2 | The UE supports inter-band RF requirements (with DL power imbalance as specified in caluse 7.10A in TS 38.101-3) and an (NG)EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in TS 38.133 and NE-DC MTTD/MRTD according to clause 7.5.5/7.6.5 in TS 38.133 | interBandMRDC-WithOverlapDL-Bands-r16, requirementTypeIndication-r18 |
| Type 4 | The UE supports inter-band RF requirements (with DL power imbalance as specified in caluse 7.10A in TS 38.101-3) and an (NG)EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in TS 38.133 and NE-DC MTTD/MRTD according to clause 7.5.5/7.6.5 in TS 38.133 | TBD |

Proposal: Introduce table 1 and table 2 UE type definitions into 38.101-1 and 18.101-3 respectively.

# 3		Conclusion

In this contribution we have discussed how to introduce Rel-19 non-collocated UE type information into RAN4 specs.

Proposal: Introduce table 1 and table 2 UE type definitions into 38.101-1 and 18.101-3 respectively.

Table 1: NR CA power imbalance UE capability types

| NR CA power imbalance UE capability types | Type description | dedicated UE capability |
| --- | --- | --- |
| Type 1 | It supports intra-band RF requirements (with a maximum power spectral density imbalance between downlink carriers, within 6 dB) and MRTD requirements according to clause 7.6.4-1 in 38.133 | N/A |
| Type 2 | The UE supports the inter-band RF requirements (with DL power imbalance as specified in clause 7.10A.2) and MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133 | intraBandNR-CA-non-collocated-r18 |
| Type 4 | The UE supports the inter-band RF requirements (with DL power imbalance as specified in clause 7.10A.2) and MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133 | TBD |

Table 2: EN-DC power imbalance UE capability types

| Power imbalance UE types | Type description | Dedicated UE capability |
| --- | --- | --- |
| Type 1 | It supports intra-band RF requirements (with a maximum power spectral density imbalance between downlink carriers, within 6 dB) and an (NG)EN-DC/NE-DC MTTD/MRTD according to clause 7.5.3/7.6.3 in TS 38.133. | N/A |
| Type 2 | The UE supports inter-band RF requirements (with DL power imbalance as specified in caluse 7.10A in TS 38.101-3) and an (NG)EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in TS 38.133 and NE-DC MTTD/MRTD according to clause 7.5.5/7.6.5 in TS 38.133 | interBandMRDC-WithOverlapDL-Bands-r16, requirementTypeIndication-r18 |
| Type 4 | The UE supports inter-band RF requirements (with DL power imbalance as specified in caluse 7.10A in TS 38.101-3) and an (NG)EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in TS 38.133 and NE-DC MTTD/MRTD according to clause 7.5.5/7.6.5 in TS 38.133 | TBD |

# 4		References

[1] R4-2414454, WF on Rel-18 clarifications of RF requirements for different UE types, RAN4#112