3GPP TSG-RAN WG4 Meeting #112-bis	R4-2415378

# Hefei, China, 14th Oct 2024 – 18th Oct 2024

Source:	ISSDU Inc.

Title:	Discussion on ACLR for SU-MIMO and MU-MIMO with Co-existence Simulation

Agenda item:	6.2.4.1 Co-existence simulations

Document for:	Discussion

Introduction

In RAN4#112, a WF was agreed in [1] where a few agreements were captured for the 14800 – 15350 MHz frequency range, with some issues still requiring further discussion. Given that MIMO modes for beamforming have become standard for IMT parameters, the influence of MIMO on the required values of ACLR and ACS still requires further analysis. In this contribution, we provide our findings on single-user MIMO (SU-MIMO) and multi-user MIMO (MU-MIMO) under co-existence simulation in the 14800 – 15350 MHz frequency range.

Discussion

It is observed that the use of beamforming can relax the ACLR requirements, as reported in [2] but also send the distortions in un-intended directions that are in turn dependent on SU-MIMO and MU-MIMO. In addition, in the specification of satellite communications, the required values of ACLR are different for different MIMO modes [3]. These therefore motivate the discussions on whether the required values of ACLR should also be determined by the MIMO modes used for transmissions.

Our goal is to verify how the introduction of MIMO technology can impact ACLR limits.  Transmit powers could be reduced to ensure the PA operates in linear region but this will impact coverage. Reductions of Tx power could reduce out of band radiations. There is a need to understand the impact of MIMO technology on ACLR and the relationship with MIMO especially SU-MIMO and MU-MIMO.

There is a need to understand how MIMO technology and PA nonlinearities affect out-of-band emissions, particularly in both SU-MIMO and MU-MIMO cases.

ACLR Calculation Procedure

Figure 2.1-1: ACLR Calculation Procedure

Step 1: Set test model, bandwidth, subcarrier spacing and duplex mode

In [1], the following was agreed:

Agreement: - For LS about 15GHz, choose the single value out of 100MHz, 200MHz or 400MHz as baseline with understanding that the other channel bandwidths are not precluded.

After discussions involving various proposals, it was agreed to include 100 MHz as one of the standard options, despite an initial focus on 200 MHz as the baseline. It is recommended that the simulation parameters for the 14.8 - 15.35 GHz range reference the frequency range 10 - 10.5 GHz, according to [4].

Table 2.1.1-1: IMT parameters for 14.8 - 15.35 GHz

| Parameters | Setting |
| --- | --- |
| Frequency Range | 14.8-15.35GHz |
| Modulation | OFDM-QPSK |
| Bandwidth | 100MHz |
| Subcarrier Spacing | 30kHz |
| Duplex Mode | TDD |

Figure 2.1.1-1: Initial Resource Allocation

Note that beamforming is not required for the CSI-RS, and we consider each time slot to be independent. Then we have

Xk=Ik+iQk(QAM symbol at the k-th subcarrier)

- Subcarrier spacing of 30 kHz or larger should be considered for bandwidth higher than 100 MHz.

Step 2: Filtering (Windowing)

The leakage resulting from the superposition of sinc functions becomes severe. Therefore, we will first filter out the energy radiated outside the band.

Let lk for k=0, 1, …, Q-1 be the k-th sample of the filter weight, then set the filtered symbol

Xk'=lkXk, for k=0,1,...,Q-1

Xk=Xk',0≤k<Q0    ,Q≤k<N

Figure 2.1.2-1: Filtering

Step 3: Beamforming and oversampling

Apply a beamforming weight wk(f) on the symbol, then perform an IFFT to convert the signal from the frequency domain back to the time domain.

IFFTwk(f)Xk=1Nk=0N-1wk(f)Xkei2πmkN=xm,  m=0,1,...,N

Oversampling helps to create a waveform that better approximates the desired signal. This reduces spectral leakage and ensures that the signal stays within its allocated bandwidth, thus improving spectral efficiency. If we oversample by a factor of two, then

xj=xj2             ,j is even or zero12⋅xj+12+xj-12,j is odd, j=0,1,…,2N-1

- Beamforming should not be use on resource elements of CSI-RS.

Step 4: Nonlinear high-power amplifier

After passing the time-domain signal through a nonlinear amplifier (Modified Rapp model), the signal after the digital beamforming is obtained. For example, if we let PA start to saturate at 43 dBm, we have the following input-output relationship.

xj,out=FAM/AM×e(FAM/PM×i)

FAM/AM(xj)=glin×xj(1+glin×xjVsat2p)1/2pFAM/PM(xj)=A×xjq(1+xjB)q+angle(u),where glin: Linear power gain (dB)Vsat: Output saturation levelp: Magnitude smoothness factorA: Phase gain (rad)B: Phase saturationq: Phase smoothness factor

Frequency dependent nonlinearity model for high-power amplifier should be further considered. As the PAs are distributed, a model for PA nonlinearity variation across the PAs need to be agreed.

Step 5: Antenna pattern and analog beamforming

Finally, by combining the effect of analog beamforming and the antenna pattern in TR 38.803 with the time-domain signal xj,out, the ACLR can be computed.

MIMO Method in Simulations

In the MIMO simulation, we utilize the channel matrix generated in R4-2411720. This channel matrix is based on the channel model defined in TR 38.901 and the antenna pattern specified in TR 38.803, accounting for the impact of different antenna configurations on transmission characteristics.

This channel matrix provides an accurate representation of channel characteristics in a multi-user environment, allowing us to apply precise beamforming weight calculations for both single-user MIMO (SU-MIMO) and multi-user MIMO (MU-MIMO) simulations. By employing this modeling approach, we can effectively compare the performance differences between Maximum Ratio Combining (MRC) beamforming and Zero Forcing (ZF) beamforming under various antenna patterns and multipath propagation conditions. Using this matrix, the required maximal ratio combining (MRC) and zero-forcing (ZF) beamforming weights can be calculated.

H[j]=FFT{Hτ,θ,φ*sinct|t=j2N}

wMRC(f)=HH

wZF(f)=H+qκH+q, where H+q is q-th column of H+, and H+=HH(HHH)-1

    - SU-MIMO

In the case of SU-MIMO, we employ MRC beamforming using the beamforming weights derived from the channel matrix. This approach optimizes the received signal quality for the intended user by maximizing the signal-to-noise ratio (SNR).

    - MU-MIMO

For MU-MIMO, individual data streams are generated for each user, and Zero Forcing (ZF) beamforming is employed to mitigate inter-user interference. Utilizing the previously derived channel matrix, ZF beamforming weights are calculated to ensure minimal interference between multiple users, thus optimizing the performance in a shared multi-user environment.

- Both SU-MIMO and MU-MIMO cases should be considered.

Simulation Results

The following simulations consider four transmit antennas under 100 MHz bandwidth whose subcarrier spacing is 30 kHz. In addition, parameters in Table 2.3.1 are considered. The PA with nonlinearity starts to saturate when the output power is larger than 37 dBm.

Table 2.1.3: simulation parameters for 14.8 - 15.35 GHz

Single-user SISO/MIMO

The results of SU-SISO case below are considered as the benchmark. It can be observed that when PA nonlinearity exists, the ACLR degrades significantly.

Under ideal conditions, an ACLR value of approximately 45 dB can be achieved, meeting current requirements. However, PA nonlinearity causes significant degradation in ACLR.

With filtering applied, an ACLR exceeding 45 dB can be achieved in the absence of PA nonlinearity.

The results of SU-MIMO are shown below (the total transmit power reduces from 43 dBm to 37 dBm). It can be observed that when MIMO is adopted. The ACLR can recover to its normal value. The fundamental reason behind this is that as the total power remain fixed at 43 dBm, the use of multiple antennas can result in the situation that the output power of each antenna can be reduced by a factor of 4 to 37 dBm.

SU-MIMO implementation can enhance ACLR performance.

Filtering is effective in improving ACLR only if the current ACLR value is not severely degraded.

Multi-user MIMO

Simulations below consider four users. Each user has a single data stream. The results of MU-MIMO PA nonlinearity are shown below. Similar results obtained for SU-MIMO cases are observed.

Without PA nonlinearity, ZF beamforming does not affect the ACLR performance.

MU-MIMO implementation can enhance ACLR performance.

Filtering improves ACLR only when the ACLR value is not severely degraded.

The ACLR values and behaviors for SU-MIMO and MU-MIMO cases are similar when their total number of data streams, antenna patterns and TX configurations are identical.

- The evaluation of ACLR for SU-MIMO and MU-MIMO cases should consider different number of data streams and TX configurations.

- Both uplink MIMO and downlink MIMO cases should be considered.

- We need to consider FR-1 like and FR-2 like UEs for uplink MIMO cases.

Conclusion

Within this contribution we have discussed and introduced the Adjusting ACLR Requirements for 14.8-15.35 GHz.

In this contribution, the following Proposals were made:

- Subcarrier spacing of 30 kHz or larger should be considered for bandwidth higher than 100 MHz.

- Beamforming should not be use on resource elements of CSI-RS

- Both SU-MIMO and MU-MIMO cases should be considered.

- The evaluation of ACLR for SU-MIMO and MU-MIMO cases should consider different number of data streams and TX configurations.

- Both uplink MIMO and downlink MIMO cases should be considered.

- We need to consider FR-1 like and FR-2 like UEs for uplink MIMO cases.

In the previous sections we made the following observations:

- There is a need to understand how MIMO technology and PA nonlinearities affect out-of-band emissions, particularly in both SU-MIMO and MU-MIMO cases.

- Frequency dependent nonlinearity model for high-power amplifier should be further considered. As the PAs are distributed, a model for PA nonlinearity variation across the PAs need to be agreed.

- Under ideal conditions, an ACLR value of approximately 45 dB can be achieved, meeting current requirements. However, PA nonlinearity causes significant degradation in ACLR.

- With filtering applied, an ACLR exceeding 45 dB can be achieved in the absence of PA nonlinearity.

- SU-MIMO implementation can enhance ACLR performance.

- Filtering is effective in improving ACLR only if the current ACLR value is not severely degraded.

- Without PA nonlinearity, ZF beamforming does not affect the ACLR performance.

- MU-MIMO implementation can enhance ACLR performance.

- Filtering improves ACLR only when the ACLR value is not severely degraded.

- The ACLR values and behaviors for SU-MIMO and MU-MIMO cases are similar when their total number of data streams, antenna pattern and TX configuration are identical.

- References

- R4-2412822, “Topic summary for [112][120] FS_NR_IMT_part2” , Maastricht Netherland, August 2024.

- Christopher Mollén, Erik G. Larsson, Ulf Gustavsson, Thomas Eriksson and Robert W. Heath, Jr., 
“Out-of-Band Radiation from Large Antenna Arrays,”, April 6, 2017

- EN302.574-1 v2.1.1 (2016-06), “Satellite Earth Stations and Systems (SES); Harmonised Standard for Mobile Earth Stations (MES) operating in the 1980 MHz to 2010 MHz (earth-to-space) and 2170 MHz to 2200 MHz (space-to-earth) frequency bands covering the essential requirements of article 3.2 of the Directive 2014/53/EU”

- 3GPP TR 38.921 v18.0.1 (2024-05), “Study on International Mobile Telecommunications (IMT) 
parameters for 6.425-7.025 GHz, 7.025-7.125 GHz and 10.0-10.5 GHz (Release 18)”.

- R4-2411720, “Discussion on ACLR Calculation Procedure for 14800 - 15350 MHz with Beamforming and MIMO ”, Maastricht Netherland, August 2024.

- 3GPP TR 38.901 v18.0.0 (2024-04), “Study on channel model for frequencies from 0.5 to 100 GHz (Release 18)”.

- 3GPP TR 38.803 v14.4.0 (2024-07), “Study on new radio access technology: Radio Frequency (RF) and co-existence aspects (Release 14)”.