3GPP TSG-RAN WG4 Meeting #112-bis	R4-2415383

Hefei, China, 14th Oct 2024 – 18th Oct 2024

Agenda item:	6.14.2

Source:	MediaTek Inc.

Title:	Simulation results of Spatial Channel Modelling

Document for:	Discussion

# Introduction

In RAN4#112, RAN4 agreed set of alignment simulations for Spatial Channel Modelling [1]. In this contribution, we provide our simulation results for Spatial Channel Modelling.

# Simulation results

In this Chapter, we provide our simulation setup and results for Spatial Channel Modelling. In RAN4#112, RAN4 agreed set of alignment simulations follow existing 38.101-4 8RX TDD requirement specified in Chapter 5.2.4.2.1 [2]. Common parameters are listed in Table 1. We have assumed 0% TxEVM and LMMSE MIMO demodulation.

Table 1: Common test parameters of 8RX tests (Table 5.2.4.2.1-2 in [2])

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 for rank <= 4 2 for rank > 4 |
| Codebook configuration | CodebookType |  | typeI-SinglePanel |
|  | CodebookMode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (2,1) for 4Tx (4,1) for 8Tx |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,1) |
| CSI-RS for tracking | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of HARQ Processes |  |  | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |

Test specific reference channels with common system bandwidth, subcarrier spacing, and TDD UL-DL pattern are listed in Table 2. User defined MCS13 follows R.PDSCH.2-3.8 TDD definition except MCS is 13.

Table 2: Test parameters of 8RX tests

| Test | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern |
| --- | --- | --- | --- | --- |
| Rank 2 | R.PDSCH.2-3.1 TDD | 40 / 30 | 64QAM, 0.50 (MCS19) | FR1.30-1 |
| Rank 4 | R.PDSCH.2-3.7 TDD | 40 / 30 | 64QAM, 0.43 (MCS17) | FR1.30-1 |
| Rank 8 | User defined MCS13 | 40 / 30 | 16QAM, 0.48 (MCS13) | FR1.30-1 |
| Rank 8 | R.PDSCH.2-3.8 TDD | 40 / 30 | 64QAM, 0.43 (MCS17) | FR1.30-1 |

CDL channel parameters are listed in Table 3 for general parameters and in Table 4 for transmit beamforming related parameters. In addition to the fixed transmit beamforming model, we also simulated for reference CDL channel without transmit beamforming so that the number of transmit antennas equals to the PDSCH Rank (2, 4, 8).

Table 3: CDL channel parameters

| Carrier frequency | 3.5 GHz |
| --- | --- |
| UE speed | 30 km/h |
| Channel profile | UMa CDL-C |
| Fixed transmit beamforming | Option 1: Used (128 TX antennas) Option 2: Not used (# TX antennas match PDSCH Rank) |

Table 4: CDL transmit beam directions

| The strongest beam | Ranks 2, 4, 8 | AOD: -7.27° | ZOD: 100° |
| --- | --- | --- | --- |
| The 2nd strongest beam | Ranks 2, 4 | AOD: -21.82° | ZOD: 100° |
| The 3rd strongest beam | Rank 8 | AOD: -36.36° | ZOD: 100° |
| The 4th strongest beam | Rank 8 | AOD: -50.91° | ZOD: 100° |

We also simulated Multi-cluster TX-RX beam steering, which is the TDL extension model proposed by MediaTek in discussion Tdoc [3]. The related channel parameters are listed in Table 5 for 3-cluster configuration and in Table 6 for 4-cluster configuration. Note that TX BS is defined only for the 1st (N1) dimension because CodebookConfig-N2 and CodebookConfig-O2 are configured as 1 in these tests, as listed in Table 1, corresponding to 1D antenna array. Furthermore, note that there is no transmit beamforming and that the number of transmit antennas equals to the PDSCH Rank (2, 4, 8).

Table 5: Multi-cluster TX-RX beam steering channel parameters with 3 clusters

| Cluster powers: p1, p2, p3 | 4/7, 2/7, 1/7 (relative powers: 0dB, -3dB, -6dB) |
| --- | --- |
| Cluster TX BS: ϴ1(1), ϴ1(2), ϴ1(3) | 0, 2π/3, 4π/3 |
| Cluster RX BS: ϴrx(1), ϴrx(2), ϴrx(3) | 0, 2π/3, 4π/3 |
| TDL channel model per cluster | TDLA-30 / 10 Hz Doppler / X-pol high correlation |

Table 6: Multi-cluster TX-RX beam steering channel parameters with 4 clusters

| Cluster powers: p1, p2, p3, p4 | 8/15, 4/15, 2/15, 1/15 (relative powers: 0dB, -3dB, -6dB, -9dB) |
| --- | --- |
| Cluster TX BS: ϴ1(1), ϴ1(2), ϴ1(3), ϴ1(4) | 0, π/2, π, 3π/2 |
| Cluster RX BS: ϴrx(1), ϴrx(2), ϴrx(3), ϴrx(4) | 0, π/2, π, 3π/2 |
| TDL channel model per cluster | TDLA-30 / 10 Hz Doppler / X-pol high correlation TDLC-300 / 100 Hz Doppler / X-pol high correlation |

In Figure 1 we show performance plots of tests listed in Table 2. In upper left corner is Rank 2 with MCS19. In upper right corner is Rank 4 with MCS17. In lower left corner is Rank 8 with MCS13. In lower right corner is Rank 8 with MCS17.

Figure 1: Simulation result plots of 8RX TDD Rank 2, 4, 8 with different channel models

From the results we can make the following observations:

Observation #1: TDL channel with ULA Medium B correlation is not suitable for Ranks higher than 2.

Observation #2: CDL-C UMa channel without transmit beamforming is not feasible for Rank 8.

Observation #3: CDL-C UMa channel with transmit beamforming with Rank 8 and MCS17 cannot reach maximum throughput.

Observation #4: The feasibility of TX-RX BS channel can be easily adjusted to match desired Rank and MCS.

In Table 7 we list required SNR in each test to achieve 70% relative throughput. If result is marked as “N/A”, it means that 70% relative throughput was not achieved. If result is marked as “-“, it was not simulated.

Table 7: Simulation results of 8RX TDD Rank 2, 4, 8 with different channel models

|  | Rank 8 | Rank 8 | Rank 4 | Rank 2 |
| --- | --- | --- | --- | --- |
|  | MCS17 | MCS13 | MCS17 | MCS19 |
| CDL-C-UMa / No Fixed TX-BF | N/A | 21.8dB | 14.6dB | 7.5dB |
| CDL-C-UMa / Fixed TX-BF | 29.5dB | 22.5dB | 12.0dB | 8.2dB |
| TDL-A-30-10 ULA LOW | 19.3dB | 14.0dB | 10.0dB | 7.1dB |
| TDL-A-30-10 ULA MED-B | N/A | N/A | 31.4dB | 9.3dB |
| TDL-C-300-100 ULA LOW | 24.2dB | 16.4dB | 11.3dB | 7.8dB |
| TDL-C-300-100 ULA MED-B | N/A | N/A | N/A | 12.6dB |
| TDL-A-30-10 TX-RX BS with 3 clusters | 26.7dB | 20.2dB | - | - |
| TDL-A-30-10 TX-RX BS with 4 clusters | 18.7dB | 14.0dB | - | - |
| TDL-C-300-100 TX-RX BS with 4 clusters | 21.9dB | 15.8dB | - | - |

# Conclusion

In this paper we provided our simulation setup and results for Spatial Channel Modelling. The following observations are made:

Observation #1: TDL channel with ULA Medium B correlation is not suitable for Ranks higher than 2.

Observation #2: CDL-C UMa channel without transmit beamforming is not feasible for Rank 8.

Observation #3: CDL-C UMa channel with transmit beamforming with Rank 8 and MCS17 cannot reach maximum throughput.

Observation #4: The feasibility of TX-RX BS channel can be easily adjusted to match desired Rank and MCS.

# Reference

- R4-2413606, Way Forward for [112][327] FS_NR_demod_SCM, Nokia

- 3GPP TS 38.101-4 V18.5.0 (2024-09), User Equipment (UE) radio transmission and reception, Part 4: Performance requirements

- R4-2415382, Discussion on Spatial Channel Modelling, MediaTek Inc.