3GPP TSG-RAN WG4 Meeting #112-bis	R4-2415386

Hefei, China, 14th Oct 2024 – 18th Oct 2024

Agenda item:	6.17.3.1

Source:	MediaTek Inc.

Title:	Discussion on CSI prediction testability and interoperability

Document for:	Discussion

# Introduction

In RAN4#112, RAN4 agreed to continue discussion on CSI prediction testability and interoperability issues [1]. In this contribution, we provide our views on CSI prediction testability and interoperability issues.

# Discussion

In this Chapter, we provide our views on CSI prediction testability and interoperability issues.

Issue 4-5: CSI prediction accuracy metric

Agreement:

Use relative throughput as requirement metric for CSI prediction.

- companies are invited to bring contribution on how the requirement could be defined:

  - what is the baseline that throughput would be compared to

CSI prediction accuracy should be tested with PMI reporting test. Test metric of these tests has always been relative throughput compared to random PMI of Type I codebook. We see that using the same approach to test CSI prediction accuracy is sufficient.

Observation #1: Existing PMI reporting tests use random PMI of Type I codebook as reference throughput.

Proposal #1: We propose to use random PMI of Type I codebook as reference throughput in test metric for CSI prediction.

In Release 18 we introduced new PMI requirements for eType II doppler codebook MIMO evolution work item that is CSI prediction without AI/ML. During this work RAN4 observed great difficulties finding suitable test configuration that would enable significantly measurable PMI prediction gains compared to legacy non-predicting eType II reporting. Therefore, we propose to use legacy non-predicting eType II codebook as reference when designing any new PMI reporting tests to confirm test configuration enables any measurable prediction gains.

Observation #2: When defining Release 18 PMI prediction test requirements, it was challenging to find test configuration that enables measurable PMI prediction gains over legacy baseline.

Proposal #2: We propose to use eType II non-predicting reporting as reference in test case definition to ensure test feasibility.

# Conclusion

In this contribution we provide the view on the CSI prediction testability and interoperability issues. The following observations and proposals are made:

Observation #1: Existing PMI reporting tests use random PMI of Type I codebook as reference throughput.

Proposal #1: Use random PMI of Type I codebook as reference throughput in test metric for CSI prediction.

Observation #2: When defining Release 18 PMI prediction test requirements it was challenging to find test configuration that enables measurable PMI prediction gains over legacy baseline.

Proposal #2: We propose to use eType II non-predicting reporting as reference in test case definition to ensure test feasibility.

# Reference

- R4-2414323, WF on AI/ML, Qualcomm